ISBN 0-9547136-5-6

| UK Greenhouse Gas Inventory, 1990 to 2003 |  |
| --- | --- |
| Annual Report for submission under the Framework Convention on Climate Change |  |
| Main authors | Baggott SL, Brown L, Milne R, Murrells TP, Passant N, Thistlethwaite G, Watterson JD |
| With contributions from | Adams M, Dore C, Downes, M, Goodwin J, Hobson M, Lawrence H, Manning A, Smith A, Walker, C |
| April 2005 |  |
| This work forms part of the Global Atmosphere Research Programme of the Department for Environment, Food and Rural Affairs. |  |

| UK Greenhouse Gas Inventory, 1990 to 2003 |  |
| --- | --- |
| Annual Report for submission under the Framework Convention on Climate Change |  |
| Main authors | Baggott SLa, Brown Lb, Milne Rc, Murrells TPa, Passant Na, , Thistlethwaite Ga, Watterson JDa |
| With contributions from | Adams Ma, Dore Ca, Downes, Ma, Goodwin Ja, Hobson Ma, Lawrence Ha, Manning Ad, Smith Aa, Walker, Ca |
| April 2005 |  |
| a National Environmental Technology Centre (NETCEN), AEA Technology plc, Building 551, Harwell, Didcot, Oxon., OX11 0QJ, UK. b Institute of Grassland and Environmental Research (IGER), North Wyke Research Station, Okehampton, Devon, EX20 2SB, UK. c Centre for Ecology and Hydrology (CEH), Bush Estate, Pennicuik, Midlothian, EH26 OQB, UK. d The Met Office, FitzRoy Road, Exeter, Devon, EX1 3PB, UK. |  |

| Title | UK Greenhouse Gas Inventory 1990 to 2003: Annual Report for submission under the Framework Convention on Climate Change |
| --- | --- |

| Customer | Department for Environment, Food and Rural Affairs |
| --- | --- |

| Customer reference | RMP/2106 |
| --- | --- |

| Confidentiality, copyright and reproduction | Copyright AEA Technology plc. All rights reserved. Enquiries about copyright and reproduction should be addressed to the Commercial Manager, AEA Technology plc. Published by AEA Technology plc. |
| --- | --- |

| File reference | N:\naei03\8_ghgi\2_NIR 2005\1_Report_and_appendices\ukghgi_90-03_Issue 1.0.doc |
| --- | --- |
| NAEI contacts database reference | 48209101/2003/CD3393/JW |

| Reference number | AEAT/ENV/R/1971 |
| --- | --- |
| ISBN | ISBN 0-9547136-5-6 |

| Issue number | Issue 1 |
| --- | --- |

- AEA Technology

National Environmental Technology Centre (NETCEN)

Building 551

Harwell

Didcot

Oxon 	OX11 0QJ

- Telephone		0870 190 6594

- Facsimile 	0870 190 6607

- AEA Technology is the trading name of

- AEA Technology plc

- AEA Technology is certified to ISO9001

| Report Manager | Name | JD Watterson |
| --- | --- | --- |

| Approved by | Name | JD Watterson |
| --- | --- | --- |

|  | Signature |  |
| --- | --- | --- |

|  | Date | 29.04.2005 |
| --- | --- | --- |

- Preface

- This is the United Kingdom’s National Inventory Report (NIR) submitted in the year 2005 to the United Nations Framework Convention on Climate Change (UNFCCC).  It contains national greenhouse gas emission inventories for the period 1990 to 2003, and the descriptions of the methods used to produce the estimates.  A CD on the back flap of the report contains tabular data in the Common Reporting Format (CRF) covering the United Kingdom’s greenhouse gas emissions for the same period.

The report is compiled in accordance with the new UNFCCC Inventory Reporting Guidelines agreed by the Conference of Parties at its eighth session (COP8), which took place in New Delhi from 23 October to 1 November 2002.  As agreed at COP8 in Decision 18/CP8, 2004 was the first year during which Parties were expected to use these Guidelines, which are set out in document FCCC/CP/2002/8.  The UK used these Guidelines for its NIR submitted in the year 2004 and the present report follows the same reporting structure as that used in the 2004 NIR.

The greenhouse gas inventory (GHGI) is based on the same data sets used by the National Atmospheric Emissions Inventory (NAEI) for reporting atmospheric emissions under other international agreements.  The GHGI is therefore consistent with that of the NAEI where they overlap.

- This inventory is compiled on behalf of the UK Department for Environment, Food & Rural Affairs (Defra) Global Atmosphere Division, by the National Environmental Technology Centre (NETCEN).  We acknowledge the positive support and advice from Defra throughout the work, and we are grateful for the help of all those who have contributed to this NIR.  A list of the contributors can be found in Chapter 12.

- The GHGI is compiled according to IPCC Good Practice Guidance (IPCC, 2000).  Each year the inventory is updated to include the latest data available.  Improvements to the methodology are made and are backdated to ensure a consistent time series.  Methodological changes are made to take account of new data sources, or new guidance from IPCC, relevant work by CORINAIR, new research, or specific research programmes sponsored by Defra.

- Units and Conversions

Emissions of greenhouse gases presented in this report are given in Gigagrammes (Gg), Million tonnes (Mt) and Teragrammes (Tg).  GWP weighted emissions are also provided.  To convert between the units of emissions, use the conversion factors given below.

Prefixes and multiplication factors

| Multiplication factor | Abbreviation | Prefix | Symbol |
| --- | --- | --- | --- |
| 1,000,000,000,000,000 | 1015 | peta | P |
| 1,000,000,000,000 | 1012 | tera | T |
| 1,000,000,000 | 109 | giga | G |
| 1,000,000 | 106 | mega | M |
| 1,000 | 103 | kilo | k |
| 100 | 102 | hecto | h |
| 10 | 101 | deca | da |
| 0.1 | 10-1 | deci | d |
| 0.01 | 10-2 | centi | c |
| 0.001 | 10-3 | milli | m |
| 0.000,001 | 10-6 | micro |  |

1 kilotonne (kt)	=	103 tonnes	=	1,000 tonnes

1 Million tonne (Mt)	=	106 tonnes	=	1,000,000 tonnes

1 Gigagramme (Gg)	=	1 kt

1 Teragramme (Tg)	=	1 Mt

Conversion of carbon emitted to carbon dioxide emitted

To covert emissions expressed in weight of carbon, to emissions in weight of carbon dioxide, multiply by 44/12.

Conversion of Gg of greenhouse gas emitted into Gg CO2 equivalent

Gg (of GHG) * GWP = Gg CO2 equivalent

The GWP is the Global Warming Potential of the greenhouse gas.  The GWPs of the greenhouse gases are given in Table 1.1 of Chapter 1.

- Abbreviations for Greenhouse Gases and Chemical Compounds

| Type of greenhouse gas | Formula or abbreviation | Name |
| --- | --- | --- |
| Direct | CH4 | Methane |
| Direct | CO2 | Carbon dioxide |
| Direct | N2O | Nitrous oxide |
| Direct | HFCs | Hydrofluorocarbons |
| Direct | PFCs | Perfluorocarbons |
| Direct | SF6 | Sulphur hexafluoride |
| Indirect | CO | Carbon monoxide |
| Indirect | NMVOC | Non-methane volatile organic compound |
| Indirect | NOX | Nitrogen oxides (reported as nitrogen dioxide) |
| Indirect | SO2 | Sulphur dioxide |

HFCs, PFCs and SF6 are collectively known as the ‘F-gases’

- ES.1

The United Kingdom ratified the United Nations Framework Convention on Climate Change (UNFCCC) in December 1993, and the Convention came into force in March 1994.  Parties to the Convention are committed to develop, publish and regularly update national emission inventories of greenhouse gases (GHGs).

This report is the UK’s National Inventory Report (NIR) submitted in 2005.  It contains greenhouse gas emissions estimates for the period 1990 to 2003, and describes the methodology on which the estimates are based.  The report and the attached Common Reporting Format (CRF) have been compiled in accordance with UNFCCC reporting guidelines on annual inventories contained in document FCCC/CP/2002/8 and Decision 18/CP8 of the Conference of Parties.

The UK’s greenhouse gas (GHG) inventory is compiled under contract to the UK Department for the Environment, Food and Rural Affairs (Defra) by the National Environmental Technology Centre (NETCEN) of AEA Technology.  NETCEN also compiles the UK’s National Atmospheric Emissions Inventory, used for reporting emissions to other international agreements.  Most of the underlying information is held on common databases and this helps ensure consistency between the inventories.  Emissions from the agricultural sector (Sector 4) are provided by Defra’s Land Management Improvement Division on the basis of a contract with the Institute of Grassland and Environmental Research (IGER), and estimates for Land-use Change and Forestry (LUCF) (Sector 5) are produced on behalf of Defra by the UK Centre for Ecology and Hydrology (CEH).  Defra also funds research contracts to provide improved emissions estimates for certain sources such as fluorinated gases, landfill methane, and to provide estimates for previously unreported sub-sectors such as methane from closed coal mines, included for the first time in the UK inventory in this submission.

The inventory covers the six direct greenhouse gases under the Kyoto Protocol.  These are:

- Carbon dioxide

- Methane

- Nitrous oxide

- Hydrofluorocarbons (HFCs)

- Perfluorocarbons (PFCs)

- Sulphur hexafluoride (SF6)

These gases contribute directly to climate change owing to their positive radiative forcing effect.  Also reported are four indirect greenhouse gases:

- Nitrogen oxides (reported as NO2)

- Carbon monoxide

- Non-Methane Volatile Organic Compounds (NMVOC)

- Sulphur dioxide

The structure of the 2005 National Inventory Report follows that of the 2004 report.  Chapter 1 provides an introduction and background information on greenhouse gas inventories.  Chapter 2 provides a summary of the emission trends for aggregated greenhouse gas emissions by source and gas.  Chapters 3 to 9 discuss each of the main source categories in detail and Chapter 10 presents information on recalculations, improvements and a summary of responses to review processes.  There are also Annexes to provide key source analysis and other detailed information as set out in the Guidelines.

The UK’s inventory is used to provide historical data consistent with projections of GHG emissions for the UK’s Climate Change Programme, most recently updated in the Consultation Paper produced for the Review of the UK Climate Change Programme published in December 2004.  The programme sets out policies to ensure that the UK delivers its legally binding target under the Kyoto Protocol to reduce emissions of the basket of the six greenhouse gases to 12.5% below 1990 levels over the first commitment period 2008-2012, and to move the UK towards its domestic goal of a 20% reduction in carbon dioxide emissions below 1990 levels by 2010.  The Energy White Paper published in 2003 announced the goal of putting the UK on a path to cut CO2 by 60% by 2050, with real progress by 2020.  The programme is currently under formal review and is expected to be revised by summer 2005.

- ES.2

Tables ES1 and ES2 summarise the emissions of each greenhouse gas expressed in terms of carbon dioxide and carbon equivalent emissions respectively.

| Table ES 1 | Mt CO2 equivalent |  |  |  |  |  |  |  |  |  |  |  |  |  |  | % changes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Base year | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 1990-2003 | Base year-2003 |
| CO2 (Emissions including LUCF emissions) | 606.3 | 606.3 | 613.1 | 597.5 | 582.9 | 575.7 | 567.1 | 588.7 | 564.5 | 566.6 | 556.4 | 560.3 | 577.1 | 559.8 | 572.2 | -5.6% | -5.6% |
| CH4 | 77.5 | 77.5 | 76.7 | 75.6 | 73.0 | 66.3 | 66.0 | 64.1 | 61.0 | 57.6 | 53.7 | 49.9 | 47.0 | 45.0 | 40.6 | -47.6% | -47.6% |
| N2O | 67.9 | 67.9 | 66.0 | 59.1 | 55.4 | 58.6 | 57.1 | 59.1 | 60.8 | 58.1 | 45.0 | 44.9 | 42.6 | 41.0 | 40.4 | -40.5% | -40.5% |
| HFCs | 15.5 | 11.4 | 11.9 | 12.3 | 13.0 | 14.0 | 15.5 | 16.7 | 19.2 | 17.3 | 10.8 | 9.1 | 9.7 | 10.4 | 10.7 | -5.9% | -30.9% |
| PFCs | 0.5 | 1.4 | 1.2 | 0.6 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.4 | 0.4 | 0.5 | 0.4 | 0.4 | 0.4 | -72.9% | -17.6% |
| SF6 | 1.3 | 1.1 | 1.1 | 1.2 | 1.2 | 1.2 | 1.3 | 1.3 | 1.3 | 1.3 | 1.5 | 1.9 | 1.5 | 1.6 | 1.6 | 44.1% | 20.8% |
| Total (Emissions) | 768.9 | 765.6 | 769.9 | 746.3 | 726.1 | 716.3 | 707.3 | 730.4 | 707.3 | 701.4 | 667.8 | 666.6 | 678.3 | 658.2 | 665.8 | -13.0% | -13.4% |
| CO2 (Removals) | -14.9 | -14.9 | -15.1 | -15.3 | -15.6 | -15.9 | -15.9 | -15.9 | -15.8 | -15.8 | -15.9 | -15.8 | -15.9 | -16.0 | -16.3 | 9.1% | 9.1% |
| CO2 (Net Emissions) | 591.4 | 591.4 | 598.0 | 582.1 | 567.3 | 559.8 | 551.1 | 572.8 | 548.7 | 550.8 | 540.4 | 544.5 | 561.3 | 543.8 | 555.9 | -6.0% | -6.0% |
| Total (Net Emissions) | 754.0 | 750.6 | 754.9 | 731.0 | 710.4 | 700.4 | 691.4 | 714.6 | 691.4 | 685.5 | 651.9 | 650.8 | 662.5 | 642.2 | 649.6 | -13.5% | -13.9% |
| CO2 (Emission excluding LUCF) | 588.8 | 588.8 | 595.4 | 580.1 | 566.4 | 559.1 | 550.3 | 572.2 | 548.4 | 551.2 | 541.0 | 545.3 | 562.3 | 545.3 | 557.5 |  |  |
| Total (Emissions excluding LUCF) | 751.4 | 748.0 | 752.3 | 728.9 | 709.5 | 699.7 | 690.6 | 713.9 | 691.2 | 685.9 | 652.4 | 651.5 | 663.5 | 643.7 | 651.1 |  |  |
| CO2 emissions from LUCF | 17.6 | 17.6 | 17.7 | 17.4 | 16.5 | 16.6 | 16.7 | 16.5 | 16.0 | 15.5 | 15.4 | 15.0 | 14.9 | 14.5 | 14.7 |  |  |
| CO2 removals from LUCF | -14.9 | -14.9 | -15.1 | -15.3 | -15.6 | -15.9 | -15.9 | -15.9 | -15.8 | -15.8 | -15.9 | -15.8 | -15.9 | -16.0 | -16.3 |  |  |
| CH4 emissions from LUCF | 0.015 | 0.015 | 0.013 | 0.010 | 0.011 | 0.012 | 0.015 | 0.017 | 0.014 | 0.015 | 0.027 | 0.020 | 0.021 | 0.016 | 0.013 |  |  |
| N2O emissions from LUCF | 0.0015 | 0.0015 | 0.0013 | 0.0010 | 0.0012 | 0.0012 | 0.0015 | 0.0017 | 0.0014 | 0.0015 | 0.0028 | 0.0021 | 0.0021 | 0.0017 | 0.0013 |  |  |
| Under Kyoto Protocol Article 3.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CO2 (Emissions) (see footnote 4) | 0.74 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total (Emissions) (see footnote 5) | 0.76 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- One Mt equals one Tg, which is equal to 1012 g (1,000,000,000,000 g) or one million tonnes

- The base year is the sum of 1990 totals for CO2, CH4 and N2O and 1995 totals for HFC, PFC and SF6

- Net Emissions are reported in the Common Reporting Format

- Under Article 3.7 of the Kyoto Protocol the UK must include emissions due to deforestation in its base year gross emissions.  For CO2 these emissions (0.20 MtC) are a subset of all LUCF emissions (4.8 MtC) in the base year.  The deforestation emissions cannot be derived from other values in this table and is entered here for information.

- Includes additional non-CO2 emissions due to deforestation

| Table ES 2 | Mt C equivalent |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | % changes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Base year | 1990 |  | 1991 |  | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 1990-2003 | Base year-2003 |
| CO2 (Emissions including LUCF emissions) | 165.4 | 165.4 |  | 167.2 |  | 162.9 | 159.0 | 157.0 | 154.7 | 160.6 | 153.9 | 154.5 | 151.7 | 152.8 | 157.4 | 152.7 | 156.1 | -5.6% | -5.6% |
| CH4 | 21.1 | 21.1 |  | 20.9 |  | 20.6 | 19.9 | 18.1 | 18.0 | 17.5 | 16.6 | 15.7 | 14.7 | 13.6 | 12.8 | 12.3 | 11.1 | -47.6% | -47.6% |
| N2O | 18.5 | 18.5 |  | 18.0 |  | 16.1 | 15.1 | 16.0 | 15.6 | 16.1 | 16.6 | 15.8 | 12.3 | 12.2 | 11.6 | 11.2 | 11.0 | -40.5% | -40.5% |
| HFCs | 4.2 | 3.1 |  | 3.2 |  | 3.4 | 3.5 | 3.8 | 4.2 | 4.6 | 5.2 | 4.7 | 3.0 | 2.5 | 2.7 | 2.8 | 2.9 | -5.9% | -30.9% |
| PFCs | 0.1 | 0.4 |  | 0.3 |  | 0.2 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | -72.9% | -17.6% |
| SF6 | 0.4 | 0.3 |  | 0.3 |  | 0.3 | 0.3 | 0.3 | 0.4 | 0.4 | 0.3 | 0.4 | 0.4 | 0.5 | 0.4 | 0.4 | 0.4 | 44.1% | 20.8% |
| Total (Emissions) | 209.7 | 208.8 |  | 210.0 |  | 203.5 | 198.0 | 195.4 | 192.9 | 199.2 | 192.9 | 191.3 | 182.1 | 181.8 | 185.0 | 179.5 | 181.6 | -13.0% | -13.4% |
| CO2 (Removals) | -4.1 | -4.1 |  | -4.1 |  | -4.2 | -4.3 | -4.3 | -4.3 | -4.3 | -4.3 | -4.3 | -4.3 | -4.3 | -4.3 | -4.4 | -4.4 | 9.1% | 9.1% |
| CO2 (Net Emissions) | 161.3 | 161.3 |  | 163.1 |  | 158.8 | 154.7 | 152.7 | 150.3 | 156.2 | 149.6 | 150.2 | 147.4 | 148.5 | 153.1 | 148.3 | 151.6 | -6.0% | -6.0% |
| Total (Net Emissions) | 205.6 | 204.7 |  | 205.9 |  | 199.4 | 193.8 | 191.0 | 188.6 | 194.9 | 188.6 | 187.0 | 177.8 | 177.5 | 180.7 | 175.2 | 177.2 | -13.5% | -13.9% |
| CO2 (Emissions excluding LUCF) | 160.6 | 160.6 |  | 162.4 |  | 158.2 | 154.5 | 152.5 | 150.1 | 156.1 | 149.6 | 150.3 | 147.5 | 148.7 | 153.3 | 148.7 | 152.0 |  |  |
| Total (Emissions excluding LUCF) | 204.9 | 204.0 |  | 205.2 |  | 198.8 | 193.5 | 190.8 | 188.3 | 194.7 | 188.5 | 187.1 | 177.9 | 177.7 | 180.9 | 175.6 | 177.6 |  |  |
| CO2 emissions from LUCF | 4.8 | 4.8 |  | 4.8 |  | 4.8 | 4.5 | 4.5 | 4.6 | 4.5 | 4.4 | 4.2 | 4.2 | 4.1 | 4.1 | 4.0 | 4.0 |  |  |
| CO2 removals from LUCF | -4.1 | -4.1 |  | -4.1 |  | -4.2 | -4.3 | -4.3 | -4.3 | -4.3 | -4.3 | -4.3 | -4.3 | -4.3 | -4.3 | -4.4 | -4.4 |  |  |
| CH4 emissions from LUCF | 0.004 | 0.004 |  | 0.003 |  | 0.003 | 0.003 | 0.003 | 0.004 | 0.005 | 0.004 | 0.004 | 0.007 | 0.006 | 0.006 | 0.004 | 0.004 |  |  |
| N2O emissions from LUCF | 0.0004 | 0.0004 |  | 0.0003 |  | 0.0003 | 0.0003 | 0.0003 | 0.0004 | 0.0005 | 0.0004 | 0.0004 | 0.0008 | 0.0006 | 0.0006 | 0.0005 | 0.0004 |  |  |
| Under Kyoto Protocol Article 3.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CO2 (Emissions) (see footnote 4) | 0.20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total (Emissions) (see footnote 5) | 0.21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- One Mt equals one Tg, which is equal to 1012 g (1,000,000,000,000 g) or one million tonnes

- The base year is the sum of 1990 totals for CO2, CH4 and N2O and 1995 totals for HFC, PFC and SF6

- Net Emissions are reported in the Common Reporting Format

- Under Article 3.7 of the Kyoto Protocol the UK must include emissions due to deforestation in its base year gross emissions.  For CO2 these emissions (0.20 MtC) are a subset of all LUCF emissions (4.8 MtC) in the base year. The deforestation emissions cannot be derived from other values in this table and is entered here for information.

- Includes additional non-CO2 emissions due to deforestation.

- ES.3

In the UK emissions arise from the following sectors:

- Energy

- Industrial Processes

- Solvents

- Agriculture

- Land-use Change and Forestry (LUCF)

- Waste

Table ES3 summaries the trends in aggregated direct greenhouse gas emissions covered by FCCC/CP/2002/8 by sector for the years 1990-2003.

Table ES3	Aggregated emission trends per source category (Mt CO2 equivalent)

| Source Category | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Energy | 610.4 | 619.6 | 604.7 | 589.7 | 575.6 | 567.7 | 588.3 | 564.1 | 565.5 | 554.2 | 557.8 | 576.0 | 559.7 | 567.3 |
| 2. Industrial Processes | 56.5 | 52.9 | 46.9 | 44.1 | 48.4 | 48.4 | 51.5 | 54.5 | 50.2 | 30.9 | 29.8 | 27.9 | 25.7 | 26.8 |
| 3. Solvents and other product usea | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4. Agriculture | 53.4 | 52.9 | 51.3 | 50.6 | 51.5 | 51.4 | 51.8 | 52.5 | 51.6 | 51.0 | 49.1 | 46.1 | 46.4 | 45.8 |
| 5. Land-use Change and Forestry (emissions) | 17.6 | 17.7 | 17.4 | 16.5 | 16.6 | 16.8 | 16.5 | 16.0 | 15.5 | 15.4 | 15.1 | 14.9 | 14.5 | 14.7 |
| 5. Land-use Change and Forestry (removals) | -14.9 | -15.1 | -15.3 | -15.6 | -15.9 | -15.9 | -15.9 | -15.8 | -15.8 | -15.9 | -15.8 | -15.9 | -16.0 | -16.3 |
| 6. Waste | 27.8 | 26.9 | 26.0 | 25.0 | 24.1 | 23.1 | 22.2 | 20.2 | 18.6 | 16.3 | 14.8 | 13.4 | 11.9 | 11.1 |
| 7. Other | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| Total (emissions only) | 765.6 | 769.9 | 746.3 | 726.1 | 716.3 | 707.3 | 730.4 | 707.3 | 701.4 | 667.8 | 666.6 | 678.3 | 658.2 | 665.8 |
| Total (net CO2 – sum of emissions and removals) | 750.6 | 754.9 | 731.0 | 710.4 | 700.4 | 691.4 | 714.6 | 691.4 | 685.5 | 651.9 | 650.8 | 662.5 | 642.2 | 649.6 |

Footnotes:

a  Solvents and other product use emissions occur as NMVOC and so do not appear in this Table which covers direct greenhouse gases

The largest contribution to greenhouse gas emissions arises from the energy sector.  In 2003 this contributed 85% to the total emissions1.  Emissions of CO2, CH4 and N2O arise from this sector.  Since 1990, emissions from the energy sector have declined by about 7%.

The second largest source of greenhouse gases in the UK is the agricultural sector.  Emissions from this sector arise for both CH4 and N2O.  Since 1990, emissions from this sector have declined by 14%, due to a decline in emissions from enteric fermentation and agricultural waste disposal  (related to lower livestock numbers) and agricultural soils (due to changes in agricultural practices, including a decline in emissions from enteric fermentation, and a decline in the emissions from the use of synthetic fertiliser).

Industrial processes sector makes up the third largest source of greenhouse gases in the UK, contributing 4% to the national total in 2003.  Emissions of all six direct greenhouse gases occur from this sector.

Land-use Change and Forestry contributes 2% to the national total in 2003.  Emissions from this source occur for CO2, N2O and CH4.  LUCF contains sinks as well as sources of CO2 emissions.

The remaining source that contributes to direct greenhouse gas totals is waste.  In 2003 this contributed 1.7% to the national total.  Emissions arise for CO2, CH4 and N2O, with emissions occurring from waste incineration, solid waste disposal on land and wastewater handling. Emissions from this sector have steadily declined, and in 2003 are estimated to have been 40% of those in 1990.

- ES.4

ES.4 lists the indirect greenhouse gases for which the UK has made emissions estimates.  Nitrogen oxides, carbon monoxide and NMVOCs are included in the inventory because they can produce increases in tropospheric ozone concentrations and this increases radiative forcing.  Sulphur dioxide is included because it contributes to aerosol formation.

Table ES4	Emissions of Indirect Greenhouse Gases in the UK, 1990-2003 (in kt)

| Gas | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOx | 2827 | 2704 | 2621 | 2449 | 2376 | 2240 | 2165 | 2003 | 1935 | 1822 | 1737 | 1660 | 1577 | 1569 |
| CO | 8306 | 8140 | 7720 | 7289 | 6878 | 6330 | 6177 | 5716 | 5278 | 4961 | 4106 | 3809 | 3325 | 2757 |
| NMVOC | 2419 | 2336 | 2257 | 2144 | 2108 | 1965 | 1868 | 1796 | 1645 | 1469 | 1334 | 1239 | 1165 | 1087 |
| SO2 | 3711 | 3521 | 3443 | 3098 | 2663 | 2354 | 2014 | 1653 | 1598 | 1219 | 1194 | 1118 | 1002 | 979 |

Since 1990, emissions of all indirect gases have decreased significantly.  The largest source of emissions for all the indirect gases is the energy sector.  For NOx, CO and SO2, over 90% of emissions for each gas arise from activities within this sector.  For NMVOC, 51% of emissions are energy related, with other significant contributions from both the industrial processes and solvent sectors.

- Contacts

This work forms part of the Global Atmosphere Research Programme of the Department for Environment, Food and Rural Affairs (Contract RMP/2106).  The Land Use Change and Forestry estimates were provided by the Centre for Ecology and Hydrology (CEH) Edinburgh (Contract CPEG 1).  The Institute of Grassland and Environmental Research (IGER) provide the estimates of agricultural emissions.

Technical enquiries should be directed to:

Dr J D Watterson

National Environmental Technology Centre (NETCEN)

AEA Technology plc

Building 551

Harwell

Didcot

Oxfordshire OX11 0QJ

Tel: +44 (0) 870 190 6594

Fax: +44 (0) 870 190 6607
E-mail: John.D.Watterson@aeat.co.uk

A copy of this report and related data may be found on the website maintained by NETCEN for Defra: http://www.naei.org.uk/

Further copies of this report are available from:

Defra Publications

Admail 6000

London

SW1A 2XX

Tel: +44 (0) 8459 556 000

Fax: +44 (0) 20 8957 5012

Email: defra@iforcegroup.com

- Contents

Preface	iv

Units and Conversions	v

Abbreviations for Greenhouse Gases and Chemical Compounds	vi

Exectutive Summary 1	vii

Exectutive Summary 2	ix

Exectutive Summary 3	x

Exectutive Summary 4	xii

Contacts	xiii

1.	Introduction	1

1.1	Background Information on Greenhouse Gas Inventories	1

1.1.1	Reporting of the UK greenhouse gas inventory	1

1.1.2	Nomenclature of the inventory period reported	1

1.1.3	Structure of the UK National Inventory Report	1

1.1.4	Reporting of greenhouse gas emissions and background data in the CRF	2

1.1.5	Reporting of CO2 emissions from Land Use Change and Forestry	2

1.1.6	Greenhouse gases reported in the UK inventory	2

1.1.7	Global Warming Potentials of the greenhouse gases	3

1.1.8	Climate change – The UK programme	4

1.2	Institutional arrangements for inventory preparation	4

1.2.1	The UK Greenhouse Gas National Inventory System (UK NIS)	4

1.2.2	UK NIS - Organisational Structure	7

1.2.3	UK NIS - Key Data Sources	8

1.3	Process of inventory preparation	10

1.4	Methodologies and Data sources	12

1.5	description of key source categories	14

1.6	QA/QC plan	15

1.6.1	Description of the QA/QC current system	15

1.6.2	Special QA/QC activities undertaken in 2004-2005	19

1.6.3	Future development of the QA/QC system	20

1.6.4	Compliance of National Statistical Agencies	20

1.6.5	Documentation and review	20

1.6.6	External Peer Review	21

1.6.7	Verification	22

1.6.8	Treatment of confidentiality	22

1.7	General Uncertainty Evaluation	23

1.8	General Assessment of Completeness	23

1.9	Geographical coverage of the UK greenhouse gas inventory	24

2.	Trends in Greenhouse Gas Emissions	25

2.1	Emission trends for aggregated greenhouse gas emissions	25

2.2	Emission trends by gas	25

2.3	Emission trends by source	26

2.4	Emission trends for indirect greenhouse gases and SO2	27

3.	Energy (CRF sector 1)	31

3.1	Overview of sector	31

3.1.1	Carbon Factor Review	31

3.2	source category 1A1 – Energy Industries	32

3.2.1	Source category description	32

3.2.2	Methodological issues	32

3.2.3	Source-specific QA/QC and verification	34

3.2.4	Source-specific recalculations	34

3.2.5	Source-specific planned improvements	36

3.3	Source Category 1A2 – Manufacturing Industries and Construction	37

3.3.1	Source Category Description	37

3.3.2	Methodological Issues	37

3.3.3	Uncertainties and time-series consistency	38

3.3.4	Source-specific QA/QC and verification	38

3.3.5	Source-specific recalculations	38

3.3.6	Source-specific planned improvements	40

3.4	Source Category 1A3 – Transport	41

3.4.1	Source Category Description	41

3.4.2	Methodological Issues	41

3.4.3	Uncertainties and time-series consistency	41

3.4.4	Source-specific QA/QC and verification	42

3.4.5	Source-specific recalculations	42

3.4.6	Source-specific planned improvements	43

3.5	Source Category 1A4 – Other Sources	43

3.5.1	Source Category Description	43

3.5.2	Methodological Issues	43

3.5.3	Uncertainties and time-series consistency	44

3.5.4	Source-specific QA/QC and verification	44

3.5.5	Source-specific recalculations	44

3.5.6	Source-specific planned improvements	46

3.6	Source Category 1A5 – Other	46

3.6.1	Source Category Description	46

3.6.2	Methodological Issues	46

3.6.3	Uncertainties and time-series consistency	46

3.6.4	Source-specific QA/QC and verification	47

3.6.5	Source-specific recalculations	47

3.6.6	Source-specific planned improvements	47

3.7	International Bunker Fuels (memo item)	47

3.8	Feedstocks and non-energy use of Fuels	47

3.9	Capture and storage of CO2 from Flue gases	47

3.10	Comparison of Sectoral and Reference approaches	48

3.11	Country specific issues	48

3.12	Source category 1B1 – Solid Fuels	48

3.12.1	Source Category Description	48

3.12.2	Methodological Issues	48

3.12.3	Uncertainties and time-series consistency	49

3.12.4	Source-specific QA/QC and verification	49

3.12.5	Source-specific recalculations	49

3.12.6	Source-specific planned improvements	50

3.13	Source category 1B2 – Oil And Natural Gas	50

3.13.1	Source Category Description	50

3.13.2	Methodological Issues	51

3.13.3	Uncertainties and time-series consistency	52

3.13.4	Source-specific QA/QC and verification	52

3.13.5	Source-specific recalculations	52

3.13.6	Source-specific planned improvements	52

3.14	general comments on QA/QC	53

3.14.1	DTI Energy Balance Data	53

3.14.2	Industrial Point-Source Emissions Data	53

3.15	General comments on time series consistency	54

4.	Industrial Processes (CRF sector 2)	55

4.1	Overview of Sector	55

4.2	Source Category 2A1 – Cement Production	55

4.2.1	Source category description	55

4.2.2	Methodological issues	55

4.2.3	Uncertainties and time-series consistency	56

4.2.4	Source-specific QA/QC and verification	56

4.2.5	Source-specific recalculations	56

4.2.6	Source-specific planned improvements	56

4.3	Source Category 2A2 – Lime Production	56

4.3.1	Source category description	56

4.3.2	Methodological issues	56

4.3.3	Uncertainties and time-series consistency	57

4.3.4	Source-specific QA/QC and verification	57

4.3.5	Source-specific recalculations	57

4.3.6	Source-specific planned improvements	57

4.4	Source Category 2A3 – Limestone & Dolomite use	57

4.4.1	Source category description	57

4.4.2	Methodological issues	58

4.4.3	Uncertainties and time-series consistency	58

4.4.4	Source-specific QA/QC and verification	58

4.4.5	Source-specific recalculations	58

4.4.6	Source-specific planned improvements	58

4.5	Source Category 2A4 – Soda Ash Use	58

4.5.1	Source category description	58

4.5.2	Methodological issues	59

4.5.3	Uncertainties and time-series consistency	59

4.5.4	Source-specific QA/QC and verification	59

4.5.5	Source-specific recalculations	59

4.5.6	Source-specific planned improvements	59

4.6	Source Category 2A5 – Asphalt Roofing	60

4.7	Source Category 2A6 – Road paving with Asphalt	60

4.7.1	Source category description	60

4.7.2	Methodological issues	60

4.7.3	Uncertainties and time-series consistency	60

4.7.4	Source-specific QA/QC and verification	60

4.7.5	Source-specific recalculations	61

4.7.6	Source-specific planned improvements	61

4.8	Source Category 2A7 – Other Mineral Products	61

4.8.1	Source category description	61

4.8.2	Methodological issues	61

4.8.3	Uncertainties and time-series consistency	62

4.8.4	Source-specific QA/QC and verification	62

4.8.5	Source-specific recalculations	62

4.8.6	Source-specific planned improvements	62

4.9	Source Category 2B1 – Ammonia Production	62

4.9.1	Source category description	62

4.9.2	Methodological issues	63

4.9.3	Uncertainties and time-series consistency	64

4.9.4	Source-specific QA/QC and verification	64

4.9.5	Source-specific recalculations	64

4.9.6	Source-specific planned improvements	64

4.10	Source Category 2B2 – Nitric Acid Production	64

4.10.1	Source category description	64

4.10.2	Methodological issues	65

4.10.3	Uncertainties and time-series consistency	66

4.10.4	Source-specific QA/QC and verification	66

4.10.5	Source-specific recalculations	66

4.10.6	Source-specific planned improvements	66

4.11	Source Category 2B3 – Adipic Acid Production	67

4.11.1	Source category description	67

4.11.2	Methodological issues	67

4.11.3	Uncertainties and time-series consistency	67

4.11.4	Source-specific QA/QC and verification	67

4.11.5	Source-specific recalculations	68

4.11.6	Source-specific planned improvements	68

4.12	Source Category 2B4 – Carbide Production	68

4.13	Source Category 2B5 – Other	68

4.13.1	Source category description	68

4.13.2	Methodological issues	69

4.13.3	Uncertainties and time-series consistency	70

4.13.4	Source-specific QA/QC and verification	70

4.13.5	Source-specific recalculations	70

4.13.6	Source-specific planned improvements	71

4.14	Source Category 2C1 – Iron and Steel Production	71

4.14.1	Source category description	71

4.14.2	Methodological issues	73

4.14.3	Uncertainties and time-series consistency	73

4.14.4	Source-specific QA/QC and verification	73

4.14.5	Source-specific recalculations	73

4.14.6	Source-specific planned improvements	73

4.15	Source Category 2C2 – Ferroalloys Production	73

4.16	Source Category 2C3 – Aluminium Production	74

4.16.1	Source category description	74

4.16.2	Methodological issues	74

4.16.3	Uncertainties and time-series consistency	74

4.16.4	Source-specific QA/QC and verification	75

4.16.5	Source-specific recalculations	75

4.16.6	Source-specific planned improvements	75

4.17	Source Category 2C4 – SF6 used in Aluminium and Magnesium Foundries	75

4.17.1	Source category description	75

4.17.2	Uncertainties and time-series consistency	76

4.17.3	Source-specific QA/QC and verification	76

4.17.4	Source-specific recalculations	76

4.17.5	Source-specific planned improvements	76

4.18	Source Category 2C5 – Other Metal Production	77

4.18.1	Source category description	77

4.18.2	Methodological issues	77

4.18.3	Uncertainties and time-series consistency	77

4.18.4	Source-specific QA/QC and verification	77

4.18.5	Source-specific recalculations	77

4.18.6	Source-specific planned improvements	77

4.19	Source Category 2D1 – Pulp and Paper	78

4.19.1	Source category description	78

4.19.2	Methodological issues	78

4.19.3	Uncertainties and time-series consistency	78

4.19.4	Source-specific QA/QC and verification	78

4.19.5	Source-specific recalculations	78

4.19.6	Source-specific planned improvements	78

4.20	Source Category 2D2 – Food and Drink	79

4.20.1	Source category description	79

4.20.2	Methodological issues	79

4.20.3	Uncertainties and time-series consistency	79

4.20.4	Source-specific QA/QC and verification	80

4.20.5	Source-specific recalculations	80

4.20.6	Source-specific planned improvements	80

4.21	Source Category 2E – Production of Halocarbons and SF6	80

4.21.1	Source category description	80

4.21.2	Methodological issues	80

4.21.3	Uncertainties and time-series consistency	81

4.21.4	Source-specific QA/QC and verification	81

4.21.5	Source-specific recalculations	81

4.21.6	Source-specific planned improvements	81

4.22	Source Category 2F1 – Refrigeration and Air Conditioning Equipment	81

4.22.1	Source category description	81

4.22.2	Methodological issues	82

4.22.3	Uncertainties and time-series consistency	83

4.22.4	Source-specific QA/QC and verification	83

4.22.5	Source-specific recalculations	83

4.22.6	Source-specific planned improvements	83

4.23	Source Category 2F2 – Foam Blowing	83

4.23.1	Source category description	83

4.23.2	Methodological issues	84

4.23.3	Uncertainties and time-series consistency	84

4.23.4	Source-specific QA/QC and verification	84

4.23.5	Source-specific recalculations	84

4.23.6	Source-specific planned improvements	84

4.24	Source Category 2F3 – Fire Extinguishers	85

4.24.1	Source category description	85

4.24.2	Methodological issues	85

4.24.3	Uncertainties and time-series consistency	86

4.24.4	Source-specific QA/QC and verification	86

4.24.5	Source-specific recalculations	86

4.24.6	Source-specific planned improvements	86

4.25	Source Category 2F4 – Aerosols/ Metered Dose Inhalers	86

4.25.1	Source category description	86

4.25.2	Methodological issues	87

4.25.3	Uncertainties and time-series consistency	87

4.25.4	Source-specific QA/QC and verification	88

4.25.5	Source-specific recalculations	88

4.25.6	Source-specific planned improvements	88

4.26	Source Category 2F5 – Solvents	88

4.26.1	Source category description	88

4.26.2	Methodological issues	88

4.26.3	Uncertainties and time-series consistency	89

4.26.4	Source-specific QA/QC and verification	89

4.26.5	Source-specific recalculations	89

4.26.6	Source-specific planned improvements	89

4.27	Source Category 2F6 – Semiconductor Manufacture	89

4.27.1	Source category description	89

4.28	Source Category 2F7 – Electrical Equipment	90

4.28.1	Source category description	90

4.29	Source Category 2F8a – One Component Foams	90

4.29.1	Source category description	90

4.29.2	Methodological issues	90

4.29.3	Uncertainties and time-series consistency	90

4.29.4	Source-specific QA/QC and verification	91

4.29.5	Source-specific recalculations	91

4.29.6	Source-specific planned improvements	91

4.30	Source Category 2F8b – Semiconductors, Electrical and production of trainers)	91

4.30.1	Source category description	91

4.30.2	Methodological issues	92

4.30.3	Uncertainties and time-series consistency	92

4.30.4	Source-specific QA/QC and verification	92

4.30.5	Source-specific recalculations	93

4.30.6	Source-specific planned improvements	93

5.	Solvent and Other Product Use (CRF sector 3)	94

5.1	Overview of Sector	94

5.2	Source Category 3A – Paint Application	94

5.2.1	Source category description	94

5.2.2	Methodological issues	95

5.2.3	Uncertainties and time-series consistency	95

5.2.4	Source-specific QA/QC and verification	95

5.2.5	Source-specific recalculations	95

5.2.6	Source-specific planned improvements	95

5.3	Source Category 3B – Degreasing and Dry Cleaning	95

5.3.1	Source category description	95

5.3.2	Methodological issues	96

5.3.3	Uncertainties and time-series consistency	96

5.3.4	Source-specific QA/QC and verification	97

5.3.5	Source-specific recalculations	97

5.3.6	Source-specific planned improvements	97

5.4	Source Category 3C  – Chemical Products, Manufacture and Processing	97

5.4.1	Source category description	97

5.4.2	Methodological issues	97

5.4.3	Uncertainties and time-series consistency	98

5.4.4	Source-specific QA/QC and verification	98

5.4.5	Source-specific recalculations	98

5.4.6	Source-specific planned improvements	98

5.5	Source Category 3D - Other	98

5.5.1	Source category description	98

5.5.2	Methodological issues	99

5.5.3	Uncertainties and time-series consistency	100

5.5.4	Source-specific QA/QC and verification	100

5.5.5	Source-specific recalculations	100

5.5.6	Source-specific planned improvements	100

6.	Agriculture (CRF sector 4)	101

6.1	Overview of Sector	101

6.2	Source Category 4A – Enteric Fermentation	101

6.2.1	Source category description	101

6.2.2	Methodological issues	101

6.2.3	Uncertainties and time-series consistency	102

6.2.4	Source-specific QA/QC and verification	102

6.2.5	Source-specific recalculations	102

6.2.6	Source-specific planned improvements	102

6.3	Source Category 4B – Manure Management	102

6.3.1	Source category description	102

6.3.2	Methodological issues	102

6.3.3	Uncertainties and time-series consistency	104

6.3.4	Source-specific QA/QC and verification	104

6.3.5	Source-specific recalculations	104

6.3.6	Source-specific planned improvements	104

6.4	Source Category 4C – Rice cultivation	104

6.5	Source Category 4D – Agricultural Soils	105

6.5.1	Source category description	105

6.5.2	Methodological issues	105

6.5.3	Uncertainties and time-series consistency	106

6.5.4	Source-specific QA/QC and verification	107

6.5.5	Source-specific recalculations	107

6.5.6	Source-specific planned improvements	107

6.6	Source Category 4E – Prescribed burning of Savannas	107

6.7	Source Category 4F – Field Burning of Agricultural Residues	107

6.7.1	Source category description	107

6.7.2	Methodological issues	107

6.7.3	Uncertainties and time-series consistency	107

6.7.4	Source-specific QA/QC and verification	108

6.7.5	Source-specific recalculations	108

6.7.6	Source-specific planned improvements	108

6.8	Source Category 4G - Other	108

6.9	general comments on QA/QC	108

7.	Land Use Change and Forestry (CRF sector 5)	109

7.1	Overview of Sector	109

7.2	Previous Source Category 5A2, new CRF categories 5A and 5G – Temperate Forests: Changes in Forests and other woody biomass stocks	110

7.2.1	Source/sink category description	110

7.2.2	Methodological issues	110

7.2.3	Uncertainties and time-series consistency	110

7.2.4	Source-specific QA/QC and verification	110

7.2.5	Source-specific recalculations	110

7.2.6	Source-specific planned improvements	112

7.2.7	Equivalent categories in the new CRF	112

7.3	Previous Source Category 5B2, new CRF category 5E – Temperate forests: Forest and Grassland conversion	112

7.3.1	Source/sink category description	112

7.3.2	Methodological issues	113

7.3.3	Uncertainties and time-series consistency	113

7.3.4	Source-specific QA/QC and verification	113

7.3.5	Source-specific recalculations	113

7.3.6	Source-specific planned improvements	114

7.3.7	Equivalent categories in the new CRF	114

7.4	Previous Source Category 5C, new CRF categories 5A to E – Abandonment of managed lands	114

7.5	Source Category 5D1 and 5D2 – emissions and removals from soils – cultivation of mineral & organic soils	114

7.5.1	Source category description	114

7.5.2	Methodological issues	114

7.5.3	Uncertainties and time-series consistency	116

7.5.4	Source-specific QA/QC and verification	116

7.5.5	Source-specific recalculations	116

7.5.6	Source-specific planned improvements	116

7.5.7	Source-specific planned improvements	116

7.6	Previous Source Category 5D3, new CRF categories 5B and 5C – CO2 emissions and removals from soils – liming of agricultural soils	117

7.6.1	Source category description	117

7.6.2	Methodological issues	117

7.6.3	Uncertainties and time-series consistency	117

7.6.4	Source-specific QA/QC and verification	117

7.6.5	Source-specific recalculations	117

7.6.6	Source-specific planned improvements	117

7.6.7	Equivalent categories in the new CRF	118

7.7	Previous Source Category 5D4, New CRF category 5a – CO2 emissions and removals from soils – forest soils	118

7.7.1	Source category description	118

7.7.2	Equivalent categories in the new CRF	118

7.8	previous Source Category 5D5, new crf category 5b – CO2 emissions and removals from soils – Other	118

7.8.1	Source category description	118

7.8.2	Methodological issues	119

7.8.3	Uncertainties and time-series consistency	119

7.8.4	Source-specific QA/QC and verification	119

7.8.5	Source-specific recalculations	119

7.8.6	Source-specific planned improvements	119

7.8.7	Equivalent categories in the new CRF	119

7.9	previous Source Category 5E, new crf categories 5G and 5C – Other: Changes in non-forest Biomass, Peat Extraction	119

7.9.1	Source category description	119

7.9.2	Methodological issues	120

7.9.3	Uncertainties and time-series consistency	120

7.9.4	Source-specific QA/QC and verification	120

7.9.5	Source-specific recalculations	120

7.9.6	Source-specific planned improvements	120

7.9.7	Equivalent GPG LULUCF format categories	120

7.10	Separate reporting of emissions and removals	120

7.11	general comments on QA/QC	125

8.	Waste (CRF sector 6)	126

8.1	Overview of Sector	126

8.2	Source Category 6A – Solid Waste Disposal On land	126

8.2.1	Source category description	126

8.2.2	Methodological issues	126

8.2.3	Uncertainties and time-series consistency	129

8.2.4	Source-specific QA/QC and verification	129

8.2.5	Source-specific recalculations	129

8.2.6	Source-specific planned improvements	129

8.3	Source Category 6B – Wastewater Handling	130

8.3.1	Source category description	130

8.3.2	Methodological issues	130

8.3.3	Uncertainties and time-series consistency	132

8.3.4	Source-specific QA/QC and verification	132

8.3.5	Source-specific recalculations	132

8.3.6	Source-specific planned improvements	132

8.4	Source Category 6C – Waste Incineration	132

8.4.1	Source category description	132

8.4.2	Methodological issues	132

8.4.3	Uncertainties and time-series consistency	133

8.4.4	Source-specific QA/QC and verification	133

8.4.5	Source-specific recalculations	134

8.4.6	Source-specific planned improvements	134

8.5	Source Category 6D – Other	134

8.5.1	Source category description	134

9.	Other (CRF sector 7)	135

10.	Recalculations and Improvements	136

10.1	RECALCULATIONS, and Explanations and justifications for recalculations	137

10.2	Implications for emission levels	144

10.3	Implications for emission trends, including time series consistency	144

10.4	Recalculations	149

10.4.1	Recalculations	149

10.4.2	Response to the review process	149

10.4.3	Major improvements to the current inventory	151

11.	References	152

11.1	Chapter 1 [Introduction]	152

11.2	Chapter 3 [Energy (CRF sector 1)]	153

11.3	Chapter 4 [Industrial Processes (CRF sector 2)]	154

11.4	Chapter 4 [Industrial Processes (CRF sector 2 - F-gases)]	155

11.5	Chapter 5 [Solvent and Other Product Use (CRF sector 3)]	156

11.6	Chapter 6 [Agriculture (CRF sector 4)]	156

11.7	Chapter 7 [Land Use Change and Forestry (CRF sector 5)]	157

11.8	Chapter 8 [Waste (CRF sector 6)]	160

11.9	Chapter 10 [Recalculations and improvements]	161

11.10	Annex 3, Sector 1, 1A	161

11.11	Annex 3, Sector 1, 1B	168

11.12	Annex 3, Sectors 2 and 3	171

11.13	Annex 3, Sector 4	174

11.14	Annex 3, Sector 5	174

11.15	Annex 4	174

11.16	annex 6	174

11.17	Annex 7 [Uncertainties]	174

11.18	Annex 8 [Verification]	175

12.	Acknowledgements	177

A1.	ANNEX 1: Key Sources	179

A1.1	Key Source Analysis	179

A2.	ANNEX 2: Detailed discussion of methodology and data for estimating CO2 emissions from fossil fuel combustion	184

A3.	ANNEX 3: Other Detailed Methodological Descriptions	185

A3.1	Background information on FUELS DATA	185

A3.2	NAEI SOurce CAtegories and IPCC Equivalents	188

A3.3	Energy (CRF sector 1)	195

A3.3.1	Basic combustion module	196

A3.3.2	Conversion of energy activity data and emission factors	206

A3.3.3	Energy Industries (1A1)	207

A3.3.4	Manufacturing Industries and Construction (1A2)	213

A3.3.5	Transport (1A3)	214

A3.3.6	Other Sectors (1A4)	245

A3.3.7	Other (1A5)	246

A3.3.8	Fugitive Emissions From fuels (1B)	250

A3.3.9	Stored carbon	261

A3.4	INdustrial Processes (crf sector 2)	263

A3.4.1	Mineral Processes (2A)	263

A3.4.2	Chemical Industry (2B)	264

A3.4.3	Metal Production (2C)	265

A3.4.4	Production of Halocarbons and SF6 (2E)	269

A3.4.5	Consumption of Halocarbons and SF6 (2F)	269

A3.5	solvent and other product use (crf sector 3)	269

A3.6	agriculture (crf sector 4)	270

A3.6.1	Enteric Fermentation (4A)	270

A3.6.2	Manure Management (4B)	273

A3.6.3	Agricultural Soils (4D)	278

A3.6.4	Field Burning of Agricultural Residues (4F)	283

A3.7	Land Use Change and Forestry (CRF Sector 5)	284

A3.7.1	Changes in Forest and Other Woody Biomass Stocks - Temperate Forests (5A2)	284

A3.7.2	Forest and Grassland Conversion - Temperate Forests (5B2)	287

A3.7.3	CO2 Emissions and Removals from Soil (5D1 and 5D2)	288

A3.7.4	Liming of Agricultural Soils (5D3)	295

A3.7.5	Other: Lowland Drainage (5D5)	296

A3.7.6	Other: Changes in Non-forest Biomass, Peat Extraction (5E)	296

A3.8	Waste (CRF sector 6)	298

A4.	ANNEX 4: Comparison of CO2 Reference and Sectoral Approaches	299

A4.1	Estimation of CO2 from the reference approach	299

A4.2	Discrepancies between the IPCC Reference and Sectoral APPROACH	299

A4.3	Time series of differences IN THE IPCC REFERENCE AND SECTORAL INVENTORIES	300

A5.	ANNEX 5: Assessment of Completeness	301

A6.	ANNEX 6: Additional Information - Quantitative Discussion of 2003 Inventory	302

A6.1	Energy Sector (1)	302

A6.1.1	Carbon Dioxide	302

A6.1.2	Methane	303

A6.1.3	Nitrous Oxide	303

A6.1.4	Nitrogen Oxides	304

A6.1.5	Carbon Monoxide	304

A6.1.6	Non Methane Volatile Organic Compounds	305

A6.1.7	Sulphur Dioxide	305

A6.2	Industrial Processes sector (2)	308

A6.2.1	Carbon Dioxide	308

A6.2.2	Methane	308

A6.2.3	Nitrous Oxide	308

A6.2.4	Hydrofluorocarbons	308

A6.2.5	Perfluorocarbons	308

A6.2.6	Sulphur Hexaflouride	309

A6.2.7	Nitrogen Oxides	309

A6.2.8	Carbon Monoxide	309

A6.2.9	Non Methane Volatile Organic Compounds	309

A6.2.10	Sulphur Dioxide	309

A6.3	Solvents and Other Product Use sector (3)	315

A6.4	Agriculture Sector (4)	317

A6.4.1	Methane	317

A6.4.2	Nitrous Oxide	317

A6.4.3	Nitrogen Oxides	317

A6.4.4	Carbon Monoxide	317

A6.4.5	Non-Methane Volatile Organic Compounds	317

A6.5	Land Use Change and forestry (5)	323

A6.5.1	Carbon Dioxide	323

A6.5.2	Methane	323

A6.5.3	Nitrous Oxide	323

A6.5.4	Nitrogen Oxides	323

A6.5.5	Carbon Monoxide	323

A6.6	Waste (6)	326

A6.6.1	Carbon Dioxide	326

A6.6.2	Methane	326

A6.6.3	Nitrous Oxide	326

A6.6.4	Nitrogen Oxides	326

A6.6.5	Carbon Monoxide	326

A6.6.6	Non-Methane Volatile Organic Compounds	327

A6.6.7	Sulphur Dioxide	327

A7.	ANNEX 7: Uncertainties	330

A7.1	Estimation of Uncertainty by Simulation	330

A7.1.1	Carbon Dioxide Emission Uncertainties	331

A7.1.2	Methane Emission Uncertainties	334

A7.1.3	Nitrous Oxide Emission Uncertainties	335

A7.1.4	Halocarbons and SF6	337

A7.1.5	GWP Weighted emissions	338

A7.1.6	Sectoral Uncertainties	341

A7.2	Estimation of Uncertainties using a Tier 1 Approach	344

A8.	ANNEX 8: Verification	347

A8.1	Modelling approach used for the verification of the UK GHGI	347

A8.2	Results	347

A9.	ANNEX 9: IPCC Sectoral Tables of GHG Emissions	349

- List of Tables and Figures

| - Figures (in the main report) |  | - Section |
| --- | --- | --- |
| - Figure 1.1a | - Main elements for the preparation of the UK greenhouse gas inventory | - 1.2.1 |
| - Figure 1.1b | - Overview of the organisational structure of the UK National Inventory System | - 1.2.2 |
| - Figure 1.2 | - Data collection for the UK greenhouse gas Inventory | - 1.3 |
| - Figure 1.3 | - Data flow through the UK greenhouse gas Inventory | - 1.4 |
| - Figure 1.4 | - System of referencing and documentation used within UK greenhouse gas inventory | - 1.6.1 |
| - Figure 1.5 | - Summary of the system of data checks used within the UK greenhouse gas inventory | - 1.6.1 |
| - Figure 2.1 | - UK Emissions of Greenhouse Gases Weighted by GWP | - 2.4 |
| - Figure 2.2 | - UK Emissions of Greenhouse Gases by Source | - 2.4 |
| - Figure 2.3 | - UK Emissions of Indirect Greenhouse Gases, 1990-2003 | - 2.4 |
| - Figure 10.1 | - Time series of changes in GWP emissions between the inventory presented in the current and the previous NIR, according to IPCC source sector | - 10.3 |
| - Figure 10.2 | - Time series of percentage changes in GWP emissions between the inventory presented in the current and the previous NIR, according to IPCC source sector | - 10.3 |
| - Figure 10.3 | - Time series of changes in total net GWP emissions, and percentage changes in total net GWP emissions, between the inventory presented in the current and the previous NIR | - 10.3 |

| Figures (in the Annexes) |  |  |  |
| --- | --- | --- | --- |
| Figure | Title | Annex | Section |
| A6.1 | UK emissions of direct Greenhouse Gases from IPCC sector 1 1990-2003 | Annex 6 | A6.1.7 |
| A6.2 | UK emissions of indirect Greenhouse Gases from IPCC sector 1 1990-2003 | Annex 6 | A6.1.7 |
| A6.3 | UK emissions of direct Greenhouse Gases from IPCC sector 2 1990-2003 | Annex 6 | A6.2.10 |
| A6.4 | UK emissions of indirect Greenhouse Gases from IPCC sector 2 1990-2003 | Annex 6 | A6.2.10 |
| A6.5 | UK emissions of NMVOC from IPCC sector 3 1990-2003 | Annex 6 | A6.3 |
| A6.6 | UK emissions of direct Greenhouse Gases from IPCC sector 4 1990-2003 | Annex 6 | A6.4.5 |
| A6.7 | UK emissions of indirect Greenhouse Gases from IPCC sector 4 1990-2003 | Annex 6 | A6.4.5 |
| A6.8 | UK emissions and removals of direct Greenhouse Gases from IPCC sector 5 1990-2003 | Annex 6 | A6.5.5 |
| A6.9 | UK emissions of indirect Greenhouse Gases from IPCC sector 5 1990-2003 | Annex 6 | A6.5.5 |
| A6.10 | UK emissions of direct Greenhouse Gases from IPCC sector 6 1990-2003 | Annex 6 | A6.6.7 |
| A6.11 | UK emissions of indirect Greenhouse Gases from IPCC sector 6 1990-2003 | Annex 6 | A6.6.7 |

| - Tables (in the main report) |  | - Section |
| --- | --- | --- |
| - Table 1.1 | - GWP of Greenhouse Gases on a 100 Year Horizon used in the UK | - 1.1.7 |
| - Table 1.2 | - Summary of methods used to estimate emissions of the direct greenhouse gases | - 1.4 |
| - Table 1.3 | - Summary of sources of activity data used to estimate greenhouse gas emissions | - 1.4 |
| - Table 1.4 | - Key source categories | - 1.5 |
| - Table 1.5 | - Schedule of QA/QC Activities | - 1.6.6 |
| - Table 1.6 | - Sources not considered in the UK greenhouse gas inventory | - 1.8 |
| - Table 3.1 | - Methods used for deriving emission estimates for direct and indirect greenhouse gases for CRF Source Category 1A1 | - 3.2.2 |
| - Table 3.2 | - Time series consistency of emission factors (EFs) of direct GHGs used in source category 1A1 | - 3.2.3 |
| - Table 3.3 | - Method for calculation of direct and indirect greenhouse gas emissions from 1A2 | - 3.3.2 |
| - Table 3.4 | - Time series consistency of emission factors of direct GHGs used in source category 1A2 | - 3.3.3 |
| - Table 3.5 | - Time series consistency of emission factors of direct GHGs used in source category 1A3 | - 3.4.3 |
| - Table 3.6 | - Time series consistency of emission factors of direct GHGs used in source category 1A4 | - 3.5.3 |
| - Table 3.7 | - Time series consistency of emission factors of direct GHGs used in source category 1A5 | - 3.6.3 |
| - Table 5.1 | - Paints and their applications in the UK | - 5.2.1 |
| - Table 7.1 | - Emissions of carbon from deep peat due to ploughing for afforestation | - 7.2.5 |
| - Table 7.2 | - Categories used for reporting soils emissions and removals in National Inventory Report and UNFCCC Common Reporting Format | - 7.10 |
| - Table 7.3 | - Emissions and removals of carbon dioxide by activities in Land Use Change and Forestry Sector | - 7.10 |
| - Table 7.4a | - Emissions and removals in categories with the Land Use Change and Forestry Sector as reported in the format used for the National Inventory Report | - 7.10 |
| - Table 7.4b | - Emissions and removals in categories with the Land Use Change and Forestry Sector as reported in the format used for the UNFCCC Common Reporting Format | - 7.10 |
| - Table 8.1 | - Specific Methane Emission Factors for Sludge Handling | - 8.3.2 |
| - Table 8.2 | - Time-Series of Methane Emission Factors for Emissions from Wastewater Handling, based on Population (kt CH4 / million people) | - 8.3.2 |
| - Table 10.1 | - Recalculations of direct GHG emissions in the UK 2005 NIR (2003 inventory) | - 10.1 |
| - Table 10.2 | - Brief details of improvements to the NIR and the inventory in response to reviews | - 10.4.2 |
| - Table 12.1 | - Contributors to this National Inventory Report and the CRF | - 12 |

| Tables (in the Annexes) |  |  |  |
| --- | --- | --- | --- |
| Table | Title | Annex | Section |
| A1.1 | Key Source Analysis Based on Level of Emissions | Annex 1 | A.1.1 |
| A1.2 | Key Source Analysis Based on Trend in Emissions | Annex 1 | A.1.1 |
| A1.3 | Key Source Category Analysis Summary | Annex 1 | A.1.1 |
| A3.1.1 | Mapping of fuels used in the GHGI and the NAEI | Annex 3 | A3.1 |
| A3.2.1 | Mapping of IPCC Source Categories to NAEI Source Categories: IPCC Table 1A | Annex 3 | A3.2 |
| A3.2.2 | Mapping of IPCC Source Categories to NAEI Source Categories: IPCC Table 1B | Annex 3 | A3.2 |
| A3.2.3 | Mapping of IPCC Source Categories to NAEI Source Categories: IPCC Table 2 | Annex 3 | A3.2 |
| A3.2.4 | Mapping of IPCC Source Categories to NAEI Source Categories: IPCC Table 3 | Annex 3 | A3.2 |
| A3.2.5 | Mapping of IPCC Source Categories to NAEI Source Categories: IPCC Table 4 | Annex 3 | A3.2 |
| A3.2.6 | Mapping of IPCC Source Categories to NAEI Source Categories: IPCC Table 5 | Annex 3 | A3.2 |
| A3.2.7 | Mapping of IPCC Source Categories to NAEI Source Categories: IPCC Tables 6 & 7 | Annex 3 | A3.2 |
| A3.3.1 | Emission Factors for the Combustion of Liquid Fuels | Annex 3 | A3.3.1 |
| A3.3.2 | Emission Factors for the Combustion of Coal | Annex 3 | A3.3.1 |
| A3.3.3 | Emission Factors for the Combustion of Solid Fuels | Annex 3 | A3.3.1 |
| A3.3.4 | Emission Factors for the Combustion of Gaseous Fuels | Annex 3 | A3.3.1 |
| A3.3.5 | Conversion Factors for Gross to Net Energy Consumption | Annex 3 | A3.3.2 |
| A3.3.6 | Emission Factors for Power Stations | Annex 3 | A3.3.3.1 |
| A3.3.7 | Components of Emissions Included in Reported Emissions from Civil Aviation | Annex 3 | A3.3.5.1 |
| A3.3.8 | Aircraft Movement Data | Annex 3 | A3.3.5.1 |
| A3.3.9a | Carbon Dioxide and Sulphur Dioxide Emission Factors for Civil and Military Aviation (kg/t) | Annex 3 | A3.3.5.1 |
| A3.3.9b | Non-CO2 Emission Factors for Civil and Military Aviation | Annex 3 | A3.3.5.1 |
| A3.3.10 | Railway Emission Factors | Annex 3 | A3.3.5.2 |
| A3.3.11 | Fuel-Based Emission Factors for Road Transport | Annex 3 | A3.3.5.3.2 |
| A3.3.12 | Fuel Consumption Factors for Road Transport | Annex 3 | A3.3.5.3.3 |
| A3.3.13 | Average Traffic Speeds in Great Britain | Annex 3 | A3.3.5.3.3 |
| A3.3.14a | Vehicle Types and regulation classes | Annex 3 | A3.3.5.3.3 |
| A3.3.14b | Emission Degradation rates permitted for Euro III and IV Light-Duty Vehicles by Directive 98/69/EC | Annex 3 | A3.3.5.3.3 |
| A3.3.15 | Scale Factors for Emissions from a Euro II Bus Running on Ultra-Low Sulphur Diesel and fitted with an oxidation catalyst or CRT | Annex 3 | A3.3.5.3.3 |
| A3.3.16 | Scale Factors for Emissions from a Euro II HGV Running on Ultra-Low Sulphur Diesel and Fitted with an Oxidation Catalyst or CRT | Annex 3 | A3.3.5.3.3 |
| A3.3.17 | NMVOC Emission Factors for Road Transport | Annex 3 | A3.3.5.3.3 |
| A3.3.18 | NOX Emission Factors for Road Transport | Annex 3 | A3.3.5.3.3 |
| A3.3.19 | CO Emission Factors for Road Transport | Annex 3 | A3.3.5.3.3 |
| A3.3.20 | Methane Emission Factors for Road Transport | Annex 3 | A3.3.5.3.3 |
| A3.3.21 | N2O Emission Factors for Road Transport | Annex 3 | A3.3.5.3.3 |
| A3.3.22 | Equations for diurnal, hot soak and running loss evaporative emissions from vehicles with and without control systems fitted | Annex 3 | A3.3.5.3.3 |
| A3.3.23 | Aggregate Emission Factors for Off-Road source categories in 2003 | Annex 3 | A3.3.7.1 |
| A3.3.24 | Methane Emissions for Coal Mining | Annex 3 | A3.3.8.1 |
| A3.3.25 | Emission Factors used for Coke and Solid Smokeless Fuel Production | Annex 3 | A3.3.8.1 |
| A3.3.26 | Aggregate Emission Factors for Offshore Gas Flaring | Annex 3 | A3.3.8.2 |
| A3.3.27 | Activity Data for Offshore Gas Flaring | Annex 3 | A3.3.8.2 |
| A3.3.28 | Aggregate Emission Factors for Offshore Own Gas Use | Annex 3 | A3.3.8.2 |
| A3.3.29 | Aggregate Emission Factors for Offshore Well Testing | Annex 3 | A3.3.8.2 |
| A3.3.30 | Aggregate Emission Factors used for Emission from Platforms and Terminals | Annex 3 | A3.3.8.2 |
| A3.3.31 | Aggregate Emission Factors for Crude Oil loading and unloading | Annex 3 | A3.3.8.2 |
| A3.3.32 | Methane and NMVOC Composition of Natural Gas | Annex 3 | A3.3.8.2 |
| A3.4.1a | Emission Factors for Cement Kilns based on Fuel Consumption | Annex 3 | A3.4.1.1 |
| A3.4.1b | Emission Factors for Cement Kilns based on Clinker Production | Annex 3 | A3.4.1.1 |
| A3.4.1c | Direct GHG Emission Factors for Lime Kilns | Annex 3 | A3.4.1.2 |
| A3.4.1d | Indirect GHG Emission Factors for Lime Kilns | Annex 3 | A3.4.1.2 |
| A3.4.2 | Summary of Nitric Acid Production in the UK | Annex 3 | A3.4.2.1 |
| A3.4.3a | Emission Factors for Blast Furnaces (BF), Electric Arc Furnaces (EAF) and Basic Oxygen Furnaces (BOF) | Annex 3 | A3.4.3.1.2 |
| A3.4.3b | Emission Factors for Aluminium Production | Annex 3 | A3.4.3.2 |
| A3.4.3c | NMVOC Emission Factors for Food and Drink Processing | Annex 3 | A3.4.3.4 |
| A3.6.1 | Livestock Population Data for 2003 by Animal Type | Annex 3 | A3.6.1 |
| A3.6.2 | Methane Emission Factors for Livestock Emissions | Annex 3 | A3.6.1 |
| A3.6.3 | Dairy Cattle Methane Emission Factors | Annex 3 | A3.6.1 |
| A3.6.4 | Beef and Other Cattle Methane Emission Factors | Annex 3 | A3.6.1 |
| A3.6.5 | Cattle Manure Management Systems in the UK | Annex 3 | A3.6.2.1 |
| A3.6.6 | Nitrogen Excretion Factors for Animals in the UK | Annex 3 | A3.6.2.2 |
| A3.6.7 | Nitrogen Excretion Factors for Dairy Cattle | Annex 3 | A3.6.2.2 |
| A3.6.8 | Distribution of Animal Waste Management Systems used for Different Animal Types | Annex 3 | A3.6.2.2 |
| A3.6.9 | Nitrous Oxide Emission Factors for Animal Waste Handling Systems | Annex 3 | A3.6.2.2 |
| A3.6.10 | Dry Mass Content and Residue Fraction of UK Crops | Annex 3 | A3.6.3.3 |
| A3.6.11 | Emission Factors for Field Burning | Annex 3 | A3.6.4 |
| A3.7.1 | Afforestation rate and age distribution of conifers and broadleaves in the United Kingdom since 1922 | Annex 3 | A3.7.1 |
| A3.7.2 | Main parameters for forest carbon flow model for species used to estimates carbon uptake by planting of forests of Sitka spruce in the United Kingdom | Annex 3 | A3.7.1 |
| A3.7.3a | Grouping of MLC land cover types for soil carbon change modelling | Annex 3 | A3.7.3 |
| A3.7.3b | Grouping of Countryside Survey Broad Habitat types for soil carbon change modelling | Annex 3 | A3.7.3 |
| A3.7.4 | Soil carbon stock for depths to 1 m in different land types in the UK | Annex 3 | A3.7.3 |
| A3.7.5a | Sources of land use change data in Great Britain for different periods in estimation of changes in soil carbon | Annex 3 | A3.7.3 |
| A3.7.5b | Sources of land use change data in Northern Ireland for different periods in estimation of changes in soil carbon | Annex 3 | A3.7.3 |
| A3.7.6a | Annual changes in land use in England in matrix form for 1990 to 1999 | Annex 3 | A3.7.3 |
| A3.7.6b | Annual changes in land use in Scotland in matrix form for 1990 to 1999 | Annex 3 | A3.7.3 |
| A3.7.6c | Annual changes in land use in Wales in matrix form for 1990 to 1999 | Annex 3 | A3.7.3 |
| A3.7.6d | Annual changes in land use in Northern Ireland in matrix form for 1990 to 1999 | Annex 3 | A3.7.3 |
| A3.7.7a | Weighted average change in equilibrium soil carbon density to 1m deep for changes between different land types in England | Annex 3 | A3.7.3 |
| A3.7.7b | Weighted average change in equilibrium soil carbon density to 1m deep for changes between different land types in Scotland | Annex 3 | A3.7.3 |
| A3.7.7c | Weighted average change in equilibrium soil carbon density to 1m deep for changes between different land types in Wales | Annex 3 | A3.7.3 |
| A3.7.7d | Weighted average change in equilibrium soil carbon density to 1m deep for changes between different land types in Northern Ireland | Annex 3 | A3.7.3 |
| A3.7.8 | Rates of change of soil carbon for land use change transitions | Annex 3 | A3.7.3 |
| A3.7.9 | Range of times for soil carbon to reach 99% of a new value after a change in land use in England, Scotland and Wales | Annex 3 | A3.7.3 |
| A3.7.10 | Area and carbon loss rates of UK fen wetland in 1990 | Annex 3 | A3.7.5 |
| A3.7.11 | Emission Factors for Peat Extraction | Annex 3 | A3.7.6.2 |
| A4.1 | Modified comparison of the IPCC Reference Approach and the National Approach | Annex 4 | A4.3 |
| A5.1 | GHGs and sources not considered in the UK GHG inventory | Annex 5 | A5 |
| A6.1 | % changes 1990-2003 within sector 1 | Annex 6 | A6.1.7 |
| A6.2 | % changes 2002-2003 within sector 1 | Annex 6 | A6.1.7 |
| A6.3 | % contribution to sector 1 | Annex 6 | A6.1.7 |
| A6.4 | % contribution to overall pollutant emissions | Annex 6 | A6.1.7 |
| A6.5 | % changes 1990-2003 within sector 2 | Annex 6 | A6.2.10 |
| A6.6 | % changes 2002-2003 within sector 2 | Annex 6 | A6.2.10 |
| A6.7 | % contribution to sector 2 | Annex 6 | A6.2.10 |
| A6.8 | % contribution to overall pollutant emissions | Annex 6 | A6.2.10 |
| A6.9 | % changes 1990-2003 within sector 3 | Annex 6 | A6.3 |
| A6.10 | % changes 2002-2003 within sector 3 | Annex 6 | A6.3 |
| A6.11 | % contribution to sector 3 | Annex 6 | A6.3 |
| A6.12 | % contribution to overall pollutant emissions | Annex 6 | A6.3 |
| A6.13 | % changes 1990-2003 within sector 4 | Annex 6 | A6.4.5 |
| A6.14 | % changes 2002-2003 within sector 4 | Annex 6 | A6.4.5 |
| A6.15 | % contribution to sector 4 | Annex 6 | A6.4.5 |
| A6.16 | % contribution to overall pollutant emissions | Annex 6 | A6.4.5 |
| A6.17 | % changes 1990-2003 within sector 5 | Annex 6 | A6.5.5 |
| A6.18 | % changes 2002-2003 within sector 5 | Annex 6 | A6.5.5 |
| A6.19 | % contribution to sector 5 | Annex 6 | A6.5.5 |
| A6.20 | % contribution to overall pollutant emissions | Annex 6 | A6.5.5 |
| A6.21 | % changes 1990-2003 within sector 6 | Annex 6 | A6.6.7 |
| A6.22 | % changes 2002-2003 within sector 6 | Annex 6 | A6.6.7 |
| A6.23 | % contribution to sector 6 | Annex 6 | A6.6.7 |
| A6.24 | % contribution to overall pollutant emissions | Annex 6 | A6.6.7 |
| A7.1 | Estimated Uncertainties in Carbon Dioxide Emissions | Annex 7 | A7.1.1 |
| A7.2 | Estimated Uncertainties in Methane Emissions | Annex 7 | A7.1.2 |
| A7.3 | Estimated Uncertainties in Nitrous Oxide Emissions | Annex 7 | A7.1.3 |
| A7.4a | Summary of Tier 2 Uncertainty Estimates – 1990 to latest year | Annex 7 | A7.1.5 |
| A7.4b | Summary of Tier 2 Uncertainty Estimates – base year to latest year | Annex 7 | A7.1.5 |
| A7.4c | Sectoral uncertainty estimates | Annex 7 | A7.1.5 |
| A7.5a | Summary of Tier 1 Uncertainty Estimates | Annex 7 | A7.2 |
| A7.5b | Summary of Tier 1 Uncertainty Estimates | Annex 7 | A7.2 |
| A8.1 | Verification of the UK emission inventory estimates for methane in Gg yr-1 for 1995-2004 (two-year averages) | Annex 8 | A8.2 |
| A8.2 | Verification of the UK emission inventory estimates for nitrous oxide in Gg yr-1 for 1995-2004 (two-year averages) | Annex 8 | A8.3 |
| A8.3 | Verification of the UK emission inventory estimates for nitrous oxide in Gg yr-1 for 1995-2004 (two-year averages). The NAME emission estimates include the western edge of the North Sea | Annex 8 | A8.3 |
| A8.4 | Verification of the UK emission inventory estimates for HFC-134a in Gg yr-1 for 1995-2004 (two-year averages) | Annex 8 | A8.4.1 |
| A8.5 | Verification of the UK emission inventory estimates for HFC-152a in Gg yr-1 for 1995-2004 (two-year averages) | Annex 8 | A8.4.2 |
| A8.6 | Verification of the UK emission inventory estimates for HFC-125 in Gg yr-1 for 1995-2004 (two-year averages) | Annex 8 | A8.4.3 |
| A9.1.1 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 1990 | Annex 9 |  |
| A9.1.2 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 1991 | Annex 9 |  |
| A9.1.3 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 1992 | Annex 9 |  |
| A9.1.4 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 1993 | Annex 9 |  |
| A9.1.5 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 1994 | Annex 9 |  |
| A9.1.6 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 1995 | Annex 9 |  |
| A9.1.7 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 1996 | Annex 9 |  |
| A9.1.8 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 1997 | Annex 9 |  |
| A9.1.9 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 1998 | Annex 9 |  |
| A9.1.10 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 1999 | Annex 9 |  |
| A9.1.11 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 2000 | Annex 9 |  |
| A9.1.12 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 2001 | Annex 9 |  |
| A9.1.13 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 2002 | Annex 9 |  |
| A9.1.14 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 2003 | Annex 9 |  |
| A9.2.1 | Sectoral report for energy - 1990 | Annex 9 |  |
| A9.2.2 | Sectoral report for industrial processes - 1990 | Annex 9 |  |
| A9.2.3 | Sectoral report for solvent and other product use - 1990 | Annex 9 |  |
| A9.2.4 | Sectoral report for agriculture - 1990 | Annex 9 |  |
| A9.2.5 | Sectoral report for Land-Use Change and Forestry - 1990 | Annex 9 |  |
| A9.2.6 | Sectoral report for waste | Annex 9 |  |
| A9.2.7 | Summary report for national greenhouse gas inventories (IPCC Table 7A) - 1990 | Annex 9 |  |
| A9.2.8 | Sectoral report for energy - 2003 | Annex 9 |  |
| A9.2.9 | Sectoral report for industrial processes - 2003 | Annex 9 |  |
| A9.2.10 | Sectoral report for solvent and other product use - 2003 | Annex 9 |  |
| A9.2.11 | Sectoral report for agriculture - 2003 | Annex 9 |  |
| A9.2.12 | Sectoral report for Land-Use Change and Forestry - 2003 | Annex 9 |  |
| A9.2.13 | Sectoral report for waste | Annex 9 |  |
| A9.2.14 | Summary report for national greenhouse gas inventories (IPCC Table 7A) – 2003 | Annex 9 |  |
| A9.3.1 | Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1990 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF | Annex 9 |  |
| A9.3.2 | Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1991 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF | Annex 9 |  |
| A9.3.3 | Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1992 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF | Annex 9 |  |
| A9.3.4 | Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1993 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF | Annex 9 |  |
| A9.3.5 | Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1994 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF | Annex 9 |  |
| A9.3.6 | Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1995 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF | Annex 9 |  |
| A9.3.7 | Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1996 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF | Annex 9 |  |
| A9.3.8 | Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1997 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF | Annex 9 |  |
| A9.3.9 | Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1998 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF | Annex 9 |  |
| A9.3.10 | Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1999 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF | Annex 9 |  |
| A9.3.11 | Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 2000 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF | Annex 9 |  |
| A9.3.12 | Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 2001 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF | Annex 9 |  |
| A9.3.13 | Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 2002 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF | Annex 9 |  |
| A9.3.14 | Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 2003 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF | Annex 9 |  |

- Document revision history

| - Issue | - Revision history |
| --- | --- |
| - Issue - Draft | - Review by Defra, IGER and CEH - Revised based on comments received |
| - Issue 1.0 | - First issue - Submitted to UNFCCC |

# Introduction

## Background Information on Greenhouse Gas Inventories

### Reporting of the UK greenhouse gas inventory

The United Nations Framework Convention on Climate Change (UNFCCC) was ratified by the United Kingdom in December 1993 and came into force in March 1994.  Parties to the Convention are committed to develop, publish and regularly update national emission inventories of greenhouse gases (GHGs).

- This report provides annual emission estimates submitted by the UK to the UNFCCC for the period 1990 to 2003 in accordance with Decision 18/CP8 and other relevant decisions of the Conference of Parties.  This report and the attached CRF tables comprise the UK's National Inventory Report (NIR).  The UK also reports emissions under other international agreements.  These estimates are provided in the UK’s National Atmospheric Emissions Inventory (NAEI), which is also compiled by NETCEN.  The greenhouse gas inventory and the NAEI share underlying data, which are extended as necessary to cover the additional sources required for UNFCCC reporting.  This ensures consistency between the inventories.  Emissions and removals from land use change and forestry are provided by the Centre of Ecology and Hydrology (CEH) Edinburgh, and agricultural emissions by the Institute of Grassland and Environmental Research (IGER), both under separate contracts to Defra.  Defra also funds research contracts to provide improved emissions estimates for certain sources such as fluorinated gases, landfill methane, and to provide estimates for previously unreported sub-sectors such as methane emissions from closedcoal mines, which are included for the first time in the UK inventory in this submission.

This report and the CRF tables have been prepared according to UNFCCC guidelines contained in FCCC/CP/2002/8 and are provided to fulfil the UK’s reporting obligations to UNFCCC.  The estimates are consistent with the IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories (IPCC, 1997a, b, c) and Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories (IPCC, 2000).

### Nomenclature of the inventory period reported

This report is the UK National Inventory Report for 2005.  It contains the UK greenhouse gas inventory for 2003.  The 2003 inventory contains emissions from 1990 to 2003 inclusive.

### Structure of the UK National Inventory Report

The structure of this report meets the specification set out by the UNFCCC in document FCCC/CP/2002/8.  This document specifies guidelines on reporting and review of greenhouse gas inventories from parties included in Annex I to the Convention.  An Annex of FCCC/CP/2002/8 specifies the sections that should be included in a National Inventory Report, and the contents of each of the sections.

- The main part of the report presents greenhouse gas emissions for the years 1990-2003, and discusses the reasons for the trends and any changes in the estimates due to revisions made since the last inventory.  Tables in Annex 9 present the UK summary emissions for these years and the IPCC Sectoral Tables are also given for the individual years 1990 and 2003.  The Annexes provide supplementary detail of the methodology of the estimates, and explain how the Greenhouse Gas Inventory relates to the IPCC Guidelines and the NAEI.  It contains mappings between IPCC, NAEI source categories and fuel types as well as some emission factors and references to the technical literature.  The Annexes also include sections on the estimation of uncertainties and atmospheric verification of the inventory, and additional detail of the methods used to estimate emissions of GHGs.  The IPCC Good Practice Guidance (IPCC, 2000) requires that certain sets of activity data are reported as well as the Common Reporting Format Tables.  These datasets are included on a CD ROM attached to this report.

- The structure of the 2005 National Inventory Report follows that of the 2004 report, which was changed compared to that of previous National Inventory Reports due to the adoption of FCCC/CP/2002/8.

### Reporting of greenhouse gas emissions and background data in the CRF

- The CRF consists of a series of detailed spreadsheets, with one set for each year.  The CRF reports much more detail than the IPCC Sectoral Tables, in that it contains additional tables of activity data as well as updated versions of the IPCC Sectoral Tables.  A copy of the CRF accompanies this report on a CD ROM.

### Reporting of CO2 emissions from Land Use Change and Forestry

- Carbon dioxide emissions and removals are discussed separately and carbon dioxide removals are given with a negative sign.  Where required by the CRF, carbon dioxide is reported as net emissions (emissions minus the magnitude of removals).  Land Use Change and Forestry Data in both formats are discussed in Chapter 7, Section 7.10.

### Greenhouse gases reported in the UK inventory

The greenhouse gases reported are:

Direct Greenhouse Gases

- Carbon dioxide	(CO2)

- Methane	(CH4)

- Nitrous oxide	(N2O)

- Hydrofluorocarbons	(HFCs)

- Perfluorocarbons	(PFCs)

- Sulphur hexafluoride	(SF6)

Indirect Greenhouse Gases

- Nitrogen oxides	(NOx, as NO2)

- Carbon monoxide	(CO)

- Non-Methane Volatile Organic Compounds	(NMVOC)

- Sulphur dioxide	(SO2)

- These indirect gases have indirect effects on radiative forcing and are requested by the UNFCCC guidelines.

- Emissions estimates are made using methodologies corresponding mostly to the detailed sectoral Tier 2/3 methods in the IPCC Guidelines.

- Most sources are reported in the detail required by the CRF.  The main exceptions are the emissions of individual halocarbon species, which cannot always be reported individually because some of these are considered commercially sensitive data.  Consequently, emissions data have been aggregated to protect this information.  It is however possible to report the total global warming potential of these gases and hence the total global warming potential of all UK greenhouse gases.

### Global Warming Potentials of the greenhouse gases

The direct greenhouse gases have different effectiveness in radiative forcing.  The Global Warming Potential (GWP) is a means of providing a simple measure of the relative radiative effects of the emissions of the various gases.  The index is defined as the cumulative radiative forcing between the present and a future time horizon caused by a unit mass of gas emitted now, expressed relative to that of CO2.  It is necessary to define a time horizon because the gases have different lifetimes in the atmosphere.  Table 1.1 shows GWPs defined on a 100-year horizon (IPCC, 1996).  These are the GWP values required by FCCC/CP/2002/8, consistent with Decision 2/CP3.

Table 1.1	GWP of Greenhouse Gases on a 100-Year Horizon used in the UK NIR

| Gas | GWP |
| --- | --- |
| Carbon Dioxide | 1 |
| Methane | 21 |
| Nitrous Oxide | 310 |
| HFCs | 140-11,700 |
| PFCs | 6,500-9,200 |
| SF6 | 23,900 |

A range of GWP values is shown for HFCs and PFCs because these refer to a number of species, each with its own GWP.  By weighting the emission of a gas with its GWP it is possible to estimate the total contribution to global warming of UK greenhouse gas emissions.

GWPs of certain greenhouse gases have been updated in the IPCC Third Assessment Report (IPCC, 2001).  However, it has been agreed internationally that these will not apply to the Kyoto targets under the first commitment period.  All calculations and inventory submissions throughout this period will be based on the GWPs given in the Second Assessment Report (IPCC, 1996).

### Climate change – The UK programme

The UK’s Climate Change Programme published in November 2000 (Cm4913, 2000), updated in the Third National Communication of October 2001 (Defra, 2001), describes measures to ensure that the UK delivers its legally binding target under the Kyoto Protocol to reduce emissions of the basket of the six greenhouse gases to 12.5% below 1990 levels over the first commitment period 2008-2012, and to move the UK towards its domestic goal of a 20% reduction in carbon dioxide emissions below 1990 levels by 2010.  The UK has additionally a long-term goal of putting itself on a path to cut CO2 emissions by 60% by 2050, with real progress by 2020.  This is described in the Energy White Paper published in February 2003, (DTI, 2003).

The UK’s Climate Change Programme was developed in partnership with the Scottish Parliament, the National Assembly for Wales and the Department of the Environment in Northern Ireland.  It is currently under review and a Consultation Paper was published in December 2004 available at: http://www.defra.gov.uk/corporate/consult/ukccp-review/ccpreview-consult.pdf.  A revised programme is expected to be available by summer 2005.

Further information on the UK’s action to tackle climate change is provided to the public through the Defra website on www.defra.gov.uk/environment/climatechange

## Institutional arrangements for inventory preparation

The UK Greenhouse Gas Inventory is compiled and maintained by the National Environmental Technology Centre (NETCEN) of AEA Technology plc – the Inventory Agency - under contract with the Global Atmosphere Division (GAD) in the UK Department for Environment, Food & Rural Affairs (Defra).  NETCEN is directly responsible for producing the emissions estimates for CRF categories Energy (CRF sector 1), Industrial Processes (CRF sector 2), Solvent and Other Product Use (CRF sector 3), and Waste (CRF Sector 6).  NETCEN is also responsible for inventory planning, data collection, QA/QC and inventory management and archiving.  Agricultural sector emissions (CRF sector 4) are produced by the Defra’s Land Management Improvement Division by means of a contract with the Institute of Grassland and Environmental Research (IGER).  Land-Use Change and Forestry emissions (CRF sector 5) are calculated by the UK Centre for Ecology and Hydrology (CEH), under separate contract to GAD.

### The UK Greenhouse Gas National Inventory System (UK NIS)

The Marrakesh Accords of the Kyoto Protocol (Decision 20/CP7) define the requirements for National Inventory Systems (NIS), including the need to establish legal, procedural and organisational arrangements to ensure that all parties to the Protocol estimate and report their GHG emissions in accordance with relevant decisions of the COP, facilitate UNFCCC Reviews and improve the quality of their inventories.  Under related EU legislation set out in Decision 280/2004/EC the UK is required to have in place its NIS by 31st December 2005.  Many aspects of the UK NIS already meet the UNFCCC’s requirements.  The development of more formal agreements between Defra and Key Data Providers (KDPs) is ongoing and will specify the framework of data supply e.g. data quality, format, timeliness and security to underpin the GHG inventory.

Figure 1.1a shows the main elements the UK National Inventory System, including provision of data to the European Union under the terms of the EU Monitoring Mechanism. Defra is the Single National Entity responsible for submitting the UK's greenhouse gas inventory (GHGI) to the UNFCCC.  NETCEN compiles the GHGI on behalf of Defra, and produces disaggregated estimates for the Devolved Administrations within the UK.

Key Data Providers include other Government Departments such as Department for Trade and Industry (DTI) and Department for Transport (DfT), Non-Departmental Public Bodies such as the Environment Agency for England and Wales (EA) and the Scottish Environmental Protection Agency (SEPA), private companies such as Corus, and business organisations such as UK Petroleum Industry Association (UKPIA) and UK Offshore Oil Association (UKOOA).

Figure 1.1a	Main elements for the preparation of the UK greenhouse gas inventory

### UK NIS - Organisational Structure

Figure 1.1b provides an overview of the organisational structure of the UK National Inventory System.

Figure 1.1b	Key organisational structure of the UK National Inventory System

The following sections summarises the roles and responsibilities of key stakeholders in the UK NIS.

#### Single National Entity: Defra

As the designated Single National Entity for the UK GHG NIS, Defra has the following roles and responsibilities:

NIS Management & Planning

- Overall control of the NIS development & function;

- Management of contracts & delivery of GHG inventory;

- Definition of performance criteria for NIS key organisations.

Development of Legal & Contractual Infrastructure

- Review of legal & organisational structure;

- Implementation of legal instruments and contractual developments as required to meet guidelines.

#### Inventory Agency: Netcen

As the designated Inventory Agency for the UK GHG NIS, Netcen has the following roles and responsibilities:

Planning

- Co-ordination with Defra to deliver the NIS;

- Review of current NIS performance and assessment of required development action;

- Scheduling of tasks and responsibilities to deliver GHG inventory and NIS.

Preparation

- Drafting of agreements with key data providers;

- Review of source data & identification of developments required to improve GHG inventory data quality.

Management

- Documentation & archiving;

- Dissemination of information regarding NIS to Key Data Providers;

- Management of inventory QA/QC plans, programmes and activities.

Inventory Compilation

- Data acquisition, processing and reporting;

- Delivery of NIR (including associated CRF tables) to time and quality.

#### Key Data Providers

Within the UK GHG NIS, organisations that are Key Data Providers have the following roles and responsibilities:

Data Quality, Format, Timeliness, Security

- Delivery of source data in appropriate format and in time for inventory compilation, allowing for all required QA/QC procedures;

- Assessment of their data acquisition, processing & reporting systems, taking regard for QA/QC requirements;

- Identification of any required organisational or legal development and resources to meet more stringent NIS data requirements, notably the security of data provision in the future;

- Communication with Defra, Netcen and their peers / members to help to disseminate information regarding NIS.

### UK NIS - Key Data Sources

The GHGI is compiled using the same database as the UK National Atmospheric Emissions Inventory (NAEI).  The NAEI is used for reporting under other international agreements and includes emission estimates for greenhouse gases, regional pollutants leading to acid deposition and photochemical pollution, persistent organic pollutants and other toxic pollutants such as heavy metals.  The NAEI is available at http://www.naei.org.uk.

Energy statistics required for compilation of the NAEI and the GHGI are obtained from the Digest of UK Energy Statistics (DUKES).  DUKES is compiled and published by the Department of Trade and Industry (DTI).  DUKES is available at

http://www.dti.gov.uk/energy/inform/energy_stats/index.shtml

Information on industrial processes is provided either directly to NETCEN by the individual plant operators or from:

- the Environment Agency's Pollution Inventory (PI) for England & Wales;

- the Scottish Environmental Protection Agency’s European Pollution Emissions Register (SEPA EPER for Scotland);

- the Northern Ireland Department of Environment Inventory of Statutory Releases (ISR for Northern Ireland).

Reporting to these UK inventories for the purposes of environmental regulation is a statutory requirement for industries covered by IPPC.  The data from these inventory sources is also used to quality check data provided voluntarily by companies directly to NETCEN.

The Institute of Grassland and Environmental Research (IGER) compiles the inventory for agricultural emissions using agricultural statistics from Defra.

The Centre for Ecology and Hydrology (CEH) compiles estimates of emissions and removals from LUCF using land-use data and information on forestry from the Forestry Commission (a non-departmental public body), Defra itself, and from other sources.

Defra also funds research contracts to provide emissions estimates for certain sources.  For example, AEA Technology, in consultation with industry, provides improved emission estimates of HFCs, PFCs and SF6 (AEAT, 2003).  Landfill methane emissions estimates were compiled by an independent consultancy (Land Quality Management), in consultation with industry.  A recent Defra-funded study has provided estimates for methane emissions from closed coal mines (Kershaw, 2005). These emissions have therefore been included in the GHG inventory for the first time, reported within IPCC sector 1B1 – Solid Fuels.

The GHGI is compiled according to IPCC Good Practice Guidance (IPCC, 2000).  Each year the inventory is updated to include the latest data available.  Improvements to the methodology are made and are backdated to ensure a consistent time series.  Methodological changes are made to take account of new data sources, or new guidance from IPCC, relevant work by CORINAIR, new research, or specific research programmes sponsored by Defra.

## Process of inventory preparation

Figure 1.2 outlines the main elements of the data collection system used in the UK inventory. The data acquisition task provides the fundamental activity data from which the GHG inventory is constructed.  Starting in July, requests for data are issued.  A database of contacts is used to track progress of the data acquired.

Figure 1.2	Data collection for the UK greenhouse gas inventory

The following activities are carried out each year, in order, as the inventory is compiled:

- Method Improvement.

Improvements to calculation methods are normally implemented before the inventory is compiled.  These improvements are in part based on recommendations of UNFCCC (In Depth Reviews, In Country Reviews), peer reviews and relevant research sponsored by Defra or other organisations.

- Data Requests.

Requests for activity data and background data are issued to a wide range of data suppliers.  Each request is issued with a unique code, and a database is used to track the request and the data supplied from that request.

- Data Verification.

Activity data received are examined. Anomalies are investigated, such as time series discrepancies, or large changes in values from the previous to the current inventory year.

- Data Processing.

Data are prepared to allow emissions of direct and indirect GHG to be estimated.

- Emission Estimation.

Provisional emissions are estimated using the most recent activity data available.

- Emissions Review.

A series of internal reviews are carried out to detect anomalies in the estimates (time series variations and year to year changes).  Errors and omissions are then rectified.

- Emissions Reporting (including background data).

Estimates of emissions are prepared for the various reporting formats (e.g. IPCC, UNECE etc.).

- Report Generation.

Draft reports are written to satisfy the reporting criteria of the various agencies, e.g. the IPCC.

- Report Review.

The reports are reviewed: internally; by external contributing agencies; and by Defra.  Errors and omissions are then rectified.

- Report Publication.

Final reports and data sets are then submitted and published in print and on publicly available web sites.

- The system outlined above complies with the Tier 1 QA/QC procedures outlined in Table 8.1 of the Good Practice Guidance (IPCC, 2000).

## Methodologies and Data sources

Figure 1.3	Data flow through the UK greenhouse gas inventory

The methods used to estimate emissions are described in detail in the relevant sections of this report.  The direct and indirect GHGs reported are estimated using methodologies corresponding mostly to the detailed sectoral Tier 2/3 methods in the IPCC Guidelines.  Table 1.2 provides a brief summary of the methods used to estimate UK GHG emissions, which are described in more detail in the subsequent Chapters and Appendices.

Table 1.2	Summary of methods used to estimate emissions of the direct greenhouse gases

| CRF sector | Comments on methods |
| --- | --- |
| 1A | Basic combustion module (see Annex 3, Section A3.3.1) Transport model (see Annex 3, Section A3.3.5) |
| 1B | Carbon Balance approach (See Annex 3, Section A3.3.8.1.2) UKOOA EEMS inventory (See Annex 3, Section A3.3.8.2) Transco gas leakage model (See Annex 3, Section A3.3.8.2.6) |
| 2A | Cement production: IPCC Tier 2 approach (see Chapter 4, Section 4.4.2) |
| 2B | Emissions calculated based on data from industry and the Pollution Inventory |
| 2C | Iron and Steel - 2 stage carbon balance (see Annex 3, Section A3.3.3.3 and A3.4.3.1) Spreadsheet model to estimate emissions of F-gases |
| 2D | Emissions calculated based on USEPA Compilation of Air Emission Factors Emissions calculated based on Industry and Government data sources |
| 2E, 2F | Spreadsheet model to estimate emissions of F-gases |
| 3A | (No direct GHGs emitted from this sector) |
| 3B | (No direct GHGs emitted from this sector) |
| 3C | (No direct GHGs emitted from this sector) |
| 3D | (No direct GHGs emitted from this sector) |
| 4A | Emissions calculated based on animal population data and appropriate EFs |
| 4B | Emissions calculated based on animal population data and appropriate EFs |
| 4D | IPCC recommended methodology |
| 4F | Emissions calculated based on IPCC methodologies and USEPA EFs |
| 5 | Spreadsheet model to estimate emissions from LUCF |
| 6 | IPCC recommended methodologies and LQM Solid Waste Disposal model |
| 7 | (No emissions reported in this sector) |

The sources of data used are documented in the relevant sections of this NIR though much of the activity data are taken from the key publications listed in Table 1.3.  All sources are updated annually.

Table 1.3	Summary of sources of activity data used to estimate greenhouse gas emissions

| Source (and publisher) | Relevant activity data contained in the source |
| --- | --- |
| Digest of UK Energy Statistics (UK Department for Trade and Industry) | - Energy statistics for the UK (imports, exports, production, consumption, demand) of liquid, solid and gaseous fuels - Calorific values of fuels and conversion factors |
| Transport Statistics GB (UK Department for Transport) | - Vehicle km according to vehicle type and road type - Vehicle licensing statistics (split in vehicle km by fuel type) - Selected domestic and international civil aviation aircraft km flown |
| Northern Ireland Department of the Environment | - Traffic count and vehicle km data for Northern Ireland - Information on regulated processes in NI |
| Civil Aviation Authority | - Detailed domestic and international civil aviation aircraft km flown |
| Pollution Inventory (Environment Agency) | - Information on emissions from regulated processes in England and Wales |
| Scottish Environmental Protection Agency | - Information on regulated processes in Scotland |
| United Kingdom Petroleum Industry Association | - Refinery emissions, - Lead and sulphur contents of fuels, benzene content of petrol, RVP of petrol |
| United Kingdom Offshore Operators Association | - Detailed inventory of oil & gas emissions |
| Iron and Steel Statistics Bureau | - Energy production and consumption in the Iron and Steel industry - Other statistics regarding the Iron and Steel industry |
| United Kingdom Minerals Yearbook (British Geological Society) | - Statistical data on minerals production, consumption and trade |
| Annual Abstract of Statistics (Office for National Statistics) | - Population data |

## description of key source categories

Key sources are defined as the sources of emissions that have a significant influence on the inventory as a whole, in terms of the absolute level of the emissions, the trend, or both.  Table 1.4 summarises the key source categories derived from the IPCC Tier 1 analysis.  Details of the key source analysis are given in Annex 1.

Table 1.4	Key Source Categories

| IPCC source category | Fuel | GHG | Reason (s) |
| --- | --- | --- | --- |
| 1A | Coal | CO2 | Level |
| 1A | Natural Gas | CO2 | Level |
| 1A3b | Auto Fuel | CO2 | Level |
| 5A | Land Use Change & Forestry | CO2 | Level |
| 5D | Land Use Change & Forestry | CO2 | Level |
| 4A | Enteric Fermentation | CH4 | Level |
| 6A | Solid Waste Disposal | CH4 | Level, Trend |
| 1A2&1A2&1A4&1A5 | Other Combustion | N2O | Level, Trend |
| 1A3b | Auto Fuel | N2O | Level, Trend |
| 2B | Adipic Acid Production | N2O | Trend |
| 2B | Nitric Acid Production | N2O | Level, Trend |
| 4B | Manure Management | N2O | Level |
| 4D | Agricultural Soils | N2O | Level, Trend |
| 6B | Wastewater Handling | N2O | Level, Trend |
| 2 | Industrial Processes | HFC | Level |

## QA/QC plan

This section presents the general QA/QC plan for the UK GHGI, including verification and treatment of confidentiality issues.  The current system complies with the Tier 1 procedures outlined in the Good Practice Guidance (IPCC, 2000).  The system is being developed and the range of activities extended so that the system complies with Tier 2.

Source specific QA/QC details are discussed in the relevant sections of this NIR.  Where there is currently insufficient detail available to provide source specific QA/QC, more general information is given in the relevant section of the NIR.

### Description of the QA/QC current system

The National Atmospheric Emissions Inventory and the UK Greenhouse Gas Inventory are compiled and maintained by the National Environmental Technology Centre of AEA Technology plc.  Whilst significant parts of the inventory (i.e. agriculture, land use change and forestry) are compiled by other agencies and contractors, NETCEN is responsible for co-ordinating QA/QC activities.

The system has developed over the years.  A new on-line database system was adopted for the 1997 Inventory in 1998, and since then, developments have proceeded to build QA/QC procedures into the on-line system.  The database consists essentially of a table of activity data and a table of emission factors for the NAEI base source categories.  These are then multiplied together to produce emissions according to the IPCC and CORINAIR formats to be generated.

The Inventory has been subject to ISO 9000 since 1994 (it is now subject to BS EN ISO 9001:2000) and is audited by Lloyds and the AEA Technology internal QA auditors.  The NAEI has been audited favourably by Lloyds on three occasions in the last ten years.  The emphasis of these audits was on authorisation of personnel to work on inventories, document control, data tracking and spreadsheet checking, and project management.  As part of the Inventory management structure there is a nominated officer responsible for the QA/QC system – the QA/QC Co-ordinator. The National Environmental Technology Centre is currently accredited to BS EN ISO 9001:2000, and was last audited in May 2003 by Lloyds.

UK Defra is in the process of implementing EU Decision 280/2004/EC on a mechanism for monitoring Community greenhouse gas emissions, and for implementing Kyoto Protocol quality and reporting requirements.  This includes considering arrangements with all the major data providers which would cover QA/QC as well as the timeliness and format of data delivered.  Existing arrangements already help to ensure the secure provision of assured data, including energy balance data from DTI and industrial point source emissions data from UK regulatory sources under IPPC and EU-ETS.

Figure 1.4	System of referencing and documentation used within UK greenhouse gas inventory

The system incorporates the following activities (see Figure 1.4), which are carried out each year as the inventory is compiled:

1.	Documentation

- Data received by NETCEN are logged, numbered and should be traceable back to their source from anywhere in the system.

- The inventory is held as a database of activity data and emission factors.  Within the database these data fields are referenced to the data sources, or the spreadsheet used to calculate the data.  For fuel consumption data, the DUKES (DTI, 2004) table numbers are identified.

- There is an on-line system of manuals, which defines timetables, procedures for updating the database, document control, checking procedures and procedures for updating the methodology manual.

- There is an on-line methodology manual, which is updated as the inventory data are entered.  This contains details of the methodology used, emission factor and activity data sources, discussion of the rationale for choice of methodology and emission factors.

- An annual report outlining the methodology of the inventory, data sources and changes made is produced.

2.	Database

- The classification of source categories is controlled by a formatting table in the database, which is used to generate emissions tabulated in the IPCC format.  Other simple queries can be used to extract all emissions data contained in the database.  These can be compared against the tabulated output to check that all sources are output and that the totals are correct.  A consistency check between IPCC output and CORINAIR formatted output is made.  Data in the CRF reporting tool are checked against the database totals.

- All fields in the database are labelled automatically with an NAEI activity/fuel category, the CORINAIR SNAP code and the units used.  A comment field linked to each data entry provides further description and the data source or spreadsheet used to calculate it.

3.	Checking

- NETCEN’s QA/QC system requires that spreadsheet calculations are checked and the checks applied are described.  Also the data sources used for calculations must be referenced on the spreadsheet.

- Data entry into the database is checked in a majority of cases by a second person.

- A system has been developed to check the fuel entries in the database.  Queries will abstract and total the fuel consumption’s for each fuel.  These totals are then checked against the totals reported in the DTI publication DUKES.

- The final checks on the inventory involve a consistency check against the previous inventory for the same year.  A designated auditor identifies sources where there have been significant changes or new sources.  Inventory staff are required to explain these changes in the inventory to satisfy the auditor.

- A further final check is made on the inventory comparing the emissions of the latest year with those of the previous year (within the same version).  A designated checker identifies sources where there have been significant changes.  Inventory staff are required to explain these changes in the inventory to satisfy the checker.  This is somewhat more detailed than the recalculation explanations required by Table 8 in the CRF, because it is based on the more disaggregated source sectors used in the NAEI database.

4.	Recalculation

- When revisions are made to the methodologies of the estimates, emissions for all previous years are recalculated.

5.	Uncertainties

- Estimates are made of the uncertainties in the estimates according to Tier 1 and Tier 2 procedures.

- A ranking exercise is performed according to Tier 1 procedures to identify key source categories.

6.	Archiving

- At the end of each reporting cycle, all the database files, spreadsheets, on-line manual, electronic source data, paper source data, output files are in effect frozen and archived.  An annual report outlining the methodology of the inventory and data sources is produced.  Electronic information is stored on hard disks that are regularly backed up.  Paper information is being archived in a Lektreiver® system and there is a simple database of all items in the archive.

The system outlined in the text above complies with the Tier 1 procedures outlined in Table 8.1 of the Good Practice Guidance (IPCC, 2000).  A review of the QA/QC procedures was carried out in 2001 (Salway, 2001).

Figure 1.5	Summary of the system of data checks used within the UK greenhouse gas inventory
(The yellow vertical bars symbolise ‘gates’ through which data should not pass until the appropriate checks have been performed)

### Special QA/QC activities undertaken in 2004-2005

This section describes certain specific activities relating to QA/QC that were carried out in the last year.

Data Processing and Storage Systems of the Inventory Agency

The data processing systems and GHG inventory database used by the Inventory Agency have been significantly improved, leading to a number of QC system improvements such as: a reduction in errors made when data is entered into the database; more stable data storage and management systems; and an improved data audit trail.

Key Data Provider Meetings

Meetings have been held between Netcen and several key organisations to identify areas for GHGI improvements including:

- The DTI team that compile the UK’s consumptions of solid, liquid and gaseous fuels;

- The Environment Agency Pollution Inventory team;

- The Scottish Environmental Protection Agency EPER team;

- The CEH, to discuss the methods used to estimate emissions from LUCF and the reporting of LUCF emissions.

The programme of stakeholder meetings will continue during summer 2005, with meetings planned with the British Cement Association and Corus (a major producer of iron and steel in the UK).

Development of Agreements with Data Providers

UK Defra is the process of implementing an EU Decision 280/2004/EC on a mechanism for monitoring Community greenhouse gas emissions and for implementing the Kyoto protocol.  Specific developments to meet the requirements of the UK National Inventory System include the arrangements with key data providers to ensure quality, timely, secure data provision to the UK GHG inventory.

### Future development of the QA/QC system

The review discussed in the previous section (Salway, 2001) identified areas where the current QA/QC system could be improved and the need for additional activities to comply with Tier 2.  These developments are now included in the QA/QC plan, elements of which are described in this section.

### Compliance of National Statistical Agencies

Many of the data received by NETCEN come from other government departments, agencies, research establishments or consultants.  Some of these organisations (e.g. DTI, IGER and BGS) would qualify as the National Statistical Agencies referred to in the Guidance.  Other organisations (e.g. CEH) compile significant parts of the Inventory; data complied by other organisations are used to compile significant parts of the inventory (e.g. the Pollution Inventory).  We are contacting these organisations and inviting them to show how their QA/QC systems comply with IPCC Good Practice Guidance.

### Documentation and review

The inventory is documented in the National Inventory Report.  The NIR describes the methods used to estimate emissions and presents underlying activity and emission factor data.  The Good Practice Guidance highlights the need for review of methodologies during inventory compilation.  A list collating and prioritising improvements identified by the Inventory Agency, and from Expert and Peer Reviews, is maintained by the Inventory Agency.  These improvements to the inventory are implemented as necessary.

### External Peer Review

Tier 2 of the Good Practice Guidance requires that key sources should be subjected to external peer review.  During 2002, the UK implemented a programme of peer reviews by experts outside of the organisation responsible for the estimates.  The first peer review on CO2 emissions from fossil fuel has been completed (Simmons, 2002).  Recommendations from this Peer Review, which have now been implemented, include: an improved method for estimating emissions from domestic and international civil aviation; a review of the carbon emission factors used in the UK GHG inventory; and a review of the proportion of recycled lubricants burnt.

The second Peer Review on agriculture was carried out in March 2005.  The external reviewers were Prof. Ulrich Daemmgen (Institute of Agroecology, Germany) and Ulrike Doering (Federal Environmental Agency, Germany).  Both Prof. Ulrich Daemmgenm and Ulrike Doering are internationally recognised experts in the technical area of agriculture.  The review team also included the GHG agricultural expert from UK IGER (Lorna Brown) and John Watterson and Chris Dore from NETCEN (representing the Inventory Agency).  The review covered: the methods used to estimate agricultural emissions, including emissions from agricultural soils (N2O), manure management (N2O) and enteric fermentation (CH4); the underlying activity data and emission factors; uncertainties; and the QA/QC of the emission estimates.  The recommendations of the review will be used to help improve the accuracy of the emission estimates from the agricultural sector.

The programme for the external Peer Review is shown in Table 1.5.

Table 1.5	Schedule of QA/QC Activities

| Activity | 2004/2005 | 2005/2006 |
| --- | --- | --- |
| Special Activities |  | Full review of completeness and accuracy of 1990 and base year emissions |
| On-going Activities | On-Going Tier 1 Activities | On-Going Tier 1 Activities |
|  |  | Acid Plant Documentation (update) |
|  | Document Source Reviews | Document Source Reviews |
| External Peer Review | National Report Nitric Acid (N2O) LUCF 5A, 5D & 5E (CO2) Wastewater (N2O) Landfill (CH4) | National Report Nitric Acid (N2O) LUCF 5A, 5D & 5E (CO2) Wastewater (N2O) Landfill (CH4) Fugitive gas (CH4) |

Notes

1	Refers to period between inventory submission i.e. April to April

### Verification

Verification is covered as part of the QA/QC checks and by the background research undertaken by Defra.  In addition, GAD contributes support and analysis of the continuous high-frequency observations of the Kyoto gases at the Mace Head Atmospheric Research Station on the Atlantic Ocean coastline of Ireland.  The UK Met Office employs the Lagrangian dispersion model NAME (Numerical Atmospheric dispersion Modelling Environment) driven by 3D synoptic meteorology from the Unified Model to sort the observations made at Mace Head into those that represent northern hemisphere baseline air masses and those that represent regionally-polluted air masses arriving from Europe.  The Lagrangian dispersion model is then used to estimate the magnitude and spatial distribution of the European emissions that best support the observations.  The technique has been applied to 2-yearly rolling subsets of the data.

The complete results of this verification and a more detailed description of the modelling method used are given in Annex 8.

### Treatment of confidentiality

Nearly all of the data necessary to compile the UK inventory are publicly available.  The only exception relates to the reporting of emissions from PFCs and HFCs.  It is not possible to report emissions by PFC or HFC species as much of the data were obtained from private companies on condition such emissions remain confidential.  However, estimates of the total GWP of emissions in the main IPCC categories are provided.

The UK National Inventory Reports from the 1999 NIR onwards and estimates of emissions of GHGs are all publicly available on the web; see http://www.naei.org.uk

## General Uncertainty Evaluation

The UK GHG inventory estimates uncertainties using both the Tier 1 and Tier 2 methods described by the IPCC.  The Tier 1 approach provides estimates of uncertainties by pollutant according to IPCC sector.  The Tier 2 approach provides estimates according to GHG (1990, base year and latest reporting year) and has now been extended to provide emissions by IPCC sector.

The uncertainty in the combined GWP weighted emission of all the greenhouse gases in 2003 was estimated as 14% and in 1990 as 14% also.  The trend in the total GWP weighted emissions expressed as the fall between 1990 and 2003 is -13%, with 95% of the values found to lie within the range -11% and -16%.  The source making the major contribution to the overall uncertainty is 4D – Agricultural Soils.  This source shows little change over the years, but other sources have fallen since 1990.

A full description of the uncertainty analysis is presented in Annex 7.  The uncertainty estimates for all gases are summarised in Table A7.4a and Table A7.4b.

## General Assessment of Completeness

The UK GHG inventory aims to include all anthropogenic sources of GHGs.  Table 1.6 shows sources of GHGs that are not estimated in the UK GHG inventory, and the reasons for those sources being omitted.  This table is taken from the CRF; Table “Table9s1”.  There is more information about the completeness of the GHG inventory in Annex 5.

A new source, methane from closed coal mines, is now included in the GHG inventory following publication of the findings of a recent Defra study (Kershaw, 2005).  These emissions are accounted within sector 1B1 – Solid Fuels, and are described within Section 3.12 of this report.

Table 1.6	Sources not considered in the UK greenhouse gas inventory

| GHG | CRF sector | Source/sink category | Reason |
| --- | --- | --- | --- |
| CO2 | 2. Industrial Processes | 2A5/6 Asphalt Roofing/Paving | No methodology available |
| CO2 | 3. Solvent and Other Product Use |  | Carbon equivalent of solvent use not included in total - provided for information |
| CO2 | 5. Land-Use Change and Forestry | 5C2/5C4 Abandonment of Managed Lands | Considered negligible |
| CH4 | 2. Industrial Processes | 2B1 Ammonia Production | Manufacturers do not report emission - believed negligible |
| CH4 | 2. Industrial Processes | 2C1 Iron and Steel | EAF emission and flaring only estimated - methodology not available for other sources |
| CH4 | 2. Industrial Processes | 2C2 Ferroalloys | Methodology not available |
| CH4 | 2. Industrial Processes | 2C3 Aluminium | Methodology not available |
| CH4 | 6. Waste | 6B1 Industrial Waste Water | Activity data unavailable - most waste water treated in public system- believed small |
| N2O | 3. Solvent and Other Product Use | 3D Other -Anaesthesia | Activity not readily available - believed small |

## Geographical coverage of the UK greenhouse gas inventory

A major source of activity data for the UK inventory is provided by the UK DTI through their publication the Digest of UK Environmental Statistics (DUKES) (see Table 1.3), and the geographical coverage of DUKES helps define the geographical coverage of the inventory.

The DTI advises that the geographical coverage of the statistics is the United Kingdom (DTI, 2004).  Shipments to the Channel Islands and the Isle of Man from the United Kingdom are not classed as exports, and supplies of solid fuel and petroleum to these islands are therefore included as part of the United Kingdom inland consumption or deliveries.

The definition of the UK used by the DTI accords with that of the "economic territory of the United Kingdom" used by the UK Office for National Statistics, which in turn accords with the definition required to be used under the European System of Accounts (ESA95).

# Trends in Greenhouse Gas Emissions

## Emission trends for aggregated greenhouse gas emissions

As already described in Chapter 1, there are six direct greenhouse gases, each with different global warming potentials.  In 2003, the total direct greenhouse gas emissions (including LUCF emissions) in the UK were approximately 665.8 Mt CO2 equivalent.  This was approximately 103 Mt CO2 equivalent (13%) below the 1990 level.  There was a reduction of about 13.4% relative to the base year under the Kyoto Protocol, which includes F gas emissions for 1995 rather than 1990.

The following sections summarise the emission trends between 1990-2003 for the aggregated greenhouse gases, both by gas and by source.  For a detailed breakdown on emission trends for all gases in all sectors, the reader should refer to Annex 6.  Unless otherwise indicated, percentages quoted are relative to gross emissions rather than net emissions (i.e. emissions minus removals from LUCF).

## Emission trends by gas

The tables shown in ES2 together with Figure 2.1 show that the largest contributor to global warming is carbon dioxide at 86% of the weighted emission.  Methane	 and nitrous oxide each contribute 6%.  In spite of their high GWPs the contribution of halocarbons is small at around 1.9% of the total.  This is because their mass emissions are very small.  Overall the total weighted emission has fallen by 13% since 1990 (13.4% relative to the base year under the Kyoto Protocol), with all gases declining except SF6.

In 2003, CO2 emissions were 572 Tg CO2 equivalent, 6% below the 1990 level.  A small increase in emissions was observed between 2000 and 2001 due to reduced nuclear output, higher coal burn relative to gas and lower outside temperatures.

Methane is the second most significant greenhouse gas in the UK after CO2.  In 2003, methane emissions were 41 Tg CO2 equivalent. Unlike most of the other major pollutants in the Greenhouse Gas Inventory, fuel combustion is not the predominant source of methane.  The major sources are agriculture, waste disposal, leakage from the gas distribution system and coal mining.  Since 1990, emissions of methane have decreased by 48%.

Emissions of nitrous oxide are uncertain because there are many small sources, both natural and anthropogenic.  The main anthropogenic sources are agriculture, transport, industrial processes, and coal combustion.  In 2003, emissions of nitrous oxide were 40 Tg CO2 equivalent.  Emissions have declined 41% since 1990.  This decline is due to decreases in emissions from both the agricultural and industrial sectors.

Emissions of the F-gases (HFCs, PFCs, and SF6) totalled 12.6 Tg CO2 equivalent in 2003.  Since 1990 the overall decrease in their emissions has been 9%, although individually, emissions from SF6 have increased by 44%.  Sources of F-Gases within the UK include: refrigeration and air conditioning, halocarbon production,  aerosols and Metered Dose Inhalers (MDI), aluminium production and electronics.

## Emission trends by source

Weighted greenhouse gas emissions broken down by sector are shown in Figure 2.2.  Clearly, the largest contribution is from the energy sector, which contributes 85% to the total emissions.  Within this category the largest contributions arise from the energy industries (38%) and transport (23%).  Category 1A4 (other sectors) and 1A2 (Manufacturing, Industry and construction) also have a significant impact on the emissions of this sector. Energy sector emissions have declined by about 7% since 1990, due to fuel switching, and reduced energy intensity of the economy.

The next largest contribution comes from the agricultural sector.  This contributes approximately 7% to the total emissions.  The emissions from this sector have shown an overall decrease of 14% since 1990, reflecting trends in livestock numbers and emissions from fertiliser application.

The industrial processes sector (sector 2) contributes 4% to total greenhouse gas emissions.  Emissions from this sector include non-energy related emissions from mineral products, chemical industry and metal production as well as emissions from the F-gases.  Since 1990, this category has seen a decline in emission of 53%, mostly due to a change in the emissions from the chemical industry

Land-use change and forestry (sector 5) contributes 2% to total greenhouse gas emissions in 2003.  Emissions from this sector have decreased by 16% since 1990, due to changes in the pattern of land use. Removals due to the forest sink are currently about 3% of gross emissions.

Emissions from the waste sector contributed 1.7% to greenhouse gas emission in 2003.  Emissions consist of CO2, N2O and CH4 from waste incineration, and CH4 and N2O from both solid waste disposal on land and wastewater handling.  Overall emissions from the waste sector have decreased by 60% since 1990 and this is mostly due to the implementation of methane recovery systems.

## Emission trends for indirect greenhouse gases and SO2

The indirect greenhouse gases in the UK consist of Nitrogen Oxides (NOX), Carbon Monoxide (CO), Non-Methane Volatile Organic Compounds (NMVOC) and Sulphur dioxide (SO2).  Of these, NOx, CO and NMVOC can increase tropospheric ozone concentration and hence radiative forcing.  Sulphur dioxide contributes to aerosol formation in the atmosphere.  This is believed to have a negative net radiative forcing effect, tending to cool the surface.  Emission trends for the indirect greenhouse gases are shown in Figure 2.3.

The main source of NOX in the UK is fuel combustion.  These emissions are complex as the nitrogen can be derived from both the fuel and the combustion air.  Emissions also depend on the conditions of combustion, which can vary considerably.  In 2003, the total emissions were 1569 Gg, with over 99% of these emissions arising from the energy sector.  Since 1990, emissions have decreased by 45%, mostly as a result of abatement measures on power stations, three-way catalytic converters fitted to cars and stricter emission regulations on trucks.

Carbon monoxide arises from incomplete fuel-combustion.  In 2003, the total emissions were 2757 Gg, of which 93% were from the energy sector.  Since 1990, emissions of CO have decreased by 67%.  This is mostly as a result of the increase in use of catalytic converters although a proportion is a consequence of fuel switching from petrol cars to diesel cars.  The other significant reduction arises from in the agricultural section due to the cessation of agricultural stubble burning in 1993.

In 2003, total emissions of NMVOCs were 1087 Gg, of which 51% were from the energy sector, with other significant contributions from solvent and other product use and industrial processes.  The development of an accurate emission inventory for NMVOCs is complex.  The diversity of processes emitting NMVOC is large.  Often emissions from sources are small individually, but important collectively.  A good example of this is leakage from valves, flanges and other connections in petrochemical plants.  Since 1990, overall emissions of NMVOCs have decreased by 55%.  This decrease in emissions can, in part be attributed to the increased use of catalytic converters on cars as well as the switching from petrol to diesel cars.

Total SO2 emissions in 2003 were 979 Gg.  Of this, 96% of emissions were from the energy sector, with the remaining emissions arising from the industrial processes sector and a small proportion from the waste sector.  Since 1990, emissions of SO2 from the energy sector have decreased by 74%.  The decrease has been as a result of the increase in the proportion of electricity generated in nuclear plant and the use of Combined Cycle Gas Turbine (CCGT) stations and other gas fired plant.

Categories ‘Solvent and Other Product Use’ and ‘Other’ are not shown in Figure 2.2 as both have zero emissions for all years.

Only gross emissions are presented

# Energy (CRF sector 1)

## Overview of sector

The energy sector is the largest emitter of greenhouse gases in the U.K.  As noted in Section 2.3, in 2003, 85% of direct greenhouse gas emissions came from this sector.  Major sources include power stations, road transport, combustion from industrial sources and provision of building services. Fugitive emissions are also accounted for in this sector.  These are emissions that arise from the production, extraction of coal, oil and natural gas, and their storage, processing and distribution.

Annex 3.3 contains more detailed descriptions of the methods used to estimate emissions in this sector.

### Carbon Factor Review

During 2004, an extensive review of carbon emission factors (CEFs) within the UK inventory was carried out.  This was a key recommendation of the Peer Review carried out by Simmons (2002).  A review of some of the CEFs of liquid fuels was carried out in the late 1990s, but many of the carbon emission factors within the inventory dated from the late 1980s or early 1990s.  The CEFs selected for review were based on the contribution of those fuels to the national total carbon emission.  The major fuels that were considered in the review were: Aviation Spirit, Aviation Turbine Fuel, Coal, Coke, Diesel, Petrol, Fuel Oil, Gas Oil, Naphtha, Natural Gas, Other Petroleum Gas (OPG), Petroleum Coke, Blast Furnace Gas (BFG) and Coke Oven Gas (COG).

As a result a number of revisions to emissions have been made, mostly in the Energy sector but also in the Industrial Processes sector.

To judge the effect of the new CEFs in isolation from changes in activity data, the new factors were applied to the 1990-2002 activity data (fuels consumed), as used in the 2004 NIR.  The overall effect of applying the new emission factors was to increase estimated carbon emissions in the UK by approximately 0.8% in 1990 and 1% in 2002.

- The CEFs used within this inventory are now based on current UK data and are appropriate for the UK.  Many CEFs will now be reviewed annually, and updated if necessary.  CEFs for solid and liquid fuels are in terms of mass.  CEFs for gases fuels are in terms of the energy content of the gas consumed (g carbon / GJ gross or g carbon / Mtherm).

## source category 1A1 – Energy Industries

### Source category description

This source category includes electricity generation, and the use of fossil fuels for petroleum refining, and production of coke and solid smokeless fuels.

The main fossil fuels used by the UK electricity supply industry are bituminous coal and natural gas.  Approximately 52 million tonnes of coal was burnt at 19 power stations during 2003, while 9650 Mtherms of natural gas was consumed at 35 large power stations and a few small regional stations.  Most gas-fired stations are combined-cycle gas turbines.  Heavy fuel oil was used as the main fuel by 6 facilities, and gas oil is used by 11 small power stations.

Bio-fuels are burnt at an increasing number of power generation sites as electricity generators strive to meet Government targets for renewable energy production. Three established sites use poultry litter as the main fuel, whilst several coal-fired power stations have increased the use of other biofuels such as short-rotation coppice to supplement the use of fossil fuels.

Electricity is also generated at municipal solid waste (MSW) incineration plant.  All the UK's 17 MSW incinerators produce electricity and heat and their emissions are therefore reported under CRF source category 1A1 for MSW burnt for electricity generation and 1A4 for MSW burnt for heat generation, rather than 6C (Waste Incineration).  This has been the case since 1997; prior to that year at least some MSW was burnt in older plant without recovery of useful energy.  Emissions from these incinerators were reported under 6C.

The UK has 12 oil refineries, 3 of these being small specialist refineries employing simple processes such as distillation to produce solvents or bitumens only.  The remaining 9 complex refineries are much larger and produce a far wider range of products including refinery gases, petrochemical feedstocks, transport fuels, gas oil, fuel oils, lubricants, and petroleum coke.  The crude oils processed, refining techniques, and product mix will differ from one refinery to another and this will influence the level of emissions from the refinery, for example by dictating how much energy is required to process the crude oil.

Most UK coke is produced at coke ovens associated with integrated steelworks, although independent coke manufacturers also exist.  At the end of 2003, there were four coke ovens at steelworks and one independent coke oven.  A further three coke ovens have closed in the last two years, due to closure of associated steelworks or closure of other coke consumers.  Solid smokeless fuels (SSF) can be manufactured in various ways but only those processes employing thermal techniques are included in the inventory since these give rise to significant emissions.  Currently, there are three sites manufacturing SSF using such processes.

### Methodological issues

- Most emissions are estimated from information such as fuel consumption data and estimates for a particular source sector are calculated by applying an emission factor to an appropriate statistic (see Annex 3, Section A3.3 for details).  This method is applied to estimating emissions from this sector for direct greenhouse gases.  General fuel consumption statistics taken from DUKES (DTI, 2004) are applied to emission factors to give an estimation of the emission.  Some emissions of indirect greenhouse gases are also estimated in this way (see Table 3.1 for details).

- For some sectors, emissions data are available for individual sites, either from the Environment Agency for England and Wales  (via the Pollution Inventory), directly from the Scottish Environment Protection Agency (SEPA) for Scotland and the Department of the Environment in Northern Ireland.  In such cases, the emission for a particular sector can be calculated as the sum of the emissions from these point sources.  However, in order to make an estimate of emissions from non-point sources in the sector, an independent estimate of fuel consumption associated with these point sources needs to be made, to ensure no double counting occurs (See Annex 3, Section A3.3).  This method is applied to emissions of indirect greenhouse gases for sectors as shown in Table 3.1.  Detailed tables of emission factors for both direct and indirect greenhouse gases can be found in Annex 3, Tables A3.3.1–A3.3.4 and A3.3.6.

Table 3.1	Methods used for deriving emission estimates for direct and indirect greenhouse gases for CRF Source Category 1A1

| Pollutant | CO2 | CH4 | N2O | CO | NOx | SO2 | NMVOC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Power Stations | F | F | F | R | R | R | R |
| MSW incineration | F | F | F | R | R | R | R |
| Refineries | F | F | F | F/R | F/R | F/R | F |
| Coke ovens | F | F | F | F | F/R | R | F |
| SSF Manufacture | F | F | F | F | F | F | F |

Key:

F	national emission estimates derived from emission factors and fuel consumption statistics (mostly DUKES)

R	national emission estimates derived from emission estimates reported by process operators to regulators

F/R	national emission estimates derived from either emission factors and fuel consumption statistics or emission estimates reported by process operators to regulators, depending upon fuel type.

#### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

Most of the core activity data for this source category is derived from the DTI publication the Digest of UK Energy Statistics.  Section 3.15 provides further general information about the time series consistency of activity data in this publication, and provides more general comments on the approaches used to ensure time series consistency in source category 1A.

Combustion emissions from the NAEI category ‘Gas separation plant’ are reported under category 1A1c (see Annex 3, Table A3.2).  Background energy data for the calculation of these emissions are taken from the most up to date version of the Digest of UK Energy Statistics.  In the DUKES published in 2002, the DTI stopped collecting the activity data about oil and gas extraction previously used to estimate these emissions.  Therefore, for data from 2001 onwards, the amount of propane and ethane has been extrapolated from historical data, as advised through discussions with DTI.

Table 3.2	Time series consistency of emission factors (EFs) of direct GHGs used in source category 1A1

| GHGs | Source category | Fuel types | Comments on time series consistency |
| --- | --- | --- | --- |
| Carbon | 1A1 | All fuels | - EFs vary somewhat across the time series based on comprehensive carbon factor review in 2004. - Key sources of carbon EF data include: UKPIA, Association of Electricity Producers, Powertech, Transco. |
| CH4, N2O | 1A1 | All fuels | - Nearly all EFs are constant over the entire time series, with limited use of time-varying EFs due to fuel variability or technological developments. - Increased availability of data from emissions of combustion of poultry litter has resulted in variable EFs across the time-series for both CH4 and N2O. |

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

The core publication for Activity Data is the annual DTI publication “The Digest of UK Energy Statistics” which is produced in accordance with QA/QC requirements stipulated within the UK Government’s “National Statistics Code of Practice” and as such is subject to regular QA audits and reviews.

Where emissions data are provided by plant operators to the Environment Agency’s Pollution Inventory and then used in the UK’s GHG emission inventory, the data is subject to audit and review within the Agency’s QA procedures. The operator emission estimates are initially checked & verified locally by their main regulatory contact (Site Inspector), and then passed to a central Pollution Inventory team where further checks are conducted prior to publication. Specific checking procedures include: benchmarking across sectors, time-series consistency checks, checks on estimation methodologies and the use and applicability of emission factors used within calculations.

### Source-specific recalculations

Some recalculations for indirect greenhouse gases have been made involving actual estimation of emissions from processes where emissions are recorded by regulators as below de minimis reporting limits.  However, the impact of this change on emission estimates is small.

#### Recalculation by gas

The following section describes the main changes that have occurred in sector 1A1 per pollutant since the publication of the 2002 inventory (2004 NIR).  Comparisons are made between the current inventory (1990-2003) and the previous inventory (1990-2002) for the year 2002.

##### Carbon Dioxide (CO2)

- There has been an increase in estimated emissions of CO2 (41 Gg) from MSW combustion for electricity generation due to a change in energy statistics.

- There has been an increase in estimated emissions of CO2 (78 Gg) from combustion of blast furnace gas in coke ovens and a decrease in estimated emissions of CO2 (209 Gg) for combustion of coke oven gas in coke ovens.  There has also been a decrease of 3 Gg CO2 from coke consumed during manufacture of solid smokeless fuels.  All of these changes are due to changes in emission factors arising from the review of carbon factors and the revision of the methodology for fuel transformation processes (see Annex 3, sections A.3.3.1 and A.3.3.3.3 for more details of these changes).

- There have been several significant changes to estimated emissions of CO2 from Public Electricity Production in 2002, following the 2004 carbon factor review. Overall, increased CEFs for coal, fuel oil and gas oil have out-weighed reductions in CEFs for natural gas, leading to an overall increase in the estimate of these emissions. Minor revisions to energy statistics are also evident.

- There has been an increase in estimated emissions of CO2 (3110 Gg) from category 1A1b Petroleum refining. This is mostly due to increases in CEFs for petroleum coke, refinery gas and fuel oil following the 2004 carbon factor review.

##### Nitrogen Oxides (NOX)

- There have been decreases in estimated emissions from use of landfill gas (32.1 Gg) and sewage gas (4.7 Gg) due to emission factors being updated to reflect the most recent version of the US EPA compilation of emission factors (US EPA, 2004).

- There has been a net increase of 0.4 Gg in estimated emissions from combustion of natural gas by the gas industry.  This change results from the adoption of a revised approach for calculation of NOx emissions from combustion processes, which is described in more detail in section A.3.3.1 of Annex 3.

- There have been minor increases in estimated emissions from combustion of MSW for electricity generation and burning of colliery methane at collieries (0.1 Gg in each case), due to use of updated energy data and revised emission factors respectively.

- There has been a decrease of 37.6 Gg in estimated emissions from 1A1a Public Electricity and Heat production due to a revision of the NOx emission factor from this source.

- Estimated emissions from 1A1b Petroleum Refining decreased by 0.72 Gg due to changes in emission factors from gas oil and natural gas.

##### Carbon Monoxide (CO)

- There has been a net decrease of 4.3 Gg in estimated emissions from combustion of landfill gas and sewage gas for electricity generation.  The change is due to emission factors used for these sectors being updated to reflect the most recent version of the US EPA compilation of emission factors (US EPA, 2004).

- There has been a net decrease of 6.8 Gg in estimated emissions from combustion of coke oven gas and blast furnace gas in coke ovens.  The change is due to emission factors used for these sectors being updated to reflect the most recent version of the EMEP/CORINAIR Emission Inventory Guidebook (EMEP/CORINAIR, 2003).

- There has been a net increase of 0.4 Gg in estimated emissions from combustion of natural gas by the gas industry.  This change results from the adoption of a revised approach for calculation of CO emissions from combustion processes, which is described in more detail in section A.3.3.1 of Annex 3.

##### Sulphur Dioxide (SO2)

- There has been an overall decrease of 3.4 Gg in estimated emissions from public electricity production due to changes in energy statistics for fuel oil.

##### Volatile Organic Compounds (VOC)

- There has been a net decrease of 1.9 Gg in estimated emissions from combustion of landfill gas and sewage gas for electricity generation.  The change is due to emission factors used for these sectors being updated to reflect the most recent version of the US EPA compilation of emission factors (US EPA, 2004).

- A decrease in estimated emissions of 0.2 Gg for coke oven gas combustion in coke ovens has occurred due to the emission factor being updated to reflect the most recent version of the EMEP/CORINAIR Emission Inventory Guidebook (EMEP/CORINAIR, 2003).

### Source-specific planned improvements

Emission factors and activity data are kept under review.

The data collection system used by the DTI to collect and calculate sector-specific estimates of the use of petroleum-based fuels has been changed, and since January 2005 a new electronic system of reporting has been introduced.  It is anticipated that this development will lead to more consistent returns from petroleum industries, reducing mis-allocations and transcription errors that may have been evident under the previous paper-based system.

Sector-specific guidance regarding estimation of annual emissions by plant operators are under development by the Environment Agency.  A rolling programme of guidance publication for different sectors until 2007 is planned, and it is anticipated that this will lead to a gradual improvement of the consistency and accuracy of operator returns to the Pollution Inventory.  When SEPA and NI DoE develop reporting systems, we anticipate they will adopt these QA/QC mechanisms also.

## Source Category 1A2 – Manufacturing Industries and Construction

### Source Category Description

This source category covers the use of fossil fuels by industrial processes, including the use of fuels to generate electricity in cases where the generation of electricity is not the principle activity of the process operator (such processes are termed autogenerators).  The GHGI separately reports emissions from autogenerators, cement clinker manufacture, lime manufacture, and iron & steel processes.  Only those iron & steel industry emissions from the use of fossil fuels in boilers and heat treatment furnaces, the use of coke in sinter plant and the use of coke oven gas, blast furnace gas and natural gas in the hot stoves used to heat air for blast furnaces are reported under 1A2.  Other sources such as emissions of carbon from basic oxygen furnaces are reported under 2C1.  Emissions from fuel used by other industrial sectors (e.g. chemicals, non-ferrous metals, food & drink) are reported as 'other industry'.

An estimate of CO emissions from manufacture of soda ash is also reported under 1A2.  This emission arises due to the burning of coke as part of the process but, due to the nature of that process, CO emissions are considerably higher than would be the case for burning of coke in conventional combustion plant.

### Methodological Issues

Emissions from direct greenhouse gases are estimated using the principles of the basic combustion model, as described in Annex 3, Section A3.3.1.  The DUKES publication is used to obtain relevant activity statistics, as well as data collected from industry.  There are a number of sources of emission factors and these can be found in Annex 3, Tables A3.3.1–A3.3.4.  Methods used to calculate emission estimates for both direct and indirect gases are summarised in Table 3.3.

Table 3.3	Methods for calculation of direct and indirect greenhouse gas emissions from 1A2

| Sector/pollutant | CO2 | CH4 | N2O | CO | 2NOx | SO2 | NMVOC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cement Fuel Combustion | Emission factors and fuel consumption data |  |  | No emissions reported |  |  |  |
| Cement Clinker production | No emissions reported |  |  | Emissions data reported by process operators to regulators |  |  |  |
| Lime Manufacture | Emission factors and fuel consumption data |  |  | Emissions data from regulators |  | Emission factors and fuel consumption data |  |
| Autogenerators1 | Emission factors and fuel consumption data |  |  |  |  |  |  |
| Other Industry | Emission factors and fuel consumption data |  |  |  |  |  |  |
| Sinter Plant | Emission factors and fuel consumption data |  |  | Emissions estimates for individual sites provided by process operators |  |  |  |

1   For the largest coal fired autogenerator, emissions data from the Pollution Inventory is used for CO, NOx, SO2

2   Emission estimated for NOx based on a combination of reported data for large combustion plant and literature based emissions factors and fuel consumption for small plant

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

Most of the core activity data for this source category is derived from the DTI publication the Digest of UK Energy Statistics.  Section 3.15 provides further general information about the time series consistency of activity data in this publication, and provides more general comments on the approaches used to ensure time series consistency in source category 1A.

Table 3.4 summarises the time series consistency of emission factors used in source category 1A2.

Table 3.4	Time series consistency of emission factors of direct GHGs used in source category 1A2

| GHGs | Source category | Fuel types | Comments on time series consistency |
| --- | --- | --- | --- |
| Carbon | 1A2 | All fuels | - EFs vary somewhat across time series based on comprehensive carbon factor review in 2004, with UKPIA providing new CEF data for many fuels used in this sector. |
| CH4, N2O | 1A2 | All fuels | - Nearly all EFs are constant over the entire time series, with limited use of time-varying EFs due to fuel variability or technological developments. |

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

Many of revisions listed below for the indirect GHGs reflect updating of emission factors in the EMEP/CORINAIR Emission Inventory Guidebook (EMEP/CORINAIR, 2003) and the most recent versions of the US EPA compilation of emission factors (US EPA, 2004).  Many of these changes are relatively small and have been grouped together in the following text to avoid unnecessary repetition.

#### Recalculation by gas

The following section describes the main changes that have occurred in sector 1A2 per pollutant since the publication of the 2002 inventory.  Comparisons are made between the current inventory (1990-2003) and the previous inventory (1990-2002) for the year 2002.

##### Carbon Dioxide (CO2)

- Emission estimates for carbon from fuel transformation processes and from related iron & steel industry processes have been extensively revised following a review of the methodology.  Details are given in Annex 3, Section A.3.3.3.3.  Estimated emissions from 1A2 have decreased by 825 Gg in 2002 as a result of these revisions.

- Estimated emissions for fuel combustion from the cement industry have decreased for waste oils (267 Gg CO2) but increased for waste solvents (269 Gg CO2) following the separation out of waste solvents from waste oils – previously both were included under waste oils.

- Estimated emissions from combustion of waste lubricants by the ‘other industry’ sector have decreased (329 Gg CO2).  The methodology for estimating usage of waste lubricants as fuels has been revised leading to a number of revisions to emission estimates for 1A2 and elsewhere.

- Estimated emissions from combustion of coal and natural gas by autogenerators have increased.  The CEF from coal combustion has been reduced following the 2004 carbon factor review, but significant increases in energy use data for both coal and natural gas in this sector have led to an overall increase in emission estimates.

##### Nitrogen Oxides (NOx)

- Estimated emissions from coal-fired autogenerators and cement kilns decreased by 1.5 Gg and 0.3 Gg respectively and estimated emissions from gas-fired lime kilns increased by 0.3 Gg due to minor data updates.

- A net increase in estimated emissions from industrial combustion sources of 18.1 Gg has resulted from the revised approach for estimating emissions of NOx from industrial combustion sources (see Annex 3, section A.3.3.1 for details).

- An estimated decrease in emissions of 1.3 Gg occurred for industrial combustion of waste lubricants, OPG and wood due to use of updated emission factors from US EPA (2004) and EMEP/CORINAIR (2003).

- Emission estimates from Iron & Steel combustion plant have increased by almost 9 Gg, mainly due to a significant increase in the NOX emission factor from natural gas combustion in this sector.

- Emission estimates from autogeneration plant have increased, mainly due to a significant increase in the NOX emission factor from natural gas combustion in this sector, partly offset by increases in both NOX emission factor and fuel use data for coal-firing in this sector.

##### Carbon Monoxide (CO)

- Estimated emissions from combustion of coal, coke, fuel oil, gas oil, and natural gas by autogenerators, iron & steel combustion plant, and other industry were revised up by 34.4 Gg due to changes in the methodology for estimating emissions of CO from industrial combustion sources (see Annex 3, section A.3.3.1 for details).

- Estimated emissions from coke and natural gas use in lime production were revised up by 0.1 and 1.4 Gg respectively, due to revisions to the estimates for consumption of these fuels as well as revisions to the emission factors.

- An estimated decrease of 1.8 Gg occurred from coke oven gas use in blast furnaces and iron & steel industry combustion due to a revision to the emission factor.

- Estimated emissions from cement works decreased by 0.5 Gg due to minor revisions to data.

##### Sulphur Dioxide (SO2)

- A net decrease in estimated emissions from industrial combustion sources of 2.8 Gg has resulted from revisions to energy statistics.

- Estimated emissions from secondary lead processes increased by 0.1 Gg while those from cement kilns decreased by 0.6 Gg due to minor data updates.

- Updated activity statistics and other data caused a 3.2 Gg increase in estimated emissions from Fletton brickworks.

##### Volatile Organic Compounds (VOC)

- An estimated decrease in emissions of 0.3 Gg occurred for combustion of natural gas and coke oven gas by the iron & steel industry due to use of updated emission factors from US EPA (2004) and EMEP/CORINAIR (2003).

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 1A3 – Transport

### Source Category Description

This source category reports the emissions of pollutants from transport.  Emissions from aviation, railways, road transport, and shipping are covered by this category.  Aircraft support vehicles are also covered under 1A3e.  Road transport is by far the largest contributor to transport emissions and estimations are made for a wide variety of vehicle types using both petrol and diesel fuel.

The UK GHGI reports emissions from both stationary and mobile sources for railways.  Stationary emissions are reported under category 1A4a.  Mobile emissions, which are reported under 1A3c cover estimates from diesel trains as freight, intercity and regional.

Emission estimates from the navigation section (1A3d) cover coastal shipping and international marine.

### Methodological Issues

The IPCC requires an estimate of emissions from 1A3ai International Aviation and 1A3Aii  Domestic to include emissions from the cruise phase of the flight as well as the LTO.  Emissions from aviation comprise emissions from the landing and take-off phases and the cruise phase of the flight.  An improved technique to estimate emissions and fuel use for civil aircraft in the UK has been developed and is now used. The approach follows the IPCC Tier 3 method.  The method estimates emissions from both domestic and international aviation. Details can be found in Annex 3, Section A3.3.5.1.

Emissions from road transport are calculated either from a combination of total fuel consumption data and fuel properties or from a combination of drive related emission factors and road traffic data.  Details are discussed in Annex 3, Section 3.3.5.3.

Details on emission estimates from railways can be found in Annex 3, Section 3.3.5.2.

Emission estimates for coastal shipping are estimated according to the base combustion module (Annex 3, Section A3.3.1) using emission factors given in Table A3.9.  For International marine, fuel consumption data are assumed to be the marine bunkers total minus the naval consumption.  Emission factors are used from Table A3.9.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

Some of the core activity data for this source category are derived from the DTI publication the Digest of UK Energy Statistics.  Section 3.15 provides further general information about the time series consistency of activity data in this publication, and provides more general comments on the approaches used to ensure time series consistency in source category 1A.  Other important sources of activity data are UK Department for Transport publication Transport Statistics Great Britain and fuel consumption data supplied by the Ministry of Defence (Defence Fuels Group).  Transport Statistics Great Britain is an established publication and the compilers of the activity data strive to use consistent methods to produce the activity data.

Table 3.5 summarises the time series consistency of emission factors used in source category 1A3.

Table 3.5	Time series consistency of emission factors of direct GHGs used in source category 1A3

| GHGs | Source category | Fuel types | Time series consistency |
| --- | --- | --- | --- |
| Carbon | 1A3 | Liquid fuels and gaseous fuels | - Time-series of EFs used based on carbon content of UK fuels available for each year from 1990 from UK sources and so appropriate for the UK |
| CH4, N2O | 1A3 | Fuel types used in the UK | - For road transport, time varying EFs used appropriate to emission standards in force and age profile of vehicle fleet |

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

#### Recalculation by gas

The following section describes the main changes that have occurred in sector 1A3 per pollutant since the publication of the 2002 inventory.  Comparisons are made between the current inventory (1990-2003) and the previous inventory (1990-2002) for the year 2002.

##### Carbon Dioxide (CO2)

- Overall, estimated emissions from category 1A3b Road transport increased by 2711 Gg CO2.  This is due to a) a revision in the fuel consumption data; b) a change in the carbon factor for road diesel fuel and c) including emissions due to combustion of lubricating oil.  The allocation of emissions due to combustion of lubricating oil between categories 1A3b and 1A2f (Manufacturing Industry & Construction: Other) is uncertain and further research is planned on this.  The sum of emissions between the categories should not be affected.

- Estimated emissions from International Marine (1A3di) showed an increase of 28.8 Gg CO2 due to a revision in carbon emission factors for gas oil and fuel oil following the 2004 UK carbon factor review.

- Emissions from category domestic aviation 1A3a have declined by 847 Gg.  This is due to a revision in the method used to estimate emissions from aviation (now improved to Tier 3) and a revision in the fuel consumption data.  There has been an increase in emissions allocated to aviation bunkers as a result of the Tier 3 method implemented.

##### Methane (CH4)

- Estimated emissions for road transport have been revised down by 0.2 Gg due to revisions in methodology, vehicle kilometre data and fleet composition data.

##### Nitrogen Oxides (NOx)

- Estimated emissions from road transport (1A3b) have decreased by 20.6 Gg.  This change is due revisions in methodology, vehicle kilometre data and fleet composition data.

##### Carbon Monoxide (CO)

- Estimated emissions for road transport have been revised down by 312 Gg due to revisions in methodology, vehicle kilometre data and fleet composition data.

##### Non-Methane Volatile Organic Compounds (NMVOC)

- Estimated emissions from road transport have decreased by 15 Gg due to revisions in methodology, vehicle kilometre data and fleet composition data.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 1A4 – Other Sources

### Source Category Description

The emissions that are included in this source category arise from the following sectors:

- Commercial/Institutional – emissions from fuel combustion in commercial and institutional buildings

- Residential – emissions from fuel combustion in households

- Agriculture/Forestry/Fishing – emissions from fuel combustion in these sectors

Emissions from the burning of MSW to generate heat are reported under CRF source category 1A4. Emissions from stationary railway sources are reported under 1A4a Commercial/Institutional.  Stationary railway sources include emissions from the combustion of burning oil, fuel oil and natural gas used by the railway sector.

### Methodological Issues

The methodology used for emissions from the burning of MSW to generate heat is identical to that used for burning of MSW to generate electricity (see Section 3.2.2) and the emission factors are therefore the same.

A re-allocation of emissions from the 1B1b sector (Solid fuel transformation) to sector 1A4b (Residential) has been introduced in this inventory.  Previously, it had been assumed that emissions from the combustion of petroleum coke occurred in the fuel transformation sector.  However, a review of this fuel use indicates that the emissions from the combustion of this fuel occur when small amounts of petroleum coke are incorporated into some grades of domestic smokeless fuel produced within the fuel transformation process.  This fuel is then burnt by householders and hence is now included within sector 1A4b and removed from 1B1b.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

Most of the core activity data for this source category is derived from the DTI publication the Digest of UK Energy Statistics.  Section 3.15 provides further general information about the time series consistency of activity data in this publication, and provides more general comments on the approaches used to ensure time series consistency in source category 1A.

Table 3.6 summarises the time series consistency of emission factors used in source category 1A4.

Table 3.6	Time series consistency of emission factors of direct GHGs used in source category 1A4

| GHGs | Source category | Fuel types | Comments on time series consistency |
| --- | --- | --- | --- |
| Carbon | 1A2 | All fuels | - EFs vary somewhat across time series based on the UK carbon factor review in 2004. |
| CH4, N2O | 1A2 | All fuels | - Nearly all EFs are constant over the entire time series, with limited use of time-varying EFs due to fuel variability or technological developments. |

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

Some recalculations for indirect greenhouse gases have been made involving actual estimation of emissions from processes where emissions are recorded by regulators as below de minimis reporting limits.  The impact of this change on emission estimates is small.

#### Recalculation by gas

The following section describes the main changes that have occurred in sector 1A4 per pollutant since the publication of the 2002 inventory.  Comparisons are made between the current inventory (1990-2003) and the previous inventory (1990-2002) for the year 2002. Across the sector, the revisions of carbon emission factors following the 2004 carbon factor review programme have had a significant impact on emission estimates. In addition, small revisions to fuel use data are evident within the 1A4a “Commercial / Institutional” and 1A4ci “Agriculture / Forestry / Fishing: Stationary” sectors.

##### Carbon Dioxide (CO2)

- Revision of the natural gas carbon EF following the 2004 carbon factor review have reduced the estimated emissions from 1A4 sectors as follows: 1A4b domestic (by 1040 Gg CO2), 1A4a industrial-commercial (by 369 Gg CO2), 1A4a public sector (by 183 Gg CO2).

- Estimated emissions from 1A4b domestic coal use have increased (by 77 Gg CO2) due to a revision in carbon EF following the 2004 carbon factor review.

- Estimated emissions from 1A4b burning oil use have decreased by 362 Gg CO2 due to a revision of fuel use statistics.

- Increases in carbon EFs for both gas oil and fuel oil use have lead to a total increase of 104 Gg CO2 emissions from across the 1A4 sector.

- A re-allocation of emissions from the 1B1b sector (Solid fuel transformation) to sector 1A4b (Residential) has increased the estimated CO2 emissions from this sector by 546 Gg.  Previously, it had been assumed that emissions from the combustion of petroleum coke occurred in the fuel transformation sector. However, a review of this fuel use indicates that the emissions from the combustion of this fuel occur when small amounts of petroleum coke are incorporated into some grades of domestic smokeless fuel produced within the fuel transformation process. This fuel is then burnt by householders. Emissions from the consumption of this petroleum coke are now assigned to the IPCC fuel  "Other Bituminous Coal".

##### Methane (CH4)

- Minor revisions of fuel use data have led to an estimated decrease in emissions from 1A4b Residential by 0.02 Gg across all fuels, and a similar reduction across all fuels in the 1A4a Commercial / Institutional sector.

##### Nitrogen Oxides (NOX)

- Estimated emissions from combustion of coal, coke, burning oil, gas oil, and natural gas by residential and commercial/institutional combustion plant were revised up by 35.5 Gg due to changes in the methodology for estimating emissions of NOx from combustion sources (see Annex 3, section A.3.3.1 for details), together with changes in energy statistics.

- Estimated emissions from combustion of other fuels by residential and commercial/institutional combustion plant were revised up by 0.4 Gg due to the use of updated emission factors from US EPA (2004) and EMEP/CORINAIR (2003), together with changes in energy statistics.

- There has been a minor decrease in estimated emissions from combustion of MSW for heat generation (0.1 Gg) due to use of updated energy.

##### Carbon Monoxide (CO)

- Estimated emissions from combustion of coal, coke, burning oil, gas oil, and natural gas by residential and commercial/institutional combustion plant were revised up by 186.1 Gg due to changes in the methodology for estimating emissions of CO from combustion sources (see Annex 3, section A.3.3.1 for details), together with changes in energy statistics.

- Estimated emissions from combustion of other fuels by residential and commercial/institutional combustion plant were revised up by 139.0 Gg due to the use of updated emission factors from US EPA (2004) and EMEP/CORINAIR (2003), together with changes in energy statistics.

##### Non-Methane Volatile Organic Compounds (NMVOC)

- Estimated emissions from combustion of natural gas, sewage gas, wood, straw & burning oil by residential and commercial/institutional combustion plant were revised up by 4.1 Gg due to the use of updated emission factors from US EPA (2004) and EMEP/CORINAIR (2003), together with changes in energy statistics.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 1A5 – Other

### Source Category Description

This category includes emissions from military aircraft and naval vessels.  Both are reported under category 1A5b: mobile emissions.

### Methodological Issues

Methods of estimation for both military aircraft and naval vessel emissions are discussed in the transport section of Annex 3 (Section A3.3.5).

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

Military fuel consumption data are supplied by the Ministry of Defence Fuels Group.  The MOD have supplied a time series of fuel consumption data since 1990 and we believe the time series consistency of the fuel use data is good.

Table 3.7	Time series consistency of emission factors of direct GHGs used in source category 1A5

| GHGs | Source category | Fuel types | Comments on time series consistency |
| --- | --- | --- | --- |
| Carbon | 1A2 | All fuels | - EFs vary somewhat across time series based on the UK carbon factor review in 2004. |
| CH4, N2O | 1A2 | All fuels | - Nearly all EFs are constant over the entire time series, with limited use of time-varying EFs due to fuel variability or technological developments. |

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

#### Recalculation by gas

The following section describes the main changes that have occurred in sector 1A5 per pollutant since the publication of the 2002 inventory.  Comparisons are made between the current inventory (1990-2003) and the previous inventory (1990-2002) for the year 2002.

##### Carbon Dioxide (CO2)

- Estimated emissions from category 1A5b have increased by 12 Gg CO2 due to a revised carbon emission factor for gas oil usage figure for naval shipping, following the 2004 UK carbon factor review

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## International Bunker Fuels (memo item)

International bunker emissions (international aviation and shipping) are not included in the national total but are reported separately.  For the U.K. they are approximately 6% of the national total of CO2.  In 2003, the shipping emission contributed 15% to total bunker emissions, with aviation contributing the remaining 85%.  Since 1990, estimated emissions from international aviation have risen by 90%.  These estimates are consistent with the revised Tier 3 method now adopted for aviation and described in Annex 3 section under Appendix 3, Section 3.3.5.1.

## Feedstocks and non-energy use of Fuels

Natural gas is used as a feedstock for the manufacture of ammonia (for fertiliser), methanol and acetic acid.  This process is described in Section 4.9.1.

## Capture and storage of CO2 from Flue gases

Currently in the UK, CO2 emitted from flue gases is not captured and stored.

## Comparison of Sectoral and Reference approaches

This comparison is documented and described in Annex 4.

Summary Table 7B includes the IPCC Reference Inventory total for carbon dioxide.  This is a ‘top-down’ inventory calculated from national statistics on production, imports, exports and stock changes of fossil fuels.  All other Sectoral Tables report emissions of pollutants estimated using a ‘bottom-up’ approach with emissions estimated from activity statistics (mostly fuel consumption) in the various economic sectors and processes.

- In principle the IPCC Reference Total can be compared with the IPCC Table 1A Total plus the emissions arising from fuel consumption in 1B1 Solid Fuel Transformation and Table 2 Industrial Processes (Iron and Steel and Ammonia Production).  The IPCC Reference totals range between 1% lower to 3 % higher than the comparable bottom up totals, largely because they are based on a different set of statistics.  Reasons for the discrepancies between the two estimates are discussed in Annex 4.

- Over the period (1990 to 2003), emissions estimated by the Reference Approach have fallen by 3.5 % compared with 4.5 % for the comparable ‘bottom-up’ totals.

- A detailed comparison between the IPCC Reference Inventory, the UK Greenhouse Gas Inventory and the UK Inventory based on the IPCC Default Methodology is given by Salway (1998a).

## Country specific issues

Country specific issues have been identified under other headings or as they occur.

## Source category 1B1 – Solid Fuels

### Source Category Description

This source category covers emissions which occur during the production, transportation or use of solid fuels but which are not due to the combustion of those fuels to support of a productive activity.  These emissions will include the release of methane contained within coal and emissions of carbon and organic compounds during the transformation of coal into coke and solid smokeless fuels.  Emissions will also occur from the flaring of any waste gases from coke or SSF manufacture.

### Methodological Issues

Carbon emissions from coke ovens are based on a carbon balance approach (discussed in Annex 3, Section A3.3.8.1.2.1) with calculations arranged so that the total carbon emission corresponds to the carbon content of the input fuels.  For process emissions from coke ovens for other pollutants, emissions are estimated either on the basis of total production of coke or the coal consumed.  Emission factors are provided in Annex 3, Table A3.3.25.

Emissions of carbon from Solid Smokeless Fuel (SSF) production are also based on a carbon balance approach, as discussed in Annex 3, Section A3.3.8.1.2.2.  For other pollutants, estimates are made based on production data and emission factors as provided in Table A3.3.25.

A re-allocation of emissions from the 1B1b sector (Solid fuel transformation) to sector 1A4b (Residential) has been introduced in this inventory.  Previously, it had been assumed that emissions from the combustion of petroleum coke occurred in the fuel transformation sector. However, a review of this fuel use indicates that the emissions from the combustion of this fuel occur when small amounts of petroleum coke are incorporated into some grades of domestic smokeless fuel produced within the fuel transformation process.  This fuel is then burnt by householders and hence is now included within sector 1A4b and removed from 1B1b.

A new source, methane from closed coal mines, is now included in sector 1B1a of the UK inventory following publication of the findings of a recent study funded by Defra (Kershaw, 2005). The study has derived emission estimates for the years 1990 to 2004 using a relationship between emission and the quantity of the underlying methane gas within the abandoned mine workings.  More details of the estimation methodology are provided in Annex 3, Section A3.3.8.1.1.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

Most of the core activity data for this source category is derived from the DTI publication the Digest of UK Energy Statistics.  Section 3.15 provides further general information about the time series consistency of activity data in this publication, and provides more general comments on the approaches used to ensure time series consistency in source category 1B.

The time series consistency of emission factors used in this source category is discussed in Annex 3, Section A3.3.8.1.1.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

#### Recalculation by gas

The following section describes the main changes that have occurred in sector 1B1 per pollutant since the publication of the 2002 inventory.  Comparisons are made between the current inventory (1990-2003) and the previous inventory (1990-2002) for the year 2002.

##### Carbon Dioxide (CO2)

- Estimated emissions from category 1B1 have decreased by 19 Gg CO2 due to a revised carbon emission factor for coke oven gas usage figure for iron & steel flaring, due to a change in iron & steel carbon balance methodology.

- A very significant reduction in emissions from category 1B1 of 1890 Gg CO2 due to a combination of improvements to the method used to estimate emissions from iron & steel and associated fuel transformation, and a re-allocation of emissions from 1B1 to 6A1. This change is a consequence of the revision to the method used to estimate emissions from the manufacture of iron & steel.  In previous inventories emissions from the combustion of these compounds was included as part of the emission from coal use in coke ovens - however these compounds are not burnt in this sector.  Emissions from the decomposition of these fossil fuel derived preservatives have now been allocated to sector 6A1.

##### Methane (CH4)

- Estimated emissions from category 1B1 have increased by 57.4 Gg CH4 due to the introduction of the new source of methane emissions from abandoned mines.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source category 1B2 – Oil And Natural Gas

### Source Category Description

This source category covers emissions which occur during the production, transportation, or use of liquid and gaseous fuels but which are not due to the combustion of those fuels to support a productive activity.  Emissions occur from oil and gas production facilities, gas and oil terminals, gas processing facilities, oil refineries, gas transmission networks, and storage and distribution of petrol.

Oil & gas production facilities are sources of CO2, CH4, CO, N2O, NOx, SO2, and VOC.  Organic pollutants are emitted as a result of venting from processes for reasons of safety and from leakages from process plant.  Flaring of waste streams gives rise to emissions of all seven pollutants.  Most of the UK's oil and gas production occurs offshore but there are a number of mostly small onshore production sites as well.

Offshore oil and gas has to be transported to processing plant and pipelines are used for gas and a proportion of the oil produced.  The remaining oil is transferred to shore using marine tankers and emissions of CH4 and VOC occur during loading of oil into the ship's tanks.  Some oil transported to shore by pipeline is subsequently reloaded into marine tankers for distribution to refineries and emissions of CH4 and VOC will occur during this loading stage as well.  Emissions of VOC occur from storage tanks located at oil terminals.

Emissions of carbon, CH4, CO, N2O, NOx, SO2, and VOC occur at refineries due to venting of process plant for reasons of safety, from flaring of waste products, leakages from process plant, evaporation of organic contaminants in refinery wastewater, regeneration of catalysts by burning off carbon fouling, and storage of crude oil, intermediates, and products at refineries.

Petrol distribution begins at refineries where petrol may be loaded into rail or road vehicles.  Petrol is distributed to approximately 60 petrol terminals where it is stored prior to loading into road tankers for distribution to petrol stations.  At petrol stations it is stored and then dispensed into the fuel tanks of road vehicles.  Emissions of VOC occur from each storage stage and from each transfer stage.

### Methodological Issues

Emissions estimates for the offshore industry are based on data provided by the trade organisation, UKOOA, through their annual emissions reporting mechanism to UK regulators, the Environmental Emissions Monitoring System (EEMS).  This system provides a detailed inventory of point source emissions estimates, based on operator returns for the years 1995-2003.  Additional, more detailed data on CO2 emissions from some offshore combustion processes has become available as a result of the development of the industry’s National Allocation Plan pertaining to the EU Emission Trading Scheme.  Therefore, for the main combustion sources in the offshore oil & gas sector, the UKOOA data from 1998 onwards is sourced via NAP estimates, superseding any historic estimates previously reported via EEMS.

For years prior to 1995 (i.e. pre-EEMS), emission totals are based on an internal UKOOA summary report produced in 1998.  These data were revised and reported with the 1995-2003 datasets in the UKOOA 2005 submission to the inventory.

The 1998 UKOOA report presented data from detailed industry studies in 1991 and 1995 to derive emission estimates for 1990 from available operator estimates.  Emission estimates for 1991-1994 were then calculated using production-weighted interpolations.  Only limited data was available from operators in 1990-1994, and emission totals could only be estimated in broadly aggregated sectors of: drilling (offshore), production (offshore), loading (offshore) and total emissions onshore.  Estimates of the more detailed oil & gas processing source sectors for 1990-1994 are therefore based on applying the fraction of total emissions derived from the 1995 data from EEMS.

Emission estimates for onshore oil and gas facilities are based on emissions data reported by process operators for the Pollution Inventory.  Historically, data have not been obtained for facilities in Scotland although this situation is likely to change soon.

Emission estimates for all pollutants from the nine complex UK refineries (see Section 3.2.1) are provided annually by the UK Petroleum Industry Association (UKPIA) and are incorporated directly into the GHGI.  The UKPIA estimates are compiled by the refinery operators using agreed industry standard methods.

Petrol distribution emissions are calculated using petrol sales data taken from the Digest of UK Energy Statistics and emission factors calculated using the UK Institute of Petroleum's protocol on estimation of emissions from petrol distribution.  This protocol requires certain other data such as average temperatures, Reid Vapour Pressure (RVP) of petrol and details of the level of abatement in place.  Central England Temperature (CET) data, obtained from the Met Office, is used for the temperature data, while UKPIA supply RVP estimates for summer and winter blend petrols and estimates of the level of control are based on statistics given in the Institute of Petroleum's annual petrol retail survey.

For further details on all processes covered under 1B2 including emission factors and detailed methodological descriptions, see Annex 3, Section 3.3.8.2.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

The emission estimates for the offshore industry are based on the UKOOA EEMS dataset for 1995-2003. Emission estimates from 1990-1994 (i.e. pre-EEMS) are estimated from specific UKOOA studies of 1991 and 1998, using production data as a basis for interpolation of data between 1990 and 1995. The new dataset provided in 2005 by UKOOA now provides a more consistent time-series of data for the range of activities within this sector. Full details are given in Annex 3 A3.3.8.2.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6. UKOOA provides emission estimation guidance for all operators to assist in the completion of EEMS returns to the UK environmental regulators, including the provision of appropriate default emission factors for specific activities, where installation-specific factors are not available.

### Source-specific recalculations

The 2005 UKOOA dataset incorporates minor recalculations of CO2 emissions data between 1998-2002, where carbon emission factors have been revised in light of the EU-ETS National Allocation Plans.

### Source-specific planned improvements

The extensive revision of UKOOA data in this inventory cycle has highlighted a number of areas where further clarifications are required. In particular, significant step-changes in emission factors are apparent for some pollutants (notably methane) across the time series, suggesting that updated guidance and emission factors have been provided to plant operators, but historic recalculations may not have been thoroughly conducted. In addition, there are a number of gaps in plant-specific data within the EEMS dataset. In some cases, interpolation estimates have been used, but further consultation with UKOOA is required.

## general comments on QA/QC

### DTI Energy Balance Data

The DTI provides the majority of the energy statistics required for compilation of the NAEI and the GHGI.  These statistics are obtained from the DTI publication “The Digest of UK Energy Statistics” which is produced in accordance with QA/QC requirements stipulated within the UK Government’s “National Statistics Code of Practice” and as such is subject to regular QA audits and reviews.

The DTI includes a number of steps to ensure the energy statistics are reliable.  At an aggregate level, the energy balances are the key quality check with large statistical differences used to highlight areas for further investigation.  Prior to this, the DTI tries to ensure that individual returns are as accurate as possible.  A two-stage process is used to achieve this.  Initially the latest data returns are compared with those from previous months or quarters to highlight any anomalies.  Where data are of a seasonal nature comparison is also made with corresponding data for the same month or quarter in the previous year.  The DTI also uses an energy balance approach to verify that individual returns are sensible.  Any queries are followed up with the reporting companies.  The DTI depends on data from a range of companies, and work closely with these reporting companies to ensure returns are completed as accurately as possible and in good time for the annual publications of statistics.

The data collection system used by the DTI to collect and calculate sector-specific estimates of the use of petroleum-based fuels has been changed, and since January 2005 a new electronic system of reporting has been introduced.  This development should lead to more consistent returns from petroleum industries, reducing mis-allocations and transcription errors that may have occurred under the previous paper-based system.

### Industrial Point-Source Emissions Data

Where emissions data are provided by plant operators to the Environment Agency’s Pollution Inventory and then used in the UK’s GHG emission inventory, the data is subject to audit and review within the Agency’s QA procedures. The operator emission estimates are initially checked & verified locally by their main regulatory contact (Site Inspector), and then passed to a central Pollution Inventory team where further checks are conducted prior to publication. Specific checking procedures include: benchmarking across sectors, time-series consistency checks, checks on estimation methodologies and the use and applicability of emission factors used within calculations.

Sector-specific guidance regarding estimation of annual emissions by plant operators are under development by the Environment Agency.  A rolling programme of guidance publication for different sectors until 2007 is planned, and it is anticipated that this will lead to a gradual improvement of the consistency and accuracy of operator returns to the Pollution Inventory.  The development of the SEPA and NI DoE reporting systems is anticipated to adopt these QA/QC mechanisms also.

## General comments on time series consistency

The UK GHG inventory seeks to ensure time series consistency of its emission estimates. In general, the time series consistency of emissions will depend on:

- consistency in the techniques used to compile activity data

- correct choice of source and fuel specific emission factors for each year of the inventory

- consistency in the techniques used to estimate emissions from the activity data and emission factors.

Much of the core activity data for the sources reported in CRF sector 1 (Energy) is derived from the DTI publication the Digest of UK Energy Statistics.  This is a long running publication and the compilers of the activity data for DUKES strive to use consistent methods to produce the activity data.  This helps to ensure good time-series consistency.  Revisions of activity data may be made up to two years behind the latest reported year, but such revisions are clearly noted in DUKES and are incorporated into the GHG inventory when the inventory is updated each year.  Where activity data other than that presented in DUKES are required for a source category, we have made quantitative and qualitative comments about the quality of the time series if possible.

The emission factors used are typically fuel - and source-specific, and any comments on the time series consistency of the emission factors are made in the sections on “uncertainties and time-series consistency” in this chapter. Comments are restricted to the emission factors of the direct greenhouse gases.

In nearly all cases in the UK GHGI, a single method is used to estimate a time series of emissions from a specific source category.  The technique of splicing two or more methods is rarely used.  If a more sophisticated method is used to replace a simpler one, the entire time series of emissions is updated using the new method.  Occasionally, there are insufficient data to produce a complete time series of emissions from the chosen method.  Here, extrapolations and interpolations, use of surrogate data, and use of constant estimates of emission factors or activity data may be used to provide a complete time series.  The same options can be used when splicing methodologies, and in addition, it may also be necessary to overlap of methodologies (Rypdal et al., 2000).

# Industrial Processes (CRF sector 2)

## Overview of Sector

- UK industry includes many processes that give rise to direct or indirect greenhouse gases.  Important sectors include cement and lime production, glass manufacture, steel production, secondary non-ferrous metal production, chemicals manufacture and food and drink manufacture.  Primary non-ferrous metal production is now limited to the production of primary aluminium at three sites and the UK paper and pulp industry is relatively small compared with many other Northern European countries.

Annex 3.4 contains more detailed descriptions of the methods used to estimate emissions in this sector.

## Source Category 2A1 – Cement Production

### Source category description

Cement is produced by grinding a mixture of calcium carbonate (CaCO3), silica, alumina and iron oxides, either in a wet or dry process, and then heating the ground material in a kiln.  In the kiln, the calcium carbonate breaks down into calcium oxide (CaO) and carbon dioxide (a process known as calcination).  The calcium oxide subsequently reacts with the other raw materials to form clinker.  The clinker is cooled and, after addition of other raw materials, ground to make cement.

Emissions of carbon dioxide result both from calcination of the calcium carbonate, but also from fuels burnt to provide the heat for calcination and clinkering.  Fuels used include coal and petroleum coke plus waste materials and some natural gas.  Emissions of CO2 from fuel combustion are reported under CRF source category 1A2f while emissions from calcination are reported under category 2A1.

Fuel combustion also gives rise to emissions of NOX and N2O while emissions of methane, NMVOC, SO2 and CO also occur, both due to fuel combustion but also due to the evaporation of organic or sulphurous components present in the raw materials.  The UK GHGI currently reports emissions both from fuels and from raw materials under 1A2f.

The UK had 18 sites producing cement clinker during 2003.

### Methodological issues

The methodology used for estimating CO2 emissions from calcination is the IPCC Tier 2 approach (IPCC, 2000).  The emission was estimated from the annual UK production of clinker (British Cement Association, 2004).  The British Cement Association also provided an estimate of the average CaO content of cement clinker (63%) and that the use of non-carbonate CaO can be assumed to be zero.  The clinker production data are revised up to take account of losses in the form of kiln dust, by assuming that these losses are 2% of clinker production.  This is also based on an estimate provided by the British Cement Association.  Based on these data, an emission factor of 137.6 t C/kt clinker was calculated according to the IPCC Tier 2 method.  This is around 2.5% lower than the emission factor used in the 1999 Inventory, which was based on IPCC default values.

### Uncertainties and time-series consistency

The emission was estimated from the annual UK production of clinker, with data provided by the British Cement Association.  The time-series consistency of these activity data is very good due to the continuity in data provision by the British Cement Association.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

No recalculations have been required for this version of the inventory.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 2A2 – Lime Production

### Source category description

Lime (CaO) is manufactured by the calcination of limestone (CaCO3) and dolomite (CaCO3MgCO3) in kilns fired by coal, coke or gas.  The calcination results in the evolution of carbon dioxide.  It is necessary to distinguish between processes where lime is produced for use off-site and where carbon dioxide is emitted to atmosphere, and those processes where lime is produced so that the carbon dioxide and lime can be used on-site in the process.  In these processes, which include sugar refining and the production of sodium carbonate using the Solvay process, most of the carbon dioxide is not emitted to atmosphere.

Lime was produced at 15 UK sites during 2003.  Two of these produce lime for use on-site in the Solvay process and five produce lime for use on-site in sugar manufacturing.

### Methodological issues

The UK bases estimation of lime production on limestone and dolomite consumption data, which are readily available (British Geological Survey, 2004).  The use of consumption data rather than production data is simpler and probably more reliable since it is not necessary to consider the different types of lime produced.  An emission factor of 120 t carbon/kt limestone was used based on the stoichiometry of the chemical reaction assuming pure limestone.  For dolomite an emission factor of 130 t carbon/kt dolomite would have been appropriate; however dolomite calcination data are not given explicitly but included in the limestone data, which will have caused a small under-estimate of emissions.  Dolomite calcination is believed to be a small proportion of the total hence the underestimate is unlikely to be significant.  The limestone calcination data used exclude limestone calcined in the chemical industry since a large proportion of this is use in the Solvay process, which does not release CO2.  The calcination of limestone in the sugar industry is also excluded.

### Uncertainties and time-series consistency

Uncertainty in both the activity data and emission factor used for this source are judged to be low.  The use of an emission factor applicable to limestone calcination for estimating emissions of both limestone and dolomite will lead to a slight underestimate in emissions.  The exclusion of limestone used by the chemicals industry and sugar production will also lead to a small underestimate since not all CO2 is consumed by the processes and, in the case of chemicals, some lime may be used in processes other than the Solvay process.  Time-series consistency of activity data is very good due to the continuity in data provided by the British Geological Survey.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

No recalculations have been required for this version of the inventory

### Source-specific planned improvements

Section 4.3.3 describes possible areas for improvement, though the impact on the trend is likely to be relatively small.

## Source Category 2A3 – Limestone & Dolomite use

### Source category description

Limestone and dolomite are added to sinter where they are calcined, the products subsequently acting as slag formers in blast furnaces.  Some limestone or dolomite may be added directly to blast furnaces instead of sintered first, but this is ignored for the GHGI with all emissions being assumed to occur from the sinter strand instead.  Limestone and dolomite are also used as sources of CaO and MgO in the manufacture of soda-lime glasses, as fluxing agents for basic oxygen furnaces, and for the liming of soils by the agricultural sector.  Agricultural use is covered in Chapter 5 of this report.  Use of limestone and dolomite in both sinter production, basic oxygen furnaces and glass manufacture results in the evolution of carbon dioxide, which is emitted to atmosphere.  Limestone is also used in flue-gas desulphurisation (FGD) plant, used to abate SO2 emissions from combustion processes.  The limestone reacts with the SO2 and is converted to gypsum, with CO2 being evolved.

The UK had three operational steel-making sites during 2002 and approximately 18 large glassworks manufacturing soda-lime type glasses.  FGD was operational on two UK power stations.

### Methodological issues

Emissions are calculated using emission factors of 120 t carbon/kt limestone and 130 t carbon/kt dolomite, in the case of processes involving calcinations, and 69 t carbon/kt gypsum produced in the case of FGD processes.  These factors are based on the assumption that all of the carbon dioxide is released to atmosphere.  Data on the usage of limestone and dolomite for glass and steel production are available from the British Geological Survey (2004) and the Iron & Steel Statistics Bureau (2004), respectively and gypsum produced in FGD plant is available from the British Geological Survey (2004).

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

Uncertainty in both the activity data and emission factor used for this source are judged to be low.  Time-series consistency is also very good due to the continuity in data provision by the British Geological Survey and the Iron & Steel Statistics Bureau.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

Some revised data were available for use of limestone by the glass industry in 2002, leading to an estimated decrease in emissions of 5.4 Gg carbon in 2002.  A revision was made to the reporting of emissions from limestone and dolomite by integrated steelworks.  Previously, emissions were reported as occurring from the blast furnaces, whereas emissions are now reported more correctly as occurring from sinter production and from basic oxygen furnaces.  The overall quantity emitted has, however, not changed.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 2A4 – Soda Ash Use

### Source category description

Soda ash (sodium carbonate, Na2CO3) is used in the manufacture of soda-lime glasses.  The soda ash decomposes in the melt to Na2O, which is incorporated into the glass, and CO2, which is released to atmosphere.  Other uses of soda ash can also result in the emission of CO2, including use in food and drink manufacture and pharmaceuticals, however the consumption of soda ash for these applications is small.  Only the emissions from soda-lime glasses are reported in 2A4.

The UK has approximately 18 large glassworks manufacturing soda-lime type glasses.

### Methodological issues

Emission estimates are based on estimates of the consumption of soda ash in the production of soda-lime glass (British Glass, 2004).  This is assumed to be 20% of the mass of glass produced.  An emission factor of 113 kt carbon/Mt soda ash is used. The estimate of soda ash consumption is based on the production of container glass, flat glass and domestic glass.  Other types of glass, such as glass fibres, glass wool and special glasses are not soda-lime glasses and do not involve the use of large quantities of soda ash.  The glass production data are corrected for the amount of recycled glass (cullet) and the soda ash consumption is therefore estimated as 20% of the new glass melted and not total glass melted.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

The calculation of soda ash consumption is subject to uncertainties linked to:

- glass production data, which are themselves estimates subject to moderate uncertainty

- estimate of the rate of soda ash production per tonne of glass, which is an approximate figure.

The emission factor is based on the stoichiometry of the chemical reaction undergone by the soda ash and will be accurate.  The time-series required some interpolation of data from year to year.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

The consumption of soda ash in 2002 has been recalculated for this version of the inventory.  This is due to the availability of new data on glass production from British Glass (2004) leading to changes in the estimates of production of flat glass and domestic glass in 2002.  This leads to an estimated decrease in emissions of 0.8 Gg carbon in 2002.

### Source-specific planned improvements

Estimates for this sector could be improved either through collection of actual soda ash consumption data or through more detailed estimation of soda ash consumption at sub-sector level (e.g. separately for flat glass, container glass etc. using glass composition data.)  Currently the sector is probably not a priority for further improvements, since emissions are relatively minor compared to the UK total.

## Source Category 2A5 – Asphalt Roofing

Emissions of CO2 are not estimated from this source as there is no methodology available.  Emissions from this source category are likely to be extremely small in relation to national emissions.

## Source Category 2A6 – Road paving with Asphalt

### Source category description

Bitumen is used in the preparation of road surfaces.  Different types of surface dressing are used and some contain kerosene as well as bitumen.  The kerosene partially evaporates and is emitted to atmosphere.  Emissions are reported under 2A6.

### Methodological issues

Emissions of CO2 are not estimated from this source, as there is no methodology available.  Emissions from this source category are likely to be extremely small in relation to national emissions.

The inventory reports emissions of NMVOC from the use of bitumen emulsions, cut-back bitumens, and cut-back fluxes used in road construction using emission factors of 7, 87.5 and 700 kg NMVOC/ tonne for each component respectively (Refined Bitumen Association, 1990).  These estimates are based on the assumption that only 70% of the kerosene is emitted, the remainder being fixed in the road material.  Estimates of the usage of these surface dressings are based on a set of consumption data for one year only, provided by the Transport and Road Research Laboratory (1989) and are extrapolated to other years using data for annual bitumen consumption given in the Digest of UK Energy Statistics (DTI, 2004).

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

The estimates of NMVOC from road paving are quite uncertain, due particularly due the long-term extrapolation of a single set of consumption data.  The use of bitumen consumption may well represent a good indicator of road construction but it is unclear whether the rate of use of the specialised products containing kerosene will change to a similar extent.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

A recalculation has been required for this version of the inventory, due to revisions in the bitumen consumption data given by DTI, 2004.  This leads to an estimated increase in emissions of 1.6 Gg NMVOC in 2002.

### Source-specific planned improvements

Emissions are small compared with national totals and the sector has been a low priority for further improvement.  Nonetheless, new solvent consumption data currently being collected may provide more accurate estimates of recent kerosene usage for road construction, allowing the time series to be improved for the next version of the inventory.

## Source Category 2A7 – Other Mineral Products

### Source category description

Emissions from Fletton brickworks and manufacture of glass fibres and glass wool are reported under 2A7.  Emissions of NMVOC from the manufacture of coated roadstone have been included in the GHGI for the first time with the 2003 version.

Fletton bricks are manufactured at three sites in Southern England using the Lower Oxford clay.  This clay contains a high level of carbonaceous material, which acts as a fuel during firing, leading to emissions of carbon dioxide, carbon monoxide, methane, and NMVOC.  The clay also contains sulphurous material, which can result in SO2 emissions as well.

Glass fibres are manufactured at two sites in the UK and glass wool at nine sites.  Both process types involve the attenuation of molten glass into fine fibres, which are then cooled and coated with organic materials.  The coating processes give rise to some emissions of NMVOC.

### Methodological issues

Emissions data for Fletton brickworks during recent years are available from the Pollution Inventory (Environment Agency, 2004).  These data will include emissions both from the burning of the carbonaceous and sulphurous material in the clay but also from the burning of coal and gas used as support fuel.  Emissions from the clay materials were estimated by estimating the likely emissions from coal and gas combustion in the manufacture of the bricks and then subtracting these estimates, which are included in source category 1A2f from the emissions reported in the Pollution Inventory.  This gave an estimated of emission from the clay which are reported here.  The recent emissions data are extrapolated back using estimates of Fletton brick production.  The sole company involved in the manufacture of Fletton bricks has been approached but did not provide any additional data; this will increase the uncertainty of the estimates of emissions from earlier years.

Emissions of NMVOC from glass fibre and glass wool processes in recent years are also available from the Pollution Inventory, although these do not include the two glass wool producers located in Scotland.  The Pollution Inventory data are used to calculate emission factors, based on estimates of glass production and emissions can then be calculated both to include all processes and, by extrapolation, to include other years.

Emissions of NMVOC during manufacture of coated roadstone are estimated using production data from TSO, 2004 and an emission factor of 8.73 g/t coated roadstone, which is the average of emission factors given by US EPA, 2004 for various types of batch roadstone coating plant.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

The estimates for both Fletton brickworks and glass processes are very uncertain.  The glass processes are very minor sources of VOC and are not considered further.  Estimates for Fletton bricks, carbon in particular, are sensitive to the assumptions made about supplementary fuel use and so the estimates could be improved were fuel consumption data available.

The time-series involves some extrapolation of data using brick production estimates and this will introduce further uncertainty within the earlier part of the time series.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

Due to changes regarding the carbon content of coal resulting from the 2004 review of UK carbon factors, estimated emissions from Fletton brickworks have increased by 0.5 Gg carbon in 2002.  Emissions of NMVOC from manufacture of coated roadstone, a source not previously included, are estimated as 0.2 Gg NMVOC in 2002.

### Source-specific planned improvements

Emissions estimates related to Fletton brick production could be significantly improved on the basis of better information on the usage of supplementary fuels and this will be sought.

## Source Category 2B1 – Ammonia Production

### Source category description

Ammonia is produced using the Haber process, which starts with the steam reforming of natural gas to make hydrogen.  The simplified reactions are:

CH4	 + 	H2O 		 CO 	+ 	3H2

CO	 +	 H2O	 	 CO2	 + 	H2

The hydrogen is then reacted with nitrogen from air to form ammonia

N2	 +	 3H2	 	 2NH3

If there is no use of the by-products CO and CO2 formed, then these are emitted to atmosphere.  Ammonia plants can be integrated with methanol and/or acetic acid manufacture for greater efficiency.  Thus, hydrogen formed as a by-product from acetic acid manufacture is used as the feedstock for ammonia manufacture.  Some carbon monoxide and carbon dioxide from the reforming process is used to manufacture methanol:

CO	 + 	2H2	 	 CH3OH

One ammonia plant sells CO2 to the food industry and nuclear industry.  Because this CO2 is still ultimately emitted to atmosphere, it is included in the emissions reported here.  This is judged more reliable than trying to identify carbon emissions at the point of final use since CO2 will also be emitted from other processes such as fermentation.

Ammonia was being produced at four UK sites by the end of 2003, one of which also produced acetic acid.  Methanol production, which was carried out at a different UK site, ceased in 2001.

### Methodological issues

Emissions from ammonia production and the associated production of methanol and acetic acid are reported under two inventory source categories.  The first category is reserved for emissions of CO2 from natural gas used as a feedstock in the ammonia and other processes.  The second category includes emissions of CO2 and other pollutants from the combustion of natural gas to produce the heat required by the reforming process.

Emissions of CO2 from feedstock use of natural gas were calculated by combining reported data on CO2 produced, emitted and sold by the various ammonia processes.  Where data were not available, they have been calculated from other data such as plant capacity or natural gas consumption.  The ammonia plant utilising hydrogen by-product from acetic acid manufacture does not need to be included since there are no process emissions of CO2.

A correction has to be made for CO2 produced at one site where some of this CO2 is subsequently 'recovered' through sequestration in methanol.  This carbon is calculated from methanol capacity data based on the stoichiometry of the chemical reaction. (This only applies to estimates for 1990-2001, after which this correction is not required as the methanol plant ceased operation.)

The use of natural gas as a feedstock was calculated by combining:

- natural gas equivalent to carbon sequestrated in methanol (see above)

- natural gas equivalent to the CO2  emitted from ammonia manufacture

- natural gas usage of the acetic acid plant, available from the process operator

For the first two parts of the calculation, the default carbon emission factor for natural gas was used to convert between carbon and natural gas.  The total feedstock use of natural gas was estimated as the sum of items 1-3 and a CO2 emission factor can be calculated from the CO2 emission estimate already generated.

Emissions of CO2 and other pollutants from natural gas used as a fuel are calculated using estimates of natural gas usage as fuel supplied by the operators and emission factors.  Factors for NOx are back-calculated from reported NOx emissions data, while emission factors for carbon, methane, CO, N2O and NMVOC are default emission factors for industrial gas combustion.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

A consistent time series of activity data has usually been reported from the manufacturers of ammonia, and this results in good time series consistency of emissions.  For 2001 to 2003, no new ammonia production data were received from one plant operator. Production estimates from 2000 and annual plant emissions data from the Environment Agency Pollution Inventory have been used to estimate production & emissions from this plant in 2001-2003.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

No recalculations of historic data have been made in the production of the 2003 inventory.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.  A programme to examine the emissions from ammonia, nitric acid, adipic acid and acetic acid production plant is scheduled for summer 2005, and any corrections to historic data will be identified and resolved prior to the production of the 2004 inventory.

## Source Category 2B2 – Nitric Acid Production

### Source category description

Nitric acid is produced by the catalytic oxidation of ammonia:

4NH3 + 5O2  4NO + 6H2O

2NO + O2  2NO2

3NO2	 + H2O  2HNO3 + NO

Nitrous oxide is also formed by oxidation of ammonia:

4NH3	 + 3O2  2N2O + 6H2O

Nitrous oxide is emitted from the process as well as a small percentage of the NOx.  Nitric acid was being manufactured at 4 UK sites at the end of 2003.  One plant fitted abatement equipment to reduce the N2O emissions in 1998. The abatement system is a thermal oxidation unit and is reported by the operators to be 99.99% efficient at N2O destruction. In 2003 it was operational 95.4% of the time (when compared to plant operation).

### Methodological issues

Across the 1990-2003 time-series, the availability of emissions and production data for UK Nitric Acid (NA) plant is inconsistent, and hence a range of methodologies have had to be used to provide estimates and derive emission factors for this sector.

For plant in England, emissions data are available for all sites from 1998 onwards. For the plant (now closed) in Northern Ireland, emissions data became available from 2001.

Site-specific production estimates are largely based on production capacity reported directly by the plant operators.  This approach may overestimate actual production.  No data are available for two sites operating between 1990 and 1994, and production at these sites is calculated based on the difference between estimates of total production and the sum of production at the other sites.

Emission estimates for N2O are derived for each NA site using:

- emissions data provided by the process operators directly or via the Pollution Inventory (1998 onwards for plant in England, 2001 onwards for plant in N Ireland);

- site-specific emission factors derived from reported emissions data for the same site for another year (1990-1997 for some plant in England, 1994-1997 for other plant in England,  1990-2000 for plant in N Ireland);

- a default emission factor of 6 ktonnes N2O /Mt 100% acid produced in cases where no emissions data are available for the site (some sites in England, 1990-1993).  This default factor is the average of the range quoted in IPCC Guidelines (IPCC, 1997) for medium pressure plant.  This default emission factor is also used to calculate emissions during the years prior to fitting of abatement at the one site which now has N2O abatement.

Emissions of NOx are derived for each nitric acid site using emissions data provided by the process operators directly or via the Pollution Inventory.  No emissions data are available before 1994 and so a default NOx emission factor of 3.98 tonne NOx /ktonne of 100% acid produced and nitric acid production data (CIS, 1991) is used up to 1988 with emissions between 1989 and 1993 being calculated by linear interpolation.  The default emission factor is an aggregate factor based on CORINAIR (1989) emission factors for the different types of processes ranging from 3-12 t/kt of 100% acid produced.  The aggregate factor was based on data on UK manufacturing plant provided by the Nitric Acid Association for the year 1985 (Munday, 1990).

Some nitric acid capacity is associated with a process that manufactures adipic acid.  For the years 1990-1993, its emissions are reported combined with those from the adipic acid plant (see Section 3.10) and emissions from 1994 onwards are reported separately.  This causes some inconsistency in between reporting categories, although total emissions should not be affected.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

Emissions from nitric acid production are estimated based on a combination of emission factors and reported emissions data.  The methodology used to estimate N2O for this sector does vary from year to year depending upon the availability of data.  The calculated N2O EF for UK nitric acid production facilities varies quite significantly across the time series, and this may be a reflection of the lack of availability of a consistent time-series of emissions data.  However, the variable N2O EF for this sector is also a reflection of nitric acid production patterns across UK sites that utilise different process conditions with only one plant fitted with N2O abatement.

For all plants in England, emissions of N2O used in the GHG inventory are taken from emissions reported in the Pollution Inventory data from 1998 onwards.  For the plant in Northern Ireland, reported emission data became available from 2001 onwards.  Prior to these years in England, emissions of N2O are estimated using either plant-specific EFs (in terms of plant capacity) based on 1998 PI data and applied to known historic plant capacity, or by applying a default emission factor of 6 ktonnes N2O /Mt 100% acid produced for some plant in 1990-1993. A similar approach has been used for the nitric acid plant in Northern Ireland prior to 2001.

The nitric acid plant emissions data are considered to be reliable since they are subject to internal QA/QC checks by the plant operators and the Environment Agency before being reported in the Pollution Inventory.  More details have been obtained regarding the abatement plant and N2O monitoring methodologies at the one UK plant with N2O abatement fitted, and this has clarified some previous uncertainties regarding their process emissions.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

No recalculations have been required for this version of the inventory.

### Source-specific planned improvements

Emission factors and activity data will be kept under review. A specific research programme into the emissions from nitric acid and adipic acid production plant is scheduled for summer 2005, and any corrections to historic data will be identified and resolved prior to the production of the 2004 inventory.

## Source Category 2B3 – Adipic Acid Production

### Source category description

Adipic acid is manufactured in a multi-stage process from cyclohexane via oxidation with nitric acid.  Nitrous oxide is produced as a breakdown product from the nitric acid.  A single company produces adipic acid in the UK.

### Methodological issues

UK production figures and emission estimates have been provided by the process operator (Invista, 2004).  The emission estimates are based on extrapolation of on-line measurement of the concentration of nitrous oxide in the flue gases, conducted for regulatory compliance monitoring purposes since 1998. Prior to 1998, the operators have provided estimates of emissions using appropriate emission factor data.

A small nitric acid plant is associated with the adipic acid plant that also emits nitrous oxide.  From 1994 onwards this emission is reported as nitric acid production but prior to 1994 it is included under adipic acid production.  This will cause a variation in reported effective emission factor for these years. This allocation reflects the availability of data.  In 1998 an N2O abatement system was fitted to the plant.  The abatement system is a thermal oxidation unit and is reported by the operators to be 99.99% efficient at N2O destruction. In 2003 it was operational 95.4% of the time (when compared to plant operation), and in previous years (1998-2002) an estimate of 89% abatement plant availability has been provided.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

Emissions of N2O from adipic acid production are now taken from emissions reported in the Pollution Inventory, with more process-specific details also provided directly by the plant operators.  In the early 1990s, emissions were received direct from the plant operators.

The level of uncertainty associated with reported emissions of N2O is not currently known.  However these data are considered to be reliable since they are subject to internal QA/QC checks within the company producing the adipic acid, and QA/QC checks by the Environment Agency before being reported in the Pollution Inventory.

Fluctuations in the N2O EF from this plant are apparent since the installation of the abatement plant. The reason for these fluctuations are unknown, and a study has been commissioned to investigate this matter during 2005.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

No recalculations have been required for this version of the inventory.

### Source-specific planned improvements

Emission factors and activity data will be kept under review. A specific research programme into the emissions from nitric acid and adipic acid production plant is scheduled for summer 2005, and any corrections to historic data will be identified and resolved prior to the production of the 2004 inventory.

## Source Category 2B4 – Carbide Production

This category does not occur in the UK.

## Source Category 2B5 – Other

### Source category description

The UK has a large chemical manufacturing sector and emissions of methane, carbon monoxide, NOX, SO2, and NMVOC in the inventory are treated in some detail to reflect the many different types of process.  All of these emission sources are reported under 2B5.

Chemical manufacturing processes are a significant source of NMVOC emissions.  Due to the complexity of the sector and the difficulty of separating emissions from different chemical processes, almost all emissions are reported using a single, general, category.

Emissions of the remaining pollutants are less significant compared with national totals but are reported in more detail.

Methane emissions are reported separately for production of ethylene and production of methanol, these chemicals being suggested as sources by the IPCC Guidelines for National Greenhouse Gas Inventories.  Ethylene was manufactured on four sites at the end of 2003 while the only methanol plant closed in 2001.  The IPCC Guidelines also suggested that methane might be emitted from manufacture of carbon black, styrene and dichloroethylene however no evidence of any emissions of methane from these processes has been found and no estimates have been made.  However, methane is emitted from other UK chemical processes and these emissions are reported as third, general, source category.

Emissions of other pollutants are reported under the following source categories:

- Chemical industry  - CO, SO2, NMVOC

- Chemical industry (carbon black) - CO, SO2

- Chemical industry (nitric acid use) - NOx

- Chemical industry (pigment manufacture) - SO2

- Chemical industry (reforming) - CO

- Chemical industry (soda ash) - CO

- Chemical industry (sulphuric acid use) - SO2

- Chemical industry (titanium dioxide) - CO

- Coal, tar and bitumen processes - NMVOC

- Solvent and oil recovery - NMVOC

- Ship purging - NMVOC

- Sulphuric acid production - SO2

The first source listed is the general category used where emissions occur from processes, which do not fit elsewhere.  The remaining categories are specific and often relate to small numbers of sites.  Carbon black is produced at two sites by partially burning petroleum feedstocks to produce finely divided soot.  The categories 'chemical industry (nitric acid use) and 'chemical industry (sulphuric acid use) refer to processes using these acids and emitting NOX and SO2 respectively.  Manufacture of nitric acid (see Section 4.10) and sulphuric acid are treated separately from use.  Sulphuric acid was being produced at four sites at the end of 2003.  Pigment manufacture relates to a single plant where sulphur is burnt as part of the manufacturing process.  The sulphur oxides produced are largely consumed in the process, although some emissions do occur.

Reforming processes convert natural gas or other light hydrocarbons into hydrogen and carbon monoxide for use in further chemical processes, and can result in emissions of CO.  Soda ash manufacture also results in some emissions of CO, which is formed during the lime manufacturing stage and then passes through the chemical processes before being emitted.  These emissions are not included in the inventory category 'Lime (combustion)'.  Titanium dioxide is manufactured by two routes in the UK, but one involves the use of coke as a reductant and is carried out on two sites.  Carbon monoxide is emitted to atmosphere from the process.  The remaining three source categories are reserved for minor sources of NMVOC.  Processes involving coal-based chemicals and bitumen-based products are reported under 'coal, tar & bitumen processes', the recovery of solvents and other organic chemicals by distillation is reported under 'oil & solvent recovery', and the venting of chemical vapours from ships' tanks where cross-contamination of cargoes must be avoided, is reported under 'ship purging'.

### Methodological issues

Emissions data for chemical processes located in England and Wales are available in the Pollution Inventory (Environment Agency, 2004).  Reporting generally started in 1994 or 1995, and few data exist for the years prior to 1994.  Data for ethylene production processes in Scotland and additional data for some of the methane-emitting processes in England and Wales have been obtained from process operators.  The Scottish Environment Protection Agency has supplied some data on emissions of NMVOC from Scottish chemical processes and additional NMVOC data for processes located in both Scotland and Northern Ireland have been obtained from process operators.  The National Sulphuric Acid Association (NSAA, 2003) have provided historical emissions data for sulphuric acid production processes.  Emissions from ship purging are based on a single estimate given by Rudd et al (1996), which is applied to all years.

All of the data available are in the form of emission estimates, usually generated by the process operators and based on measurements or calculated based on process chemistry.  Emission factors and activity data are not required, although emission factors are back-calculated in the process of extrapolation of emissions back to the years prior to 1994.   The extrapolation is usually linked to changes in the level of output from the chemicals manufacturing sector as measured by the 'index of output' figures published by the Office of National Statistics (2004).  In a few cases, such as the figures for methane from ethylene production and SO2 from sulphuric acid production, actual emissions data are available or can be estimated for individual plant based on actual plant capacities.

Some gaps exist in the reported data.  For example, emissions from a given process will be reported for some years but not others, even though the process is known to have been operating.  These gaps are presumably due to the fact that either the process operator was not required to submit emissions data or that emissions data was not or could not be supplied when requested.  Most of the gaps occur in the early years of the pollution inventory.  These gaps have been filled by copying emissions data from the nearest year for which emissions data were reported.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

Emission estimates for 1994 onwards are mostly based on data reported by process operators and might therefore be considered accurate.  However, in the absence of any detailed assessment of the methods used by individual process operators to estimate emissions, it is not possible to come to a definite conclusion.  Emission estimates for NMVOC are more uncertain than the estimates for other pollutants because of the way in which these emissions are reported in the Pollution Inventory.  As a result, the data have to be interpreted using expert judgement.

Emission estimates for the period prior to 1994 are also more uncertain, with the exceptions of sulphuric acid production and methane emissions.  This is due to the need for extrapolation of emissions data for 1994, or later, backwards, using general indicators of chemical industry output.

The reliability of emission estimates from 2002 onwards may deteriorate for at least some of the sources included in this sector.  This is due to changes in the reporting requirements for the Pollution Inventory with the de minimis limits for reporting of emissions of some pollutants being raised, and greater use made of extrapolation based on drivers.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

Some recalculation of emissions has occurred since the last inventory.  This is due to a number of factors including:

- changes to the emissions data given in the Pollution Inventory and other sources;

- the influence of emissions data for 2003, available for the first time, with subsequent changes to the extrapolations necessary for filling 'gaps' in the data (for example, gaps in reported data for 2001 might previously been filled using emissions reported for 2000, whereas now the mean of the 2000 and 2002 emissions would be used).

The various recalculations have usually resulted in very small changes in emissions from these sources compared with values in the last version of the inventory. These changes are summarised below by gas.

#### Recalculation by gas

The following section describes the main changes that have occurred in sector 2B5 per pollutant since the publication of the 2001 inventory.  Comparisons are made between the current inventory (1990-2002) and the previous inventory (1990-2001) for the year 2001.

##### Sulphur Dioxide (SO2)

- Estimated emissions from 2B5 for CO have decreased by 2.0 Gg.  This increase occurs due to changes to the chemicals and manmade fibres due to updated data.

##### Non Methane Volatile Organic Compounds (NMVOCs)

- Estimated emissions from 2B5 have increased by 0.4 Gg for NMVOCs.  This is largely due to revisions in the emission factors for the chemicals and manmade fibres section of the industry due to updates in available data.

### Source-specific planned improvements

Estimates of emissions from this sector are subject to uncertainty although this uncertainty is not well understood.  The estimates of NMVOC are certainly subject to considerable uncertainty and have been considered as a priority for improvement for some years.  However, the methods used to generate emission estimates for all of these sources are complex and require a significant effort each year in terms of data gathering, data interpretation and extrapolation.  Changes in the methodology are likely to be required from year to year in order to deal with changes in the data available.  The intention behind these changes is to try to maintain the quality of estimates at current levels but improvement of the estimates would almost certainly require radical and resource-intensive changes to the procedures used to generate emission estimates.

## Source Category 2C1 – Iron and Steel Production

### Source category description

UK iron and steel production may be divided into integrated steelworks, electric arc steelworks, downstream processes such as continuous casting and rolling of steel, and iron & steel foundries.

Integrated steelworks convert iron ores into steel using the three processes of sintering, pig iron production in blast furnaces and conversion of pig iron to steel in basic oxygen furnaces.  For the purposes of the inventory, emissions from integrated steelworks are estimated for these three processes, as well as other minor processes such as slag processing.

Sintering involves the agglomeration of raw materials for the production of pig iron by mixing these materials with fine coke (coke breeze) and placing it on a travelling grate where it is ignited.  The heat produced fuses the raw materials together into a porous material called sinter.  Emissions of CO2, CH4, CO, NOx SO2, and NMVOC from sintering are reported under CRF category 1A2.

Blast furnaces are used to reduce the iron oxides in iron ore to iron.  They are continuously charged with a mixture of sinter, fluxing agents such as limestone, and reducing agents such as coke.  Hot air is blown into the lower part of the furnace and reacts with the coke, producing carbon monoxide, which reduces the iron ore to iron.  Gas leaving the top of the blast furnace has a high heat value because of the residual CO content, and is used as a fuel in the steelworks.  Molten iron and liquid slag are withdrawn from the base of the furnace.  Subsequent cooling of the slag with water can cause emissions of SO2.  The most significant greenhouse gas emissions to occur directly from the blast furnace process are the combustion gases from the 'hot stoves' used to heat the blast air.  These generally use blast furnace gas, together with coke oven gas and/or natural gas as fuels.  These emissions are reported under CRF category 1A2.  Gases emitted from the top of the blast furnace are collected and emissions should only occur when this gas is subsequently used as fuel.  These emissions are allocated to the process using them.   However, some blast furnace gas is lost and the carbon content of this gas is reported under CRF category 2C1.

Pig iron has a high carbon content derived from the coke used in the blast furnace.  A substantial proportion of this must be removed to make steel and this is done in the basic oxygen furnace.  Molten pig iron is charged to the furnace and oxygen is blown through the metal to oxidise carbon and other contaminants.  As a result, carbon monoxide and carbon dioxide are emitted from the furnace and are collected for use as a fuel.  As with blast furnace gases, some losses occur and these losses are reported with blast furnace gas losses under CRF category 2C1.

Electric arc furnaces produce steel from ferrous scrap, using electricity to provide the high temperatures necessary to melt the scrap.  Emissions of carbon dioxide occur due to the breakdown of the graphite electrodes used in the furnace and NOx is formed due to oxidation of nitrogen in air at the high temperatures within the furnace.  Emissions of NMVOC and CO occur due to the presence of organic contaminants in the scrap, which are evaporated and partially oxidised.  Emissions from electric arc furnaces are reported under CRF category 2C1.

The inventory contains estimates of NMVOC emissions from rolling mills.  Lubricants are needed and contain organic material, some of which evaporates.  These emissions are reported under 2C1.  A more significant emission from rolling mills and other downstream processing of steel are those emissions from use of fuels to heat the metal.  These emissions are reported under 1A2.

### Methodological issues

The methodology for the prediction of carbon dioxide emissions from fuel combustion, fuel transformation, and processes at integrated steelworks is based on a detailed carbon balance (this methodology is described in more detail within the section on CRF sector 1A2a).  Carbon emissions from electric arc furnaces are calculated using an emission factor provided by BISPA (1997).  For other pollutant emissions from blast furnaces, emissions are partly based on the methodology described in IPCC (1997), with some revisions made to the SO2 factors based on data available from industry.  Details of all methodologies are provided in Annex 3, Section A3.4.2, which also provides details on emissions from electric arc furnaces.  Energy related emissions from foundries are included in category 1A2a but any process emissions from foundries of direct GHGs are likely to be very small and are not estimated.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

Much of the activity data used to estimate emissions from this source category come from the Iron and Steel Statistics Bureau (ISSB) and the DTI publication DUKES.  Time-series consistency of these activity data are very good due to the continuity in data provided in these two publications.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

Estimated emissions from flaring of blast furnace gas have been increased by 27.1 Gg carbon as a result of changes to the emission factor following the review of UK carbon emission factors (Netcen, 2004).  A decrease in estimated emissions of 1.1 Gg of CO from primary aluminium production occurred due to the availability of updated data.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 2C2 – Ferroalloys Production

This category is not relevant to UK emissions.

## Source Category 2C3 – Aluminium Production

### Source category description

Aluminium is produced by the electrolytic reduction of alumina, currently at three sites in the UK.  A fourth process closed in mid 2000.  All of the operational sites use the pre-baked anode process, whereas the closed plant used the Soderberg Cell process.  This distinction is important because of large differences in emission rates for some pollutants.

Both process types make use of carbon anodes and these anodes are consumed as the process proceeds, resulting in emissions of CO2, CO, NMVOC and SO2.  The high temperatures necessary in the process mean that NOx is also emitted.  Finally, the PFC species tetrafluoromethane (CF4) and hexafluoroethane (C2F6) are formed if the alumina content of the electrolyte falls too low.  Computerised control of alumina addition to the cells is a feature of modern plant and has helped to reduce PFC emissions from aluminium production.

### Methodological issues

Emissions of carbon were estimated based on the production of aluminium for each type of process and emission factors.  The carbon emission factors reflect current practice, and higher emission factors were used for earlier years.

All emissions of PFCs occur during the aluminium smelting process.  The estimates were based on actual emissions data provided by the aluminium-smelting sector. The methodology used for estimating emissions, based on IPCC Good Practice Guidance (2000), was ‘Tier 2 Method – smelter-specific relationship between emissions and operating parameters based on default technology-based slope and over-voltage coefficients’.  Emissions estimates were based on input parameters, including frequency and duration of anode effects, and number of cells operating.  Emission factors were then used to derive the type of PFC produced.  All emissions occur during manufacturing.

The type of smelter design has a large effect on the rate of PFC emissions.  The UK industry has previously made major investment to improve their technology and all UK plants now use point feeder prebake.  A more detailed description of the methodology used to calculate emission estimates for this sector is provided in AEAT (2004).

For other pollutants, emissions data are available from the Environment Agency’s Pollution Inventory for the two largest processes in England & Wales, whilst data for the plant located in Scotland were obtained by direct contact with the plant operators or derived from emission factors calculated from the England & Wales plant emissions. From 2004 onwards, the emissions data from the Scottish plant will be available on an annual basis via the EPER inventory database of the Scottish Environmental Protection Agency (SEPA).

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

The source of activity data, from 1990 to the current inventory year, is from data compiled by the British Geological Survey (production of primary aluminium).  This is a long running publication and the compilers of the activity data strive to use consistent methods to produce the activity data.  This helps to ensure good time series consistency of the emission estimates.

For emissions of PFCs, a large decrease in implied emission factor occurs in 1992, which appears linked to a 31% reduction in the number of operating cells at the largest UK manufacturer.  These may presumably have been older, less efficient cells and their elimination from the production process would have led to lower emissions occurring per production unit.  Conversely, in 2000 the implied emission factor increases.  In that year the UK production of primary aluminium jumped by 12% compared with the previous year, however it is not readily apparent from the data received from industry why this would have led to the increased implied emission factor.  We are following this up with industry for future clarification.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

- There have been no significant recalculations made to the PFC emissions data for this sector since the previous submission.

### Source-specific planned improvements

The calculation methodology for historic emissions of CO, NOx and SO2 uses emissions data reported for two processes and extrapolates those data to a third.  The data for emissions from the Scottish plant are now available directly from the operators and in future will be reported annually via the Scottish Environment Protection Agency.

## Source Category 2C4 – SF6 used in Aluminium and Magnesium Foundries

### Source category description

SF6 is used in the magnesium alloy and casting industry as a cover gas, to prevent molten magnesium oxidising when exposed to air.  All SF6 used in this way is released to the atmosphere unless capture/recycle technologies are employed.  SF6 is non-flammable and non-toxic, and is therefore a safe gas to use.  In the UK, SF6 has been used as an alternative cover gas to SO2 in magnesium alloy production and sand and die-casting since the early 1990s.  Magnesium alloy production and casting are therefore significant emitters of SF6 in the UK.

In the UK, there is one large magnesium alloy producer and five smaller casting operators (2 die-casting and 3 sand-casting). Alloy production involves the use of primary magnesium ingots, recycled scrap material and second-generation magnesium materials (i.e. material already made into alloys) for the production of different alloys.  Both die and sand casters use these magnesium alloys to produce specific components for a wide range of industries.  For the casting industry, SF6 is used for casting specific magnesium alloys where other cover gases, such as argon, are not suitable.

For magnesium alloy production, emissions were estimated based on SF6 consumption data supplied by the main distributor, on the basis that all SF6 consumed is emitted immediately.  Data from 1998-2003 are considered accurate whilst earlier data are estimated and, based on consultation with the manufacturer.  Estimates of emissions from magnesium casting are based on those made by March (1999), based on discussion with the sector and the unavailability of more robust information.  These estimates are significantly lower than those from magnesium alloy production.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

The following information on uncertainty associated with time-series data for this sector has been derived from the study that produced the emission estimates (AEAT, 2004), and should not be confused with the formal IPCC uncertainty analysis in Annex 7.

For the period 1990-1997, the estimated uncertainty in the time series data was +/- 30%.  The main area of uncertainty is regarding emissions of SF6 from casting, which were estimated based on the previous March (1999) report, and from discussions with the sector Trade Association.  In the absence of more up-to-date information, a high uncertainty level was therefore assumed.  Data from the main magnesium alloy producer is also uncertain for this period.  For the period 1998-2003, the uncertainty of the time-series emissions is estimated to be significantly lower (+/- 10%).  The main area of uncertainty was regarding emissions of SF6 from casting, which again were estimated from the previous March (1999) inventory and discussions with the sector Association.  Data from the main magnesium alloy producer is considered to be reasonably robust and accurate.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

No recalculations have been performed in the 2003 inventory for this sector.

### Source-specific planned improvements

- Emission factors and activity data will be kept under review.

## Source Category 2C5 – Other Metal Production

### Source category description

UK production of many non-ferrous metals has been relatively small for many years and has declined further in recent years with the closure of the only primary lead/zinc producer in 2003 and the only secondary copper production process in 1999.  A number of secondary lead processes exist, although the capacity of one lead refinery has been significantly reduced following the closure of the primary lead/zinc producer, which used to supply it with lead bullion.  The two closed processes and some of the secondary lead processes use coke as a reductant and emissions from these processes should be reported under 2C5.  Currently, emissions of carbon from use of this coke are included with estimates for other industrial combustion (see Section A3.4.3) while emissions of CO from the lead/zinc smelter, copper refinery and secondary lead producers are reported under 2C5.  Two of the secondary lead producers also emit SO2 from the automotive batteries that they recover lead from.  Copper wire rod plants use natural gas burners to create a slightly reducing atmosphere in the melting furnace, which helps to maintain a high conductivity product.  Emissions of CO are reported under 2C5.  Carbon monoxide is also used by the only UK nickel refinery and is produced by reforming of butane.  Emissions from this process have been included in the NAEI estimates for chemical industry reforming processes and are reported under 2B5.

### Methodological issues

Emission estimates for these processes are derived from emissions data available from the Pollution Inventory (Environment Agency, 2004).  For earlier years, where no emissions data are available, emission estimates are made by extrapolation based on production of the relevant type of metal.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

Emissions of direct greenhouse gases from this source category will be minor and are currently not estimated.  No comments are currently made here on the time series consistency of the indirect GHGs.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

An estimated decrease in SO2 emissions of 0.1 Gg in 2002 occurred as a result of a minor change in the treatment of emission data recorded in the Pollution Inventory.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 2D1 – Pulp and Paper

### Source category description

The UK paper industry is mainly confined to the production of pulp from recycled material and the production of papers using either imported virgin pulp, recycled pulp or a combination of the two.  Production of virgin pulp is limited to a few processes producing mechanical or neutral sulphite semi-chemical pulp.  Emissions from UK paper processes consist largely of emissions from the associated combustion processes, which supply steam and power to the papermaking processes.  These emissions are reported under CRF category 1A2.  Other atmospheric emissions of greenhouse gases from UK paper and pulp processes will be minor and are currently not estimated.

Emissions of NMVOC from the manufacture of chipboard, fibreboard and oriented strand board (OSB) are reported under 2D1.  These products differ in the type of wood material that is made into board.  Chipboard is made from assorted wood shavings, dust & chippings etc., while fibreboard is made from mechanically pulped wood fibres and OSB is made from long, thin wafers of wood with fairly uniform dimensions.  All three processes involve steps for drying of the wood particles and hot pressing of the formed board and both steps give rise to some NMVOC emissions.

### Methodological issues

Emissions are estimated using emission factors derived from those available in the USEPA Compilation of Air Emission Factors (USEPA, 2004).  Production of the wood products is estimated from data published by the Office of National Statistics.  These data are given as areas or volumes of product depending upon the type of product and must be converted to a mass basis by making assumptions about the thickness and/or density of the products.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

No recalculations have been required for this version of the inventory.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 2D2 – Food and Drink

### Source category description

A number of food and drink manufacturing processes give rise to emissions of NMVOC.  Most significant are emissions of ethanol from whisky maturation.  Whisky is matured for a period of years in wooden barrels.  This process develops the character of the whisky but an inevitable consequence is that spirit evaporates from the barrel.  Other spirit manufacturing stages such as fermentation, distillation, casking (whisky only) and drying of spent grains also give rise to NMVOC emissions although these emissions are relatively small in comparison with those from maturation.  Whisky manufacture is confined mainly to Scotland, which has 7 large grain distilleries and approximately 90 smaller, malt distilleries.  There is a single small whisky distillery in Wales and a large whiskey distillery in Northern Ireland.  Scotland is also a major source of other distilled spirits such as gin and vodka, as is England.  Malt production also creates emissions of NMVOC.  Malting is occasionally carried out by distilleries but most malt, both for distillers and breweries, is produced by specialist maltsters.  Brewing processes such as fermentation and wort boiling and fermentation for production of cider and wine are all very minor sources of NMVOC.

Bread manufacture involves fermentation reactions and ethanol is released as a result.  Most bread in the UK is made in large mechanised bakeries, of which there are about 70.  The remainder is made in small ‘craft bakeries’.  Some other baked products include a fermentation stage and also emit ethanol.   Heating of food products can cause reactions that produce organic emissions, and so processes such as drying of vegetable matter, preparation of compounded animal foods and cooking of meat and fish can cause NMVOC emissions.  Finally, the processing of oils and fats is also a source of emissions, although emissions of hexane, a solvent used to extract vegetable oil from rape and other oilseeds is included in estimates of solvent use rather than as a food industry emission.

### Methodological issues

Emissions of NMVOC from food and drink manufacture are all calculated using emission factors and activity data obtained from either industry or Government sources.  In the case of whisky maturation, data are available for volumes of whisky in storage at the end of each year from the Scotch Whisky Association (2004), and so emissions can be calculated by applying an annual emission rate factor with the average volume of whisky in storage for each year.  This is more accurate than using an overall emission factor applied to whisky production since whiskies are stored for varying lengths of time and stock levels will rise or fall depending upon production, demand and changes in the length of maturation required.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

Emissions of direct greenhouse gases from this source category will be minor and are currently not estimated.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

The estimates for animal feed manufacture and production of oils and fats have both been increased by 0.1 Gg NMVOC as a result of updated activity data being available.  An additional 0.4 Gg NMVOC is estimated to be emitted from the maturation of alcoholic drinks other than Scotch Whisky – this source has not been previously included.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 2E – Production of Halocarbons and SF6

### Source category description

Emissions arise from the UK manufacture of HFCs, PFCs and HFC 23 (as a by-product formed during HCFC 22 production).  There are two single manufacturers of HFCs and PFCs respectively in the UK, and two companies currently operate HCFC 22 plants.  Species data from these sectors have been aggregated to protect commercial confidentiality.  There is no UK production of SF6.

In terms of their global warming impact (expressed as kt CO2 eq.), HFC 23 emissions are responsible for the substantial majority of emissions from this manufacturing sector.  HFC 23 is emitted as a by-product of HCFC 22 production.  It has a high GWP, and traditionally is emitted at levels of 3-5% of the amount of HCFC 22 produced. The market for HCFC 22 is presently made up of three elements:

- end user markets, refrigerants for refrigeration and air-conditioning equipment

- export markets

- feedstock for production of certain plastic products, especially PTFE.

### Methodological issues

Emissions for this sector were calculated using the same emission model as used for the UK’s previous submission.  A full description of the emission model and associated methodology used is contained in AEAT (2004).  Within the model, manufacturing emissions from UK production of HFCs, PFCs and HFC 23 (by-product of HCFC 22 manufacture) are estimated from reported data from the respective manufacturers.  Manufacturers have reported both production and emissions data, but only for certain years, and for a different range of years for different manufacturers.  Therefore the emissions model is based on implied emission factors, and production estimates are used to calculate emissions in those years for which reported data was not available.  Two of the three manufacturers are members of the UK greenhouse gas Emissions Trading Schemes.  As a requirement of participation in the scheme, their reported emissions are verified annually via external and independent auditors.

Under an agreement on confidentiality, the three UK manufacturers have provided speciated data on the condition that only aggregated data are reported.  As described in Section 4.21.1, there is only one UK manufacturer of HFCs, a different sole manufacturer of PFCs and two manufacturers of HCFC22.  The UK inventory team will investigate to establish whether it will be possible to report emissions by species in future.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

The following information on uncertainty associated with time-series data for this sector has been derived from the study that produced the emission estimates (AEAT, 2004) based on an understanding of the uncertainties within the sector and from discussion with industry, and should not be confused with the formal IPCC uncertainty analysis in Annex 7.  An uncertainty range of +/- 15% was estimated for the time-series emissions.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6, and details of verification of emissions of HFC-134a and HFC-152a are given in Annex 8.  Additionally, as described above in Section 4.21.2, two of the UK manufacturing plants also have their emissions externally validated as part of the requirements of the UK Emissions Trading Scheme.

### Source-specific recalculations

There have been no significant recalculations made to the emissions data for this sector since the previous submission.

### Source-specific planned improvements

There are no source-specific improvements planned.

## Source Category 2F1 – Refrigeration and Air Conditioning Equipment

### Source category description

HFCs and HFC blends have been widely used as replacement refrigerants across virtually all refrigeration sub-sectors.  They generally share many of the properties of CFC and HCFC refrigerants, namely low toxicity, zero and/or varying degrees of flammability and acceptable materials compatibility.  Emissions of HFCs can occur at various stages of the refrigeration/air-conditioning product life-cycle:

- during the refrigeration equipment manufacturing process;

- over the operational lifetime of the refrigeration or air-conditioning unit; and

- at disposal of the refrigeration or air-conditioning unit.

This emission category contains aggregated emission estimates from the following sector sub-divisions:

- Domestic refrigeration (including refrigerators, chest freezers, upright freezers and fridge-freezers)

- Other small hermetic refrigeration units (including through the wall air-conditioners, retail equipment, drinking water coolers etc)

- Small commercial distributed systems (including pub cellar coolers, small chill and cold stores)

- Supermarket systems

- Industrial systems

- Building air conditioning systems (direct use of refrigerant)

- Building air-conditioning chillers (indirect use of refrigerant)

- Refrigerated transport (refrigerated lorries, containers etc) using conventional refrigeration technology

- Mobile air conditioning (air-conditioning systems for cars and other vehicles).

### Methodological issues

Emissions for this sector were calculated using the same emission model as used for the UK’s previous submission. A full description of the emissions and associated methodology used is contained in AEAT (2004).  The general methodology used was based on that of March (1999).  The calculation methodology within the model is considered to provide a relatively conservative approach to the estimation of emissions.  The bank of fluid is estimated by considering the consumption of fluid in each sector, together with corrections for imports, exports, disposal and emissions.  Once the size of the bank in a given year is known, the emission can be estimated by application of a suitable emission factor.  Emissions are also estimated from the production stage of the equipment and during disposal.  The methodology corresponds to the IPCC Tier 2 'bottom-up' approach.  Data are available on the speciation of the fluids used in these applications; hence estimates were made of the global warming potential of each fluid category.

- Emissions from the domestic refrigeration sector were estimated based on the bottom-up approach using UK stock estimates of refrigerators, fridge-freezers, chest-freezers and upright freezers from the UK Market Transformation Programme (MTP, 2002).  For the commercial refrigeration sub-sectors, emissions for these sectors were based on the activity data supplied by industry and used in previous emission estimates by March (1999) and WS Atkins (2000).  Consultation with a range of stakeholders was used to determine appropriate country-specific emission factors; these generally fell within the ranges given in IPCC guidance (IPCC 2000).  A full list of emission factors and assumptions used for the domestic and commercial refrigeration sub-sectors is provided in AEAT (2004).

- Emissions of HFCs from mobile air conditioning systems were also derived based on a bottom-up analysis using UK vehicle statistics obtained from the UK Society of Motor Manufacturers and Traders, and emission factors determined in consultation with a range of stakeholders.  A full account of the assumptions and data used to derive emission estimates for the MAC sub-sector is in AEAT (2004).

### Uncertainties and time-series consistency

The following information on uncertainty associated with time-series data for this sector has been derived from the study that produced the emission estimates (AEAT, 2004), and should not be confused with the formal IPCC uncertainty analysis in Annex 7.

- Estimates of the uncertainties associated with time-series data for this sector were made in AEAT (2004), based on an understanding of the uncertainties within the sector and from discussion with industry.  An uncertainty range of +/- 20% was estimated for the aggregated time-series emissions from the domestic and commercial refrigeration sectors, and +/- 10% for the mobile air conditioning sector.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6. Details of verification of emissions of HFC-134a and HFC-152a are given in Annex 8.

### Source-specific recalculations

There have been no significant recalculations made to the emissions data for this sector since the previous submission.

### Source-specific planned improvements

- Emission factors and activity data will be kept under review.

## Source Category 2F2 – Foam Blowing

### Source category description

Prior to the Montreal Protocol, a wide range of foams was produced using CFC blowing agents.  As use of these chemicals was banned, the industry moved to alternatives including HCFCs.  For applications such as packaging and cushioning, the use of HCFCs was banned under the EC Regulation on Substances that Deplete the Ozone Layer (EC 3093/94) and these sectors moved to blowing agents such as water or CO2.  Use of HCFC was still permitted in rigid insulating foams and integral skin foams for safety applications, but a new EC Regulation on Substances that Deplete the Ozone Layer  (EC 2037/2000) has or will shortly ban all HCFC use in these remaining sectors.

Emissions of HFCs from foams can occur:

- during the manufacturing process

- over the lifetime of the foam; rigid foams are closed cell foams and the blowing agent is designed to remain in the foam and contributes to its performance.  Loss of HFCs is undesirable as it may affect the performance of the foam but is estimated to occur, albeit at a low rate

- at disposal of the foam.

- Emissions at each point vary according to the type of foam.  Typically, of the HFC used in the production process, less than 10% is emitted during manufacture (although emissions may be as high as 40 to 45 % for some types of foam), less than 1% per year over the useful lifetime of the product and the remainder on disposal.

### Methodological issues

- Emissions for this sector were calculated using the same emission model as used for the UK’s previous submission.  A full description of the emissions and associated methodology used is contained in AEAT (2004).  The emissions for the years 1990 to 2003 are based on data from March (1999).  The methodology used estimates the bank of fluid used by considering the consumption of fluid in each foam sub-sector, together with corrections for imports, exports, disposal and emissions.  Once the size of the bank in a given year is known, the emission can be estimated by application of a suitable emission factor.  Emissions are also estimated from the production stage of the equipment and during disposal.  The methodology corresponds to the IPCC Tier 2 'bottom-up' approach.

### Uncertainties and time-series consistency

- The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

- The following information on uncertainty associated with time-series data for this sector has been derived from the study that produced the emission estimates (AEAT, 2004), and should not be confused with the formal IPCC uncertainty analysis in Annex 7.

- Estimates of the uncertainties associated with time-series data for this sector were made in AEA Technology (2004), based on an understanding of the uncertainties within the sector and from discussion with industry.  Time-series data was estimated to have an uncertainty range of +/- 30% for this sector.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6. Details of verification of emissions of HFC-134a and HFC-152a are given in Annex 8.

### Source-specific recalculations

There have been no significant recalculations made to the emissions data for this sector since the previous submission.

### Source-specific planned improvements

- Emission factors and activity data will be kept under review.

## Source Category 2F3 – Fire Extinguishers

### Source category description

In the UK, manufacturers of fixed suppression systems for fire fighting have been using HFCs as an alternative to Halons for the past 7-8 years.  Fluorocarbons currently take up some 25% of the market that would have previously been covered by Halons. This is primarily due to the specific requirements of certain industries where the use of HFCs is seen as necessary to reduce fire risks.  Such systems have much faster discharge and suppression times, and do not damage equipment.

The systems are also compact and take up minimal space.  The HFCs themselves are non-toxic. It is the combination of speed, space and safety that makes HFCs important alternatives to Halon in those applications where these properties are required.  HFC-based systems are used for the protection of electronic and telecommunications equipment, and in military applications, records offices, bank vaults and oil production facilities.

The main HFC used in UK fixed systems is HFC 227, with some use of HFC 23 and HFC 125.  The majority of emissions of HFCs will occur when the system is discharged, either when triggered accidentally or during a fire.  Minimal emissions may also occur during filling or maintenance of the systems.  The rest of the market for fixed system applications uses inert gases or non-gaseous agents, such as water mist, and non-extinguishing early warning systems.

As well as HFCs being used to replace halon-based systems in the mid-1990s, a small quantity of PFC (mainly C4F10) was imported by a US company into the EU to be used as an alternative fluid in fire fighting fixed systems.  The main application of these PFC-based fixed systems is for fire protection of flooding closed rooms (e.g. control rooms).  Imports for new systems stopped in 1999, as this application of PFCs was not regarded as an essential use.  For purposes of recharge, PFCs are still supplied.  By 2010 there will probably be no fixed systems using PFCs in the EU.  Emissions of PFCs from these systems are thought to be insignificant relative to other PFC emission sources.

Portable extinguishers have moved away from Halons, with most manufacturers using water, dry powder and carbon dioxide as the replacement.  A small number of niche applications use HFCs, but emissions from such applications are thought to be insignificant.

### Methodological issues

Emissions for this sector were calculated using the same emission model as used for the UK’s previous submission. Emissions estimates were obtained from March (1999) for years 1990-1996 and for subsequent years from the representative UK trade organisation, the Fire Industry Council (FIC).  The emissions data are based on estimates of installed capacity and an annual emission rate of approximately 5% per annum.  The latter has been based on discussion with the industry, and with reference to the 1998 UNEP Halon Technical Options Committee (HTOC) report.  There are no emissions from HFC prior to 1995.  A full description of the emissions and associated methodology used is contained in AEAT (2004).

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

The following information on uncertainty associated with time-series data for this sector has been derived from the study that produced the emission estimates (AEAT, 2004), and should not be confused with the formal IPCC uncertainty analysis in Annex 7.

Estimates of the uncertainties associated with time-series data for this sector were made in AEAT (2004), based on an understanding of the uncertainties within the sector and from discussion with industry.  Uncertainties in emissions over the 1990-2003 period were estimated to be +/- 5-10%.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6. Details of verification of emissions of HFC-134a and HFC-152a are given in Annex 8.

### Source-specific recalculations

There have been no significant recalculations made to the emissions data for this sector since the previous submission.

### Source-specific planned improvements

- Emission factors and activity data will be kept under review.

## Source Category 2F4 – Aerosols/ Metered Dose Inhalers

### Source category description

In the UK, HFCs are generally used as propellants in specific aerosols where the use of HFCs is considered critical, i.e. where safe alternative propellants are not available.  Historically many types of aerosols were formulated with CFCs as propellants.  However, for the vast majority of aerosols, the use of CFCs ceased at the end of 1989 on account of concerns regarding their role in ozone destruction.  Aerosol manufacturers could then choose between a number of options to replace CFCs, including hydrocarbons, dimethyl ether (DME), compressed gases or HFCs.

The vast majority of aerosols use hydrocarbon propellants, with a relatively small proportion of the market favouring DME.  Compressed gases are used in very few aerosols since they suffer from a number of disadvantages compared with liquefied gas propellants such as DME and hydrocarbons.  HFCs are used only in a few specialist applications, which can be categorised as industrial or non-industrial.  Most of these are considered critical (as defined by BAMA (British Aerosol Manufacturers Association) and agreed by Defra) with regard to the use of HFCs as propellants.  The most important industrial applications in volume terms are air dusters and pipe freezing products; other applications include specialised lubricants and surface treatments, and specialised insecticides.  The main non-industrial applications in the UK are novelty products, such as ‘silly string’, where the use of HFC is considered critical due to the need for non-flammable propellants.  The use of HFCs for novelty applications is proposed to be eliminated in the future, under the proposed EC Regulation on fluorinated greenhouse gases.

Metered dose inhalers (MDIs) are used to deliver certain pharmaceutical products as an aerosol.  For patients with respiratory illnesses, such as asthma and chronic obstructive pulmonary disease (COPD), medication needs to be delivered directly to the lungs.  MDIs are one of the preferred means of delivering inhaled medication to patients with these illnesses.  MDIs originally used CFC propellants but, as with industrial aerosols, concern over ozone destruction led to attempts to replace CFCs with HFCs.  HFCs have been identified as the only viable replacement for CFCs in MDIs as no other compound has met the stringent criteria for a medical gas to be used for inhalation by patients.  Criteria include the need for the gas to be non-flammable, non-toxic, liquefied, chemically stable, compatible with range of medicines, acceptable to patients, and to have appropriate density and solvent properties.  This switch from CFCs to HFCs has resulted in increasing emissions of HFCs from this sector (although a saving in terms of CO2 equivalent).

### Methodological issues

Emissions for this sector were calculated using the same emission model as used for the UK’s previous submission.  A full description of the emissions and associated methodology used is contained in AEAT (2004).  Aerosol HFC emission estimates have been derived on the basis of fluid consumption data provided by BAMA.  Estimates of emissions from HFC-filled aerosols were derived by estimating the amount of fluid used annually in their manufacture.  An average product lifetime of one year for all aerosols containing HFC has been assumed, based on discussions with BAMA, although this may be shorter or longer depending on the specific aerosol application.  The number of HFC-based aerosols that are used in the UK is derived from data from BAMA, based on assumptions concerning imports and exports.  It is estimated that 1% of HFC emissions from aerosols occur during manufacture.  The majority is released during the product lifetime (97%), with end of life emissions accounting for the other 2%.  These emission factors are the same as those estimated in previous work by March (1999).  The lifetime and end of life emissions are calculated after import and exports have been taken into account.

The MDI methodology was based on a Tier 2 bottom-up analysis, deriving the number of units (inhalers) used annually and estimating the amount of HFC in each inhaler.  Although the amount of HFC in each inhaler differs between manufacturers, an average amount was assumed.  MDIs were assumed to emit 96% of total HFC contained during the lifetime usage: 2% of emissions occur during manufacture and 2% at end-of-life.  Import and export levels have been based on data provided by manufacturers, and estimates of the UK market for MDI usage.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

The following information on uncertainty associated with time-series data for this sector has been derived from the study that produced the emission estimates (AEAT, 2004), and should not be confused with the formal IPCC uncertainty analysis in Annex 7.

Estimates of the uncertainties associated with time-series data for this sector were made in AEAT (2004), based on an understanding of the uncertainties within the sector and from discussion with industry.  The uncertainty for aerosol emissions was estimated to be  +/- 15-20%, based on uncertainties surrounding the estimation of import and export markets, and reliance on estimates from previous work (March 1999).  For MDIs, the uncertainty was estimated to be +/- 30-40%, a relatively high uncertainty due to the use of approximations of the use of HFCs in MDIs for research work, and assumptions that had to be made concerning the import / export market, domestic market and number of doses used in the UK annually.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6. Details of verification of emissions of HFC-134a and HFC-152a are given in Annex 8.

### Source-specific recalculations

There have been no significant recalculations made to the emissions data for this sector since the previous submission.

### Source-specific planned improvements

- Emission factors and activity data will be kept under review.

## Source Category 2F5 – Solvents

### Source category description

- HFCs can be used as solvents in a range of applications such as precision cleaning to replace CFCs, HCFCs or 1,1,1-trichloroethane, the use of all of which have been or will be phased out as a result of the Montreal Protocol.  In recent years, HFCs have been developed that are used for precision cleaning in sectors such as aerospace and electronics. CFCs were used as solvents in precision cleaning before being replaced by certain HCFCs, namely HCFC-141-b.  As an ozone depleting substance, this HCFC has started to be replaced by HFC-43-10mee, albeit slowly.  Due to only being used as a replacement in recent years, the amount of this HFC being sold in the UK market at present is thought to be insignificant relative to other UK sources of HFCs.  However, future growth could be high, depending on their use as a replacement to HCFC-141b over the next 10 years.

### Methodological issues

Emissions for this sector were calculated using the same emission model as used for the UK’s previous submission. A full description of the emissions and associated methodology used is contained in AEAT (2004).  UK estimates of emissions from this source were based on a recent European evaluation of emissions from this sector (Harnisch and Schwarz, 2003), subsequently disaggregated to provide a top-down UK estimate.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

The following information on uncertainty associated with time-series data for this sector has been derived from the study that produced the emission estimates (AEAT, 2004), and should not be confused with the formal IPCC uncertainty analysis in Annex 7.

Estimates of the uncertainties associated with time-series data for this sector were made in AEAT (2004), based on an understanding of the uncertainties within the sector and from discussion with industry. There is a relatively high uncertainty estimated for emissions from this sector (+/- 25%).

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.  Details of verification of emissions of HFC-134a and HFC-152a are given in Annex 8.

### Source-specific recalculations

There have been no significant recalculations made to the emissions data for this sector since the previous submission.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 2F6 – Semiconductor Manufacture

### Source category description

PFCs and SF6 are released from activities in this source sector.

Emissions of PFCs from semiconductor manufacturing are combined with emissions from training shoes in source category 2F8b for reasons of commercial confidentiality.  This source category is described in Section 4.30.

Emissions of SF6 from semiconductor manufacturing are combined with emissions from training shoes and electrical insulation in source category 2F8b for reasons of commercial confidentiality.  This source category is described in Section 4.30.

## Source Category 2F7 – Electrical Equipment

### Source category description

SF6 is released from activities in this source sector.

Emissions of SF6 from electrical equipment (insulation in electrical transmission and distribution – e.g. switchgear) are combined with emissions from training shoes and semiconductor manufacture in source category 2F8b for reasons of commercial confidentiality.  This source category is described in Section 4.30.

## Source Category 2F8a – One Component Foams

### Source category description

One Component Foams (OCFs) are used by tradesmen (and in the home improvement sector to a lesser extent) to mount doors and windows, and to insulate different types of open joints and gaps.  As an insulator, OCF helps improve energy efficiency, due to the insulating properties of the PU foam and because the foam adheres to the building materials providing air tightness.  Therefore, use of OCFs could contribute to savings of CO2 through improved energy efficiency.  When used as an OCF propellant, HFC (134a, 152a) is blended with various flammable gases.  HFC escapes from the foam on application, leaving small residues, which remain in the hardened foam for up to a year.  These products are not manufactured in the UK, although they are imported.

### Methodological issues

Emissions for this sector were calculated using the same emission model as used for the UK’s previous submission.  A full description of the emissions and associated methodology used is contained in AEAT (2004).  UK estimates of emissions from this source were based on a recent European evaluation of emissions from this sector (Harnisch and Schwarz, 2003), subsequently disaggregated by GDP to provide a top-down UK estimate.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

The following information on uncertainty associated with time-series data for this sector has been derived from the study that produced the emission estimates (AEAT, 2004), and should not be confused with the formal IPCC uncertainty analysis in Annex 7.

- Estimates of the uncertainties associated with time-series data for this sector were made in AEAT (2004), based on an understanding of the uncertainties within the sector and from discussion with industry.  Emissions from this sector are estimated to fall within an uncertainty range of 10-25%.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6. Details of verification of emissions of HFC-134a and HFC-152a are given in Annex 8.

### Source-specific recalculations

There have been no significant recalculations made to the emissions data for this sector since the previous submission.

### Source-specific planned improvements

- Emission factors and activity data will be kept under review.

## Source Category 2F8b – Semiconductors, Electrical and production of trainers)

### Source category description

SF6 has been used as a cushioning agent in sports-shoes.  It is well suited to this application because it is chemically and biologically inert and its high molecular weight means it cannot easily diffuse across membranes.  This means the gas is not released until the training shoe is destroyed at the end of its useful life.  SF6 has also been used for filling tennis balls, but this practice has now ceased.

SF6 has been used in electrical transmission and distribution high and medium voltage switchgear and transformers since the mid-1960s because the physical properties of the gas make it very effective as an arc-quenching medium and as an insulator.  Consequently it has gradually replaced equipment using older technologies, namely oil filled and air blast equipment.

The electronics industry is one of the largest sources of PFC emissions in the UK, accounting for 36% of emissions in 2000.

The main uses of PFCs are:

- cleaning of chambers used for chemical vapour deposition (CVD) processes

- dry plasma etching

- vapour phase soldering and vapour phase blanketing

- leak testing of hermetically sealed components

- cooling liquids, e.g. in supercomputers or radar systems.

In addition SF6 is used in etching processes for polysilicon and nitrite surfaces, and there is some usage of CHF3 and NF3.  The first two of these processes (cleaning and etching during semiconductor manufacture) account for the majority of emissions from the sector, with cleaning accounting for around 70% and etching 30%.

### Methodological issues

- Emissions from these sectors have been combined for reasons of commercial confidentiality.  Emissions for this sector were calculated using the same emission model as used for the UK’s previous submission.  A full description of the emissions and associated methodology used is contained in AEAT (2004).  Estimates of emissions from sports-shoes were based on a bottom-up Tier 2 estimate, using activity data supplied in confidence by the manufacturer.

SF6 emission from electrical transmission and distribution were based on industry data from BEAMA (for equipment manufacturers) and the Electricity Association (for electricity transmission and distribution), who provided emission estimates based on Tier 3b, but only for recent years.  Tier 3a estimates were available for the electricity distribution and transmission industry for 1995.  In order to estimate a historical time series and projections, these emission estimates together with fluid bank estimates provided by the utilities were extrapolated using the March study methodology (March, 1999).  This involved estimating leakage factors based on the collected data and using the March model to estimate the time series.  Emissions prior to 1995 used the March SF6 consumption data to extrapolate backwards to 1990 from the 1995 estimates.

Emissions of PFC and SF6 emissions from electronics are based on data supplied by UK MEAC – the UK Microelectronics Environmental Advisory Committee.  UK MEAC gave total PFC consumption for the UK electronics sector based on purchases of PFCs as reported by individual companies.  Emissions were then calculated using the IPCC Tier 1 methodology, which subtracts the amount of gas left in the shipping container (10%), the amount converted to other products (between 20% and 80% depending on the gas) and the amount removed by abatement (currently assumed to be zero).  Emissions for previous years were extrapolated backwards assuming an annual 15% growth in the production of semiconductors in the UK up until 1999.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and fuel type.

The following information on uncertainty associated with time-series data for this sector has been derived from the study that produced the emission estimates (AEAT, 2004), and should not be confused with the formal IPCC uncertainty analysis in Annex 7.

- Estimates of the uncertainties associated with time-series data for this sector were made in AEAT (2004), based on an understanding of the uncertainties within the sector and from discussion with industry.  Estimated uncertainties in individual sectors: sports-shoes: +/- 20-50%, electronics +/- 30%, and electrical transmission and distribution +/- 20%.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.  Details of the verification of the greenhouse gas inventory are given in Annex 8.

### Source-specific recalculations

There have been no significant recalculations made to the emissions data for this sector since the previous submission.

### Source-specific planned improvements

- Emission factors and activity data will be kept under review.

# Solvent and Other Product Use (CRF sector 3)

## Overview of Sector

Solvents are used in a wide range of processes and products and the GHGI gives detailed estimates to reflect this diversity.  Significant quantities of solvent are used both for industrial applications (mainly coatings and cleaning solvents), but also for non-industrial applications (mainly aerosols, decorative paints and consumer products).

Annex 3.5 contains more detailed descriptions of the methods used to estimate emissions in this sector.

## Source Category 3A – Paint Application

### Source category description

Emissions of solvents from the use of both industrial and decorative paints are reported under CRF source category 3A.  Both types of paint are further sub-divided in the GHGI:

Table 5.1	Paints and their applications in the UK

| Type of paint | Application |
| --- | --- |
| Decorative paint: Retail decorative Trade decorative | 'DIY' decorative coatings mainly sold directly to the public 'Professional' decorative coatings mainly sold to decorating contractors |
| Industrial coatings: ACE Aircraft Coil Commercial vehicles Drum High performance Marine Metal and plastic Metal packaging OEM Vehicle refinishing Wood | Coatings for agricultural, construction and earthmoving equipment Coatings for aircraft & aircraft components Coatings for steel and aluminium coil Coatings for new, non-mass produced vehicles Coatings for new and reclaimed metal drums Coatings for large structures such as bridges, offshore installations etc. Coatings for the exteriors and interiors of ships and yachts including both new and old vessels Coatings for metal and plastic substrates not covered elsewhere Coatings for food and beverage cans and other small metal packaging Coatings for new mass-produced road vehicles Coatings for the refinishing of road vehicles Coatings for wooden substrates |

### Methodological issues

Emission estimates for most types of coatings are based on annual consumption data and emission factors provided by the British Coatings Federation (BCF, 2003).  Emission estimates for drum coatings, metal packaging and OEM coatings are estimated instead using a combination of consumption data and emission factors and estimates made on a plant by plant basis using information supplied by the Metal Packaging Manufacturers Association (MPMA, 2000) and the regulators of individual sites.

### Uncertainties and time-series consistency

This source does not affect the overall total or trend in UK emissions of direct greenhouse gases and is not included in the Tier 1 or Tier 2 uncertainty analysis.

The data used to estimate emissions from paint application are mostly provided by the British Coating Federation (BCF) and are available for all years except 2003.  Data from 2002 are used instead for that year.  With this exception, the time series is consistent.  Estimates for the drum coating, car coating, and metal packaging coating sectors are based on emissions data collected from regulators for the latter part of the time series with extrapolation to earlier years on the basis of BCF coating consumption data.  This extrapolation is not thought likely to introduce significant problems with the accuracy of estimates.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

A recalculation was necessary in order to eliminate an error in the calculation of emissions from coating of metal packaging.  A net estimated decrease of 1.6 Gg of NMVOC occurred due to this change.

It should be noted that in this section and the corresponding sections in 3B-3D, emissions occur for NMVOC only.  Comparisons refer to the year 2002 and reflect the effect that changes made to the underlying data and methodology can have on emission estimates.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.  Efforts will be made to obtain updated activity data for 2003.

## Source Category 3B – Degreasing and Dry Cleaning

### Source category description

This sector covers the use, predominantly of chlorinated solvents, for cleaning and degreasing of surfaces, dry cleaning of clothing and textiles and degreasing of leather.

Chlorinated solvents, including trichloroethene, tetrachloroethene and dichloromethane are widely used in industry to clean metallic, plastic and other surfaces, often using the process of vapour degreasing.  Objects to be cleaned are suspended above boiling solvent.  Solvent vapour condenses on the object and removes grease and other surface contamination.   Cooling tubes at the top of the tank minimise emissions but some solvent is emitted.  Cold cleaning is also used with objects being dipped in cold solvent and larger objects may be hand cleaned with solvent-soaked cloths.  Historically, 1,1,1-trichloroethane was also used as a cleaning solvent but this was prohibited due to this solvent's contribution to ozone depletion and use ceased by 1999.

Hydrocarbons and oxygenated solvents are also used as cleaning solvents, generally being used for hand cleaning or cold cleaning of objects.

Dry cleaning involves the use of tetrachloroethene to clean clothes and textiles in special equipment.  The solvent is largely recovered and recycled within the machine but emissions do occur, especially in older 'open' machines, where the final drying stage involves venting of solvent-laden vapour to atmosphere.

Sheepskins must be degreased due to their high fat content.  Degreasing can be done using either hydrocarbon or chlorinated solvents.

### Methodological issues

Emission estimates for surface cleaning processes are based on estimates of annual consumption and emission factors.  Consumption estimates are based on data from UK industry sources and UK and European trade associations, together with some published data.  Some extrapolation of data is necessary, using Index of Output data produced annually by the Office for National Statistics (ONS), although this is not expected to introduce significant uncertainty into the estimates.  Emission factors assume that all hydrocarbon and oxygenated solvent is emitted, while emission factors for chlorinated solvents are lower, reflecting the fact that some solvent is sent for disposal rather than emitted.

Emission estimates for dry cleaning are based on estimates of solvent consumption by the sector.  Industry-sourced data are available for some years and estimates for the remaining years are based on a model of the sector, which takes account of changes in the UK population and the numbers of machines of different types and with different emission levels.

Emission estimates for leather degreasing are based on a single estimate of solvent use extrapolated to all years using the Index of Output for the leather industry, which is produced annually by the ONS.

### Uncertainties and time-series consistency

This source does not affect the overall total or trend in UK emissions of direct greenhouse gases and is not included in the Tier 1 or Tier 2 uncertainty analysis.

The time series for degreasing emissions uses a consistent methodology, although the activity data used are not of uniform quality for each year, some extrapolation of data being required.  This extrapolation is not thought likely to introduce significant problems with the accuracy of estimates.  Although perhaps more uncertain than estimates for 3A and 3C, the estimates for source category are still expected to be good.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

Some recalculations were necessary for this sector in order to incorporate revised data from ONS.   A net estimated increase of 0.4 Gg of NMVOC occurred due to this change.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 3C  – Chemical Products, Manufacture and Processing

### Source category description

This sector includes the manufacture of coatings, the coating of films, leather, paper and textiles, and the use of solvents in the manufacture of tyres and other rubber products.

Coating manufacture includes the manufacture of paints, inks, and adhesives, plus specialist coatings for films, leather, paper and textiles.

Film coating includes the manufacture of photographic film, data storage films, hot stamping films and other specialist products.  Processes manufacturing hot stamping films are especially large users of solvents.

Leather is generally coated with products that are waterborne, although more solvent borne coatings were used historically.  Coatings are used to provide protection or to enhance the appearance by improving colour or glossiness.

Textile coating processes include the manufacture of textiles, manufacture of tarpaulins and other heavy-duty textiles, and coating of textiles with rubber.

Solvents are used in the manufacture of tyres and other rubber products such as hose, belting and sports goods.  The solvent is used for cleaning and also to increase the tackiness of the rubber during joining operations.

### Methodological issues

Emission estimates for coating of film, leather, and textiles as well as estimates for tyre manufacture are based on plant-by-plant emission estimates, made on the basis of information available from regulators.

Emissions from coating manufacture are calculated from the solvent contained in coatings produced in the UK, by assuming that an additional 2.5% of solvent was lost during manufacture.

Emissions from the manufacture of rubber goods other than tyres are based on solvent consumption estimates provided by the British Rubber Manufacturers Association (BRMA), which are extrapolated to other years on the basis of the Index of Output figures for the rubber industry which are published each year by the ONS.

### Uncertainties and time-series consistency

This source does not affect the overall total or trend in UK emissions of direct greenhouse gases and is not included in the Tier 1 or Tier 2 uncertainty analysis.

Estimates for sources covered by source category 3C are estimating using a consistent methodology with relatively little extrapolation of data.  As with the estimates for source categories 3A and 3B, extrapolation of data is not thought likely to introduce significant problems with the accuracy of estimates.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

Some minor recalculations of estimates for coating manufacture have been made in order to incorporate new data for a small number of processes.  Revised data from the ONS have also been incorporated.  This has lead to an estimated decrease of 0.2 Gg of NMVOC.

### Source-specific planned improvements

Emission factors and activity data for the category will be kept under review.  Some additional data are being gathered for the estimation of emissions from the production of other rubber products.  This may lead improvements to the estimates for the next version of the inventory.

## Source Category 3D - Other

### Source category description

This category covers a diverse group of sources including paper coating, printing processes, adhesives use, seed oil extraction, wood impregnation, agrochemicals use, aerosols, consumer products and miscellaneous solvent use.

Paper coating processes include solvent used in the manufacture of wallpapers, together with coating of other specialist paper products such as vehicle air filters or colour cards.

Printing processes differ in their requirement for solvent-borne inks and chemicals.  Most solvent use occurs from the printing of flexible packaging using flexography and rotogravure printing with solvent-borne inks.  Publication gravure printing for magazines and catalogues etc. also uses high solvent inks.  Heatset web offset printing, coldset web offset, and sheetfed offset, used for printing magazines, newspapers and other publications, employ paste inks that contain high boiling point hydrocarbons which are driven off and burnt in the case of heatset web offset or absorb into the printed substrate in the case of the other two processes.  Offset presses may use solvents in the 'damping solutions', which are used to ensure accurate reproduction of the image.  Letterpress printing also uses paste inks that dry by adsorption and is little used now.  Paper & board packaging are printed using flexography, rotogravure and offset although, unlike flexible packaging, the flexographic and gravure inks used are generally waterborne.  Screen printing, used for high quality colour printing such as art reproduction, textile printing and point of sale printing can use either water or solvent-based inks.  Other, specialist printing processes include printing of roll labels and printing of securities both of which use a variety of printing techniques including offset, letterpress, copperplate (a form of gravure printing with paste inks), flexography, and screen printing.  Solvent-borne varnishes may be applied over some printed materials.

Adhesives are used by many industries, although solvent-borne adhesives are becoming increasingly confined to a small number of industry sectors.  Construction and pressure-sensitive tapes and labels are the largest users of solvent-borne adhesives.  Other sectors include footwear, abrasives, and some furniture manufacture.

Seed oil extraction involves the use of hexane to extract vegetable oil from rape and other seed oils.  The solvent is recovered and reused in the process.

Solvents are used in some wood preservatives, although consumption has fallen markedly in the last ten years.  Emissions from use of creosote, which does not contain solvent, are also reported under 3D.

Agrochemicals can be supplied in many forms including solid or solutions and some are dissolved in organic solvents, which are emitted when the agrochemical is applied.

Aerosols use organic chemicals both as propellants and as solvents.  All use of volatile organic materials in aerosols is reported under CRF source category 3D.  Non-aerosol consumer products which contain or can contain significant levels of solvents include fragrances, nail varnish and nail varnish remover, hair styling products, slow release air fresheners, polishes, degreasers, screen wash, and de-icers.

Miscellaneous solvent use includes solvent usage not covered elsewhere and, current, little information is available on the types of uses included.  However, it will include applications such as pharmaceutical processes, acetylene storage, flavour extraction, foam blowing, production of asbestos-based products, oil-field chemicals and foundry chemicals.

### Methodological issues

Emission estimates are based on one of three approaches:

- Estimates are made based on activity data and emission factors supplied by industry sources (printing processes, consumer products, wood preservation)

- Estimates are made for each process in a sector based on information provided by regulators or process operators (seed oil extraction, pressure sensitive tapes, paper coating)

- Estimates are based on estimates of solvent consumption supplied by industry sources (adhesives, aerosols, agrochemicals, miscellaneous solvent use).

### Uncertainties and time-series consistency

This source does not affect the overall total or trend in UK emissions of direct greenhouse gases and is not included in the Tier 1 or Tier 2 uncertainty analysis.

Estimates for sources covered by source category 3D are estimating using a consistent methodology with relatively little extrapolation of data.  Some extrapolation of activity data is required for some sources included in source category 3D as this will limit the accuracy of emission estimates for these sources e.g. industrial adhesives, other solvent use.  Other sources included in 3D, including emission estimates for printing and paper coating are likely to be comparable in quality to the estimates for paint application or chemical products (source categories 3A and 3C).  Overall, however, the estimate for source category 3D is likely to be more uncertain than those for 3A, 3B and 3C.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

Some recalculations were necessary in order to eliminate an error in the calculation of emissions from printing.  A net estimated decrease of 4.1 Gg of NMVOC occurred due to these changes.

Recalculation of estimates was required for aerosols to reflect updated data available from industry sources.  This led to an overall estimated increase in emissions of 1.6 Gg NMVOC. Other, minor, revisions were made due to the availability of revised data for non-aerosol products, wood preservatives, printing processes, industrial adhesives and paper coating processes and these changes lead to an overall estimated increase of 0.7 Gg NMVOC.

### Source-specific planned improvements

NETCEN has an on-going programme of research to collect more data which may allow improvements to be made to the estimates for a number of sectors including miscellaneous solvent use, adhesives and certain printing processes.

# Agriculture (CRF sector 4)

## Overview of Sector

Emissions of GHGs from this sector include all anthropogenic emissions, except for emissions from fuel combustion and sewage.  These emissions are included in Energy 1A and Waste 6B, respectively.  Emissions from enteric fermentation, manure management, and agricultural soils are included in this CRF sector.  Historical emissions from the field burning of agricultural residues are included here also, but field burning ceased in the UK in 1993.

Annex 3.6 contains more detailed descriptions of the methods used to estimate emissions in this sector.

## Source Category 4A – Enteric Fermentation

### Source category description

Methane is produced as a by-product of enteric fermentation.  Enteric fermentation is a digestive process whereby carbohydrates are broken down by micro-organisms into simple molecules.  Both ruminant animals (e.g. cattle and sheep), and non-ruminant animals (e.g. pigs and horses) produce CH4, although ruminants are the largest source per unit of feed intake.

### Methodological issues

A more detailed description of the method used and emission factors can be found in Annex 3, Section A3.6.1.

Emissions from enteric fermentation are calculated from animal population data collected in the June Agricultural Census and the appropriate emission factors.  Data for earlier years are often revised so information was taken from the Defra agricultural statistics database.  Apart from cattle, lambs and deer, the methane emission factors are IPCC Tier I defaults (IPCC, 1997) and do not change from year to year.  The dairy cattle emission factors are estimated following the IPCC Tier 2 procedure (IPCC, 1997) and vary from year to year.  For dairy cattle, the calculations are based on the population of the ‘dairy breeding herd’ rather than ‘dairy cattle in milk’.  The former definition includes ‘cows in calf but not in milk’.  The emission factors for beef and other cattle were also calculated using the IPCC Tier 2 procedure but do not vary from year to year.  The enteric emission factors for beef cattle were almost identical to the IPCC Tier I default so the default was used in the estimates.  The emission factor for lambs is assumed to be 40% of that for adult sheep (Sneath et al. 1997).  In using the animal population data, it is assumed that the reported number of animals are alive for that whole year.  The exception is the treatment of sheep where it is normal practice to slaughter lambs and other non-breeding sheep after 6 to 9 months.  Hence it is assumed that breeding sheep are alive the whole year but that lambs and other non-breeding sheep are only alive 6 months of a given year (based on Smith and Frost, 2000).

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category.

Emissions are calculated from animal population data and appropriate emission factors.  The animal population data are collected in an annual census, published by Defra.  This is a long running publication and the compilers of the activity data strive to use consistent methods to produce the activity data.  The time-series consistency of these activity data is very good due to the continuity in data provided.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC procedures, which are discussed in Section 6.9.

### Source-specific recalculations

There have been no source specific recalculations.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 4B – Manure Management

### Source category description

This category reports emissions of methane from animal manures as well as emissions from their manures arising during its storage.

### Methodological issues

#### Methane emissions from animal manures

A more detailed description of the method used and emission factors can be found in Annex 3, Section A3.6.

Methane is produced from the decomposition of manure under anaerobic conditions.  When manure is stored or treated as a liquid in a lagoon, pond or tank it tends to decompose anaerobically and produce a significant quantity of methane.  When manure is handled as a solid or when it is deposited on pastures, it tends to decompose aerobically and little or no methane is produced.  Hence the system of manure management used affects emission rates.  Emissions of methane from animal manures are calculated from animal population data (Defra, 2004a) in the same way as the enteric emissions.  Apart from cattle, lambs and deer, these are all IPCC Tier I defaults (IPCC, 1997) and do not change from year to year.  The emission factors for lambs are assumed to be 40% of that for adult sheep (Sneath et al. 1997).  Emission factors for dairy cattle were calculated from the IPCC Tier 2 procedure.  There was a revision (in 2002) of the allocation of manure to the different management systems based on new data. This is detailed in Section 6.3.2.2.  For dairy cattle, the calculations are based on the population of the ‘dairy breeding herd’ rather than ‘dairy cattle in milk’ used in earlier inventories.  The former includes ‘cows in calf but not in milk’.  The waste factors used for beef and other cattle are now calculated from the IPCC Tier 2 procedure but do not vary from year to year.

#### Nitrous Oxide emissions from Animal Waste Management Systems

Animals are assumed not to give rise to nitrous oxide emissions directly, but emissions from their manures during storage are calculated for a number of animal waste management systems (AWMS) defined by IPCC.  Emissions from the following AWMS are reported under the Manure Management IPCC category:

- Flushing anaerobic lagoons. These are assumed not to be in use in the UK.

- Liquid systems

- Solid storage and dry lot (including farm-yard manure)

- Other systems (including poultry litter, stables)

According to IPCC (1997) guidelines, the following AWMS are reported in the Agricultural Soils category:

- All applied animal manures and slurries

- Pasture range and paddock

Emissions from the combustion of poultry litter for electricity generation are reported under power stations.

The IPCC (1997) method for calculating emissions of N2O from animal waste management is followed.

The UK application of the methodology assumes that 20% of the total N emitted by livestock volatilises as NOx and NH3 and therefore does not contribute to N2O emissions from AWMS.  This is because in the absence of a more detailed split of NH3 losses at the different stages of the manure handling process it has been assumed that NH3 loss occurs prior to major N2O losses.  Thus, the Nex factors used in the AWMS estimates exclude the fraction of N volatilising and are 20% less than if they were reported on the same basis as the ‘total’ Nex factors reported in the IPCC Guidelines.  Values of total N excreted shown in the Common Reporting Format are not corrected in this way and are estimates of total N excreted from livestock.  Nex factors for dairy cattle take account of the assumed growth in the average cow weight by 1% per annum (1990–2001).

The conversion of excreted N into N2O emissions is determined by the type of manure management system used.  The distributions used were revised for cattle and poultry in the 2000 Inventory.  The change related to the way that data on ‘no significant storage capacity’ of farmyard manure (FYM) were allocated.  This could have a large effect on emissions because it amounted to around 50% of manure and the ‘Daily spread (DS)’ category has an emission factor of zero, compared to 0.02 for the ‘Solid storage and dry lot (SSD)’ category.  Assigning this ‘stored in house’ manure to ‘daily spread’ is acceptable only if emissions from the housing phase are thought to be very small.  Calculations were performed with the N2O Inventory of Farmed Livestock to compare housing and storage phases (Sneath et al. 1997).  For pigs and poultry, the emission factor for housing is the same as or greater than that of storage.  It would therefore lead to significant underestimation to use the daily spread emission factor.  The FYM in this case has therefore been re-allocated to SSD or ‘other’ as appropriate.

For dairy and non-dairy cattle, the emission factor for the housing phase is around 10% of the storage phase, so the non-stored FYM has been split between SSD and DS to account for this.

Emissions from grazing animals (pasture range and paddock) and daily spread are calculated in the same way as the other AWMS.  However, emissions from land spreading of manure that has previously been stored in a) liquid systems, b) solid storage and dry lot and c) other systems, are treated differently.  These are discussed in Annex 3, Section A3.6.3.7.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category.

Emissions are calculated from animal population data and appropriate emission factors.  The animal population data are collected in an annual census, published by Defra.  This is a long running publication and the compilers of the activity data strive to use consistent methods to produce the activity data.  The time-series consistency of these activity data is very good due to the continuity in data provided.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC procedures which are discussed in Section 6.9.

### Source-specific recalculations

There have been no recalculations made for this category.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 4C – Rice cultivation

This source is not relevant in the UK.

## Source Category 4D – Agricultural Soils

### Source category description

Direct emissions of nitrous oxide from agricultural soils are estimated using the IPCC recommended methodology (IPCC, 1997) but incorporating some UK specific parameters.  The IPCC method involves estimating contributions from:

- The use of inorganic fertilizer

- Biological fixation of nitrogen by crops

- Ploughing in crop residues

- Cultivation of Histosols (organic soils)

- Spreading animal manures on land

- Manures dropped by animals grazing in the field

In addition to these, the following indirect emission sources are estimated:

- Emission of N2O from atmospheric deposition of agricultural NOx and NH3

- Emission of N2O from leaching of agricultural nitrate and runoff

Descriptions of the methods used are described in Section 6.5.2.

### Methodological issues

A more detailed description of the method used and emission factors can be found in Annex 3, Section A3.6.3.

#### Inorganic Fertiliser

Emissions from the application of inorganic fertilizer are calculated using the IPCC (1997) methodology and IPCC default emission factors.

Annual consumption of synthetic fertilizer is estimated based on crop areas (Defra, 2004a) and fertilizer application rates (BSFP, 2004).

#### Biological Fixation of Nitrogen by crops

Emissions of nitrous oxide from the biological fixation of nitrogen by crops are calculated using the IPCC (1997) methodology and IPCC default emission factors.

The data for the ratio residue/crop are default values found under Agricultural Soils or derived from Table 4.17 in Field Burning of Agricultural Residues (IPCC, 1997).  Crop production data are taken from Defra (2004a, 2004b). The total nitrous oxide emission reported also includes a contribution from improved grass calculated using a fixation rate of 4 kg N/ha/year (Lord, 1997).

#### Crop Residues

Emissions of nitrous oxide from the ploughing in of crop residues are calculated using the IPCC (1997) methodology and IPCC default emission factors.

Production data of crops are taken from Defra (2004a, 2004b).  Field burning has largely ceased in the UK since 1993.  For years prior to 1993, field-burning data were taken from the annual MAFF Straw Disposal Survey (MAFF, 1995).

#### Histosols

Emissions from Histosols were estimated using the IPCC (2000) default factor of 8 kg N2O-N/ha/yr.  The area of cultivated Histosols is assumed to be equal to that of eutric organic soils in the UK and is based on a FAO soil map figure supplied by SSLRC (now NSRI).

#### Grazing Animals

Emissions from manure deposited by grazing animals are reported under agricultural soils by IPCC.  The method of calculation is the same as that for AWMS (Section 6.3.2.2), using factors for pasture range and paddock.

#### Organic Fertilizers

Emissions from animal manures and slurries used as organic fertilizers are reported under agricultural soils by IPCC.  The calculation involves estimating the amount of nitrogen applied to the land and applying IPCC emission factors.

The summation is for all animal types and manure previously stored in categories defined as a) liquid, b) solid storage and dry lot and c) other.

#### Atmospheric deposition of NOX and NH3

Indirect emissions of N2O from the atmospheric deposition of ammonia and NOx are estimated according to the IPCC (1997) methodology but with corrections to avoid double counting N.  The sources of ammonia and NOx considered are synthetic fertiliser application and animal manures applied as fertiliser.

The method used corrects for the N content of manures used as fuel but no longer for the N lost in the direct emission of N2O from animal manures as previously.  The nitrogen excretion data in Table A3.6.6 already exclude volatilisation losses and hence a correction is included for this.

#### Leaching and runoff

Indirect emissions of N2O from leaching and runoff are estimated according the IPCC methodology but with corrections to avoid double counting N.  The sources of nitrogen considered, are synthetic fertiliser application and animal manures applied as fertiliser.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category.

Emissions are calculated from a range of activity data and appropriate emission factors (see A3.6.3).  Emissions of N2O from the use of fertilizers are important in this source category.  The annual consumption of synthetic fertilizer is estimated based on crop areas (crop area data reported by annually Defra) and fertilizer application rates (reported annually in another Defra publication, the British Survey of Fertiliser Practice).  These are both long running datasets and the compilers of the activity data strive to use consistent methods to produce the activity data.  The time-series consistency of these activity data is very good due to the continuity in data provided.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC procedures, which are discussed in Section 6.9.

### Source-specific recalculations

There have been no recalculations in this category.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 4E – Prescribed burning of Savannas

This source is not relevant in the UK.

## Source Category 4F – Field Burning of Agricultural Residues

### Source category description

This sector covers the emissions of non-CO2 greenhouse gases from the burning (in the field) of crop residue and other agricultural waste on site.

### Methodological issues

The National Atmospheric Emissions Inventory reports emissions from field burning under the category agricultural incineration.  The estimates are derived from emission factors calculated according to IPCC (1997) and from USEPA (1997).

The estimates of the masses of residue burnt of barley, oats, wheat and linseed are based on crop production data (e.g. Defra, 2004a) and data on the fraction of crop residues burnt (MAFF, 1995; ADAS, 1995b).  Field burning ceased in 1993 in England and Wales.  Burning in Scotland and Northern Ireland is considered negligible, as is grouse moor burning, so no estimates are reported from 1993 onwards.  The carbon dioxide emissions are not estimated because these are part of the annual carbon cycle.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category.

Field burning ceased in 1994, and emissions are reported as zero after this date.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC procedures, which are discussed in Section 6.9.

### Source-specific recalculations

There have been no recalculations.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 4G - Other

There are no emissions reported in the UK under this category

## general comments on QA/QC

The livestock activity data used for constructing the inventory are supplied annually from the June census by the Defra Economics and Statistics Group, who adhere to documented QA procedures.  Activity data on mineral fertiliser are calculated using application rates from Defra's annual British Survey of Fertiliser Practice (BSFP, 2004) multiplied by crop areas in Defra's Survey of Farming Incomes (June Census).  Data from the June Census, in the form of *.PDF files, can be downloaded from the Defra website (www.defra.gov.uk) and incorporated into inventory spreadsheets without the need for manual data entry, eliminating the need for “double entry” procedures.  Annual comparisons of emission factors and other coefficients used are made by contractors compiling the inventory on behalf of Defra and by Defra itself.  Any changes are documented in the spreadsheet and in the accompanying chapter of the National Inventory Report.  Hardcopies of the submitted inventories, associated emails and copies of activity data are filed in Government secure files adhering to Government rules on document management.

Defra contractors who work on compiling the agricultural inventory, IGER, operate strict internal quality assurance systems with a management team for each project overseen by an experienced scientist with expertise in the topic area.  A Laboratory Notebook scheme provides quality control through all phases of the research and these are archived in secure facilities at the end of the project.  All experiments are approved by a consultant statistician at each of the planning, data analysis and interpretation and synthesis stages.  A range of internal checks exists to ensure that projects run to schedule, and internal and external (viz. visiting group procedures, etc.) reviews ensure the quality of the outputs.

# Land Use Change and Forestry (CRF sector 5)

## Overview of Sector

This sector differs from others in that it contains both sources and sinks of carbon dioxide.  The sinks, (or removals), are presented as negative quantities and are reported separately from emissions in the inventory tables.  Emissions from land use change and forestry were approximately 2.5% of the UK total in 2003 and are declining gradually.

The estimates for Land Use Change and Forestry are from work carried out by the Centre for Ecology & Hydrology.  The data have been submitted using both the new Common Reporting Format tables agreed at the 9th Conference of Parties to the UNFCCC and contained in FCCC/SBSTA/2004/8, and the previous Common Reporting Format under IPCC 1996 Guidelines.

Extensive revision of the data and methods used for this Sector has been made for the 2005 NIR, starting from the approaches described by Cannell et al. (1999) and Milne and Brown (1999).  These revisions have taken into account the recommendations of the IPCC Good Practice Guidance on Land Use, Land Use Change and Forestry (GPG LULUCF) (IPCC, 2003), particularly with respect to land use categories.  Annex 3.7 contains more detailed descriptions of the methods used to estimate emissions in this Sector.

The structure of this Chapter, of Annex 3.7 and of the main submission of CRF Tables is based on the Categories of the IPCC 1996 Guidelines.  Each section below includes an account of how the estimates have been used to report under the categories of the new Common Reporting Format.  The Sectoral Report Tables 5 in the new CRF format for each year from 1990 to 2003 are presented in Annex 9.3 and all the CRF Tables for Sector 5 (LULUCF) have been submitted using the CRF (LULUCF) Software v1.0.  The UK also provides data for the entire forest sink together and hence non-forest emissions and removals from soils separately.  The activity data and the different groupings are discussed in more detail in Section 7.10.

Net emissions in 1990 are now estimated to be 2645 Gg CO2 compared to 9050 Gg CO2 in the 2002 National Inventory Report.  For 2002 a net removal of –1489 Gg CO2 is estimated here compared to a net emission of 1903 Gg CO2 in the 2002 GHGI.

## Previous Source Category 5A2, new CRF categories 5A and 5G – Temperate Forests: Changes in Forests and other woody biomass stocks

### Source/sink category description

In the UK all forests can be classified as temperate and about 65% of these have been planted since 1920 on land that had not been forested for many decades.  The forests in existence since before 1920 are considered not to have significant long term changes in biomass stock. This is probably a conservative assumption.  The estimates of changes in carbon stock of the forests established since 1920 are based on activity data in the form of annual planting areas of forest published by the UK Forestry Commission and the Northern Ireland Department of Agriculture.  In the previous CRF the Removals to litter and soil for the afforested areas are reported under CRF Category 5D4 (Forest Soils) but in this, and earlier, UK NIRs they have been included in the 5A2 Category with changes in forest biomass stocks.

### Methodological issues

The carbon uptake by the forests planted since 1920 is calculated by a carbon accounting model (C-Flow) as the net change in the pools of carbon in standing trees, litter, soil and products from harvested material for conifer and broadleaf forests.  The method can be described as Tier 3, as defined in the GPG LULUCF (IPCC, 2003).  The model calculates the masses of carbon in the pools of new even-aged plantations that were clearfelled and then replanted at the time of Maximum Area Increment.

A detailed description of the method used and emission factors can be found in Annex 3, Section 3.7.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and gas.

Activity data are obtained consistently from same national forestry sources, which helps ensure time series consistency of estimated removals.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC procedures, which are discussed in Section 7.11.

### Source-specific recalculations

The method used for this category has been revised for this Report and recalculations for each year since 1990 have been made. This results in an increase in removals of about 40% for each year with some variation due the pattern of activity.

The main revision was to improve estimates of emissions from soils due to the disturbance and drainage to allow forest establishment.  Analysis of measurements taken at a deep peat moorland locations covering afforestation of peat from 1 to 9 years previously and at a 26 year old conifer forest have been published (Hargreaves et al. 2003) and suggest that long term losses from afforested peatlands are not as great as had been previously thought, settling to about 0.3 tC ha-1 a-1 thirty years after afforestation.  In addition a short burst of regrowth of moorland plant species occurs before forest canopy closure.  The pattern of carbon loss and gain from afforested deep peat moorland is summarized in Table 7.1.

Table 7.1	Emissions of carbon from deep peat due to ploughing for afforestation.
(Negative values mean uptake of carbon from the atmosphere.  Here this is due to temporary re-growth of moorland plants between ploughing and forest canopy closure.  (Based on work of Hargreaves et al. 2003)).

| Years after afforestation | Carbon loss (tC ha-1 a-1) |
| --- | --- |
| 0 | 0.0 |
| 1 | 2.2 |
| 2 | 3.8 |
| 3 | 2.5 |
| 4 | 1.1 |
| 5 | -0.3 |
| 6 | -1.2 |
| 7 | -1.6 |
| 8 | -1.6 |
| 9 | -1.3 |
| 10 | -1.1 |
| 15 | -0.2 |
| 20 | 0.1 |
| 25 | 0.2 |
| 30 | 0.3 |

The data reported for forest biomass and soils have therefore been revised in order to include this new understanding of long-term carbon loss.  Previously emissions and removals of carbon from soils after afforestation were treated differently for broadleaves and conifers. For broadleaves the reported data included the new carbon accumulating in the afforested area and assumed that emissions from pre-existing soil layers were negligible.  For conifers on peaty and mineral soils it was assumed that the newly accumulating soil carbon would exactly balance the emissions due to disturbance from pre-existing soil layers but that there would be an additional emission from afforested areas of deep peat.  This latter emission was previously reported under Category 5D or 5E as “Upland drainage”.

The carbon incorporated into the soil under all new conifer forests has now been included and losses from pre-existing soil layers are described by the general pattern measured for afforestation with conifers of deep peat.  The revision has taken into account estimates of the areas of deep peat afforested in the decades since 1920.  For planting on organo-mineral and mineral soils it is assumed that the pattern of emissions after planting will follow that measured for peat but the emissions from the pre-existing soil layers will broadly be in proportion to the initial soil carbon density of the top 30 cm relative to that same depth of deep peat.  A simplified approach was taken to deciding on the proportionality factors and it is assumed that emissions from pre-existing soil layers will be equal to that from the field measurements for all planting in Scotland and Northern Ireland and for conifer planting on peat in England and Wales.  Broadleaf planting in England and Wales is however assumed to proceed at half the rate of the field measurements.  These assumptions are based on consideration of mean soil carbon densities for non-forest in the fully revised UK soil carbon database (See Section 3.5).  The temporary re-growth of grasses is however assumed to occur at the same rate as in afforested peat moorland for all planting.  This assumption agrees with qualitative field observations in plantings on agricultural land in England.  When the smaller estimates of loss from pre-existing soil are introduced with the enhanced removals to conifer forests there is a significant increase in the net level of removals of atmospheric carbon dioxide to forest soils.

The pattern of increase in stemwood volume between planting and first thinning has also been revised.  Early growth now follows a smooth curve from planting to first thinning rather than a linear trend.  The new formulation begins with an exponential pattern but progresses to a linear trend that merges with the pattern in forest management tables after first thinning (Hargreaves et al. 2003).

### Source-specific planned improvements

The method for estimating removals and emissions due to afforestation is being developed to provide data for grid cells of 20 x 20 km.  Periodically updated forest inventory data will be used rather than annual planting data to drive the new version.  This approach is being developed to meet the requirements of the Kyoto Protocol for data more geographically explicit than the national area for reporting removals due to afforestation and deforestation under Article 3.3.  In addition the effect of deviations from standard management and externally imposed disturbances will be taken into account by this approach.

### Equivalent categories in the new CRF

The data from this Category have been entered in Sectoral Background Table 5.A in the FCCC/SBSTA/2004/8 format.  In Table 5.A.2 (Land converted to Forest Land) the data are disaggregated into afforestation of Cropland, Grassland and Settlements and further by a) the four geographical areas of England, Scotland, Wales and Northern Ireland and b) three time periods, 1920 – 1949, 1950-1979 & 1980 onwards.  The removals in this Category due to carbon stock changes in harvested wood products are entered into Sectoral Report Table 5.G (Other) as “Harvested Wood Products” in the FCCC/SBSTA/2004/8 format.

## Previous Source Category 5B2, new CRF category 5E – Temperate forests: Forest and Grassland conversion

### Source/sink category description

In National Inventory Reports and CRF submissions prior to 2002, it was assumed that permanent conversion of forest to non-forest in the UK was negligible.  This assumption was based on government guidelines against deforestation, including the need for approval for any permanent forest felling from the Forestry Commission or equivalent in Northern Ireland.  Review of this assumption suggested that some deforestation was occurring, and several data sources were examined to estimate the rate quantitatively (Levy and Milne, 2004).

### Methodological issues

Levy and Milne (2004) suggested the approach of combining Forestry Commission unconditional felling licence data for rural areas with Ordnance Survey data for non-rural areas, to reduce suspected biases and inconsistencies in the available data sources.  A mean deforestation rate of 1633 ha a-1 was estimated for the period 1990 to 1999 and the associated emissions for 1990 to 2002 in the GHG Inventory submitted in 2004 were each derived from this mean.  This approach has now been revised to provide annual figures for the period 1990 to 2003.  The Ordnance Survey data come from a continuous rolling survey programme, both on the ground and from aerial photography.  The changes reported each year may have actually occurred in any of the preceding 1-5 years (the survey frequency varies among areas, and can be up to 10 years for moorland/mountain areas).  Consequently, a three-year moving average was applied to the data to smooth out the between-year variation appropriately, to give a suitable estimate with annual resolution.  This is combined with the Forestry Commission annual unconditional felling licence data to give an estimate of total deforestation.  The felling licence data area mostly only available for England but from 1999 to 2001 they are also available for GB and these parallel series were used to scale up the annual total of deforestation for England to values for GB.

The annual area loss rate was used in the method described in the IPCC 1996 guidelines (IPCC, 1997a, b, c) to estimate emissions of CO2, CH4 and N2O.  Deforestation is not estimated for Northern Ireland, as no data are available.  Further details of the application of the method to the UK can be found in Annex 3, Section 3.7.2.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and gas.

A time series of emissions from this sector is only medium given that the two constituent data series are not both available for each year and the values for the period several years are partially derived from data in one region.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC procedures, which are discussed in Section 7.11.

### Source-specific recalculations

Emissions from this sector were not included in the UK GHGI prior to 2002.  In 2002, estimates of emissions of CO2, CO, CH4, and N2O were made based on a mean deforestation rate for 1990 to 2001.  Here, annual values are estimated for 1990 to 2002, as described in Annex 3, Section 3.7.2.

### Source-specific planned improvements

Future improvements of the method should include (i) collating Forestry Commission unconditional felling licence data for Scotland and Wales, and (ii) analysing possible causes for the high deforestation rates estimated by OS data for rural areas, which are currently considered too high to be realistic.

### Equivalent categories in the new CRF

The data on carbon stock change in biomass from this Category are entered into Sectoral Background Table 5.E.2.1 (Forest Land converted to Settlements) and emissions of CH4, N2O, NOx and CO into the Sectoral Report Table 5 in the CRF tables of the FCCC/SBSTA/2004/8 format.  It is assumed that all deforestation is for the purpose establishment of Settlements.  Carbon stock change in soils due to deforestation is dealt with in Category 5D below.  In Sectoral Report Table 5 in the FCCC/SBSTA/2004/8 format the Information item “Forest Land converted to other Land-Use Categories” includes both changes in carbon stock in biomass and soils under “Net CO2 emissions/removals”.

## Previous Source Category 5C, new CRF categories 5A to E – Abandonment of managed lands

This does not occur in the UK.

## Source Category 5D1 and 5D2 – emissions and removals from soils – cultivation of mineral & organic soils

### Source category description

In this category, changes in soil stocks due to land use change are estimated.  All forms of land use change except afforestation are considered together and both mineral and organic soils are included.  Removals to soils due to afforestation are considered separately using the forest carbon accounting model described in Section 7.2.  The net emissions due to land use change are reported in the previous CRF under Category 5D1 & 5D2 (CO2 Emissions and Removals from Soils – Cultivation of Mineral & Organic Soils), combined in this and earlier UK NIRs with other Emissions from soils.  For this NIR , emissions due to liming of agricultural land are reported separately.  Removals due to the effect of Set Aside on soils are now fully reflected in revised land use data and therefore no longer separately estimated or reported.

### Methodological issues

The basic method for assessing changes in soil carbon due to land use change is to link a matrix of change from surveys of land to a dynamic model of change of carbon.  Matrices from the Monitoring Landscape Change project for 1947 and 1980 and the ITE/CEH Countryside Surveys of 1984, 1990 and 1998 are used.  Land use in the UK was placed into 4 broad groups – Forestland, Grassland, Cropland, and Settlements by combining the more detailed categories for the two surveys.  Area change data exist for the period up to 1998 and those from 1990 to 1998 are used to extrapolate forward for the years 1999 to 2003.  The land use categories for this Report and Submission have been changed from those used previously to allow a closer match with the broad land type recommended in the IPCC LULUCF GPG (IPCC, 2003).

In Northern Ireland, less data are available to build matrices of land use change, but for 1990 to 1998 a matrix for the whole of Northern Ireland was available from the Northern Ireland Countryside Survey (Cooper and McCann, 2002).  The only data available pre-1990 for Northern Ireland is land use areas from The Agricultural Census and The Forest Service and processed by Cruickshank and Tomlinson (2000).  Matrices of land use change had then to be estimated for 1970-80 and 1980-90 using area data.  The basis of the method devised was to assume that the relationship between the matrix of land use transitions for 1990 to 1998 and the area data for 1990 is the same as the relationship between the matrix and area data for each of two earlier periods – 1970-79 and 1980-89.  The matrices developed by this approach were used to extrapolate areas of land use transition back to 1950 to match the start year in the rest of the UK.

A database of soil carbon density for the UK based on information on soil type, land cover and carbon content of soil cores has been available since about 1995.  These densities included carbon to a depth of 1 m or to bedrock, whichever was the shallower, for mineral and peaty/mineral soils.  Deep peat in the North of Scotland was identified separately and depths to 5 m are included.  For this Report a complete re-evaluation of the database was carried out (Bradley et al., In prep).  There are three soil survey groups covering the UK and the field data, and soil classifications and laboratory methods were harmonized to reduce uncertainty in the final data.  The depth of soil considered was also restricted to 1 m at maximum as part of this process.  This restriction may influence estimates of changes in soil carbon stocks, but carbon deeper than 1 m (in deep peat) should not be greatly affected by surface processes.  The availability of the new depth resolved data has been used to remove any effect on the calculations.

In the dynamic model of changes in soil carbon, the change is required in equilibrium carbon density from the initial to the final land use during a transition.  These are calculated for each land use category as averages for Scotland, England, Northern Ireland and Wales.

The rate of loss or gain of carbon is dependent on the type of land use transition.  For previous Reports, ranges of possible times for completion of different transitions were chosen from the literature, and a Monte Carlo approach was taken by varying the rate of change separately for England, Scotland and Wales.  Estimates for Northern Ireland were made separately.  In this Report, the Monte Carlo approach has been extended to include the area activity data and to the values for changes in soil carbon from equilibrium under the initial land use to equilibrium under a new land use, and to use the same methodology for Northern Ireland.  The mean carbon flux for each region resulting from these imposed random choices was then reported as the estimate for the Inventory.  A detailed description of the method be found in Annex 3, Section 3.3.7.  An adjustment was made to these calculations for each country to remove increases in soil carbon due to afforestation, as a better value for this is found from the C-Flow model used for the Changes in Forests and Other Woody Biomass Stocks Category (See Section 7.2.2).

Prior to the availability of land use change data for the period after 1990, an additional and separate estimate was made of the effect of Set Aside policy of soil carbon stock because this mechanism only became common after that date.  The new land use data should include this effect; therefore the separate Set Aside data is no longer reported.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and gas.

Land use change activity data are obtained from several sources.  The periods 1947-1980 and from 1984-1998 for Great Britain have separate good internal consistency, but there is poorer consistency between these periods and with the data for Northern Ireland.  There may be carry-over effects on emission/removal estimates for the reported years due to the long time response of soil systems.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC procedures, which are discussed in Section 7.11.

### Source-specific recalculations

The estimated emission for each year from 1990 to 2002 in the NIR/CRF has been recalculated following the methodological revision.  This has resulted in a reduction of net emissions in this category, compared to the 2002 inventory, of about 3% for 1990, but an increase of about 25% for 2002.

### Source-specific planned improvements

In the long term, the UK is planning to implement the use of a process-based model for estimating Emissions and Removals from soils.  This method is unlikely to be available for a few years, hence the enhancement of the existing approach.

### Source-specific planned improvements

The data on carbon stock change in soils from this Category are entered into Sectoral Background Table 5.B.2 (Forest Land converted to Cropland), 5.C.2 (Forest Land converted to Grassland) and 5.E.2 (Forest Land converted to Settlements) in the CRF tables of the FCCC/SBSTA/2004/8 format.  The data are reported as aggregate values for all land converted to Cropland, Grassland and Settlements i.e. they are not disaggregated by the original land category.  The aggregate data are however disaggregated by a) the four geographical areas of England, Scotland, Wales and Northern Ireland and b) by changes that occurred in the two time periods, 1950 to 1979 and 1980 onwards.  Soil carbon stock changes due to deforestation were identified within the aggregate data in 5.E.2 (Forest Land converted to Settlements) are included with carbon stock changes in biomass from 5.E.2.1 to provide the basis for “Net CO2 emissions/removals” in the Information item “Forest Land converted to other Land-Use Categories” in Sectoral Report Table 5 in the FCCC/SBSTA/2004/8 format.

## Previous Source Category 5D3, new CRF categories 5B and 5C – CO2 emissions and removals from soils – liming of agricultural soils

### Source category description

Emissions of carbon dioxide from the application of limestone, chalk and dolomite to agricultural soils were estimated using the method described in the IPCC 1996 Guidelines (IPCC, 1997a, b, c).  Data on the use of limestone, chalk and dolomite for agricultural purposes is reported in BGS (2004).  They also include ‘material for calcination’.  In agriculture all three minerals are applied to the soil, and CO2 emissions, weight for weight, from limestone and chalk will be identical since they have the same chemical formula.  Dolomite, however, will have a slightly higher emission due to the presence of Mg.  Estimates of the individual materials had to be made this year as only their total was published because of commercial confidentiality rules for small quantities.  It is assumed that all the carbon contained in the materials applied is released in the year of use.

The emissions due to this activity are identified separately in the previous CRF under Category 5D4 (CO2 Emissions and Removals from Soils – Liming of Agricultural Soils) but in this UK NIR they are combined with emissions from soils due to land use change.

### Methodological issues

Further information on application of the method used and emission factors can be found in Annex 3, Section 3.7.4.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and gas.

Uncertainty in both the activity data and emission factor used for this source are judged to be low.  The main source of uncertainty in the estimates is the caused by non-publication of some data due to commercial restrictions although these are not judged to be very significant.  Time-series consistency is underpinned by continuity in data source.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC procedures, which are discussed in Section 7.11.

### Source-specific recalculations

An arithmetic error in the estimated emissions for 2002 reported in the CRF for 2002 (2004 NIR) has been corrected.  This correction reduces the emission for that year by 13 Gg CO2.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

### Equivalent categories in the new CRF

The emissions in this Category are entered into Sectoral Background Table 5 (IV) (Carbon emissions from agricultural lime application) in the CRF tables of the FCCC/SBSTA/2004/8 format.  The data in that table are disaggregated by application of limestone and dolomite separately on either Cropland or Grassland.

## Previous Source Category 5D4, New CRF category 5a – CO2 emissions and removals from soils – forest soils

### Source category description

Removals associated with increases in soil carbon under areas of the UK afforested since 1920 are estimated by the carbon accounting method described in Annex 3, Section A3.7.1.  In the previous CRF these removals to soil are identified under previous CRF Category 5D4 (Forest Soils), but in this and earlier UK NIRs they have been included in the 5A2 Category with changes in forest biomass stocks. This latter approach is consistent with that in the GPG for LULUCF and the new CRF.

### Equivalent categories in the new CRF

The emissions in this Category are entered into Sectoral Background Table 5.A.2 (Land converted to Forest Land) in the CRF tables of the FCCC/SBSTA/2004/8 format. The data in that table are disaggregated as described above for Source Category 5A.

## previous Source Category 5D5, new crf category 5b – CO2 emissions and removals from soils – Other

### Source category description

Two source and one sink activities are considered for this category.

#### Upland drainage

This source, due to the ploughing and drainage of deep peat for the purposes of establishment of new forests, is no longer reported.  Losses from deep peat afforestation are now estimated within the forest carbon accounting model as described in Section 7.2.5.

#### Lowland Drainage

Fenland areas of England were drained many decades ago for agriculture.  These areas are still emitting CO2.  This management activity is not modelled by the broad scale approach to land use change described in Section 7.5, and separate estimates of recent emissions have been included here.  The emissions due to this activity are reported in the CRF under Category 5D5 (CO2 Emissions and Removals - Other) but are reported in this and earlier UK NIRs under Category 5E (Other).

#### Set Aside

Various schemes for arable land to be set aside from agricultural production have been in place in the UK since 1990.  A separate estimate was made of the changes in stocks of soil carbon (a net sink) due to set aside of arable land in previous UK GHG Inventories because the land use change data available were extrapolated from data collected before 1990.  The effect of this activity is now estimated within the methods of Section 7.5 for assessing the effect of all land use change on soil carbon stocks.  This has become possible due to the availability of post-1990 land use change data.

### Methodological issues

A detailed description of the methods used can be found in Annex 3, Section 3.7.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and gas.

The emissions due to lowland drainage are obtained from as model driven by activity data from a single source that provides good time series consistency.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC procedures, which are discussed in Section 7.11.

### Source-specific recalculations

No recalculations required.

### Source-specific planned improvements

No planned improvements.

### Equivalent categories in the new CRF

The emissions in this Category (due to lowland drainage) are entered into Sectoral Background Table 5.B.1 (Cropland remaining Cropland) in the CRF tables of the FCCC/SBSTA/2004/8 format.

## previous Source Category 5E, new crf categories 5G and 5C – Other: Changes in non-forest Biomass, Peat Extraction

### Source category description

Two activities are included in this category.

#### Changes in non-forest biomass

This includes annual changes in the biomass of vegetation in the UK due to all land use change but excludes forests and woodland.  Much of this change involves changes to or from agricultural crops, hence the previous use of the term “crop biomass” for this activity.

#### Peat extraction

Peat is extracted in the UK for use as either a fuel or in horticulture.  Estimates are made separately for each of these end uses.  Peat is not a “renewable” fuel but has not been included in the Energy Sector of the UK Inventory.

### Methodological issues

A detailed description of the methods used and emission factors can be found in Annex 3, Section 3.7.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category.

Changes in non-forest biomass are reported as a constant average value in each year.  Activity data for peat extraction come from a number of sources, only some of which are reliable, which will have some effect on time series consistency.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC procedures, which are discussed in Section 7.11.

### Source-specific recalculations

No recalculations were required.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

### Equivalent GPG LULUCF format categories

In the CRF tables of the FCCC/SBSTA/2004/8 format a) the emissions in this Category due to peat extraction are entered into Sectoral Background Table 5.C.1 (Grassland remaining Grassland) and b) the removals in this Category due to carbon stock changes in non-forest biomass are entered into Sectoral Report Table in 5.G (Other) as “Changes in non-forest biomass”.

## Separate reporting of emissions and removals

The UK has also provided data for the entire forest sink together and non-forest emissions and removals from soils in a separate group.  This provides a broad separation of sinks and sources within the LUCF sector, Tables 7.2, 7.3, 7.4a and 7.4b show the activities concerned and how they have been combined in different ways.  The reported totals for emissions and removals for the LUCF Sector are not affected.

The UK has prepared the data in the format and tables described in FCCC/SBSTA/2004/8 as adopted at COP9 and based the GPG LULUCF (IPCC, 2003).  The Sectoral Report Tables for 1990 and 2003 in this new CRF format are included here as Tables A9.3.1 to A9.3.14.

Table 7.2	Categories used for reporting soils emissions and removals in National Inventory Report and UNFCCC Common Reporting Format

| Process | National Inventory Report | Common Reporting Format |
| --- | --- | --- |
| Removals to forest soils and litter | 5A2 (Removal) | 5D (Removal) |
| Emissions from soils due to lowland drainage | 5E (Emission) | 5D (Emission) |

Table 7.3	Emissions and removals of carbon dioxide by activities in Land Use Change and Forestry Sector.  The reporting categories used in the National Inventory Report and for the UNFCCC Common Reporting Format are also shown. (IE - Included Elsewhere.)

| Activity | Gg CO2 | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | NIR | CRF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperate forest | Removal | -6014 | -6486 | -6950 | -7215 | -7561 | -7245 | -7137 | -6927 | -6827 | -7171 | -6856 | -7776 | -8916 | -9808 | 5A2 | 5A2 |
| Harvested wood | Removal | -1587 | -1344 | -1130 | -1059 | -942 | -1123 | -1098 | -1195 | -1289 | -1161 | -1314 | -743 | -133 | 248 | 5A5 | 5A5 |
| Deforestation | Emission | 164 | 137 | 107 | 124 | 132 | 161 | 185 | 152 | 159 | 297 | 223 | 228 | 180 | 141 | 5B | 5B |
| Mineral soils | Emission | 13522 | 13326 | 13139 | 12961 | 12791 | 12630 | 12475 | 12327 | 12186 | 12050 | 11922 | 11798 | 11679 | 11565 | 5D | 5D |
| Mineral soils | Removal | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | 5D | 5D |
| Organic soils | Emission | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | 5D | 5D |
| Organic soils | Removal | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | 5D | 5D |
| Liming | Emission | 1430 | 1772 | 1810 | 1130 | 1270 | 1529 | 1515 | 1346 | 1058 | 887 | 794 | 725 | 739 | 918 | 5D | 5D |
| Forest soils | Removal | -6211 | -6131 | -6168 | -6263 | -6297 | -6483 | -6524 | -6601 | -6633 | -6494 | -6556 | -6233 | -5859 | -5610 | 5A2 | 5D |
| Lowland Drainage | Emission | 1650 | 1613 | 1577 | 1540 | 1503 | 1467 | 1430 | 1393 | 1357 | 1320 | 1283 | 1261 | 1239 | 1217 | 5E | 5D |
| Peat extraction | Emission | 792 | 803 | 792 | 781 | 889 | 950 | 869 | 815 | 704 | 822 | 816 | 855 | 683 | 894 | 5E | 5E |
| Changes in non-forest biomass | Removal | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | 5E | 5E |

Table 7.4a	Emissions and removals in categories with the Land Use Change and Forestry Sector as reported in the format used for the National Inventory Report. (IE - Included Elsewhere.)

| NIR | Gg CO2 | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | Category |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperate forest | Removal | -12226 | -12616 | -13118 | -13478 | -13858 | -13727 | -13661 | -13527 | -13459 | -13665 | -13411 | -14009 | -14775 | -15418 | 5A2 | Sum of Removals due to Changes in Forest biomass, Forest litter & soils |
| Harvested wood | Removal | -1587 | -1344 | -1130 | -1059 | -942 | -1123 | -1098 | -1195 | -1289 | -1161 | -1314 | -743 | -133 | 248 | 5A5 | Removals to Harvested wood |
| Deforestation | Emission | 164 | 137 | 107 | 124 | 132 | 161 | 185 | 152 | 159 | 297 | 223 | 228 | 180 | 141 | 5B | Emissions (CO2) due to Deforestation |
| Soils | Emission | 14952 | 15098 | 14948 | 14091 | 14061 | 14159 | 13990 | 13674 | 13244 | 12937 | 12716 | 12522 | 12418 | 12482 | 5D | Sum of Emissions from soils and Removals to soils due to Land use change (not forestry), and Liming of agricultural land |
| Soils | Removal | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |  | 5D | Included in Emission |
| Other | Emission | 2442 | 2416 | 2368 | 2321 | 2392 | 2417 | 2299 | 2208 | 2060 | 2142 | 2099 | 2116 | 1922 | 2111 | 5E | Sum of Emissions from soils due to Lowland drainage and Peat extraction |
| Other | Removal | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | 5E | Removals due to changes in non-forest biomass |
| Total | Emission | 17558 | 17650 | 17424 | 16536 | 16585 | 16737 | 16474 | 16034 | 15463 | 15376 | 15038 | 14866 | 14520 | 14734 | 5 | Gross LUCF Emissions |
| Total | Removal | -14913 | -15061 | -15348 | -15637 | -15900 | -15950 | -15859 | -15823 | -15849 | -15926 | -15826 | -15852 | -16008 | -16270 | 5 | Gross LUCF Removals |
| Total | Net | 2645 | 2590 | 2076 | 899 | 685 | 787 | 616 | 211 | -385 | -550 | -787 | -986 | -1489 | -1536 | 5 | Net LUCF Emissions |

Table 7.4b	Emissions and removals in categories with the Land Use Change and Forestry Sector as reported in the format used for the UNFCCC Common Reporting Format. (IE - Included Elsewhere.)

| CRF | Gg CO2 | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | Category |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperate forest | Removal | -6014 | -6486 | -6950 | -7215 | -7561 | -7245 | -7137 | -6927 | -6827 | -7171 | -6856 | -7776 | -8916 | -9808 | 5A2 | Removals due to Changes in forest biomass. |
| Harvested wood | Removal | -1587 | -1344 | -1130 | -1059 | -942 | -1123 | -1098 | -1195 | -1289 | -1161 | -1314 | -743 | -133 | 248 | 5A5 | Removals to Harvested wood |
| Deforestation | Emission | 164 | 137 | 107 | 124 | 132 | 161 | 185 | 152 | 159 | 297 | 223 | 228 | 180 | 141 | 5B | Emissions (CO2) due to Deforestation |
| Soils | Emission | 16602 | 16711 | 16525 | 15631 | 15565 | 15626 | 15420 | 15067 | 14601 | 14257 | 13999 | 13784 | 13657 | 13700 | 5D | Sum of Emissions from soils due to Land use change on agricultural soils (net emissions), Lowland drainage and liming of agricultural land |
| Soils | Removal | -6211 | -6131 | -6168 | -6263 | -6297 | -6483 | -6524 | -6601 | -6633 | -6494 | -6556 | -6233 | -5859 | -5610 | 5D | Removals to Forest litter & soils. |
| Other | Emission | 792 | 803 | 792 | 781 | 889 | 950 | 869 | 815 | 704 | 822 | 816 | 855 | 683 | 894 | 5E | Emissions from soils due to Peat extraction |
| Other | Removal | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | -1100 | 5E | Removals due to changes in non-forest biomass |
| Total | Emission | 17558 | 17650 | 17424 | 16536 | 16585 | 16737 | 16474 | 16034 | 15463 | 15376 | 15038 | 14866 | 14520 | 14734 | 5 | Gross LUCF Emissions |
| Total | Removal | -14913 | -15061 | -15348 | -15637 | -15900 | -15950 | -15859 | -15823 | -15849 | -15926 | -15826 | -15852 | -16008 | -16270 | 5 | Gross LUCF Removals |
| Total | Net | 2645 | 2590 | 2076 | 899 | 685 | 787 | 616 | 211 | -385 | -550 | -787 | -986 | -1489 | -1536 | 5 | Net LUCF Emissions |

## general comments on QA/QC

CEH has put in place high quality assurance standards, and selects subcontractors from professional organisations who meet those standards.  The general standards are:

- The use of professionally qualified staff

- The application of rigorous quality control procedures

- The use of modern equipment

- The use of validated methods

- The quality control and curation of databases

- The establishment of management procedures to ensure compliance

The particular quality control measures relevant to this report are as follows:

Databases

The databases used to calculate carbon sources and sinks are all quality controlled at source by the responsible organisation, e.g. CEH for land use, NSRI and MI for soils and FC for forestry statistics.

Models

All modelling is done by trained staff who now have many years’ experience of simulating changes in soil and biomass carbon.  The output of models is checked against quality assured data.  Predictions of future sources and sinks are bench marked against predictions made by other researchers in Europe through COST E21, other research meetings and the scientific literature.

Output

The integrity of results, the quality of the reports, the relationship to contracted deliverables and the punctuality of reporting are all subject to management vetting and tracking within CEH, through the Heads of Sections, Directors of Sites and the Finance Administration.  Additionally, all staff are required to publish as much non-confidential scientific information in the peer reviewed scientific literature, with the prior approval of the customer and customer acknowledgement.

Field measurements of sources and sinks

All fieldwork, sampling and data handling is carried out by experienced and trained staff to defined protocols agreed to meet the objectives of the work.  The procedures for flux measurement are fully documented and instruments are calibrated directly with primary standards.

Chemical analysis

All chemical analyses are made at CEH Lancaster and are supported by full quality assurance and control procedures under BS 5750.  The integrity of results is checked by conducting bi-monthly inter-laboratory comparisons (Aquachecks and the International Soil Exchange Scheme).

# Waste (CRF sector 6)

## Overview of Sector

Emissions of GHGs from this sector occur from the disposal of waste.  Solid wastes can be disposed of through landfilling, recycling, composting, incineration or waste-to-energy.  The most important GHG produced in this sector is methane.  In addition to methane, solid waste disposal sites can produce substantial amounts of CO2 and NMVOCs.  The primary source of CO2 release from waste is the decomposition of organic matter derived from biomass sources (e.g. crops and forests).  These biomass sources are re-grown annually, and so CO2 emissions are not treated as net emissions from waste in the IPCC methodology.  Emissions of CO2 derived from fossil carbon by waste incineration are covered, together with other greenhouse gases, under category 6C (Waste incineration) or 1A if there is energy recovery.

Wastewater treatment produces methane and N2O.  Emissions of NMVOCs are covered under secotr 6B.

## Source Category 6A – Solid Waste Disposal On land

### Source category description

Methane (CH4) is emitted during the anaerobic decomposition of organic waste disposed of in solid waste disposal sites (SWDS).  Organic waste decomposes at a diminishing rate, but takes many years to decompose completely.

The NAEI category Landfill maps directly on to IPCC category 6A1 Landfills (managed waste disposal on land) for methane emissions.  Emissions are reported from managed landfills only, as open dumps and unmanaged landfills (unmanaged waste disposal sites) are considered insignificant sources in the UK.

Neither the GHGI nor the NAEI reports carbon dioxide emissions from the anaerobic decay of landfilled waste since this is considered to be part of the carbon cycle and is not a net source.

### Methodological issues

The AEA Technology model of methane generation from landfill sites, used until 2002 (Brown et al, 1999), was updated and revised for Defra by the consultants Land Quality Management (LQM, 2003).

The UK method uses a first order decay (Tier 2) methodology based on estimates and historical data on waste quantities, composition and disposal practices over several decades.

The UK method is based on equations 4 and 5 in the Revised 1996 IPCC guidelines (IPCC, 1997) (pp 6.10-6.11), which are compatible with equations 5.1 and 5.2 in the Good Practice Guidance (IPCC, 2000).  A slightly different version of equation 5.1 is used, which takes into account the fact that the model uses a finite time interval (one year).  The full derivation of the equations used is given in Appendix 2 of Brown et al. (1999).

The UK method divides the waste stream into four categories of waste: rapidly degrading, moderately degrading, slowly degrading and inert waste.  These categories each have a separate decay rate.  The decay rates have been revised slightly for the 2002 version of the model (LQM, 2003). They now range from 0.046 (slowly degrading waste) to 0.116 (rapidly degrading waste), within the range of 0.030 to 0.200 quoted in the Good Practice Guidance.

The model uses data extending back to 1945, which gives a time period of around 4 half lives for the slowest of the three decay rates (0.05, half life 14 years).  This lies within the range of 3 to 5 half-lives recommended by the Good Practice Guidance.

As recommended, the model attempts to take into account changes in landfill practice over past decades by modifying the gas collection rate over time where appropriate.

The estimates of historical waste disposal and composition data are based on various data sources, described fully in Brown et al (1999) and LQM (2003).  As recommended in the Good Practice Guidance, estimates for municipal waste are based on population where data are absent. Up until 1994 the waste arisings data are the same as that used for the AEA model (Brown et al., 1999) and is based on waste surveys in the UK using actual data.  After 1994, data are based on a new study carried out by a UK consultancy ERM for input to the LQM model, which uses updated waste survey data gathered by the Environment Agency for 1999.  Years between 1995 and 1998 inclusive are extrapolated backwards from the 1999 data and years ahead of 1999 are extrapolated based on a projected scenario of waste disposal.

As recommended in the Good Practice Guidance, the estimates of waste disposal quantities include commercial and industrial waste, demolition and construction waste and sewage sludge, as well as municipal waste.  For industrial and commercial waste, the data are based on national estimates from a recent survey.  The data were extrapolated to cover past years based on employment rates in the industries concerned.

All sites in the UK are managed, and therefore have a methane correction factor of 1.0.

Degradable organic carbon (DOC) was estimated assuming that the DOC arises solely from the cellulose and hemi-cellulose content of the waste.  The LQM report states that “Cellulose and hemicellulose are known to make up approximately 91% of the degradable fraction, whilst other potential degradable fractions which may have a small contribution (such as proteins and lipids) are ignored.”  The proportion of cellulose and hemi-cellulose in each waste component and the degradability of these fractions were based on a study by Barlaz et al. (1997).

Each waste component (paper, food, etc) was assigned a DOC value based on the cellulose and hemi-cellulose content. The component was then split into four fractions: rapidly degrading, moderately degrading, slowly degrading and inert, each of which was assigned the appropriate degradation rate.  For example, paper was assumed to be 25% moderately degrading and 75% slowly degrading.  The DOC value for both components was assumed to be equal to the percentage by weight of cellulose and hemi-cellulose multiplied by a factor of 72/162 (to account for the carbon content).  This was around 22% for household paper waste.

The fraction of degradable organic carbon dissimilated (DOCF) is also derived from an analysis of the laboratory study by Barlaz et al. (1997).  It varies from 35% (newspaper) to 98% (white office paper) depending on the particular component in the waste. The majority of the waste components are between 50% and 64% degradable.  The IPCC recommended range quoted in the guidance is 50-60%.

The fraction of CH4 in landfill gas is generally taken to be 50%, which is in line with the Guidance.  For old shallow sites it is taken to be 30% to reflect a higher degree of oxidation.

The fraction of methane recovered was derived from a survey of statistics on gas use for power generation, and a survey of installed flare capacity, assuming that flares operate at full capacity except for 15% downtime.  In 2002 the estimates were that 24% of generated methane was utilised and 45% was flared.  The estimates are not derived from metering data, as recommended by the Guidance, as such data were not readily available at the time of the study.

The oxidation factor is based on a model developed by LQM, which is parameterised using a survey of empirical field and laboratory observations of soil methane oxidation capacity.  This model incorporates a fissure fracture term, which has a default of 10% but is relatively insensitive within the range 1% (very low fissuring) to 30% (very high degree of fissuring).  It also represents the oxidation efficiency for the methane migrating through the intact cap, up to the oxidising capacity of the cap.  The oxidising efficiency has a default value of 75% but this can vary between 50% and 100% and not significantly deviate from the field validation estimates of Milton et al. (1997).  The combination of the fissure fracture and oxidation efficiency parameters give a higher overall methane oxidation factor (67.5%) compared to the IPCC default of 10%.  For the year 2002, for all currently operating landfill sites, all the methane which is not recovered and does not escape through fissures is assumed to be oxidised.  This is equivalent to a methane oxidation factor of 90% for those sites.  The principal reason for this high residual methane oxidising capacity is due to the high proportion of active gas management (flaring and utilisation) in the UK.  This removes from the surface emissions flux a substantial quantity of the methane, which would otherwise pass through the cap unoxidised (because the oxidising capacity of the cap would be exceeded).  The gas which does pass through the cap does so at a rate substantially less that than the methane oxidising capacity of the microbial populations in the cap, and so a significantly higher percentage oxidation can take place.  The value of the oxidation factor used in the model is therefore a direct function of the relatively high percentage of gas abstraction technology employed in the UK.

Recovered methane is subtracted before applying the oxidation factor.  This is in line with the Guidance.

The emissions of pollutants from the flare stacks were not estimated but those from electricity generation and heat generation were.  Emissions from electricity generation are considered under Power Stations and emissions from heat generation are included under Miscellaneous and are discussed in Appendix 1.

An estimate of NMVOC emissions from landfills was made using an emission factor of 0.01 t NMVOC/t methane produced which is equivalent to 5.65g NMVOC/m3 landfill gas (Passant, 1993).

Neither the GHGI nor the NAEI reports carbon dioxide emissions from the anaerobic decay of landfilled waste since this is considered to be part of the carbon cycle and not a net source.

The estimates include the contribution of sewage disposed of to landfill.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and gas.

The estimates for all years have been calculated from the LQM model and thus the methodology is consistent throughout the time series.  Estimates of waste composition and quantities have been taken from different sources – prior to 1995 they are from Brown et al. 1999, and after 1995 new estimates were made as part of the LQM study, based on more recent data.  This has led to some discontinuity between the two sets of estimates – there is an increase in estimated MSW arisings from households from 25 million tonnes in 1994 to 28 million tonnes in 1995, against a background trend of an annual increase of around 1 million tonnes before and after 1995.  Similarly this has led to a discontinuity in the estimates of industrial and commercial waste arising – from 108 million tonnes in 1995 to reach 169 million tonnes by 1999 (assuming a linear increase over this period), although the arisings are roughly constant in the years before 1995 and after 1999.

### Source-specific QA/QC and verification

There are many uncertainties inherent in estimating methane emissions from landfill sites.  The model is particularly sensitive to the values assumed for the degradable organic carbon (DOC) present in different fractions of waste, and the amount of this that is dissimilable, as well as to the oxidation factor.  Both of these parameters are poorly understood, and field and experimental observations exhibit wide variation, so uncertainties are inevitably high, and the uncertainty estimates in Annex 7 reflect this.

### Source-specific recalculations

The methodology is identical to that used in the 2002 GHG inventory, as estimates are still based on the LQM model using the same parameter set.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 6B – Wastewater Handling

### Source category description

Emissions from this category cover those released from wastewater handling.  Emissions are included for industrial, domestic and commercial wastewater.

Methane and nitrous oxide are produced from anaerobic decomposition of organic matter by bacteria in sewage facilities and from food processing and other industrial facilities during wastewater handling.  Nitrous oxide may also be released from wastewater handling and human waste.

### Methodological issues

The NAEI category Sewage is mapped on to the IPCC category 6B2 Domestic and Commercial Wastewater.  There is no estimate made of emissions from private wastewater treatment plants operated by companies prior to discharge to the public sewage system or rivers.  The NAEI estimate is based on the work of Hobson et al (1996) who estimated emissions of methane for the years 1990-95.  Subsequent years are extrapolated on the basis of population.  Sewage disposed to landfill is included in landfill emissions.

The methodology of the UK model differs in some respects from the IPCC default methodology.  The main differences are that it considers wastewater and sewage together rather than separately.  It also considers domestic, commercial and industrial wastewater together rather than separately.  Emissions are based on empirical emission factors derived from the literature expressed in kg CH4/tonne dry solids rather than the BOD default factors used by IPCC.  The model however complies with the IPCC Good Practice Guidance as a national model (IPCC, 2000).

The basic activity data are the throughput of sewage sludge through the public system.  The estimates are based on the UK population connected to the public sewers and estimates of the amount of sewage per head generated.  From 1995 onwards the per capita production is a projection (Hobson et al, 1996).  The main source of sewage activity data is the UK Sewage Survey (DOE, 1993).  Emissions are calculated by disaggregating the throughput of sewage into 14 different routes.  The routes consist of different treatment processes each with its own emission factor.  The treatment routes and emission factors are shown in Table 8.1.  The allocation of sludge to the treatment routes is reported for each year on the CRF tables attached to this report as a CD ROM.

The model accounts for recovery of methane and its subsequent utilization and flaring by estimating the proportion of anaerobic digester emissions that are recovered.

Table 8.1	Specific Methane Emission Factors for Sludge Handling (kg CH4/Mg dry solids, Hobson et al (1996))

| Sludge Handling System | Gravity Thickening1 | Long term storage | Anaerobic Digestion2 | Agricultural Land | Landfill |
| --- | --- | --- | --- | --- | --- |
| Anaerobic digestion to agriculture | 0.72 |  | 143 | 5 |  |
| Digestion, drying, agriculture | 0.72 |  | 143 | 5 |  |
| Raw sludge, dried to agriculture | 0.72 |  |  | 20 |  |
| Raw sludge, long term storage (3m), agriculture | 0.72 | 36 |  | 20 |  |
| Raw sludge, dewatered to cake, to agriculture | 0.72 |  |  | 20 |  |
| Digestion, to incinerator | 0.72 |  | 143 |  |  |
| Raw sludge, to incinerator | 0.72 |  |  |  |  |
| Digestion , to landfill | 0.72 |  | 143 |  | 0 |
| Compost, to agriculture | 0.72 |  |  | 5 |  |
| Lime raw sludge, to agriculture | 0.72 |  |  | 20 |  |
| Raw Sludge , to landfill | 0.72 |  |  |  | 0 |
| Digestion , to sea disposal | 0.72 |  | 143 |  |  |
| Raw sludge to sea disposal | 0.72 |  |  |  |  |
| Digestion to beneficial use (e.g. land reclamation) | 0.72 |  | 143 | 5 |  |

- An emission factor of 1 kg/tonne is used for gravity thickening.  Around 72% of sludge is gravity thickened hence an aggregate factor of 0.72 kg CH4/Mg is used.

- The factor refers to methane production, however it is assumed that 121.5 kg CH4/Mg is recovered or flared

Table 8.2	Time-Series of Methane Emission Factors for Emissions from Wastewater Handling, based on Population (kt CH4 / million people)

| Year | CH4 Emission (kt) | CH4 EF | Units |
| --- | --- | --- | --- |
| 1990 | 33.38 | 0.580 | Kt CH4 / million people |
| 1991 | 31.27 | 0.544 | Kt CH4 / million people |
| 1992 | 34.76 | 0.604 | Kt CH4 / million people |
| 1993 | 34.46 | 0.597 | Kt CH4 / million people |
| 1994 | 35.96 | 0.622 | Kt CH4 / million people |
| 1995 | 34.33 | 0.593 | Kt CH4 / million people |
| 1996 | 35.27 | 0.608 | Kt CH4 / million people |
| 1997 | 36.21 | 0.623 | Kt CH4 / million people |
| 1998 | 37.15 | 0.637 | Kt CH4 / million people |
| 1999 | 36.02 | 0.616 | Kt CH4 / million people |
| 2000 | 36.89 | 0.629 | Kt CH4 / million people |
| 2001 | 37.13 | 0.628 | Kt CH4 / million people |
| 2002 | 37.35 | 0.630 | Kt CH4 / million people |
| 2003 | 37.58 | 0.631 | Kt CH4 / million people |

Nitrous oxide emissions from the treatment of human sewage are based on the IPCC (1997c) default methodology.  The average protein consumption per person is based on the National Food Survey (Defra, 2004).  The food survey is based on household consumption of food and so may give a low estimate.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and gas.

The same methodology has been used to estimate emissions for all years, providing a good time series consistency.  The population data needed to estimate emissions are provided by the Office of National Statistics (ONS).  Time-series consistency of activity data is very good due to the continuity in data provided by the ONS.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

There have been no significant recalculations to historic emissions from this sector.

### Source-specific planned improvements

Emission factors and activity data will be kept under review.

## Source Category 6C – Waste Incineration

### Source category description

This source category covers the incineration of wastes, excluding waste-to-energy facilities.  For the UK, this means that all MSW incineration is excluded, being reported under CRF source category 1A instead.  Incineration of chemical wastes, clinical wastes, sewage sludge and animal carcasses is included here.  In-situ burning of agricultural waste e.g. crop residue burning is reported under category 4F.

There are approximately 70 plant incinerating chemical or clinical waste or sewage sludge and approximately 2600 animal carcass incinerators. Animal carcass incinerators are typically much smaller than the incinerators used to burn other forms of waste.

This source category also includes emissions from crematoria.

### Methodological issues

Emissions of carbon, CO, NOx, SO2, and VOC from chemical waste incinerators are estimated based on analysis of data reported to the Environment Agency's Pollution Inventory.  This only covers England and Wales and there may be some significant emissions from plant in Scotland and Northern Ireland but no emissions data are available from these plant.  Emissions data are not available for all pollutants for all sites and so some extrapolation of data from reporting sites to non-reporting sites has been done, using estimates of waste burnt at each site as a basis.  The gaps in reported data are usually for smaller plant and the extrapolation of data is unlikely to seriously reduce the quality of the estimates.

Emissions of CH4, CO, N2O, NOx, SO2 and VOC from sewage sludge incinerators are estimated from a combination of data reported to the Environment Agency's Pollution Inventory (2004), supplemented with the use of literature-based emission factors for those pollutants where the Pollution Inventory does not give information sufficient to derive estimates.  Emissions of NOx are estimated using Pollution Inventory data while emissions of all other direct and indirect greenhouse gases are estimated from literature-based emission factors.  The factor for N2O is the default factor given in the IPCC good practice guidance for UK sewage sludge incineration. Emission factors for other pollutants are taken from the EMEP/CORINAIR Emission Inventory Guidebook.  The quantity of waste burnt annually is estimated, these estimates being based on estimates given in the literature.

Emissions of carbon, CH4, CO, N2O, NOx, SO2, and VOC from clinical waste incinerators are estimated using literature-based emission factors.  The factor for carbon is the default factor given in the IPCC good practice guidance, while the factor for N2O is the default for UK MSW incineration given in the same source.  Emission factors for other pollutants are largely taken from the EMEP/CORINAIR Emission Inventory Guidebook.  The quantity of waste burnt annually is also estimated, these estimates being based on information given in literature sources.

Emission estimates for animal carcass incinerators are taken directly from a Defra-funded study (AEA Technology, 2002) and are based on emissions monitoring carried out at a cross section of incineration plant.  No activity data are available and so the emission estimates given in this report are assumed to apply for all years.

Emissions of CO, NOx, SO2 and VOC from crematoria are based on literature-based emission factors, expressed as emissions per corpse, and taken from the US EPA compilation of emission factors (2004).  Data on the annual number of cremations is available from the Cremation Society of Great Britain (2004), a group who promote and establish the practice of cremation.

### Uncertainties and time-series consistency

The Tier 1 uncertainty analysis in Annex 7, shown in Table A7.5a and Table A7.5b, provides estimates of uncertainty according to IPCC source category and gas.

### Source-specific QA/QC and verification

This source category is covered by the general QA/QC of the greenhouse gas inventory in Section 1.6.

### Source-specific recalculations

No recalculations have been required for this version of the inventory.

### Source-specific planned improvements

Emission estimates for chemical waste incineration currently do not include the burning of chemical wastes in flares or the burning of chemical wastes as fuels.  The estimates therefore underestimate actual emissions from combustion of chemical wastes.  Further development of the methodology should allow at least some of these omitted sources to be included although this will not completely address this issue.  The resulting underestimation is, however, likely to be a fairly trivial issue, given the relative insignificance of this source.  Other possible areas for improvement include activity data for clinical waste, animal carcass and sewage sludge incineration.

Improvements in the reporting of data from plant in Scotland (i.e. the ongoing development of an annual EPER database of process emissions by SEPA) and improved communications with the Department of Environment in Northern Ireland are anticipated to improve the coverage of industrial waste incineration plant from these constituent countries of the UK.

## Source Category 6D – Other

### Source category description

This category covers the release of greenhouse gas from other waste handling activities.

There are no emissions from this source category in the UK.

# Other (CRF sector 7)

There are no emissions from the UK reported in this sector.

# Recalculations and Improvements

This section of the report summarises the recalculations and improvements made to the UK GHG inventory since the 2004 NIR (2002 inventory) was issued.  It summarises material that has already been presented and discussed in more detail in Chapter 3 to Chapter 9.  Table 8(b) of the CRF for each year also contains a summary of the recalculations since the previous inventory was submitted.  For a quantitative discussion of emissions estimated in the 2003 GHG inventory, please see Annex 6.

Each year, the UK greenhouse inventory is:

- updated	existing activity data and/or emissions factors may be revised;

- extended	the inventory includes a new inventory year.

Updating often entails revision of emission estimates, most commonly because of revision to the core energy statistics presented in the Digest of UK Energy Statistics (DUKES).  The inventory also makes use of other datasets (see Table 1.3 for a summary), and these too may also be revised.  Updating will also reflect adoption of revised methodologies, as has happened with the estimates of emissions of fluorinated compounds in this submission.  Updating, particularly involving revised methodologies, may affect the whole time series, so estimates of emissions for a given year may differ from estimates of emissions for the same year reported previously.  Therefore comparisons between submissions should take account of whether there have been changes to:

- the methodology used to estimate emissions, or

- the base activity data.

There is normally a comment in the report to indicate where such changes have occurred.

The time series of the inventory is extended by including a new inventory year - for example, the previous report covered the years up to and including 2002; this report gives emission estimates for 2002, and includes estimates for the year 2003 also.

The inventory may also be expanded to include 	emissions from additional sources if a new source has been identified within the context of the IPCC Guidelines and Good Practice Guidance, and there are sufficient activity data and suitable emission factors.

## RECALCULATIONS, and Explanations and justifications for recalculations

Table 10.1 summarises the recalculations that have occurred in estimates of the direct GHGs since the 2004 NIR (2002 inventory) was issued.  It contains brief comments on the reasons behind the recalculations, and shows if a revision of the entire time series has occurred.  The changes in emissions are net changes (the sum of any increases and decreases) in the source category, for the year 2002.

Table8(a)s1 and Table8(a)s2 of the CRF also present details of recalculations of emissions between the current and the previous inventory.  The emissions are expressed as GWP emissions, but are not shown to the sectoral detail in Table 10.1.

The percentage change, due to recalculation with respect to the previous submission, has been calculated as

Percentage change = 100% x [(LS-PS)/PS]

Where

LS = Latest Submission (2003 inventory; 2005 NIR)

PS = Previous Submission (2002 inventory, 2004 NIR)

The percentages expressed in this way are consistent with those calculated in the CRF in Table8(a)s1 and Table8(a)s1.

For changes in earlier years data the corresponding CRF tables for that year should be referred to.

Table 10.1	Recalculations of direct GHG emissions for the year 2002 in the UK 2005 NIR (2003 inventory)

| Source category and GHG | Change in emissions (Gg)1 (Emissions in 2003 inventory minus emissions in 2002 inventory) | Change in emissions (%) (Percentage change relative to the 2002 inventory) | Brief description of reasons for recalculation |
| --- | --- | --- | --- |
| 1A1 |  |  |  |
| CO2 | +2554.3 | +4.8 | - Carbon emission factors for solid, liquid and gaseous fuels have been updated following a review of the CEFs in the UK GHG inventory. - Method improvement as a result of an on-going task to improve the accuracy of fuel use estimates for the cement industry, the lime industry, and the electricity supply industry. This necessarily affects estimates from other industrial combustion. This is therefore a re-allocation of emissions. - Changes in fuel use statistics for MSW, sour gas, OPG, natural gas, LPG, gas oil, fuel oil, naphtha and petroleum coke |
| CH4 | -0.4 | -0.9 | - Method of estimating emissions from iron and steel production improved. The new method updates the existing approach and is still based on a carbon balance for integrated steelwork processes, including the associated fuel transformation. - Revision in the emission factor for petroleum coke - Revision to fuel use statistics for fuel oil, MSW, Natural gas, sour gas, sewage gas, gas oil, naphtha, OPG, blast furnace gas and coke oven gas |
| N2O | +0.1 | +1.1 | - Revision to fuel statistics for fuel oil, MSW, Natural gas, sour gas, sewage gas, gas oil, petroleum coke |
| 1A2 |  |  |  |
| CO2 | -225.1 | -1.0 | - Method of estimating emissions from iron and steel production improved. The new method updates the existing approach and is still based on a carbon balance for integrated steelwork processes, including the associated fuel transformation. There has been a re-allocation of certain fuels which affects the emissions in 1A2a, 1A2f and 2C1. - Method improvement as a result of continuing task to improve the accuracy of fuel use estimates for the cement industry, the lime industry, and the electricity supply industry. This necessarily affects estimates from other industrial combustion. This is a re-allocation of fuel use and therefore emissions. - Improvement to model used to estimate emissions from industrial off-road mobile machinery - Carbon emission factors for solid, liquid and gaseous fuels updated following a review of the CEFs in the UK GHG inventory. - Revision to activity data for fuel oil, gas oil, natural gas, coal, petroleum coke and coke |
| CH4 | -0.05 | -0.5 | - Method of estimating emissions from iron and steel production improved. The new method updates the existing approach and is still based on a carbon balance for integrated steelwork processes, including the associated fuel transformation. There has been a re-allocation of certain fuels which affects the emissions in 1A2a, 1A2f and 2C1 - Method improvement as a result of continuing task to improve the accuracy of fuel use estimates for the cement industry, the lime industry, and the electricity supply industry. This necessarily affects estimates from other industrial combustion. This is a re-allocation of fuel use and therefore emissions. - Revision to emission factor for petroleum coke use in cement production - Emission factor revision for gas oil in 1A2fii - Revision to activity data for petroleum coke, natural gas, coal, waste oils, coked, fuel oil, gas oil, LPG, lubricants |
| N2O | -0.3 | -7.7 | - Method of estimating emissions from iron and steel production improved. The new method updates the existing approach and is still based on a carbon balance for integrated steelwork processes, including the associated fuel transformation. There has been a re-allocation of certain fuels which affects the emissions in 1A2a, 1A2f and 2C1. - Method improvement as a result of continuing task to improve the accuracy of fuel use estimates for the cement industry, the lime industry, and the electricity supply industry. This necessarily affects estimates from other industrial combustion. This is a re-allocation of fuel use and therefore emissions. |
| 1A3 |  |  |  |
| CO2 | +1914 | 1.6 | - Major improvement to the method used to estimate GHG emissions from domestic and international aviation - a model based on individual aircraft movements is now used, corresponding to a Tier 3 approach. The split between domestic and international emissions has changed - domestic emissions have declined, with a corresponding increase in emissions allocated to aviation bunkers. - Model to estimate emissions of GHGs from road transport revised. Improvements to the methodology include more detailed information on traffic speeds on different types of roads and information on the composition of the vehicle fleet. Now includes estimate of GHGs released when lubricating oil is burnt inside car engines. - Improvement to estimate of emissions from rail transport. Re-allocation of emissions associated with freight, intercity and regional rail movements. - Carbon emission factors for liquid and gaseous fuels updated following a review of the CEFs in the UK GHG inventory. - Carbon emission factor for aviation spirit revised. - Carbon factor for fuel oil and gas oil revised - Activity data revision |
| CH4 | -4.4 | -1.8 | - Changes in the emission factor and activity data for gas oil - Major improvement to the method used to estimate GHG emissions from domestic and international aviation - a model based on individual aircraft movements is now used, corresponding to a Tier 3 approach. - Model to estimate emissions of GHGs from road transport revised. Improvements to the methodology include more detailed information on traffic speeds on different types of roads and information on the composition of the vehicle fleet. Now includes estimate of GHGs released when lubricating oil is burnt inside car engines. - Minor revisions by DfT in the vehicle km data |
| N2O | 76.9 | 1.6 | - Model to estimate emissions of GHGs from road transport revised. Improvements to the methodology include more detailed information on traffic speeds on different types of roads and information on the composition of the vehicle fleet. Now includes estimate of GHGs released when lubricating oil is burnt inside car engines. - Major improvement to the method used to estimate GHG emissions from domestic and international aviation - a model based on individual aircraft movements is now used, corresponding to a Tier 3 approach. - Minor revisions by DfT in the vehicle km data - Activity data and emission factor revisions |
| 1A4 |  |  |  |
| CO2 | -305.9 | -1.0 | - Carbon emission factors for solid, liquid and gaseous fuels updated following a review of the CEFs in the UK GHG inventory. - Re-allocation of emissions from sector 1B1b_Solid_Fuel_Transformation to sector 1A4b_Residential. Previously, it was assumed that emissions from the combustion of petroleum coke occurred in the fuel transformation sector. - Revision to activity data |
| 1A5 |  |  |  |
| CO2 | +3.2 | +0.4 | - Carbon emission factors for solid, liquid and gaseous fuels updated following a review of the CEFs in the UK GHG inventory. |
| CH4 | -0.001 | -2.3 | - Ef revision to aviation turbine fuel |
| 1B1 |  |  |  |
| CO2 | -1891 | -94.4 | - Revision to emission factors for Coke Oven Gas (COG) following a review of the CEFs in the UK GHG inventory. |
| CH4 | 57.4 | 23.5 | - Addition of emission estimate for closed coal mines. - Method of estimating emissions from iron and steel production improved. The new method updates the existing approach and is still based on a carbon balance for integrated steelwork processes, including the associated fuel transformation. |
| 1B2 |  |  |  |
| CH4 | 49.8 | 0.6 | Methods of estimating emissions from onshore and offshore oil and gas sectors revised. |
| 2A3 |  |  |  |
| CO2 | 213 | New source | - Method to estimate emissions of CO2 from flue gas desulphurisation (FGD) introduced - emissions assigned to sector 2A3. |
| 2C1 |  |  |  |
| CO2 | -282.7 | -19.4 | - Method of estimating emissions from iron and steel production improved. The new method updates the existing approach and is still based on a carbon balance for integrated steelwork processes, including the associated fuel transformation. - Revision to emission factor for Blast Furnace Gas (BFG) following a review of the CEFs in the UK GHG inventory. - Revision to activity data used for iron and steel flaring |
| 4A1 |  |  |  |
| CH4 | -866 | -5.1 | - Emission factor revision affecting agriculture livestock - dairy cattle enteric |
| 4B1 |  |  |  |
| CH4 | 542 | 26.1 | - Emission factor revision affecting emissions from agriculture livestock - dairy cattle wastes |
| 5D |  |  |  |
| CO2 | -2708 | -25.8 | - Removals & Emissions associated with disturbance of soils at afforestation now included explicitly. "Upland drainage" reported previously now included within this new estimate. - Net emissions due to changes of stock in soils due to land use change other than afforestation have been extensively revised. "Set Aside" reported previously now included within this new estimate. - Afforestation areas revised. |
| 6A |  |  |  |
| CO2 | 550 | 113.2 | - Methodology change to now report CO2 emissions in sector 6A1_Managed_Waste_Disposal_on_Land from the disposal of wood treated with preservatives derived from benzols and tars . This change is a consequence of the revision to the method used to estimate emissions from the manufacture of iron & steel. In previous inventories emissions from the combustion of these compounds was included as part of the emission from coal use in coke ovens - however these compounds are not burnt in this sector. Emissions from the decomposition of these fossil fuel derived preservatives have now been allocated to sector 6A1. |
| 6C |  |  |  |
| CO2 | -5.6 | -1.1 | - emission factor and activity data revised |
| CH4 | 1.8 | 118.7 | - Addition of new category accounting for emissions from accidental vehicle fires. |

## Implications for emission levels

The implications for emission levels in the year 2002 are summarised by sector in Table 10.1, and the overall effect for individual years is shown in Figure 10.2 and Figure 10.3.

## Implications for emission trends, including time series consistency

The effects of the recalculations and improvements made in the 2003 inventory are summarised in this section in a series of charts.  The charts show the changes in the time series of emissions, or percentage changes in emissions, since the 2002 inventory.

Figure 10.1 summarises the effect of the recalculations in the 2004 NIR (2002 inventory) in terms of the time series of GWP emissions.  The chart shows the time series of differences in the annual GWP emissions of the basket of the 6 Kyoto GHGs between the inventories of 2002 and 2003, according to IPCC source sector.  A negative difference indicates a decline in GWP emission between the inventory presented in the 2005 NIR (2003 inventory), and the inventory presented in the 2004 NIR (2002 inventory).

Figure 10.2 summarises the effect of the recalculations in the 2005 NIR in terms of the percentage changes in the time series of GWP emissions.  The chart shows the time series of percentage changes in the annual GWP emissions of the basket of the 6 Kyoto GHGs between the inventories of 2002 and 2003, according to IPCC source sector.

The percentage change, due to recalculation with respect to the previous submission, has been calculated as

Percentage change = 100% x [(LS-PS)/PS]

Where

LS = Latest Submission (2003 inventory; 2005 NIR)

PS = Previous Submission (2002 inventory, 2004 NIR)

The percentages expressed in this way are consistent with those calculated in the CRF in Table8(a)s1 and Table8(a)s1.

Figure 10.3 summarises the effect of the recalculations in the 2005 NIR in terms of

- changes in the time series of total net UK GWP emissions (sum of emissions and removals), and,

- percentage changes in the time series of GWP emissions.

The chart shows the time series of changes in the basket of the 6 Kyoto GHGs between the inventories of 2002 and 2003.

The method used to calculate the percentage change is given above.

The current inventory incorporates a number of important revisions to both the methods used to estimate GHG emissions, and the emission factors used in these methods.  The changes in the time series of GWP emissions in Figure 10.3 reflect these enhancements.  There is no single factor that is responsible for the changes seen in GWP emissions in 1990 and 2002 and summary of the key reasons for the changes are given below.  More detailed information is given in the sections describing the source-specific recalculations given in Chapters 3 to 8.

Reasons for changes in GWP emissions in 1990

- The changes in GWP emissions are dominated by the changes in emissions of carbon.

- Following a review of the carbon emission factors in the inventory, many of the factors have been revised.  The net result of these changes was to increase the total emissions of CO2.

- Revised methods have been used to estimate emissions from LUCF and this has affected both estimates of emissions and removals.  Data on rates of change of land use have been improved in both quality and consistency.  Data on soil carbon density has been extensively revised to improve quality and treatment of variation with depth.  Net emissions due to disturbance of soil for afforestation is now based on field measurements, hence removals to soil carbon in conifer forest are now better estimated.

- The method to used estimate emissions from the consumption of waste lubricants has been revised, and emissions from the use of this fuel have increased.

- The method used to estimate emissions from civil aviation has been improved to correspond to an IPCC Tier 3 approach.  The method has improved the estimate of the allocation of fuels consumed by domestic and international aviation, and incorporates revised emission factors also.  The effect of these changes is that emissions allocated to domestic aviation have declined and those allocated to international aviation (bunkers) have increased.

- The use of petcoke in certain domestic fuels has now been included in the current inventory.  This was previously omitted.

- Estimates of emissions of methane from closed coal mines are now included in the inventory.

Reasons for changes in GWP emissions in 2002

- The main reasons for the changes in 2002 are as given above for 1990, with the addition of the points below.

- The changes in GWP emissions are dominated by the changes in emissions of carbon.

- There were a number of significant changes to the underlying activity data (energy statistics).

- Emissions from flue gas desulphurisation (FGD) were not previously considered in the inventory.  They are now significant enough to include, and likely to increase as new power stations in the UK are fitted with FGD.

Figure 10.1	Time series of changes in GWP emissions between the inventory presented in the current and the previous NIR, according to IPCC source sector

Figure 10.2	Time series of percentage changes in GWP emissions between the inventory presented in the current and the previous NIR, according to IPCC source sector

Figure 10.3	Time series of changes in total net GWP emissions, and percentage changes in total net GWP emissions, between the inventory presented in the current and the previous NIR

## Recalculations

This section gives details of recalculations, including the response to the UNFCCC review process, and planned improvements to the inventory.

### Recalculations

Recalculations are summarised in Section 10.1 to Section 10.3.  Details of recalculations are given in Sections 3 to Sections 9, and are also shown in CRF Table8(a)s2 and Table8(b) for each year.

### Response to the review process

The UNFCCC secretariat conducted a Centralised Review of the UK's 2004 National Inventory Report (Bonn, 18 to 22 October 2004).  This was the Third Centralised Review of the UK greenhouse gas inventory (FCCC ERT, 2005).  This review follows on from the Second Centralised Review of the 2003 NIR (Bonn, 15-19 September 2003).

Many of the changes suggested in the Third Centralised Review have already been incorporated within this NIR.  A few of the tasks suggested by the Expert Review Team (ERT) in the Second Centralised Review are being carried over to 2006.

Table 10.2 provides an overview of the actions taken to improve the NIR and the inventory in response to the comments made by the ERTs.  Some comments have been carried over from the second Centralised Review of 2003, because of the time needed for the major improvements which have been incorporated in the current inventory.

Table 10.2	Brief details of improvements to the NIR and the inventory in response to FCCC reviews

| ERT comment | Actions |
| --- | --- |
| Second Centralised Review |  |
| Further explanation requested about emissions from aviation bunkers | There has been a complete revision of the method used to estimate emissions from domestic and international civil aviation. Description of the method is given in Annex 3. |
| Further explanation requested about feedstocks and non-energy use of fuels | Further information is being prepared, and will be included in the 2006 NIR. |
| Negative emissions used in Iron and Steel Production | There has been a complete revision of method to estimate emissions from the iron and steel sector and associated fuel transformation. The previous method contained negative emission factors and these were used as artificial devices to ensure that carbon balances for processes in steelworks balanced correctly. These negative emission factors are no longer necessary in the current method. |
| Third Centralised Review |  |
| Improve documentation of method & results of key source analysis | Two additional tables of information have been included to support the Tier 1 uncertainty analysis: the key source analysis based on level of emissions, and the key source analysis based on trend in emissions. The Tier 2 analysis has now been refined to present estimated uncertainties at the sectoral level as well as by gas. |
| Recommendation to strengthen NIR text on recalculations, include comments on changes to time series | Extended information has been included in this NIR. |
| Include in NIR description of how AD and emissions data reported by companies are verified (e.g. description of QA of Pollution Inventory data) | Additional information has been included in this NIR. A programme of meetings involving the Key Data Providers, UK Defra and the Inventory Agency continues. The aim of these meetings is understand QA/QC activities in place and suggest improvements where necessary. |
| Present a time series of carbon emission factors and GCVs. | A time series of carbon emission factors and GCVs is included as background information to this NIR, in a spreadsheet, on the accompanying CD. |
| Solid fuel transformation - check that there is no double-count between 1B1b and 1A1c | The method used to estimate emissions from fuel transformation has been revised this year. |
| Inconsistent reporting of CH4 emissions from venting in offshore oil installations. | Previous reporting of emissions was limited by the data available. The method used to estimate emissions from the offshore and onshore oil industry has been revised this year, and a time series of estimates of emissions from venting is now presented. |
| Observation regarding variability of implied emission factors of N2O from nitric and adipic acid production. | Information has been provided in this NIR to discuss the reasons for this variability. |
| Comment on lime production - data based on assumption that all lime is quicklime and that calcination of dolomite is minimal - review, confirm, improve text in NIR - provide documented evidence of assumptions. | This action will be carried forward to the 2006 NIR. Our best information at present is that the use of dolomite is small or trivial. |
| CH4 from waste-water handling – information requested on model validation and assumptions, and emission factors. | Time series of emission factors have now been included in this NIR. |

Sector specific improvements are identified in Chapters 3 to 9, and the main actions currently underway are:

- A review of the completeness of the UK GHG inventory prior to the 2006 NIR – the UK Assigned Amount will be allocated on the emissions presented in this NIR.

- A review of the emissions from stored carbon and the feedstock use of fuels.

- A review of the allocation of emissions to IPCC sectors.

### Major improvements to the current inventory

A number of major improvements have been made to this inventory.  Very briefly, these are:

- Review and revision of a number of carbon emission factors;

- Improved methods to estimate emissions from LUCF;

- Improved method to estimate emissions from civil aviation;

- Two new sources have been included: emissions from flue gas desulphurisation and closed coal mines;

- Re-assessment of the consumption of waste lubricants and the use of petcoke in selected domestic fuels.

# References

References for the main chapters and the annexes are listed here and are organised by chapter and annex.

## Chapter 1 [Introduction]

AEAT (2003) Haydock, H et al. Emissions and Projections of HFCs, PFCs and SF6 for the UK and Constituent Countries (reference EPG 1/1/128).  Study by AEA Technology Environment on behalf of Global Atmosphere Division (GAD) of the Department for UK Environment, Food and Rural Affairs (Defra).

Cm2427 (1994) Climate Change the UK Programme.  UK’s (first) report under the Framework Convention on Climate Change.  HMSO, London.

Cm3558 (1997) Climate Change the UK Programme.  UK’s (second) report under the Framework Convention on Climate Change.  HMSO, London.

Cm4913 (2000) Se 2000/209; NIA 19/00.  Climate Change the UK Programme.  UK’s (second) report under the Framework Convention on Climate Change.  HMSO, London.

Defra (2001) The UK’s Third National Communication under the United Nations Framework Convention on Climate Change.  Defra publications, Admail 6000, London, SW1A 2XX.

DTI (2003), Energy White Paper, London, The Stationery Office.

DTI (2004), Digest of United Kingdom Energy Statistics 2004, London, The Stationery Office.

FCCC (1992) Report of the Intergovernmental Negotiating Committee for a Framework Convention on Climate Change on the work of the second part of its fifth session, held at New York from 30 April to 9 May 1992.

IPCC (1996) Climate Change 1995 – The science of climate change.  Contribution of Working Group 1 to the Second Assessment Report of the Intergovernmental Panel on Climate Change, Ed. Houghton, JT., Cambridge University Press.

IPCC (1997a), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 1, Greenhouse Gas Inventory Reporting Instructions, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC (1997b), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 2, Greenhouse Gas Inventory Workbook, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC (1997c), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC (2000), Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories, ed. Penman, J, Kruger, D, Galbally, I, et al, IPCC National Greenhouse Gas Inventories Programme, Technical Support Programme Technical Support Unit, Institute for Global Environmental Strategies, Hayama, Kanagawa, Japan.

IPCC (2001), Climate change 2001 – The scientific basis.  Contribution of Working Group 1 to the Third Assessment Report of the Intergovernmental Panel on Climate Change, Cambridge University Press.

Kershaw (2005), “Development of a Methodology for Estimating Methane Emissions from Abandoned Coal Mines in the UK”, White Young Green report to Defra, 2005.

Salway, AG (2001), Accordance of UK Greenhouse Gas Inventory with IPCC Good Practice Guidance, Note prepared for DETR, NETCEN, AEA Technology, Culham.

Simmons, T (2002), Peer Review of the UK Greenhouse Gas Inventory CO2 from Fuel Combustion 1990 - 1999. Avonlog Ltd, 49 Headley Chase, Brentwood, Essex, CM14 5DH.

## Chapter 3 [Energy (CRF sector 1)]

DTI (2004), Digest of United Kingdom Energy Statistics 2004, London, The Stationery Office.

DTI (2004a), Department of Trade and Industry, Personal Correspondence (Clive Evans, Martin Young, Ian M Corrie)

EMEP/CORINAIR (2003), Atmospheric Emission Inventory Guidebook, 3rd Edition.

SEPA (2004), EPER Inventory of industrial point source emissions, Scottish Environmental Protection Agency, Personal Communication (Bob Boyce).

UKPIA (2004), Refinery production & emissions data, carbon emission factors, petroleum products data from the United Kingdom Petroleum Industry Association, Personal Communication (Ian McPherson, & Malcolm Watson)

Rypdal et al. (2000), Kristin Rypdal and Ketil Flugsrud (Statistics Norway), and William Irving (USEPA). Good Practice – Methodological Choice, Good Practice – Methodological Choice – background paper.

US EPA (2004), Compilation of Air Pollutant Emission Factors, AP-42, www.epa.gov, 2004

## Chapter 4 [Industrial Processes (CRF sector 2)]

British Cement Association (2004), UK cement clinker production data, Personal Communication (Richard Leese).

British Geological Survey (2004), UK Minerals Yearbook 2003

British Glass (2004), UK glass production and recycling data in 2003, Personal Communication (Rebecca Cocking).

CIS (1991), Chem-Facts UK, Chemical Intelligence Services, Reed Telepublishing, Dunstable, Beds, UK.

CORINAIR (1989), CORINAIR Inventory, Commission of the European Community, Default Emission Factors Handbook, 1st edition, CITEPA, Paris.

DTI (2004), Digest of United Kingdom Energy Statistics 2004, London, The Stationery Office.

Environment Agency (2004), Pollution Inventory database 2004, Personal Communication (EA PI team: Charlie Corbishley, Claudia Towner and Alex Wills)

Invista (2004), 2003 Plant-specific emissions and abatement details, Personal communication (Graham Hinton).

IPCC (1997), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC (2000), Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories, ed. Penman, J, Kruger, D, Galbally, I, et al, IPCC National Greenhouse Gas Inventories Programme, Technical Support Unit, Institute for Global Environmental Strategies, Hayama, Kanagawa, Japan.

Iron & Steel Statistics Bureau (2004), Iron and Steel Industry, Annual Statistics for the UK, 2004.  ISSB Limited.

Munday, PK (1990) UK Emissions of Air Pollutants 1970-1988, Warren Spring Laboratory, LR 764.

Netcen (2004), UK Review of Carbon Emission Factors, Baggott & Watterson 2004.

Office of National Statistics (2004), Index of Production, Population Statistics and Prodcom data for 1990-2003, www.statistics.gov.uk

Refined Bitumen Association (1990), Personal Communication.

Rudd, HJ, Mikkelsen, M, (1996), Emissions of Volatile Organic Compounds from Ship Purging in the UK, AEA Technology Plc, Report No AEAT/20034001/REMA-208/Issue 2.

Scotch Whisky Association (2004), Statistical Report 2003.

TSO (2004), Annual Abstract of Statistics, 2004.

Transport and Road Research Laboratory (1989), Personal Communication, 1989.

US EPA (2004), United States Environmental Protection Agency, Compilation of Air Pollutant Emission Factors. Vol 1, 5th Edition, AP-42, North Carolina, Internet version, 2004.

## Chapter 4 [Industrial Processes (CRF sector 2 - F-gases)]

AEAT (2004)  Emissions and Projections of HFCs, PFCs and SF6 for the UK and Constituent Countries.  Final Report prepared for the Department for Environment, Food and Rural Affairs, 2nd Edition, June 2004, AEA Technology, Culham, Oxfordshire, UK.

Harnisch, J & Schwarz, W (2003), Costs and the impact on emissions of potential regulatory framework for reducing emissions of hydrofluorocarbons, perfluorocarbons and sulphur hexafluoride.  Prepared on behalf of EC (DG ENV).

IPCC (2000), Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories.  Penman, J, Kruger, D, Galbally, I, Hiraishi, T, Nyenzi, B, Emmanuel, S, Buendia, L, Hoppaus, R, Martinsen, T, Meijer, J, Miwa, K and Tanabe, K (Eds). IPCC National Greenhouse Gas Inventories Programme Published for the IPCC by the Institute for Global Environmental Strategies, Japan. ISBN 4-88788-000-6.

March (1999), UK Emissions of HFCs, PFCs and SF6 and Potential Emission Reduction Options, March Consulting Group.

MTP (2002), UK Market Transformation Programme, Department for Environment, Food and Rural Affairs. www.mtprog.com

UNEP (1998), UNEP Halon Technical Options Committee (HTOC) report.

WS Atkins (2000), Projections of Non-CO2 Greenhouse Gas Emissions for the UK and Constituent Countries. WS Atkins Environment.

## Chapter 5 [Solvent and Other Product Use (CRF sector 3)]

British Coatings Federation (2004), UK production data, Personal Communication.

British Rubber Manufacturers Association (2001), Personal Communication, September 2001

Metal Packing Manufacturers Association (MPMA) (2000), Personal Communication.

Office of National Statistics (2004), Personal Communication, 2004.

## Chapter 6 [Agriculture (CRF sector 4)]

ADAS (1995a), Personal communication to A Croxford (MAFF).  Distribution of animal waste management systems for cattle.

ADAS (1995b), Personal communication to A Croxford (MAFF).  Linseed burning data.

Brewers Licensed Retail Association (1998), Personal communication to R Gerry (MAFF).  Estimate of dry matter content of hops.

BSFP (2004).  British Survey of Fertiliser Practice: Fertiliser Use on Farm Crops for Crop Year 2003, The BSFP Authority, Peterborough.  Data for preceding years comes from earlier versions of the same publication.

Burton (1982), Post-Harvest Physiology of Crops, Longman, London, ISBN 0-582-46038-7.

Cottrill, B (ADAS) Personal Communication

Defra (2002), Personal communications from M Rose, Land Management Improvement Division.

Defra (2004a), Agriculture in the UK 2003, The Stationery Office, London.

Defra (2004b), Basic Horticultural Statistics for the United Kingdom: Calendar and Crop Years 1991/92 – 2001/02, The Stationery Office, London.

IPCC (1997), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC (2000), Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories.

Lord, E (1997), Personal communication to S Gleave (MAFF).  Factor for biological nitrogen fixation by improved grass.

MAFF (1995), Straw Disposal Survey.  Data for earlier years taken from previous editions of the survey.  Contact: Government Buildings, Epsom Rd., Guildford, Surrey, GU1 2LD for copies.

MAFF (1997), Farm Incomes in the UK 1995/96, The Stationery Office, London, ISBN 0-11-2543026-0.

Nix, J (1997), Farm Management Pocket Book 1998, 28th ed., Wye College Press, Ashford, UK.

PGRE (1998), Personal communication to R Gerry (MAFF).  Estimate of dry matter content of field beans.

Smith, K & Frost, J (2000), Nitrogen excretion by farm livestock with respect to land spreading requirements and controlling nitrogen losses to ground and surface waters.  Part 1 Cattle and sheep. Bioresource Technology 71, 173-181.

Smith, K, Charles, D & Moorhouse, D (2000), Nitrogen excretion by farm livestock with respect to land spreading requirements and controlling nitrogen losses to ground and surface waters.  Part 2 Pigs and poultry.  Bioresource Technology 71, 183-194.

Smith, K (2002), Personal communication from K Smith, ADAS.

Sneath, RW, Chadwick DR, Phillips VR & Pain BF (1997), A UK Inventory of Methane/Nitrous Oxide Emissions from Farmed Livestock.  Contract reports (2) to MAFF, projects WA0604/5, SRI, IGER & ADAS.

Tucker, S & Canning, P (1997), Personal communication to D Wilkins (MAFF).

USEPA (1997), Compilation of Air Pollutant Emission Factors, volume 1, 5th ed., AP-42, United States Environmental Protection Agency, North Carolina.

## Chapter 7 [Land Use Change and Forestry (CRF sector 5)]

Adger, WN & Subak, S (1996), Estimating above-ground carbon fluxes from UK agricultural land, Geographical Journal, Vol. 162, pp 191-204.

Barr, CJ, Bunce, RGH, Clarke, RT, Fuller, RM, Furse, MT, Gillespie, MK, Groom, GB, Hallam, CJ, Hornung, M, Howard, DC & Ness, MJ (1993), Countryside Survey 1990, Main Report. London, Department of the Environment.

BGS (2004), United Kingdom Minerals Yearbook 1, British Geological Survey, Natural Environment Research Council.

Bradley, I (1997), Carbon loss from drained lowland fens. In: Carbon Sequestration in Vegetation and Soils (Ed. by MGR Cannell), DOE/ITE Contract EPG 1/1/3.  Final Report March 1997.  Department of Environment, London.

Bradley, RI, Milne, R, Bell, J, Lilly, A, Jordan, C and Higgins, A (In prep), The derivation and use of a UK database for modelling soil carbon fluxes and land use for the national carbon dioxide inventory (To be submitted to Soil Use and Management).

Burton, RGO (1995), Evaluating organic matter dynamics in cultivated organic topsoils – use of historical analytical data.  MAFF Contract to SSLRC No. LE0203 –81/3830.

Cannell, MGR & Dewar, RC (1995), The carbon sink provided by plantation forests and their products in Britain, Forestry, Vol. 68, No. 1, pp 35-48.

Cannell, MGR, Milne, R, Hargreaves, KJ, Brown, TAW, Cruickshank, MM, Bradley, RI, Spencer, T, Hope, D, Billett, MF, Adger, WN & Subak, S (1999), National inventories of terrestrial carbon sources and sinks: the UK experience.  Climatic Change 42, 505-530.

Cooper, A and McCann, T (2002), Technical Report of the Northern Ireland Countryside Survey.  University of Ulster.

Cruickshank, MM, Tomlinson, RW, Devine, PM & Milne, R (1997), Effects of agricultural management on carbon storage in Northern Ireland.  In: Carbon Sequestration in Vegetation and Soils (Ed. by MGR Cannell), DOE Contract EPG 1/1/3.  Final Report March 1997.  Department of Environment, London.

Cruickshank, MM & Tomlinson, RW (1997), Carbon loss from UK peatlands for fuel and horticulture. In: Carbon Sequestration in Vegetation and Soils (Ed. by MGR Cannell), DOE, Contract EPG 1/1/3.  Final Report March 1997.  Department of Environment, London.

Cruickshank, MM, Tomlinson, RW, Devine, PM & Milne, R (1998), Carbon in the vegetation and soils of Northern Ireland. Proceedings of the Royal Irish Academy, 98B, 9-21.

Cruickshank, MM and Tomlinson, RW (2000), Changes in soil carbon storage in Northern Ireland: estimated by the IPCC default and matrix methods.  In: Carbon Sequestration in Vegetation and Soils (Ed. by R Milne), DETR Contract EPG 1/1/39. Progress Report April 2000.  DETR, London.  Available at:

http://www.edinburgh.ceh.ac.uk/ukcarbon/docs/DETR%20Contract%20Report%202000.PDF

Dewar, RC & Cannell, MGR (1992), Carbon sequestration in the trees, products and soils of forest plantations: an analysis using UK examples. Tree Physiology Vol. 11, pp49-72.

DOE (1997), Climate Change: The UK Programme; United Kingdom’s Second Report under the Framework Convention on Climate Change, The Stationery Office, London.

Edwards, PN & Christie, JM (1981), Yield models for forest management, Forestry Commission Booklet 48, Forestry Commission, Edinburgh.

Forestry Commission (2002), National Inventory of Woodland and Trees,

http://www.forestry.gov.uk/forestry/hcou-54pg4d

Haines-Young, RH + 23 others (2000), Accounting for nature: assessing habitats in the UK countryside.  DETR, London.  ISBN 1 85112 460 8

Hargreaves, K & Fowler, D (1997), Short-term CO2 fluxes over peatland.  In: Carbon Sequestration in Vegetation and Soils (Ed. by MGR Cannell), DOE Contract EPG 1/1/3.  Final Report March 1997.  Department of Environment, London.

Hargreaves, KJ, Milne, R & Cannell, MGR (2003), Carbon balance of afforested peatland in Scotland.  Forestry, 76, 299-317.

Howard, PJA & Howard, DM (1994), Modelling the effects of land use change and climate change on soil organic carbon stores.  In: Carbon sequestration by soils in the UK, Report to the Department of the Environment, Contract No. PECD 7/12/80, March 1994.

IPCC (1997a), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 1, Greenhouse Gas Inventory Reporting Instructions, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC (1997b), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 2, Greenhouse Gas Inventory Workbook, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC (1997c), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC (2003) Good Practice Guidance for Land Use, Land Use Change and Forestry, IPCC National Greenhouse Gas Inventories Programme, Technical Support Programme Technical Support Unit, Institute for Global Environmental Strategies, Hayama, Kanagawa, Japan.

Levy, PE & Milne, R (2004), Estimation of deforestation rates in Great Britain.  Forestry, 77, 9-16.

Milne, R & Brown, TA (1997), Carbon in the vegetation and soils of Great Britain.  Journal of Environmental Management, 49, 413-433.

Milne, R, Brown, TAW & Murray, TD (1998), The effect of geographical variation in planting rate on the uptake of carbon by new forests of Great Britain.  Forestry, 71, 298-309.

Milne, R & Brown, TA (1999), Methods and data for Land Use Change and Forestry Sector in the 1997 IPCC Greenhouse Gas Inventory.  In.: Carbon Sequestration in Vegetation and Soils (Ed. by R. Milne), DETR Contract EPG 1/1/39.  Interim Report April 1999.

MLC (1986) Monitoring Landscape Change Vols. 1, 1A & 10.  Report prepared by Hunting Surveys & Consultants Ltd for Department of the Environment and the Countryside Commission.

Office of The Deputy Prime Minister (2004), Land Use Change Statistics Index.  http://www.planning.odpm.gov.uk/luc/lucindex.htm

Stamp, LD (1962), The Land of Britain: Its Use and Misuse. London, Longman.

## Chapter 8 [Waste (CRF sector 6)]

AEA Technology (2002), Atmospheric Emissions from Small Carcass Incinerators, Report produced for the Department for Environment, Food and Rural Affairs by AEA Technology Environment, Culham Science Centre, Abingdon, Oxon. OX14 3ED, August 2002.  Report AEAT/ENV/R/0920.  http://www.airquality.co.uk/archive/reports/cat07/aeat-env-r-0920.pdf

Barlaz et al (1997), Biodegradative analysis of municipal solid waste in laboratory scale landfills.  Prepared for US Environmental Protection Agency Office of Research and Development, Washington DC 20460.  Report EPA-600/R-97-071.  USEPA, Washington DC.

Brown, KA, Smith, A, Burnley, SJ, Campbell, DJV, King, K & Milton, MJT (1999), Methane Emissions from UK Landfills, AEA Technology, AEAT-5217, Culham.

The Cremation Society of Great Britain (2004), National Cremation Statistics 1960-2003, http://www.srgw.demon.co.uk/CremSoc4/Stats/

Defra (2004), The National Food Survey 1990-03

DOE (1993), UK Sewage Sludge Survey, CES Limited, HMSO.

Environment Agency (2004), Pollution Inventory 2004

Hobson, J, Palfrey, R, Sivil, D, Palfrey, E & Day, M (1996), Control Measures to Limit Methane Emissions from Sewage and Sludge Treatment and Disposal, WRc, Report No DOE 4118.

IPCC (1997), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC (2000), Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories, ed. Penman, J, Kruger, D, Galbally, I, et al, IPCC National Greenhouse Gas Inventories Programme, Technical Support Programme Technical Support Unit, Institute for Global Environmental Strategies, Hayama, Kanegawa, Japan.

LQM (2003), Methane emissions from landfill sites in the UK.  Final report.  January 2003.  Report for the UK Department for Environment, Food and Rural Affairs.  Authors: Gregory, RG, Gillett, AG, Bradley, D, LQM report 443/1.  Defra contract EPG 1/1/145.

Milton, MJT, Partridge, RH, Goody, BA and Andrews, AS (1997),"An estimate of methane emissions from UK landfill sites based on direct flux measurements at representative sites", Final report to Department of the Environment, NPL report QM134, November 1997.

Passant, NR (1993), Emissions of Volatile Organic Compounds from Stationery Sources in the UK: A review of Emission Factors by Species and Process (September 1993 Update), Warren Spring Laboratory, LR 990.

US EPA (2004), AP 42, Fifth Edition, Compilation of Air Pollutant Emission Factors, Volume I, Stationary Point and Area Sources.  US EPA website, 2004: http://www.epa.gov/ttn/chief/ap42/index.html

## Chapter 10 [Recalculations and improvements]

FCCC ERT (2005).  Framework Convention on Climate Change – Expert Review Team.  United Kingdom of Great Britain And Northern Ireland Report of the Individual Review Of The Greenhouse Gas Inventory Submitted in the year 2004.  FCCC/WEB/IRI/2004/GBR.  United Nations Framework Convention on Climate Change, Haus Carstanjen, Martin-Luther-King-Strasse 8, D-53175 Bonn, Germany.  See
http://unfccc.int/files/national_reports/annex_i_ghg_inventories/inventory_review_reports/application/pdf/2004_irr_centralized_review_uk.pdf

## Annex 3, Sector 1, 1A

AEP (2004), 	Joslin, A, Rodgers I, Salway G, Tabberer R, Relating fuel consumption to CO2 emissions: Review and recommendations for power station fuel and plant parameters for use in the DTI power station model.

ACEA (1995), “Evaporative Emissions Test Programme: Preliminary Report Including Diurnal Test Results”, AE137/95.

Andre, M, Hassel, D, Hickman, AJ & Joumard, R (1993), The Science of the Total Environment, 134, 171.

Barlow, TJ, Hickman, AJ & Boulter, P (2001), Exhaust Emission Factors 2001: Database and Emission Factors, TRL Report PR/SE/230/00, September 2001.

BLF/UKPIA/CORA (1994), The Use and Fate of Lubricants and Used Oils.

Boulter, PG (1996), Factors Affecting Cold-Start Emissions: A Review, Transport Research Laboratory Report, PR/SE/183/96, September 1996.

Brain, SA, Carr, CE, Davies, M, Rantell, T & Stokes, BJ (1994), Emission of Volatile Organic Compounds (VOCs) from Coal-Fired Appliances, DTI, Coal R&D, Report No COAL R033.

British Coal (1989), Personal Communication.

British Gas (1992,) Personal Communication from FE Shephard, Gas Research Centre, Loughborough, Leics.

British Gas (1994), Personal Communication from S Pearson, Gas Research Centre, Loughborough, Leics.

BR (1994), Research Report "Determination of Diesel Exhaust emissions on British Rail" Jan 1994.

CONCAWE (1986), Hydrocarbon Emissions from Gasoline Storage and Distribution Systems, Report No 85/54

CONCAWE (1987), “An Investigation into Evaporative Hydrocarbon Emissions from European Vehicles”, Report 87/60.

CONCAWE (1990), “The Effects of Temperature and Fuel Volatility on Vehicle Evaporative Emissions”, Report 90/51.

CORINAIR (1992) CORINAIR Inventory, Commission of the European Community, Default Emission Factors Handbook, 2nd ed, CITEPA, Paris.

Corus (2004), Process-specific emissions data for all UK plant, Personal Communication (Peter Quinn)

DETR (1996a), data from “Continuous Survey of Road Goods Transport” communication, June 1996.

DETR (1996b), Air and Environment Quality Division communication, May 1996.

DETR (1996c), Transport Statistics Great Britain: 1996 edition.

DETR (1997), National Road Traffic Forecasts (Great Britain) 1997.

DETR (1998), Personal Communication from Dr T McMullen

DETR (1998a), Road Travel Speeds in English Urban Areas: 1996/97, Transport Statistics Report, January 1998

DETR (1998b), “Traffic Speeds in Central and Outer London: 1996-97”, DETR Statistics Bulletin (98) 17, April 1998

DETR (1998c), “Vehicle Speeds in Great Britain: 1997”, DETR Statistics Bulletin (98) 19, June 1998

DETR (1998d), “Traffic Speeds in Inner London: 1998”, DETR Statistics Bulletin (98) 22, November 1998

DETR (1998e), Data from National Travel Survey, Transport Statistics communication, March 1998.

DETR (2000), Transport, Environment and Taxation Division communication (A Short), November 2000.

DfT (2001), New Car Fuel Consumption and Emission Figures, Vehicle Certification Agency, July 2001

DfT (2004a), Transport, Environment and Taxation Division communication (N Barter), July 2004.

DfT (2004b), Transport of Goods by Road in Great Britain: 2003, Transport Statistics Bulletin (SB(04)27), 2004.

DfT (2004c), Transport Statistics Great Britain: 2004 edition, The Stationery Office.

DfT (2004d), Vehicle Licensing Statistics: 2003, Transport Statistics Bulletin (SB(04)26), 2004

DfT (2004e), Transport, Environment and Taxation Division communication (C Parkin), February 2004.

DRDNI (2002), “Vehicle Kilometres of Travel Survey of Northern Ireland.  Annual Report 1999”, Report by Doran Consulting for Department for Regional Development, Northern Ireland, Road Services Headquarters (2002).

DRDNI (2003a), “Vehicle Kilometres of Travel Survey of Northern Ireland.  Annual Report 2001”, Report by Doran Consulting for Department for Regional Development, Northern Ireland, Road Services Headquarters (2003).

DRDNI (2003b), “Vehicle Kilometres of Travel Survey of Northern Ireland.  Annual Report 2002”, Report by Doran Consulting for Department for Regional Development, Northern Ireland, Road Services Headquarters (2003).

DRDNI (2004), “Traffic and Travel Information 2003 incorporating Annual Traffic Census and Vehicle Kilometres of Travel”, Department for Regional Development, Northern Ireland, Road Services Headquarters (2004)

DTI (2000), Scrap Tyre Working Group, Personal Communication

DTI (2004), Digest of United Kingdom Energy Statistics 2004, London, The Stationery Office.

DTLR (2002a), Transport Statistics Bulletin.  Road Traffic Statistics: 2001, SB(01)19 August 2001.

DTLR (2002b), Transport Statistics Great Britain: 2002 Edition, The Stationery Office.

DTLR (2002c), Vehicle Licensing Statistics: 2001 Data, Transport Statistics Bulletin (SB(01)15), 2001.

DTLR (2002d), “New Car Fuel Consumption and Emission Figures”, Vehicle Certification Agency, July 2002.

EMEP (1990), European Monitoring and Evaluation Programme. Proceedings of the EMEP Workshop on Emissions from Ships.  State Pollution Control Authority, Norway.

EMEP/CORINAIR (1996), Atmospheric Emission Inventory Guidebook, 1st Edition, ed. G McInnes.

EMEP/CORINAIR (1999), Atmospheric Emission Inventory Guidebook, 2nd Edition, ed. S Richardson.

EMEP/CORINAIR (2003), Atmospheric Emission Inventory Guidebook, 3rd Edition.

Environment Agency (1997), Chemical Release Inventory 1996, Personal Communication.

Environment Agency (2004), Pollution Inventory 2003, Personal Communication.

European Environment Agency (1997), COPERT II: Computer Programme to Calculate Emissions from Road Transport - Methodology and Emission Factors, P Ahlvik et al, European Topic Centre on Air Emissions, European Environment Agency, April 1997.

European Environment Agency (2000), COPERT III: Computer Programme to Calculate Emissions from Road Transport - Methodology and Emission Factors (Version 2.1), L. Ntziachristos and Z Samaras, European Topic Centre on Air Emissions, European Environment Agency, November 2000.

Fynes, G & Sage, PW (1994), Emissions of Greenhouse Gases from Coal Fired Plant, British Coal, Coal Research Establishment, CERCHAR, DMT, Delft University of Technology, TPS Termiska Processer AB, CONTRACT NO JOUF 0047-C(SMA).

Goodwin, JWL, Salway, AG, Murrells, TP, Dore, CJ & Eggleston, HS (1999), UK Emissions of Air Pollutants 1970 to 1997, AEA Technology, AEAT/EEQC/0015, Culham.

Hawkins & Coad (2004), Personal communication.  John Hawkins & Nick Coad from the Association of Train Operating Companies (ATOC).

Hickman, AJ (1998), Transport Research Laboratory, communication, February 1998.

Institute of Petroleum (2000), Protocol for the Estimation of VOC Emissions from Petroleum Refineries and Gasoline Marketing Operations

IPCC (1997a), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 1, Greenhouse Gas Inventory Reporting Instructions, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC (1997b), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 2, Greenhouse Gas Inventory Workbook, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC (1997c), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC (2000), Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories, ed. Penman, J, Kruger, D, Galbally, I, et al, IPCC National Greenhouse Gas Inventories Programme, Technical Support Programme Technical Support Unit, Institute for Global Environmental Strategies, Hayama, Kanagawa, Japan.

Leech, P, (1994) Air Pollutant Emission Inventories for Heathrow Terminal 5 Study. Vol 2. Warren Spring Laboratory, Report LR 999.

LRC (1998), London Research Centre, UK Emission Factor Database.

LT Buses (1998), “Buses: A Cleaner Future. Bus Emissions and Air Quality in London”, London Transport Buses Report, 1998.

Met Office (2004), via the internet at:

http://www.met-office.gov.uk/research/hadleycentre/obsdata/cet.html, September 2004

MOD (2004a), Defence Fuels Group, SO3 Aviation Fuel, Personal Communication (Patrick Gedge)

MOD (2004b), Defence Fuels Group, MF1b, Marine Fuels, Personal Communication (Patrick Gedge).

Murrells, TP (2000), “UK Road Transport Emission Projections:  The Assumptions Used and Results of the 1997 National Atmospheric Emissions Inventory Base Projections”, AEA Technology Report AEAT-5953, January 2000.

Munday, PK (1990), UK Emissions of Air Pollutants 1970-1988, Warren Spring Laboratory, LR 764.

National Power (1997), Environmental Performance Review 1997.

NETCEN (2004a), “Non-Road Mobile Machinery Usage, Life and Correction Factors”, Report to the Department for Transport, AEAT/ENV/R/1895, November 2004, www.airquality.co.uk/archive/reports/reports.php?report_id=304

NETCEN (2004b), “Emission factors programme Task 8c – emissions from non-road mobile machinery in the lawn and garden sector”, Report to Defra, AEAT/ENV/R/1719, April 2004.

Oakdene Hollins Ltd (2001), UK Waste Oils Market 2001, Report for DETR.

Off Highway Research Ltd (2000), International Database Service, March 2000

Official Journal (1998), Official Journal of the European Communities L59/6, 59/15, February 1998.

ONS (1995), UK Defence Statistics, 1994, Office for National Statistics.

Ogilvie, SM (1995), Opportunities and Barriers to Scrap Tyre Recycling, National Environmental Technology Centre, AEA/CS R1026/C, ISBN 0 856248647.

Passant, NR (2004), Emission factors programme Task 1 – Summary of simple desk studies (2003/4), AEA Technology Plc, Report No AEAT/ENV/R/1715/Issue 1, March 2004.

Perry, RH & Chilton, C (1973), Chemical Engineer Handbook, 5th ed.  McGraw-Hill, New York.

Petroleum Review (2004), UK Retail Marketing Survey

Powergen (1994), Personal communication from D Grist, Corporate Environment Unit.

Powergen (1997), Environmental Performance Report 1996.

PRI (1995), Communication from Precision Research International.

PRI (1998), Communication from Precision Research International.

Quick, W (2004) – Power Tech – Personnal correspondence

Recycling Advisory Unit (1999), Markets for Recycled Materials, 1999

Royal Commission on Environmental Pollution (1993), 17 the Report, Incineration of Waste, HMSO, London.

Samaras, Z & Zierock, KH (1993), Notes on the Assessment of the Emissions of Off-Road Mobile Machinery in the European Community.

Samaras, Z & Zierock, KH, (1994), Supplement to above.

Samaras, Z (1996), Personal Communication.

Scottish Coal (2004), Coal sales and elemental content data, Personal Communication (Louis Suffredini).

SMMT (1999), Society of Motor Manufacturers and Traders, communication, November 1999.

Stewart, R (1997), Gaseous and PM10 Emission Factors for Station ‘W’ - a Modern CCGT Power Plant, AEA Technology, AEA/20011002/002/Issue 1.

Stewart, R & Walker, D (1996), Emissions to Atmosphere from Fossil Fuel Power Generation in the United Kingdom, AEAT-0746, ISBN 0-7058-1753-3.

Terra Nitrogen (2004), Plant-specific production and emissions data, Personal Communication (Peter Rees).

Transco (2004) – Measurement and process report MPR046: Estimation of Carbon Dioxide emission factors for UK Natural Gas 1990-2003

TRL (1993), Evaporative Emissions from Petrol Fuelled UK Light Duty In-Service Vehicles, TJ Barlow, Transport Research Laboratory Report PR/ENV/022/93.

UK Coal (2004), Coal sales and methane emission estimates for 2003, Personal Communication (Chris McGlen)

UKPIA (1989), United Kingdom Petroleum Industry Association, Personal Communication.

UKPIA (2004), Refinery production & emissions data, carbon emission factors, petroleum products data from the United Kingdom Petroleum Industry Association, Personal Communication (Ian McPherson, & Malcolm Watson)

USEPA (1997), United States Environmental Protection Agency, Compilation of Air Pollutant Emission Factors, Vol 1, 5th Edition, AP-42, North Carolina.

US EPA (2004), Compilation of Air Pollutant Emission Factors, AP-42, via the internet at: www.epa.gov, 2004

Walker, DS, Galbraith, R & Galbraith, JM (1985), Survey of Nitrogen Oxides, Carbon Monoxide and Hydrocarbon Emissions from Industrial and Commercial Boilers in Scotland, Warren Spring Laboratory, LR 524.

Watson, M (2001, 2003), UKPIA, Personal Communication.

Watterson, JD, Walker, C and Eggleston, S (2004) Revision to the Method of Estimating Emissions from Aircraft in the UK Greenhouse Gas Inventory.  Report to Global Atmosphere Division, UK Defra.  Report number ED47052, July 2004.  NETCEN, B551, Harwell, Didcot, Oxfordshire, OX11 0QJ, UK.

Zachariadis, Th. & Samaras, Z, (1997), Int. J. of Vehicle Design, 18, 312.

## Annex 3, Sector 1, 1B

Barty, R (1995), Energy Policy and Analysis Unit, DTI, Personal Communication.

Bennet, S, Kershaw, S & Burrell, R, (1995), Control Measures for Methane Emissions from Coal Production, ETSU N/01/00006/REP.

British Gas (1993), Personal Communication from C Rose, Regional Services Engineering Dept, London.

British Gas (1994), Personal Communication from S Pearson, Gas Research Centre, Loughborough, Leics.

CIS (1991), Chem Facts UK, Chemical Intelligence Services, Reed Telepublishing, Dunstable, Beds, UK.

Coal Authority (2004), Production and Manpower Return, Personal Communication (Simon Cooke).

CONCAWE (1986), Hydrocarbon Emissions from Gasoline storage and distribution systems. Report No 85/54.

CORINAIR (1989), CORINAIR Inventory, Commission of the European Community, Default Emission Factors Handbook, 1st ed, CITEPA, Paris.

Corus (2004), Process-specific emissions data for all UK plant, Personal Communication (Peter Quinn)

DETR (1998), C McMullen, Personal Communication.

DETR (1999), Global Atmosphere Division, Personal Communication.

DTI (1997), The Energy Report 2, Oil and Gas Resources of the United Kingdom, Department of Trade and Industry, The Stationery Office.

DTI (1998), Digest of United Kingdom Energy Statistics 1998, London, The Stationery Office.

DTI (1998a), The Energy Report 2, Oil and Gas Resources of the United Kingdom, Department of Trade and Industry, The Stationery Office.

DTI (2004), Digest of United Kingdom Energy Statistics 2004, London, The Stationery Office.

DTI (2004a), Detailed fuel use data, Personal Communication (Clive Evans, Martin Young and Ian M Corrie)

DTI (2004b), Development of UK Oil and Gas Resources, 2003, Department of Trade and Industry, The Stationery Office.

E & P Forum (1994), Methods for Estimating Atmospheric Emissions from E&P Operations, The Oil Industry International Exploration & Production Forum, Report No 2.59/197.

EIPPCB (2000), European Integrated Pollution Prevention and Control Bureau, Best Available Techniques reference Document on the Production of Iron and Steel.

Environment Agency (2004), Pollution Inventory database 2004, Personal Communication (EA PI team: Charlie Corbishley, Claudia Towner and Alex Wills)

Institution of Petroleum (2000), Protocol for the Estimation of VOC Emissions from Petroleum Refineries and Gasoline Marketing Operations.

IPCC (1997), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC (2000), Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories, ed. Penman, J, Kruger, D, Galbally, I, et al, IPCC National Greenhouse Gas Inventories Programme, Technical Support Programme Technical Support Unit, Institute for Global Environmental Strategies, Hayama, Kanagawa, Japan.

Kershaw (2005), “Development of a Methodology for Estimating Methane Emissions from Abandoned Coal Mines in the UK”, White Young Green report to Defra, 2005.

Oilfield Publications (1996) North Sea Field Development Guide, 6th ed., Oilfield Publications Limited, Ledbury, Herts.

Petroleum Review (2000), 2000 UK Retail Marketing Survey. A special Supplement to Petroleum Review, Institute of Petroleum, March 2000.

Russell, P (1997), Personal Communication, Safety and Environment Directorate, BG plc.

Russell, P (2004)  “UKOOA: Offshore Industry Carbon Dioxide Calculation Requirements for Emissions Trading”

Sage, PW (2001), Methane from Abandoned Coal Mines in the UK, AEA Technology, Report AEAT/ENV/R/0500, Harwell, UK.

Transco (2004), Gas leakage, regional sales and gas content data for 2003, Personal Communication (Chris Allen, Dave Lander and Peter Osbaldstone).

UK Coal (2004), Coal sales and methane emission estimates for 2003, Personal Communication (Chris McGlen)

UKOOA (1993), Atmospheric Emissions from UK Oil and Gas Exploration and Production Facilities in the UK Continental Shelf Area. Prepared for United Kingdom Offshore Operators Association Limited, March 1993, Ref HN08-007.REP, Brown & Root Environmental, Leatherhead, Surrey.

UKOOA (1995), Guidelines on Atmospheric Emissions Inventory, UK Offshore Operators Association Limited (Environment Committee).

UKOOA (1998), Internal UKOOA study to determine pre-EEMS emissions totals and trends, 1990-1995, via Personal Communication (Peter Russell, Granherne)

UKOOA (2001), “Environmental Emissions Monitoring System (EEMS): GUIDE FOR DATA SUBMISSION Version 7.0”, UKOOA & LearnIT September 2001.

UKOOA (2005), EEMS Environmental Database for Emissions and Discharges from Offshore Installations, Atmospheric Emissions Inventory, 2003 data.  Personal communication from UKOOA via Peter Russell at Granherne. Previous editions 1995-2002.

UKPIA (2004), Refinery production & emissions data, petroleum products data from the United Kingdom Petroleum Industry Association, Personal Communication (Ian McPherson, & Malcolm Watson)

USEPA (2004), United States Environmental Protection Agency, Compilation of Air Pollutant Emission Factors. Vol 1, 5th Edition, AP-42, North Carolina. www.epa.gov

Williams, A (1993), Methane Emissions, Watt Committee Report Number 28, The Watt Committee on Energy, London.

## Annex 3, Sectors 2 and 3

Alcan (1997), Personal Communication from A Walker, Alcan International.

Alcan (2004), Personal Communication from J Clarkson, Alcan International.

BISPA (1997), Personel Comunication, M Funnell, British Iron and Steel Producers Association.

Blyth, WJ, Collingwood, JC, Pratten, NA, (1996), Estimation and Projection of NOx and SOx Emissions from UK Industrial Sectors, Phase 2. ETSU REF RYTA 18400734/Z/3

BP Chemicals (2004), Personal Communication (Dave Ingram).

Brain, SA, Carr, CE, Davies, M, Rantell, T, Stokes, BJ. (1994), Emission of Volatile Organic Compounds (VOCs) from Coal-Fired Appliances, DTI, Coal R&D, Report No COAL R033

British Cement Association (2004), UK cement clinker production data, Personal Communication (Richard Leese).

British Coal (1989), Personal Communication.

British Gas (1992), Personal Communication from FE Shephard, Gas Research Centre, Loughborough, Leics.

British Geological Survey (2004), UK Minerals Yearbook 2003

British Glass (2004), UK glass production and recycling data in 2003, Personal Communication (Rebecca Cocking).

CIS (1991), Chem-Facts UK, Chemical Intelligence Services, Reed Telepublishing, Dunstable, Beds, UK.

Collingwood, J (1997), ETSU, AEA Technology, Personal Communication.

CORINAIR (1989), CORINAIR Inventory, Commission of the European Community, Default Emission Factors Handbook, 1st ed, CITEPA, Paris.

CORINAIR (1992), CORINAIR Inventory, Commission of the European Community, Default Emission Factors Handbook, 2nd ed, CITEPA, Paris.

Corus (2004), Process-specific emissions data for all UK plant, Personal Communication (Peter Quinn)

DoENI (2004), ISR inventory of industrial point source emissions from the Department of the Environment of Northern Ireland, Personal Communication (David Bell).

DTI (2001b), Department of Trade and Industry. Construction Market Intelligence, Monthly Statistics of Building Materials and Components.

DTI (1992), Business Monitor PAS 4196.

DTI (2004), Digest of United Kingdom Energy Statistics 2004, London, The Stationery Office.

DTI (2000a), DTI Tyre Working Group, Private Communication from P Hallett

EIPPCB (2000), European Integrated Pollution Prevention and Control Bureau. Integrated Pollution Prevention and Control (IPPC):  Best Available Techniques Reference Document on the Production of Iron and Steel, March 2000.

EMEP/CORINAIR (1996), Atmospheric Emission Inventory Guidebook, 1st Edition, ed. G McInnes.

EMEP/CORINAIR (1999), Atmospheric Emission Inventory Guidebook, 2nd Edition.

Environment Agency (2004), Pollution Inventory database 2004, Personal Communication (EA PI team: Charlie Corbishley, Claudia Towner and Alex Wills)

Federation of Bakers (2000), Personal Communication from J White.

Gibson, et al (1995), Atmospheric Environment, Vol 29, No 19, p2661-2672.

Fynes, G and Sage, PW (1994), Emissions of Greenhouse Gases from Coal Fired Plant,  British Coal, Coal Research Establishment, CERCHAR, DMT, Delft University of Technology, TPS Termiska Processer AB, CONTRACT NO JOUF 0047-C(SMA).

Invista (2004), 2003 Plant-specific emissions and abatement details, Personal communication (Graham Hinton).

IPCC (1997), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC (2000), Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories, ed. Penman, J, Kruger, D, Galbally, I, et al, IPCC National Greenhouse Gas Inventories Programme, Technical Support Unit, Institute for Global Environmental Strategies, Hayama, Kanagawa, Japan.

Iron & Steel Statistics Bureau (2004), Iron and Steel Industry, Annual Statistics for the UK, 2004.  ISSB Limited.

MCG (1999), UK Emissions of HFCs, PFCs and SF6 Potential Emission Reduction Options.  A study for the Department of the Environment carried out by March Consulting Group. Telegraphic House, Waterfront Quay, Salford Quays, Manchester

Munday, PK (1990) UK Emissions of Air Pollutants 1970-1988, Warren Spring Laboratory, LR 764.

Ogilvie, SM (1995), Opportunities and Barriers to Scrap Tyre Recycling, National Environmental Technology Centre, AEA/CS R1026/C, ISBN 0 856248647

Office of National Statistics (2004), Index of Production, Population Statistics and Prodcom data for 1990-2003, www.statistics.gov.uk

ONS (2004), Annual Abstract of Statistics 2003, Office for National Statistics, The Stationery Office.

Passant, NR (1993), Emissions of Volatile Organic Compounds from Stationery Sources in the UK, LR 990, Warren Spring Laboratory.

Passant, NR (2002), Speciation of UK Emissions of Non- Volatile Organic, AEA Technology, NETCEN, AEAT/ENV/R/0545, Culham.

Rudd, HJ (1995), Emissions of Volatile Organic Compounds from Stationery Sources in the United Kingdom: Speciation, AEA Technology, National Environmental Technology Centre, AEA/CS/16419033/REMA-029/ISSUE1.

Rudd, HJ, Mikkelsen, M (1996), Emissions of Volatile Organic Componds from Ship Purging in the UK.

SEPA (2004), EPER Inventory of industrial point source emissions, Scottish Environmental Protection Agency, Personal Communication (Bob Boyce).

SWA (2004), The Scotch Whiskey Association Statistical Report 2003, Personal Communication (Pam Peacocke)

Terra Nitrogen (2004), Plant-specific production and emissions data, Personal Communication (Peter Rees).

UKPIA (2004), Refinery production & emissions data, petroleum products data from the United Kingdom Petroleum Industry Association, Personal Communication (Ian McPherson, & Malcolm Watson)

USEPA (2004), United States Environmental Protection Agency, Compilation of Air Pollutant Emission Factors. Vol 1, 5th Edition, AP-42, North Carolina. www.epa.gov

## Annex 3, Sector 4

See references under Chapter 6

## Annex 3, Sector 5

See references under Chapter 7

## Annex 4

DTI (2004), Digest of United Kingdom Energy Statistics 2004, London, The Stationery Office.

## annex 6

DTI (2004), Digest of United Kingdom Energy Statistics 2004, London, The Stationery Office.

## Annex 7 [Uncertainties]

Bennet, S, Kershaw, S, Burrell, R (1995), Control Measures for Methane Emissions from Coal Production, ETSU N/01/00006/REP

Brown, KA, Smith, A, Burnley, SJ, Campbell, DJV, King, K, Milton, MJT, (1999) Methane Emissions from UK Landfills, AEA Technology, AEAT-5217, Culham.

DTI (1996), Digest of United Kingdom Energy Statistics 1996, London, The Stationery Office.

DTI (2004), Digest of United Kingdom Energy Statistics 2004, London, The Stationery Office.

Eggleston, HS, Salway, AG, Charles, D, Jones, BMR, Milne, R (1998), Treatment of Uncertainties for National Estimates of Greenhouse Gas Emissions, National Environmental Technology Centre, AEA Technology, Report AEAT - 2688.

Fynes, G, Sage, PW (1994), Emissions of Greenhouse Gases from Coal Fired Plant, British Coal, Coal Research Establishment, CERCHAR, DMT, Delft University of Technology, TPS Termiska Processer AB, CONTRACT NO JOUF 0047-C(SMA).

Hobson, J, Palfrey, R, Sivil, D, Palfrey, E, Day, M (1996), Control Measures to Limit Methane Emissions from Sewage and Sludge Treatment and Disposal, WRc, Report No DOE 4118.

IPCC (2000), Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories, ed. Penman, J, Kruger, D, Galbally, I, et al, IPCC National Greenhouse Gas Inventories Programme, Technical Support Programme Technical Support Unit, Institute for Global Environmental Strategies, Hayama, Kanagawa, Japan.

Transco (1998), Personal Communication, I Jones, System Control.

Milne, R (1999), Centre for Ecology and Hydrology, (Edinburgh), Personal Communication.

UKOOA (1993), Atmospheric Emissions from UK Oil and Gas Exploration and Production Facilities in the UK Continental Shelf Area. Prepared for United Kingdom Offshore Operators Association Limited, March 1993, Ref HN08-007.REP, Brown & Root Environmental, Leatherhead, Surrey

UKOOA (2005), Environmental Database for Emissions & Discharges from Offshore Installations (EEMS), UKOOA via Peter Russell.

Williams, A (1993), Methane Emissions, Watt Committee Report Number 28, The Watt Committee on Energy, London.

## Annex 8 [Verification]

Cullen, MJP (1993), The Unified Forecast/Climate Model, Meteorological Magazine UK, 1449, 81-94, 1993.

Department of the Environment, Food and Rural Affairs (Defra) (2000), Climate Change: The UK Programme, Nov 2000, www.defra.gov.uk/environment/climatechange/07.htm

Manning, AJ, Ryall, DB, Derwent, RG, Simmonds, PG, O’Doherty, S (2003), Estimating European emissions of ozone-depleting and greenhouse gases using observations and a modelling back-attribution technique, in press J. of Geophysical Research (atmospheres) 2003.

Manning, AJ, Derwent, RG, Ryall, DB.  Interpretation of long-term measurements of radiatively active trace gases and ozone depleting substances.  Defra Contract number: CPEG 11.  Final Report to UK Defra, January 2004 to May 2005.  Met Office, FitzRoy Road, Exeter, Devon, EX1 3PB, United Kingdom.

Nilsson, M, Mikkela, C, Sundh, I, Granberg, G, Svensson, BH & Ranneby, B (2001), Methane emission from Swedish mires: Natural and regional budgets and dependence on mire vegetation, J. of Geophys. Res., 106, 20847-20860, 2001.

Ryall, DB & Maryon, RH (1998), Validation of the UK Met Office's NAME model against the ETEX dataset, Atmospheric Environment, 32, 4265-4276, 1998.

Simmonds, PG, Derwent, RG, McCulloch, A, O’Doherty, S, Gaudry, A (1996), Long-term trends in concentrations of halocarbons and radiatively-active trace gases in Atlantic and European air masses monitored at Mace Head, Ireland from 1987-1994.  Atmospheric Environment, 30, 4041-4063, 1996.

United Nations Framework Convention on Climate Change (UNFCCC) www.unfccc.int

# Acknowledgements

We are grateful for the contributions, advice and support from the following people during the compilation of this National Inventory Report (NIR):

Table 12.1	Contributors to this National Inventory Report and the CRF

| Person | Technical work area and responsibility |
| --- | --- |
| Main authors |  |
| Baggott, Sarah | Author of Chapter 2, Annex 6 and selected sections of Chapters 1 and 3; contributions to most chapters; assistance with compiling the CRF |
| Brown, Lorna | Author of all sections on agriculture |
| Milne, Ronnie | Author of all sections on land use change and forestry |
| Murrells, Tim | Author of all sections on road transport |
| Passant, Neil | Author of selected sections on energy, industry and uncertainties; contributions to most chapters; |
| Thistlethwaite, Glen | Author of selected sections of Chapters 1 and 2; assistance with compiling the 2003 GHG inventory; assistance with responses to the UNFCCC Centralised Review of the 2002 NIR |
| Watterson, John | Author of Chapter 1 and Chapter 10, and contributions to most chapters; internal QA checks for consistency between the CRF, IPCC reporting formats and the UK National Atmospheric Emissions Inventory database; Project Manager for the UK Greenhouse Gas Inventory with overall responsibility for the NIR and the CRF |
| Contributors |  |
| Adams, Martin | Author of sections on F-gases and assistance with the CRF |
| Dore, Chris | Assistance with compiling the UK National Atmospheric Emissions Inventory database; QA checks on time series consistency and sector consistency; assistance with the compiling the CRF |
| Goodwin, Justin | Internal QA checks for consistency between the CRF, IPCC reporting formats and the UK National Atmospheric Emissions Inventory database |
| Lawrence, Helen | Assistance with co-ordinating contributions from the sector experts and with updating Annex 6. |
| Manning, Alistair | Verification of the UK greenhouse gas inventory |
| Smith, Alison | Author of section on solid waste disposal and assistance with the CRF |
| Additional assistance |  |
| Downes, Melissa | Assistance with compiling the 2003 GHG inventory and the CRF, development of data processing and reporting systems; assistance with developing the uncertainty analysis. |
| Grice, Susannah | Assistance with compiling the 2003 GHG inventory (many industrial sectors) and the CRF |
| Hobson, Melanie | Assistance with compiling the 2003 GHG inventory (transport) and the CRF |
| Karwal, Meena | Assistance with compiling the CRF |
| Kent, Val | Editorial support |
| Penman, Jim | Suggestions and improvements to draft versions of the NIR |
| Donaldson, Susan | Suggestions and improvements to draft versions of the NIR |
| Salway, Geoff | (Formerly Netcen) – for helpful comments and advice |
| Sturman, James | Assistance with compiling the CRF |
| Wagner, Anna | Assistance with compiling the CRF and tables of EFs for the NIR |

Table 12.2	Key Data Providers to the Greenhouse Gas Inventory

Company

UKPIA

UKOOA

Environment Agency

DTI

PowerTech

British Cement Association

Transco

Corus

DFT

UK Coal

Kemira

NIDoE

SEPA

- ANNEX 1: Key Sources

  - Key Source Analysis

In the UK inventory, certain source categories are particularly significant in terms of their contribution to the overall uncertainty of the inventory.  These key source categories have been identified so that the resources available for inventory preparation may be prioritised, and the best possible estimates prepared for the most significant source categories.  We have used the method set out in Section 7.2 of the IPCC Good Practice Guidance (2000) (Determining national key source categories) to determine the key source categories.

The results of the analysis are summarised by sector and gas in Table A1.3.  The analysis is based on the Tier 1 Level analysis and Trend analysis, performed using the data shown in Tables A7.5a and A7.5 b using the same categorisation and the same estimates of uncertainty.  The table indicates whether a key source arises from the level assessment or the trend assessment.  The factors that make a source a key source are:

- A high contribution to the total

- A high contribution to the trend

- High uncertainty.

For example, transport fuel (1A3b) is a key source for carbon dioxide because it is large; landfill methane (6A) is key because it is large, has a high uncertainty and shows a significant trend.

Both the Level and the Trend assessments were completed, following the procedure set out in the IPCC Good Practice Guidance (2000).  A qualitative assessment was not conducted, but we did not anticipate that additional source categories would have been identified using such an assessment.

The results of the Level Assessment are shown in Table A1.1, with the key source categories shaded.  The emission estimates were taken from the 2003 UK GHG inventory.  The source categories (i.e. rows of the table) were sorted in descending order of magnitude based on the results of the “Level Parameter”, and then the cumulative total was included in the final column of the table.  The key source categories are those whose contributions add up to 95% of the total uncertainty in the final column after this sorting process.

The results of the Trend Assessment are shown in Table A1.2, with key source categories shaded.  The emission estimates were taken from the 2003 UK GHG inventory.  The “Trend Parameter” was calculated using absolute value of the result.  An absolute function is used since Land Use Change and Forestry contains negative sources (sinks) and the absolute function is necessary to produce positive uncertainty contributions for these sinks.  The source categories (i.e. rows of the table) were sorted in descending order of magnitude based on the results of the “Trend Parameter”, and then the cumulative total was included in the final column of the table.  The key source categories are those whose contributions add up to 95% of the total uncertainty in the final column after this sorting process.

Table A1.1	Key Source Analysis Based on Level of Emissions

Table A1.2	Key Source Analysis Based on Trend in Emissions

Table A1.3	Key Source Category Analysis Summary

- ANNEX 2: Detailed discussion of methodology and data for estimating CO2 emissions from fossil fuel combustion

- Methodology for estimating CO2 emissions from fossil fuel combustion is discussed together with the methodologies for other emissions in Annex 3.  This is because the underlying methodology for such estimates apply to a range of pollutants and not just CO2.

- ANNEX 3: Other Detailed Methodological Descriptions

- This Annex contains background information about methods used to estimate emissions in the UK GHG inventory.  This information has not been incorporated in the main body of the report because of the level of detail, and because the methods used to estimate emissions cut across sectors.

- This Annex provides:

- Background information on the fuels used in the UK GHG inventory.

- Mapping between IPCC and NAEI source categories.

- Detailed description of methods used to estimate GHG emissions, and emission factors used in those methods – presented in Section A3.3 onwards.

  - Background information on FUELS DATA

- The fuels data are taken from DUKES - the Digest of UK Energy Statistics (DTI, 2004), so the fuel definitions and the base source categories used in the NAEI reflect those in DUKES.  Base categories for non-combustion sources generally reflect the availability of data on emissions from these sources.

- IPCC Guidelines (IPCC, 1997a) lists fuels that should be considered when reporting emissions.  Table A3.1.1 lists the fuels that are used in the GHGI and indicates how they relate to the fuels reported in the NAEI.  In most cases the mapping is obvious but there are a few cases where some explanation is required.

- Aviation Fuels

UK energy statistics report consumption of aviation turbine fuel and this is mapped onto jet kerosene in the GHGI.  However aviation turbine fuel includes fuel that is correctly described as jet gasoline using IPCC terminology.  For non- CO2 gases, emissions are estimated from data on numbers of landing-takeoff cycles, so jet kerosene and aviation gasoline emissions are combined.

- Coal

The IPCC Guidelines (IPCC, 1997a) classify coal as anthracite, coking coal, other bituminous coal and sub-bituminous coal.  In mapping the UK fuel statistics to these categories it is assumed that only the coal used in coke ovens is coking coal; and the rest is reported as either coal or anthracite.  Most of the coal used in the UK is bituminous coal; anthracite is reported separately.

- Coke Oven Coke

Gas works coke is no longer manufactured in the UK so all coke and coke breeze consumption is reported as coke oven coke.

- Colliery Methane

The IPCC Guidelines do not refer to colliery methane but significant use is made of it as a fuel in the UK so emissions are included in the GHGI.

- Orimulsion

Orimulsion® is an emulsion of bitumen and water and was burnt in some power stations in the UK, however its use has now been discontinued

- Slurry

This is a slurry of coal and water used in some power stations.

- Sour Gas

Unrefined natural gas is used as a fuel on offshore platforms and in some power stations.  It has a higher carbon and sulphur content than mains gas.

- Wastes used as fuel

The following wastes are used for power generation: municipal solid waste, scrap tyres, poultry litter, meat and bone meal, landfill gas, sewage gas, and waste oils.  Some waste oils and scrap tyres are burnt in cement kilns.  Further waste oils are burnt by other industrial sectors, and it is assumed that between 40% and 50% of lubricants consumed in the UK are destroyed (burnt) in the vehicle.

Table A3.1.1	Mapping of fuels used in the GHGI and the NAEI

|  | - GHGI | - NAEI |
| --- | --- | --- |
| - Category | - Subcategory | - Subcategory |
| - Liquid | - Motor Gasoline - Aviation Gasoline - Jet Kerosene - Other Kerosene - Gas/Diesel Oil - Residual Fuel Oil - Orimulsion - Liquefied Petroleum Gas - Naphtha - Petroleum Coke - Refinery Gas - Other Oil: Other - Other Oil: Other - Lubricants | - Petrol - Aviation Spirit - Aviation Turbine Fuel1 (ATF) - Burning Oil - Gas Oil/ DERV - Fuel Oil - Orimulsion - Liquefied Petroleum Gas (LPG) - Naphtha - Petroleum Coke - Other Petroleum Gas (OPG) - Refinery Miscellaneous - Waste Oils - Lubricants |
| - Solid | - Anthracite - Coking Coal - Coal - Coal - Coke Oven Coke - Patent Fuel - Coke Oven Gas - Blast Furnace Gas | - Anthracite - Coal2 - Coal - Slurry3 - Coke - Solid Smokeless Fuel (SSF) - Coke Oven Gas - Blast Furnace Gas |
| - Gas | - Natural Gas - Natural Gas - Colliery Methane5 | - Natural Gas - Sour Gas4 - Colliery Methane |
| - Other Fuels | - Municipal Solid Waste - Industrial Waste: Scrap Tyres | - Municipal Solid Waste - Scrap Tyres |
| - Biomass | - Wood/Wood Waste - Other Solid Biomass: Straw - Other Solid Biomass: Poultry Litter, Meat & Bone Meal - Landfill Gas - Sludge Gas | - Wood - Straw - Poultry Litter, Meat & bone meal - Landfill Gas - Sewage Gas |

- 1	Includes fuel that is correctly termed jet gasoline.

- 2	Used in coke ovens.

- 3	Coal-water slurry used in some power stations

- 4	Unrefined natural gas used on offshore platforms and some power stations

- 5	Not referred to in IPCC Guidelines (IPCC, 1997a) but included in GHGI.

  - NAEI SOurce CAtegories and IPCC Equivalents

- Tables A3.2.1 to A3.2.7 relate the IPCC source categories to the equivalent NAEI base categories.  In most cases it is possible to obtain a precise mapping of an NAEI source category to a specific IPCC source category.  In some cases the relevant NAEI source category does not correspond exactly to the IPCC source category and in a few cases an equivalent NAEI source category is not estimated or is defined quite differently.  As a result, total annual emissions given in the NAEI and GHGI may differ slightly.  The source categories responsible for the differences between the GHGI and the NAEI are:

- 1A3a Civil Aviation

- 5 Land Use Change and Forestry

- Forests (NMVOC emission only reported in the NAEI)

- Tables A3.2.1 to A3.2.7 refer to NAEI base categories.  Normally the NAEI is not reported in such a detailed form but in the summary UNECE/CORINAIR SNAP97, eleven-sector format or the new NRF (Nomenclature or Reporting) system used for submission to CORINAIR.

Table A3.2.1	Mapping of IPCC Source Categories to NAEI Source Categories:
IPCC Table 1A

| - IPCC Source Category | - NAEI Source Category |
| --- | --- |
| - 1A1a Public Electricity and Heat Production | - Power Stations |
| - 1A1b Petroleum Refining | - Refineries (Combustion) |
| - 1A1ci Manufacture of Solid Fuels | - SSF Production - Coke Production |
| - 1A1cii Other Energy Industries | - Collieries - Gas Production - Gas Separation Plant (Combustion) - Offshore Own Gas Use - Production of Nuclear Fuel - Town Gas Production |
| - 1A2a Iron and Steel | - Iron and Steel (Combustion) - Iron and Steel (Sinter Plant) - Iron and Steel (Blast Furnaces) |
| - 1A2b Non-Ferrous Metals - 1A2c Chemicals - 1A2d Pulp, Paper and Print - 1A2e Food Processing, Beverages, Tobacco | - Included under Other Industry (Combustion) |
| - 1A2fi Other | - Other Industry (Combustion) - Cement (Fuel Combustion) - Cement (Non-decarbonising) - Lime Production (Combustion) - Autogenerators - Ammonia (Combustion) |
| - 1A2fii Other (Off-road Vehicles and Other Machinery) | - Other Industry Off-road |
| - 1A3a Civil Aviation | - No comparable category |
| - 1A3b Road Transportation | - Road Transport |
| - 1A3c Railways | - Railways (Freight) - Railways (Intercity) - Railways (Regional) |
| - 1A3di International Marine | - International Marine |
| - 1A3dii Internal Navigation | - Coastal Shipping |
| - 1A3e Other Transport | - Aircraft Support |
| - 1A4a Commercial/Institutional | - Miscellaneous - Public Services - Railways (Stationary Sources) |
| - 1A4bi Residential | - Domestic |
| - 1A4bii Residential Off-road | - Domestic, House & Garden |
| - 1A4ci Agriculture/Forestry/Fishing (Stationary) | - Agriculture |
| - 1A4cii Agriculture/Forestry/Fishing (Off-road Vehicles and Other Machinery) | - Agriculture Power Units |
| - 1A4ciii Agriculture/Forestry/Fishing (Fishing) | - Fishing |
| - 1A5a Other: Stationary | - No comparable category-included in 1A4a |
| - 1A5b Other: mobile | - Aircraft Military - Shipping Naval |

Table A3.2.2	Mapping of IPCC Source Categories to NAEI Source Categories:
IPCC Table 1B

| - IPCC Source Category | - NAEI Source Category |
| --- | --- |
| - 1B1a Coal Mining i Mining activities | - Deep-Mined Coal |
| - 1B1a Coal Mining ii Post mining activities | - Coal Storage & Transport |
| - 1B1a Coal Mining ii Surface Mines | - Open-Cast Coal |
| - 1B1b Solid Fuel Transformation | - Coke Production (Fugitive) - SSF Production (Fugitive) - Flaring (Coke Oven Gas) |
| - 1B1c Other | - Not Estimated |
| - 1B2a Oil i Exploration | - Offshore Oil and Gas (Well Testing) |
| - 1B2a Oil ii Production | - Offshore Oil and Gas |
| - 1B2a Oil iii Transport | - Offshore Loading - Onshore Loading |
| - 1B2a Oil iv Refining/Storage | - Refineries (drainage) - Refineries (tankage) - Refineries (Process) - Oil Terminal Storage - Petroleum Processes |
| - 1B2a Oil vi Other | - Not Estimated |
| - 1B2a Oil v Distribution of oil products | - Petrol Stations (Petrol Delivery) - Petrol Stations (Vehicle Refuelling) - Petrol Stations (Storage Tanks) - Petrol Stations (Spillages) - Petrol Terminals (Storage) - Petrol Terminals (Tanker Loading) - Refineries (Road/Rail Loading) |
| - 1B2b i Natural Gas Production | - Gasification Processes |
| - 1B2b ii Natural Gas. Transmission/Distribution | - Gas Leakage |
| - 1B2ciii Venting: Combined | - Offshore Oil and Gas (Venting) |
| - 1B2ciii Flaring: Combined | - Offshore Flaring - Refineries (Flares) |

Table A3.2.3	Mapping of IPCC Source Categories to NAEI Source Categories:
IPCC Table 2

| - IPCC Source Category | - NAEI Source Category |
| --- | --- |
| - 2A1 Cement Production | - Cement (Decarbonising) |
| - 2A2 Lime Production | - Lime Production (Decarbonising) |
| - 2A3 Limestone and Dolomite Use | - Glass Production: Limestone and Dolomite - Iron and Steel (Blast Furnace): Limestone and Dolomite - Power Stations (FGD) |
| - 2A4 Soda Ash Production and Use | - Glass Production: Soda Ash |
| - 2A5 Asphalt Roofing | - Not Estimated |
| - 2A6 Road Paving with Asphalt | - Road Construction |
| - 2A7 Other | - Brick Manufacture (Fletton) - Glass (continuous filament glass fibre) - Glass (glass wool) |
| - 2B1 Ammonia Production | - Ammonia Feedstock |
| - 2B2 Nitric Acid Production | - Nitric Acid Production |
| - 2B3 Adipic Acid Production | - Adipic Acid Production |
| - 2B4 Carbide Production |  |
| - 2B5 Other | - Sulphuric Acid Production - Chemical Industry - Chemical Industry (Carbon Black) - Chemical Industry (Ethylene) - Chemical Industry (Methanol) - Chemical Industry (Nitric Acid Use) - Chemical Industry (Pigment Manufacture) - Chemical Industry (Reforming) - Chemical Industry (Sulphuric Acid Use) - Coal, tar and bitumen processes - Solvent and Oil recovery - Ship purging |
| - 2C1 Iron and Steel | - Iron and Steel (other) - Iron and Steel (Basic Oxygen Furnace) - Iron and Steel (Electric Arc Furnace) - Iron and Steel Flaring (Blast Furnace Gas) - Rolling Mills (Hot & Cold Rolling) |
| - 2C2 Ferroalloys Productions | - No Comparable Source Category |
| - 2C3 Aluminium Production | - Non-Ferrous Metals (Aluminium Production) |
| - 2C4 SF6 Used in Aluminium and Magnesium Foundries | - SF6 Cover Gas |
| - 2C5 Other | - Non-Ferrous Metals (other non-ferrous metals) - Non-Ferrous Metals (primary lead/zinc) - Non-Ferrous Metals (secondary Copper) - Non-Ferrous Metals (secondary lead) |
| - 2D1 Pulp and Paper | - Wood Products Manufacture |
| - 2D2 Food and Drink | - Brewing (barley malting, fermentation, wort boiling) - Bread Baking - Cider Manufacture - Other Food (animal feed; cakes, biscuits, cereals; coffee, malting, margarine and other solid fats; meat, fish and poultry; sugar) - Spirit Manufacture (barley malting, casking, distillation, fermentation, maturation, spent grain drying) - Wine Manufacture |
| - 2E1 Halocarbon & SF6 By-Product Emissions - 2E2 Halocarbon & SF6 Fugitive Emissions | - Halocarbons Production (By-Product and Fugitive) |
| - 2E3 Halocarbon & SF6 Other | - Not Estimated |
| - 2F1 Refrigeration & Air Conditioning Equipment | - Refrigeration - Supermarket Refrigeration - Mobile Air Conditioning |
| - 2F2 Foam Blowing | - Foams |
| - 2F3 Fire Extinguishers | - Fire Fighting |
| - 2F2 Aerosols | - Metered Dose Inhalers - Aerosols (Halocarbons) |
| - 2F5 Solvents | - Not Occurring |
| - 2F8a One Component Foams |  |
| - 2F8 Semiconductors, Electrical and Production of Trainers | - Electronics - Training Shoes - Electrical Insulation |

Table A3.2.4	Mapping of IPCC Source Categories to NAEI Source Categories:
IPCC Table 3

| - IPCC Source Category | - NAEI Source Category |
| --- | --- |
| - 3A Paint Application | - Decorative paint (retail decorative) - Decorative paint (trade decorative) - Industrial Coatings (automotive) - Industrial Coatings (agriculture & construction) - Industrial Coatings (aircraft) - Industrial Coatings (Drum) - Industrial Coatings (coil coating) - Industrial Coatings (commercial vehicles) - Industrial Coatings (high performance) - Industrial Coatings (marine) - Industrial Coatings (metal and plastic) - Industrial Coatings (metal packaging) - Industrial Coatings (vehicle refinishing) - Industrial Coatings (wood) |
| - 3B Degreasing & Dry Cleaning | - Dry Cleaning - Surface Cleaning - Leather Degreasing |
| - 3C Chemical Products, Manufacture & Processing | - Coating Manufacture (paint) - Coating Manufacture (ink) - Coating Manufacture (glue) - Film Coating - Leather coating - Other Rubber Products - Tyre Manufacture - Textile Coating |
| - 3D Other | - Aerosols (Car care, Cosmetics & toiletries, household products) - Agrochemicals Use - Industrial Adhesives - Paper Coating - Printing - Other Solvent Use - Non Aerosol Products (household, automotive, cosmetics & toiletries, domestic adhesives, paint thinner) - Seed Oil Extraction - Wood Impregnation |

Table A3.2.5	Mapping of IPCC Source Categories to NAEI Source Categories:
IPCC Table 4

| - IPCC Source Category | - NAEI Source Category |
| --- | --- |
| - 4A1 Enteric Fermentation: Cattle | - Dairy Cattle Enteric - Other Cattle Enteric |
| - 4A2 Enteric Fermentation: Buffalo | - Not Occurring |
| - 4A3 Enteric Fermentation: Sheep | - Sheep Enteric |
| - 4A4 Enteric Fermentation: Goats | - Goats Enteric |
| - 4A5 Enteric Fermentation: Camels & Llamas | - Not Occurring |
| - 4A6 Enteric Fermentation: Horses | - Horses Enteric |
| - 4A7 Enteric Fermentation: Mules & Asses | - Not Occurring |
| - 4A8 Enteric Fermentation: Swine | - Pigs Enteric |
| - 4A9 Enteric Fermentation: Poultry | - Not Occurring |
| - 4A10 Enteric Fermentation: Other: Deer | - Deer Enteric |
| - 4B1 Manure Management: Cattle | - Dairy Cattle Wastes - Other Cattle Wastes |
| - 4B2 Manure Management: Buffalo | - Not Occurring |
| - 4B3 Manure Management: Sheep | - Sheep Wastes |
| - 4B4 Manure Management: Goats | - Goats Wastes |
| - 4B5 Manure Management: Camels & Llamas | - Not Occurring |
| - 4B6 Manure Management: Horses | - Horses Wastes |
| - 4B7 Manure Management: Mules & Asses | - Not Occurring |
| - 4B8 Manure Management: Swine | - Pigs Wastes |
| - 4B9 Manure Management: Poultry | - Broilers Wastes - Laying Hens Wastes - Other Poultry |
| - 4B9a Manure Management: Other: Deer | - Deer Wastes |
| - 4B10 Anaerobic Lagoons | - Not Occurring |
| - 4B11 Liquid Systems | - Manure Liquid Systems |
| - 4B12 Solid Storage and Dry Lot | - Manure Solid Storage and Dry Lot |
| - 4B13 Other | - Manure Other |
| - 4C Rice Cultivation | - Not Occurring |
| - 4D 1 Agricultural Soils: Direct Soil Emissions - 4D 2 Agricultural Soils: Animal Emissions - 4D 4 Agricultural Soils: Indirect Emissions | - Agricultural Soils Fertiliser - Agricultural Soils Crops |
| - 4E Prescribed Burning of Savannahs | - Not Occurring |
| - 4F1 Field Burning of Agricultural Residues: Cereals | - Barley Residue - Wheat Residue - Oats Residue |
| - 4F5 Field Burning of Agricultural Residues: Other: Linseed | - Linseed Residue |
| - 4G Other | - Not Estimated |

Table A3.2.6	Mapping of IPCC Source Categories to NAEI Source Categories:
IPCC Table 5

| - IPCC Source Category1 | - NAEI Source Category |
| --- | --- |
| - 5A Changes in Forest and Other Woody Biomass Stocks | - Not estimated |
| - 5B Forest and Grassland Conversion | - 5B2 Deforestation |
| - 5C Abandonment of Managed Lands | - Not estimated |
| - 5D CO2 Emissions and Removals from Soil | - Agricultural Soils: Limestone - Agricultural Soils: Dolomite |
| - 5E Other | - Not estimated |

- 1	Categories 5A, 5B, 5C and 5E are not included in the NAEI because a time series back to 1970 is unavailable. They are included in the Green House Gas Inventory.

Table A3.2.7	Mapping of IPCC Source Categories to NAEI Source Categories:
IPCC Tables 6 & 7

| - IPCC Source Category | - NAEI Source Category |
| --- | --- |
| - 6A1 Managed Waste Disposal on Land | - Landfill - Waste disposal – benzoles and tars |
| - 6A2 Unmanaged Waste Disposal on Land | - Not Occurring |
| - 6A3 Other | - Not Occurring |
| - 6B1 Industrial Wastewater - 6B2 Domestic and Commercial Wastewater - 6B3 Other | - Sewage Sludge Disposal |
| - 6C Waste Incineration | - Incineration: MSW - Incineration: Sewage Sludge - Incineration: Clinical - Incineration: Cremation |
| - 6D Other Waste | - Not estimated |
| - 7 Other | - Not estimated |

  - Energy (CRF sector 1)

The previous two sections defined the fuels and source categories used in the NAEI and the GHGI.  The aim of this section is to describe in detail the methodology used to estimate the emissions arising from fuel combustion for energy.  These sources correspond to IPCC Table 1A.

There is little continuous monitoring of emissions performed in the UK; hence information is rarely available on actual emissions over a specific period of time from an individual emission source.  In any case, emissions of CO2 from fuel are probably estimated more accurately from fuel consumption data.  The majority of emissions are estimated from other information such as fuel consumption, distance travelled or some other statistical data related to the emissions.  Estimates for a particular source sector are calculated by applying an emission factor to an appropriate statistic. That is:

Total Emission	=	Emission Factor		Activity Statistic

Emission factors are typically derived from measurements on a number of representative sources and the resulting factor applied to the UK environment.

For the indirect gases, emissions data are sometimes available for individual sites from databases such as the Environment Agency’s Pollution Inventory (PI).  Hence the emission for a particular sector can be calculated as the sum of the emissions from these point sources. That is:

Emission    =    	 	Point Source Emissions

However it is still necessary to make an estimate of the fuel consumption associated with these point sources, so that the emissions from non-point sources can be estimated from fuel consumption data without double counting.  In general the point source approach is only applied to emissions of indirect greenhouse gases for well-defined point sources (e.g. power stations, cement kilns, coke ovens, refineries).  Direct greenhouse gas emissions and most non-industrial sources are estimated using emission factors.

    - Basic combustion module

For the pollutants and sources discussed in this section the emission results from the combustion of fuel.  The activity statistics used to calculate the emission are fuel consumption statistics taken from DTI (2004).  A file of the fuel combustion data used in the inventory is provided on a CD ROM attached to this report.  Emissions are calculated according to the equation:

- E(p,s,f)	=	A(s,f)		e(p,s,f)

- where

- E(p,s,f)	=	Emission of pollutant p from source s from fuel f  (kg)

- A(s,f)	=	Consumption of fuel f by source s  (kg or kJ)

- e(p,s,f)	=	Emission factor of pollutant p from source s from fuel f	 (kg/kg or kg/kJ)

- The pollutants estimated in this way are:

- Carbon dioxide as carbon

- Methane

- Nitrous oxide

- NOx as nitrogen dioxide

- NMVOC

- Carbon monoxide

- Sulphur dioxide

- The sources covered by this module are:

- Domestic

- Miscellaneous

- Public Service

- Refineries (Combustion)

- Iron & Steel (Combustion)

- Iron & Steel (Blast Furnaces)

- Iron & Steel (Sinter Plant)

- Other Industry (Combustion)

- Autogenerators

- Gas Production

- Collieries

- Production of Nuclear Fuel

- Coastal Shipping

- Fishing

- Agriculture

- Ammonia (Combustion)

- Railways (Stationary Sources)

- Aircraft Military

- Shipping Naval

- The fuels covered are listed in Annex 3, Section 3.1, though not all fuels occur in all sources.

- Beginning with the 2003 inventory, a major change has been made to the estimation of CO & NOx emissions from industrial, commercial/institutional, and domestic sources.  Whereas, previously, a single emission factor would be applied for a given source/fuel combination, the new methodology allows source/fuel combinations to be further broken down by a) thermal input of combustion devices; b) type of combustion process e.g. boilers, furnaces, turbines etc.  Different emission factors are applied to these subdivisions of the source/fuel combination.   Most of these emission factors are taken from literature sources, predominantly from US EPA, (2004), EMEP/CORINAIR (2003), and Walker et al, (1985).  Some emissions data reported in the Pollution Inventory (Environment Agency, 2004) are also used to generate emission factors.

- Tables A3.3.1 to A3.3.4 list the emission factors used in this module.  Emission factors are expressed in terms of kg pollutant/tonne for solid and liquid fuels, and g/TJ gross for gases.  This differs from the IPCC approach, which expresses emission factors as tonnes pollutant/TJ based on the net calorific value of the fuel.  For gases the NAEI factors are based on the gross calorific value of the fuel.  This approach is used because the gas consumption data in DTI (2004) are reported in terms of energy content on a gross basis.

During 2004, an extensive review of carbon emission factors (CEFs) within the UK inventory was carried out.  This was a key recommendation of the Peer Review carried out by Simmons (2002).  A review of some of the CEFs of liquid fuels was carried out in the late 1990s, but many of the carbon emission factors within the inventory dated from the late 1980s or early 1990s.  The CEFs selected for review were based on the contribution of those fuels to the national total carbon emission.  The major fuels that were considered in the review were: Aviation Spirit, Aviation Turbine Fuel, Coal, Coke, Diesel, Petrol, Fuel Oil, Gas Oil, Naphtha, Natural Gas, Other Petroleum Gas (OPG), Petroleum Coke, Blast Furnace Gas (BFG) and Coke Oven Gas (COG).  As a result, a number of revisions to emissions have been made, mostly in the Energy sector but also in the Industrial Processes sector.  The overall effect of applying the new emission factors was to increase estimated carbon emissions.

- The CEFs used within this inventory are now based on current UK data and are appropriate for the UK.  Many CEFs will now be reviewed annually, and updated if necessary.  CEFs for solid and liquid fuels are in terms of mass.  CEFs for gases fuels are in terms of the energy content of the gas consumed (g carbon / GJ gross or g carbon / Mtherm).

- As required by the IPCC Guidelines, carbon dioxide emissions from biomass combustion are not included in the National Total, but emissions of other pollutants are.  The NAEI includes emissions from the combustion of wood in the industrial and domestic sectors as well as the combustion of straw in agriculture.  DTI (2004) reports estimates of wood and straw combustion for energy use and the estimate of emissions is based on these data.  Emission factors are given in Table A3.3.3.  Emissions from biogas and poultry litter/meat and bone meal combustion are included in the GHGI and are discussed in Section A3.3.3 since they occur mainly in electricity generation.

- Waste oil recycling companies produce a good quality recycled oil which is marketed or sold back to the originator.  This oil is however only a proportion of the total waste oil burnt, much of which is burnt in small waste oil burners in the commercial and construction sectors.  A revised methodology has been introduced for the 2003 GHGI to estimate consumption of waste oils by industry (see Section A.3.3.9).

- For most of the combustion source categories, the emission is estimated from fuel consumption data reported in DUKES and an emission factor appropriate to the type of combustion e.g. commercial gas fired boiler.  However the DUKES category ‘Other Industries’ covers a range of sources and types, so the Inventory disaggregates this category into a number of sub-categories, namely:

- Other Industry

- Other Industry Off-road

- Ammonia Feedstock (natural gas only)

- Ammonia (Combustion) (natural gas only)

- Cement (Combustion)

- Lime Production (non-decarbonising)

- Thus the NAEI category Other Industry refers to stationary combustion in boilers and heaters by industries not covered elsewhere (including the chemicals, food & drink, non-ferrous metal, glass, ceramics & bricks, textiles & engineering sectors).  The other categories are estimated by more complex methods discussed in the sections indicated.  For certain industrial processes (e.g. Lime production, cement production and ammonia production), the methodology is discussed in Section A3.4 as the estimation of the fuel consumption is closely related to the details of the process.  However, for these processes, where emissions arise from fuel combustion for energy production, these are reported under IPCC Table 1A.  The fuel consumption of Other Industry is estimated so that the total fuel consumption of these sources is consistent with DUKES (DTI, 2004).

- According to IPCC 1996 Revised Guidelines, electricity generation by companies primarily for their own use is autogeneration, and the emissions produced should be reported under the industry concerned.  However, most National Energy Statistics (including the UK) report emissions from electricity generation as a separate category.  The UK inventory attempts to report as far as possible according to the IPCC methodology.  Hence autogenerators would be reported in the relevant sector where they can be identified e.g. iron and steel (combustion), refineries (combustion).  In some cases the autogenerator cannot be identified from the energy statistics so it would be classified as other industry (combustion).  This means that the split between iron and steel (combustion) and other industry (combustion) may be uncertain.  Also, for certain sectors, data on fuel deliveries are used in preference to data on fuel consumption because deliveries will include autogeneration whereas consumption does not.

Table A3.3.1	Emission Factors for the Combustion of Liquid Fuels for 2003 (kg/t)

| Fuel | Source | Caj | CH4 | N2O | NOx | CO | NMVOC | SO2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ATF | Aircraft Military | 859a | 0.103ad | 0.1g | 8.5ad | 8.2ad | 1.1ad | 0.66z |
| Burning Oil | Domestic | 859a | 0.309b | 0.0265g | 3.23ay | 1.85l | 0.048f | 0.44z |
| Burning Oil (P) | Domestic | 859a | 0.309b | 0.0265g | 3.23l | 1.85l | 0.048f | 0.02z |
| Burning Oil | Other Industry | 859a | 0.0432b | 0.0265g | 3.32l | 0.19l | 0.028e | 0.44z |
| Burning Oil | Public Service, Railways (Stationary) | 859a | 0.0432b | 0.0265g | 2.05l | 0.16l | 0.048f | 0.44z |
| Burning Oil | Miscellaneous | 859a | 0.0432b | 0.0265g | 2.70l | 0.16l | 0.048f | 0.44z |
| Gas Oil | Agriculture | 870a | 0.225b | 0.026g | 2.70l | 0.38l | 0.048f | 2.78z |
| Gas Oil | Domestic | 870a | 0.303b | 0.026g | 3.19l | 1.82l | 0.048f | 2.78z |
| Gas Oil | Fishing; Coastal Shipping, Naval; International Marine | 870a | 0.288c | 0.2t | 57t | 7.4t | 2.11c | 19.4h |
| Gas Oil | Iron&Steel | 870a | 0.0432b | 0.026g | 17.04l | 6.70l | 0.028f | 2.78z |
| Gas Oil | Refineries | 870a | 0.0432b | 0.026g | 4.56k | 0.24i | 0.028f | 2.78z |
| Gas Oil | Other Industry | 870a | 0.0432b | 0.026g | 4.02l | 0.46l | 0.028f | 2.78z |
| Gas Oil | Public Service | 870a | 0.0432b | 0.026g | 2.44l | 0.38l | 0.048f | 2.78z |
| Gas Oil | Miscellaneous | 870a | 0.0432b | 0.026g | 0.64l | 0.08l | 0.048f | 2.78z |
| Fuel Oil | Agriculture | 877a | 0.122b | 0.0243g | 7.69l | 0.31l | 0.14f | 20.4z |
| Fuel Oil | Public Service | 877a | 0.122b | 0.0243g | 6.39l | 0.71l | 0.14f | 20.4z |
| Fuel Oil | Miscellaneous | 877a | 0.122b | 0.0243g | 1.56l | 0.06l | 0.14f | 20.4z |
| Fuel Oil | Fishing; Coastal Shipping, International Marine | 877a | 0.288c | 0.2t | 57t | 7.4t | 2.11c | 56.4h |
| Fuel Oil | Domestic | 877a | 0.303b | 0.0243g | 3.19l | 1.82l | 0.14f | 20.4z |
| Fuel Oil | Iron&Steel | 877a | 0.122b | 0.0243g | 6.20l | 0.74l | 0.035f | 20.4z |
| Fuel Oil | Railways (Stationary) | 877a | 0.122b | 0.0243g | 6.39l | 0.71l | 0.14f | 20.4z |
| Fuel Oil | Other Industry | 877a | 0.122b | 0.0243g | 7.00l | 0.76l | 0.035f | 20.4z |
| Fuel Oil | Refineries (Combustion) | 877a | 0.122b | 0.0243g | 3.42ag | 1.08ag | 0.035f | 11.9ag |
| Lubricants | Other Industry | 865x | 0.0432e | 0.026e | 4.56k | 0.26f | 0.14f | 11.4x |
| Naphtha | Refineries | 854a | 0.122j | 0.0182w | 4.62k | 0.24e | 0.028e | 0.2af |
| Miscellaneous | Refineries | 800y | 0.122j | 0.0247w | 4.62k | 0.24e | 0.028e | 0.2af |
| Petrol | Refineries | 855a | 0.0432e | 0.0179w | 4.62k | 0.24e | 0.028e | 0.08z |

Table A3.3.2	Emission Factors for the Combustion of Coal for 2003 (kg/t)

| - Source | - Caj | - CH4 | - N2O | - NOx | - CO | - NMVOC | - SO2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - Agriculture | - 650.5a | - 0.011o | - 0.155w | - 4.75l | - 8.25l | - 0.05o | - 19.2aa |
| - Collieries | - 685.0a | - 0.011o | - 0.061w | - 4.75l | - 8.25l | - 0.05o | - 24.1aa |
| - Domestic | - 697.0a | - 15.7o | - 0.119w | - 3.47l | - 180.7l | - 14o | - 17.1aa |
| - Iron and Steel (Combustion) | - 693.8a | - 0.011o | - 0.0704w | - IE | - IE | - 0.05o | - 19.2aa |
| - Lime Production (Combustion) | 628.8a | 0.011o | 0.213w | 46.7v | 12.7v | 0.05o | NE |
| - Miscellaneous | - 722.0a | - 0.011o | - 0.143w | - 4.73l | - 7.95l | - 0.05o | - 19.2aa |
| - Public Service | - 722.0a | - 0.011o | - 0.143w | - 4.75l | - 8.93l | - 0.05o | - 19.2aa |
| - Other Industry | - 633.5a | - 0.011o | - 0.213w | - 3.98l | - 1.65l | - 0.05o | - 19.2aa |
| - Railways | - 722.0a | - 0.011o | - 0.0753w | - 4.75l | - 8.93l | - 0.05o | - 19.2aa |
| - Autogenerators | - 628.8a | - 0.02o | - 0.0658w | - 5.94l | - 1.33l | - 0.03o | - 19.2aa |

Table A3.3.3	Emission Factors for the Combustion of Solid Fuels 2003 (kg/t)

| - Fuel | - Source | - Caj | - CH4 | - N2O | - NOx | - CO | - NMVOC | - SO2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Anthracite | - Domestic | - 820.7a | - 2o | - 0.14w | - 3.38k | - 202.8q | - 1.7o | - 13.5aa |
| - Coke | - Agriculture | - 764.8r | - 0.011p | - 0.144w | - 5.49l | - 3.77l | - 0.05p | - 19ab |
| - Coke | - Coke Production | - 764.8r | - 0.011p | - 0.221w | - IE | - IE | - IE | - IE |
| - Coke | - SSF Production | - 764.8r | - 0.011p | - 0.221w | - 4.47k | - 4.1p | - 0.05p | - 19ab |
| - Coke | Domestic | - 764.8r | - 5.8o | - 0.111w | - 3.04l | - 118.6l | - 4.9o | - 13.5aa |
| - Coke | - I&Sak (Sinter Plant) | - 764.8r | - 1.24ae | - 0.221w | - 12.28ae | - 358.4ae | - 0.29ae | - 13.4ae |
| - Coke | - I&Sak (Combustion) | - 764.8r | - 0.011p | - 0.221w | - 0.87l | - 226.2l | - 0.05p | - 19ab |
| - Coke | - Other Industry | - 764.8r | - 0.011p | - 0.221w | - 5.49l | - 3.77l | - 0.05p | - 19ab |
| - Coke | - Railways | - 764.8r | - 0.011p | - 0.144w | - 5.49l | - 3.77l | - 0.05p | - 19ab |
| - Coke | - Miscellaneous; Public Service | - 764.8r | - 0.011p | - 0.144w | - 5.49l | - 3.77l | - 0.05p | - 19ab |
| - MSW | - Miscellaneous | - 75ah | - 0.0008ac | - 0.03u | - 1.31v | - 0.12v | - 0.008v | - 0.07v |
| - Petroleum Coke | - Domestic | - 930a | - NE | - NE | - 3.95q | - 158q | - NE | - 19ab |
| - Petroleum Coke | - Refineries | - 930a | - 0.0164o | - 0.31w | - 11.6ag | - 1.14ag | - 0.075o | - 38.2ag |
| - SSF | - Agriculture; Miscellaneous; Public Service | - 766.3n | - 0.011p | - 0.143w | - 4.67k | - 4.67q | - 0.05p | - 19ab |
| - SSF | - Domestic | - 774.2n | - 5.8o | - 0.112w | - 3.11k | - 124.4q | - 4.9o | - 16ab |
| - SSF | - Other Industry | - 766.3n | - 0.011p | - 0.218w | - 4.67k | - 4.67q | - 0.05p | - 19ab |
| - Straw | - Agriculture | - 418g | - 4.28g | - 0.057g | - 1.5g | - 75g | - 9g | - 0 |
| - Wood | - Domestic | - 264g | - 3.61b | - 0.038g | - 0.5k | - 50g | - 17k | - 0.108f |

Table A3.3.4	Emission Factors for the Combustion of Gaseous Fuels 2003 (g/GJ gross)

| - Fuel | - Source | - Caj | - CH4 | - N2O | - NOx | - CO | - NMVOC | - SO2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Blast Furnace Gas | - Coke Production | - 71085a | - 0.3t | - 2.0k | - 79k | - 39.5k | - 5.6k | - 0 |
| - Blast Furnace Gas | - I&Sak (Combustion), I&Sak (Flaring) | - 71085a | - 112t | - 2.0k | - 79k | - 39.5k | - 5.6k | - 0 |
| - Blast Furnace Gas | - Blast Furnaces | - 71085a | - 112t | - 2.0k | - 48.2v | - 39.5k | - 5.6k | - 0 |
| - Coke Oven Gas | - Other Sources | - 11450a | - 7.5b+ | - 1.8k | - 80.5k | - 40.0k | - 4.35k | - 250v |
| - Coke Oven Gas | - I&Sak Blast Furnaces | - 11450a | 7.5b+ | 1.8k | 48.2v | 40.0k | 4.35k | 250v |
| - Coke Oven Gas | - Coke Production | - 11450a | 7.5b+ | 1.8k | 336v | 40.0k | 4.35k | 250v |
| - LPG | - Domestic | - 16227a | 0.92m | 0.09g | 64.8f | 8.9f | 1.55f | 0 |
| - LPG | - I&Sak, Other Industry, Refineries, Gas Production | - 16227a | 0.92m | 0.09g | 89.3f | 15.2f | 1.55f | 0 |
| - Natural Gas | - Agriculture | - 14016a | - 3.6b | - 0.09g | - 39.2l | - 2.13l | - 2.21f | - 0 |
| - Natural Gas | - Miscellaneous | - 14016a | - 3.6b | - 0.09g | - 48.9l | - 4.74l | - 2.21f | - 0 |
| - Natural Gas | - Public Service | - 14016a | - 3.6b | - 0.09g | - 47.7l | - 4.22l | - 2.21f | - 0 |
| - Natural Gas | - Coke Production, SSF Prodnal, | - 14016a | - 3.6b | - 0.09g | - 90.0b | - 2.37l | - 2.21f | - 0 |
| - Natural Gas | - Refineries | - 14016a | - 3.6b | - 0.09g | - 70.0k | - 2.37l | - 2.21f | - 0 |
| - Natural Gas | - Blast Furnaces | - 14016a | - 3.6b | - 0.09g | - 48.2v | - 2.37l | - 2.21f | - 0 |
| - Natural Gas | - Domestic | - 14016a | - 2.7b | - 0.09g | - 69.2l | - 30.8l | - 2.21f | - 0 |
| - Natural Gas | - Gas Prodnal, | - 14016a | - 3.6b | 0.09g | 105.1l | 17.4l | 2.21f | - 0 |
| - Natural Gas | - I&Sak | - 14016a | - 3.6b | 0.09g | 241.2l | 240l | 2.21f | - 0 |
| - Natural Gas | - Railways | - 14016a | - 3.6b | 0.09g | 71.2l | 33.8l | 2.21f | - 0 |
| - Natural Gas | - Other Industry, Nuclear Fuel Prodnal, Collieries | - 14016a | - 3.6b | - 0.09g | - 88.4l | - 23.0l | - 2.21f | - 0 |
| - Natural Gas | - Autogenerators | - 14016a | - 5.5g | - 3.33g | - 81.5l | - 22.6l | - 2.21f | - 0 |
| - Natural Gas | - Ammonia (Combustion) | - 14016a | - 3.6b | - 0.09g | - 143.9d | - NE | - 2.21f | - 0 |
| - OPG | - Gas production, Other Industry | - 15582a | - 3.07b+ | - NE | - 70.0k | - 2.37i | - 1.55f | - 0 |
| - OPG | - Refineries (Combustion) | - 15582a | - 3.07b+ | - NE | - 72.3z | - 5.11z | - 1.55f | - 0 |
| - Colliery Methane | - All Sources | - 13909a | - 3.6s | - 0.09g | - 70.0k | - 2.37i | - 2.21f | - 0 |
| - Sewage Gas | - Public Services | - 27404g | - 615m | - 0.09g | - 39.8f | - 4.2f | - 2.16f | - 0 |
| - Landfill Gas | - Miscellaneous | - 27404g | - 615m | - 0.09g | - 23.2f | - 73.0f | - 1.44f | - 0 |

- Footnotes to Tables A3.3.1 to A3.3.4

| - a | - Carbon Factor Review (2004), Review of Carbon Emission Factors in the UK Greenhouse Gas Inventory. Report to UK Defra. Baggott, SL, Lelland, A, Passant and Watterson, JW - (UKPIA (2004)-Liquid Fuels, Transco (2004) – Natural Gas, Quick (2004) and AEP(2004) – Power Station Coal) |
| --- | --- |
| - b | - CORINAIR (1992) |
| - b+ | - Derived from CORINAIR(1992) assuming 30% of total VOC is methane |
| - c | - Methane facto r estimated as 12% of total hydrocarbon emission factor taken from EMEP/CORINAIR(1996) based on speciation in IPCC (1997c) |
| - d | - Based on operator data: Terra Nitrogen (2004), Invista (2004), BP Chemicals (2004) |
| - e | - As for gas oil |
| - f | - USEPA (2004) |
| - g | - IPCC (1997c) |
| - h | - EMEP (1990) |
| - i | - Walker et al (1985) |
| - j | - As for fuel oil. |
| - k | - EMEP/CORINAIR (2003) |
| - l | - Netcen estimate based on application of literature emission factors at a greater level of detail than the sector listed (see Section A.3.3.1). |
| - m | - USEPA (1997) |
| - n | - British Coal (1989) |
| - o | - Brain et al, (1994) |
| - p | - As for coal |
| - q | - EMEP/CORINAIR (2004) |
| - r | - Netcen estimate based on carbon balance |
| - s | - As for natural gas |
| - t | - EMEP/CORINAIR (1996) |
| - u | - IPCC (2000) |
| - v | - Emission factor derived from emissions reported in the Pollution Inventory (Environment Agency, 2004) |
| - w | - Fynes et al (1994) |
| - x | - Passant (2004) |
| - y | - UKPIA (1989) |
| - z | - Emission factor derived from data supplied by UKPIA (2003) |
| - aa | - Emission factor for 2002 based on data provided by UK Coal (2003), Scottish Coal (2003), Celtic Energy (2000), Tower (2001), Betwys (2000) |
| - ab | - Munday (1990) |
| - ac | - Estimated from THC data in CRI (Environment Agency, 1997) assuming 3.% methane split given in EMEP/CORINAIR (1996) |
| - ad | - EMEP/CORINAIR (1999) |
| - ae | - Netcen estimate based on data from Environment Agency (2004) and Corus (2000, 2001, 2002) |
| - af | - UKPIA (2002) |
| - ag | - Netcen estimate based on data from Environment Agency (2004), UKPIA, DUKES, and other sources |
| - ah | - Royal Commission on Environmental Pollution (1993) |
| - ai |  |
| - aj | - Emission factor as mass carbon per unit fuel consumption |
| - ak | - I&S = Iron and Steel |
| - al | - Prodn = Production |
| - NE | - Not estimated |
| - IE | - Included elsewhere |

    - Conversion of energy activity data and emission factors

- The NAEI databases store activity data in Mtonnes for solid and liquid fuels and Mtherms (gross) for gaseous fuels.  Emission factors are in consistent units namely: ktonnes/Mtonne for solid and liquid fuels and ktonnes/Mtherm (gross) for gaseous fuels.  For some sources emission factors are taken from IPCC and CORINAIR sources and it is necessary to convert them from a net energy basis to a gross energy basis.  For solid and liquid fuels:

- Hn	=	m hg f

- and for gaseous fuels:

- Hn	=	Hg f

- where:

- Hn	Equivalent energy consumption on net basis				(kJ)

- m	Fuel consumption						(kg)

- hg	Gross calorific value of fuel					(kJ/kg)

- f	Conversion factor from gross to net energy consumption		(-)

- Hg	Energy Consumption on gross basis					(kJ)

- In terms of emission factors:

- em	=	en hg f

- or

- eg	=	en f

- where:

- em	Emission factor on mass basis					(kg/kg)

- en	Emission factor on net energy basis					(kg/kJ net)

- eg	Emission factor on gross energy basis				(kg/kJ gross)

- The gross calorific values of fuels used in the UK are tabulated in DTI, (2004).  The values of the conversion factors used in the calculations are given in Table A3.3.5.

Table A3.3.5		Conversion Factors for Gross to Net Energy Consumption

| - Fuel | - Conversion Factor |
| --- | --- |
| - Other Gaseous Fuels | - 0.9 |
| - Solid and Liquid Fuels | - 0.95 |
| - LPG and OPG | - 0.92 |
| - Blast Furnace Gas | - 1.0 |

- The values given for solid, liquid and other gaseous fuels are taken from IPCC Guidelines (IPCC, 1997c).  The value used for LPG is based on the calorific value for butane, the major constituent of LPG (Perry et al, 1973).  Blast furnace gas consists mainly of carbon monoxide and carbon dioxide.  Since little hydrogen is present, the gross calorific value and the net calorific values will be the same.

    - Energy Industries (1A1)

      - Electricity generation

- The NAEI category Power Stations is mapped onto 1A1 Electricity and Heat Production.  In the NAEI, the category power stations aims to report as nearly as possible emissions from electricity generation by companies whose main business is producing electricity (Major Power Producers) and hence excludes autogenerators.  The fuel consumption entries from DUKES are chosen to obtain the best match with this definition.  The coal and natural gas entries used are very close to this definition but the fuel oil entry does contain a small contribution from transport undertakings and groups of factories.  In 1999 and 2000, the fuel oil consumption reported by DUKES was significantly lower than that estimated from returns from the power generators.  In the inventory, the power generators’ figures are used otherwise the emission factors implied by the data reported to the Environment Agency would be impossibly high.  A correction is applied to other industry (combustion) so that total UK fuel oil consumption corresponds to that reported in DUKES.  The following paragraph gives further information about the action taken to resolve the inconsistencies.

Table A3.3.6	Emission Factors for Power Stations in 2003 [A time series of carbon emission factors can be found in the background energy tables on the accompanying CD]

| - Source | - Unit | - CO21 | - CH4 | - N2O | - NOx | - CO | - NMVOC | - SO2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Coal | - Kt/Mt | - 627.2a | - 0.02e | - 0.073l | - 6.366n | - 1.031n | - 0.0258n | - 12.75n |
| - Fuel Oil | - Kt/Mt | - 879a | - 0.0054f | - 0.0243h | - 8.264n | - 1.401n | - 0.0229n | - 26.83n |
| - Gas Oil | - Kt/Mt | - 870a | - 0.0432g | - 0.0260h | - 3.784n | - 0.649n | - 0.0638n | - 2.85n |
| - Natural gas | - Kt/Mth | - 1.480a | - 0.00058h | - 0.00035m | - 0.004n | - 0.0014n | - 0.00054n | - 9.77E-05n |
| - MSW | - Kt/Mt | - 75d | - NE | - NE | - 1.385o | - 0.121o | - 0.00813o | - 0.0715o |
| - Sour gas | - Kt/Mth | - 1.916c | - 2.3E-05i | - 9.5E-06j | - 0.0062n | - 0.012o | - 9.15E-05o | - 0.00101n |
| - Poultry Litter | - Kt/Mt | - NE | - 0.437j | - 0.0584j | - 0.782n | - 0.605o | - NE | - 0.478n |
| - Sewage Gas | - Kt/Mth | - NE | - NE | - 9.5E-06j | - 0.0042k | - 0.00046k | - 0.00015k | - NE |
| - Waste Oils | - Kt/Mt | - 864.84b | - NE | - NE | - 8.264n | - 1.4n | - 0.0229n | - 26.83n |
| - Landfill gas | - Kt/Mth | - NE | - 0.0649p | - 9.5E-06j | - 0.0025k | - 0.0077k | - 0.00023k | - NE |

- Footnotes to A3.3.6 (	Emission Factors for Power Stations)

| - 1 | - Emission factor as mass carbon/ unit fuel consumption |
| --- | --- |
| - a | - Baggott et al (2004) - Review of Carbon Emission Factors in the UK Greenhouse Gas Inventory. Report to UK Defra. Baggott, SL, Lelland, A, Passant and Watterson, JW - (UKPIA (2004)-Liquid Fuels, Transco (2004) – Natural Gas, Quick (2004) and AEP(2004) – Power Station Coal) |
| - b | - Passant, N.R., Emission factors programme Task 1 – Summary of simple desk studies (2003/4), AEA Technology Plc, Report No AEAT/ENV/R/1715/Issue 1, March 2004 |
| - c | - Stewart et al (1996a) Emissions to Atmosphere from Fossil Fuel Power Generation in the UK, AEAT-0746, ISBN 0-7058-1753-3 |
| - d | - RCEP (Royal Commission on Environmental Protection) 17th Report - Incineration of Waste, 1993 |
| - e | - Brain (1994) |
| - f | - Stewart et al (1996) estimated from total VOC factor assuming 27.2% is methane after USEPA(1997) |
| - g | - CORINAIR (1992) |
| - h | - IPCC (1997c) |
| - i | - EMEP/CORINAIR (1996) |
| - j | - IPCC (1997) |
| - k | - USEPA (2004) |
| - l | - Fynes et al (1994) |
| - m | - Stewart (1997) |
| - n | - Based on reported emissions data from the EA Pollution Inventory (Environment Agency, 2004), SEPA’s EPER inventory (SEPA, 2004), NI DoE’s ISR list (NI DoE, 2004) and direct communications with plant operators (Pers. Comms., 2004) |
| - o | - Environment Agency (2004) |
| - p | - USEPA (1997) |
| - NE | - Not Estimated |

- The emission factors used for Power Stations are shown in Table A3.3.6.  The NOx and SO2 emissions from coal and oil stations are based on estimates for individual power stations released by the Environment Agency (2004).  The Environment Agency emissions are reported on a power station basis so those from coal fired plant will include emissions from the fuel oil used to light up the boilers.  A correction has been applied to the data so that the coal emissions reported in the NAEI pertain only to the coal burnt, and the oil emissions apply only to the oil burnt.  This is necessary to fulfill IPCC and UNECE reporting requirements.  The CO and NMVOC emissions are reported in the Pollution Inventory (Environment Agency, 2004) on the same basis, hence a similar correction is made.  Details of the corrections made are provided in the following paragraph.

- National emission estimates for SO2, NOx and CO are based on estimates for each power station provided by the process operators via UK regulators. These data relate to total emissions from each power station and will include the emissions from more than one fuel type in many cases e.g. coal and fuel oil at coal-fired stations where fuel oil is used during start-up periods.  The process operators do not provide estimates broken down by fuel type so emissions by fuel type has to be estimated.  Consumption of each fuel type is either known or can be estimated for each station, and emission factors for each fuel type are calculated from the overall emission estimates by assuming that the calculated factors will proportional to each other by the same ratio as ratios derived for each combination of fuel types from a set of 'default', literature-based emission factors.

- From 1991 to 1997 some UK power stations burnt orimulsion, an emulsion of bitumen and water.  DTI (1998) gives the UK consumption of orimulsion.  This fuel was only used by the electricity supply industry so these data were used in the category power stations.  The carbon content of the fuel was taken from the manufacturers specification (BITOR, 1995).  The emissions of NOx, SO2, NMVOC and CO were taken from Environment Agency (1998) but emission factors for methane and N2O were derived from those of heavy fuel oil but adjusted on the basis of the gross calorific value.  The CO emission factor is based on measured data. This fuel is no longer used.

- Electricity has been generated from the incineration of municipal solid waste (MSW) for some years now, though generation capacity has recently increased markedly owing to construction and upgrading of incinerators to meet new regulations which came into force at the end of 1996.  Data are available (DTI, 2004) on the amount of waste used in heat and electricity generation and the emissions from the incinerators (Environment Agency, 2004).  In earlier inventories, these emissions were reported as waste disposal, but it is now possible to report the electricity generation component separately under power stations and the heat component under miscellaneous.  Since 1997, all MSW incinerators have generated electricity so the waste incineration category has reduced to zero.

- In addition to MSW combustion, the inventory reports emissions from the combustion of scrap tyres.  The tyre emissions are based on estimates compiled by DTI (2000) and a carbon emission factor based on the carbon content of tyres (Ogilvie, 1995).  IPCC default factors based on oil are used.  In 2000 the tyre burning plant closed down.

- Also included are emissions from three plants that burnt poultry litter and wood chips.  In 2000 one of these was converted to burn meat and bone meal.  The carbon emissions are not included in the UK total since these derive from biomass, but emissions are reported for information in the CRF. Emission factors are based on Environment Agency (2004) data and IPCC (1997) defaults for biomass. Fuel use data is provided directly by the plant operators. There is considerable variation in emission factors due to the variability of fuel composition.

- Emission estimates were made from the generation of electricity from landfill gas and sewage gas (DTI, 2004).  It was assumed that the electricity from this source was fed into the public supply or sold into non-waste sectors and hence classified as public power generation.  The gases are normally used to power reciprocating gas (or duel-fuel engines), which may be part of combined heat and power schemes.  Emission factors for landfill gas and sewage gas burnt in reciprocating engines have not been found so those for these gases burnt in gas turbines have been used instead (USEPA, 2004).  DTI (2004) reports the energy for electricity production and for heat production separately.  The emissions for electricity generation are categorised under public power whilst those for heat production are reported under miscellaneous for landfill gas and public services for sewage gas.  The carbon emissions are not included in the National Total since these derive from biomass but emissions are reported for information in the CRF.

      - Petroleum refining

- The NAEI category refinery (combustion) is mapped onto the IPCC category 1A1b Petroleum Refining.  The emission factors used are shown in Table A3.3.1.  Included in this category is an emission from the combustion of petroleum coke.  This emission arises from the operation of fluidized bed catalytic crackers.  During the cracking processes coke is deposited on the catalyst degrading its performance.  The catalyst must be continuously regenerated by burning off the coke.  The hot flue gases from the regeneration stage are used as a source of heat for the process.  Since the combustion provides useful energy and the estimated amount of coke consumed is reported (DTI, 2004), the emissions are reported under 1A1b Petroleum Refining rather than as a fugitive emission under 1B2.  Emission factors are either based on operators' data (UKPIA, 2004) or IPCC (1997) defaults for oil.  The NAEI definition of refinery (combustion) aims to include all combustion sources and includes refinery fuels, electricity generation in refineries and fuel oils burnt in the petroleum industry.  In previous inventories the consumption of LPG and OPG by gas separation plants was classified under refinery (combustion) refineries. It has become clear that these processes occur in oil terminals, reported under offshore oil and gas (see Section A3.3.8).  However, as OPG and LPG are already reported under offshore own gas use this was a double count and was removed in the 1999 Inventory.

      - Manufacture of solid fuels

- The mappings used for these categories are given in Sections A3.1-3.2 and emission factors for energy consumption in these industries are given in Tables A3.3.1-3.3.4.  The fuel consumption for these categories are taken from DTI (2004).  The emissions from these sources where it is clear that the fuel is being burnt for energy production are calculated as in the base combustion module and reported in IPCC Table 1A Energy.  Where the fuel is used as a feedstock resulting in it being transformed into another fuel, which may be burnt elsewhere, a more complex treatment is needed.  The approach used by the NAEI is to perform a carbon balance over solid smokeless fuel (SSF) production and a separate carbon balance over coke production, sinter production, blast furnaces and basic oxygen furnaces.  This procedure ensures that there is no double counting of carbon and is consistent with IPCC guidelines.  No town gas was manufactured in the UK over the period covered by these estimates so this is not considered.

- The transformation processes involved are:

- Solid Smokeless Fuel Production

- coal    SSF  +  carbon emission

- Coke Production/Sinter production/Blast furnaces/Basic oxygen furnaces (simplified)

- coal    coke  +  coke oven gas  + benzoles & tars + fugitive 	carbon emission

- coke + limestone + iron ore    sinter + carbon emission

- sinter + coke + other reducing agents    pig iron + blast furnace gas

- pig iron + oxygen  steel + basic oxygen furnace gas

- Carbon emissions from each process can be estimated by comparing the carbon inputs and outputs of each stage of the transformation.  The carbon content of the primary fuels are fixed based on the findings of the 2004 UK carbon factor review, as is the carbon content of coke oven gas, blast furnace gas, pig iron, and steel.  The carbon contents of coke, coke breeze, and basic oxygen furnace gas are allowed to vary in order to enable the carbon inputs and outputs to be balanced.  The calculations are so arranged that the total carbon emission corresponds to the carbon content of the input fuels in accordance with IPCC Guidelines.  In the case of SSF production, the carbon content of both input (coal) and output (SSF) are held constant with the difference being treated as an emission of carbon from the process (since the carbon content of the input is always greater than the output).  This procedure has been adopted because it has been assumed that some carbon would be emitted in the form of gases, evolved during the production process.  This procedure will be reviewed before the next version of the inventory is compiled.

- In reporting emissions from coke ovens and SSF manufacturing processes, emissions arising from fuel combustion for energy are reported under 1A1ci Manufacture of Solid Fuels, whilst emissions arising from the transformation process are reported under 1B1b Solid Fuel Transformation.  In the case of blast furnaces, energy emissions are reported under 1A2a Iron and Steel and process emissions under 2C1 Iron and Steel Production.

- A number of differences exist between the current method for performing the carbon balance calculations for coke ovens, and other processes at steelworks, compared with the procedure in previous years.  A number of these differences require some comment:

- the methodology has been improved, but is still based on a carbon balance approach as recommended by IPCC;

- the fixed carbon factors used for primary fuels and some secondary fuels have been reviewed as part of the carbon emission factor review;

- previously, the method made use of certain artificial devices to ensure that carbon balances for coke ovens and iron & steel processes balanced correctly.  These took the form of an additional emission source each for coke ovens and blast furnaces which could be either positive or negative and which were used to reconcile differences between calculated carbon inputs and carbon outputs.  The current methodology, by allowing the carbon contents of certain fuels to vary, does not require these devices;

- there has been significant re-allocation of emissions between various IPPC sectors, in order to model the use of fuels in the iron & steel industry more accurately and thus improve the accuracy of the carbon balance.  These re-allocations, which affect IPCC sectors 1A1c, 1A2a, 1A2f, 1B1b, and 2C1, have neither increased or decreased overall emissions except in one case (see below);

- carbon contained in tars and benzoles produced by coke ovens were previously included as emissions of carbon from the coke oven and was therefore allocated to 1B1b.  This carbon is now recognised as being removed in products sold by coke oven operators (for example wood preservatives) and is therefore not included as an emission allocated to 1B1b.  The ultimate fate of the carbon is not known with certainty and therefore the carbon has been allocated to a waste sector – IPCC sector 6A1, based on the assumption that the preservatives and other products would be ultimately consigned to landfills and that their carbon content would be released through microbial action.  This assumption will be reviewed before the next version of the inventory is compiled.

The revised methodology for fuel transformation and related steelmaking processes have brought significant differences to the emissions allocated individually to 1A1c, 1A2a, 1B1b, 2C1, and 6A1 compared with the previous version of the inventory, together with smaller changes for 1A2f.  However, the revision will not have changed overall UK emissions (although emissions from these IPCC sectors will have changed as a result of the carbon emission factor review).

      - Other energy industries

- Section A3.2 shows the NAEI source categories mapped onto 1A1cii Other Energy Industries.  All these emissions are treated according to the base combustion module using emission factors given in Tables A3.3.1 to A3.3.4.  However, the treatment of gas oil use on offshore installations is anomalous: this is included in the NAEI category Coastal Shipping and hence is mapped to 1A3dii Internal Navigation.  There are no double counts in these emissions.

- The estimation of emissions from natural gas, LPG and OPG used as a fuel in offshore installations and onshore terminals is discussed in Section A3.3.8.  These emissions are reported in category 1A1cii, but the methodology used in their estimation is closely linked to the estimation of offshore fugitive emissions.

    - Manufacturing Industries and Construction (1A2)

      - Other Industry

- In the NAEI, the autogenerators category reports emissions from electricity generation by companies primarily for their own consumption.  The Inventory makes no distinction between electricity generation and combined heat and power or heat plants.  Hence CHP systems where the electricity is fed into the public supply are classified as power stations and CHP systems where the electricity is used by the generator are classified as autogeneration.  The autogenerators category is mapped onto the IPCC category 1A2f Other Industry.  The IPCC 1A1 category also refers to CHP plant and heat plant.

    - Transport (1A3)

      - Aviation

Overview of method to estimate emissions from civil aviation

In accordance with the agreed guidelines, the UK inventory contains estimates for both domestic and international civil aviation.  Emissions from international aviation are recorded as a memo item, and are not included in national totals.  Emissions from both the Landing and Take Off (LTO) phase and the cruise phase are estimated.  The method used to estimate emissions from military aviation can be found towards the end of this section on aviation.

In 2004, the simple method previously been used to estimate emissions from aviation overestimated fuel use and emissions from domestic aircraft because only two aircraft types were considered and the default emission factors used applied to older aircraft.  It is clear that more smaller modern aircraft are used on domestic and international routes.  Emissions from international aviation were correspondingly underestimated.  A summary of the more detailed approach now used is given below, and a full description is given in Watterson et al. (2004).

The current method estimates emissions from the number of aircraft movements broken down by aircraft type at each UK airport, and so complies with the IPCC Tier 3 specification.  Emissions of a range of pollutants are estimated in addition to the reported greenhouse gases.  In comparison with earlier methods used to estimate emissions from aviation, the current approach is much more detailed and reflects differences between airports and the aircraft that use them.  Emissions from additional sources (such as aircraft auxiliary power units) are also now included.

This method utilises data from a range of airport emission inventories compiled in the last few years by Netcen.  This work includes the RASCO study (23 regional airports, with a 1999 case calculated from CAA movement data) carried out for the Department for Transport (DfT), and the published inventories for Heathrow, Gatwick and Stansted airports, commissioned by BAA and representative of the fleets at those airports in a fairly recent year.  Emissions of NOx and fuel use from the Heathrow inventory have been used to verify the results of this study.

Separate estimates have been made for emissions from the LTO cycle and the cruise phase for both domestic and international aviation.  For the LTO phase, fuel consumed and emissions per LTO cycle are based on detailed airport studies, and engine specific emission factors (from the ICAO database).  For the cruise, fuel consumed and emissions have been estimated using distances (based on great circles) travelled from each airport for a set of representative aircraft types.

Emission reporting categories for civil aviation

Table A3.3.7 below shows the emissions included in the emission totals for the domestic and international civil aviation categories currently reported to the FCCC and the UN/ECE.  Note the reporting requirements to the UN/ECE have altered recently – the table contains the most recent reporting requirements.

Table A3.3.7	Components of Emissions Included in Reported Emissions from Civil Aviation

| Organisation receiving emissions data | Category of emissions | LTO | Cruise |
| --- | --- | --- | --- |
| FCCC | Domestic |  |  |
|  | International | m | m |
| UN/ECE | Domestic |  | m |
|  | International | m | m |

Notes

	emissions included in national totals

m	memo items - emissions are estimated and reported, but are not included in national totals

Emissions from the LTO cycle include emissions within a 1000 m ceiling of landing.

Aircraft movement data (activity data)

The methods used to estimate emissions from aviation require the following activity data:

- Aircraft movements and distances travelled
Detailed activity data has been provided by the UK Civil Aviation Authority (CAA).  These data include aircraft movements broken down by: airport; 	aircraft type; 	whether the flight is international or domestic; and, the·	next/last POC (port of call) from which sector lengths (great circle) have been calculated.  A summary of aircraft movement data is given in Table A3.3.8.

- Inland deliveries of aviation spirit and aviation turbine fuel
Total inland deliveries of aviation spirit and aviation turbine fuel to air transport are given in DTI (2004).  This is the best approximation of aviation bunker fuel consumption available and is assumed to cover international, domestic and military use.

- Consumption of aviation turbine fuel by the military
Total consumption by military aviation is given in ONS (1995) and MOD (2005a) and is assumed to be aviation turbine fuel.

Table A3.3.8	Aircraft Movement Data

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Domestic LTOs (000s) | 311.4 | 307.1 | 325.1 | 332.7 | 312.6 | 325.5 | 337.5 |
| International LTOs (000s) | 410.1 | 397.4 | 432.8 | 443.6 | 461.8 | 480.7 | 507.1 |
| Domestic Aircraft, Gm flown | 97.4 | 96.3 | 101.9 | 105.7 | 101.7 | 106.9 | 112.5 |
| International Aircraft, Gm flown | 634.2 | 624.6 | 705.3 | 716.2 | 793.5 | 831.9 | 870.0 |

|  | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Domestic LTOs (000s) | 342.2 | 355.6 | 363.8 | 367.4 | 376.5 | 374.8 | 375.0 |
| International LTOs (000s) | 537.4 | 575.6 | 609.6 | 646.3 | 653.2 | 650.0 | 669.3 |
| Domestic Aircraft, Gm flown | 117.7 | 123.8 | 128.2 | 130.4 | 137.0 | 136.3 | 136.6 |
| International Aircraft, Gm flown | 946.5 | 1031.0 | 1096.7 | 1167.3 | 1181.6 | 1174.1 | 1225.3 |

Notes

Gm	Giga metres, or 109 metres

These data are taken from the method used to estimate emissions from aviation, and are based on data provided by the CAA/

International aircraft, Gm flown, calculated from total flight distances for departures from UK airports

Emission factors used

The following emission factors were used to estimate emissions from aviation.  The emissions of CO2, SO2 and metals depend on the carbon, sulphur and metal contents of the aviation fuels’.  Emissions factors for CO2, SO2 and metals have been derived from the contents of carbon, sulphur and metals in aviation fuels.  These contents are reviewed, and revised as necessary, each year.  Full details of the emission factors used are given in Watterson et al. (2004).

Table A3.3.9a	Carbon Dioxide and Sulphur Dioxide Emission Factors for Civil and Military Aviation for 2003 (kg/t)

| Fuel | CO2 | SO2 |
| --- | --- | --- |
| Aviation Turbine Fuel | 859 | 0.76 |
| Aviation Spirit | 853 | 0.76 |

Notes

Carbon and sulphur contents of fuels provided by UKPIA (2004)

Carbon emission factor as kg carbon/tonne

Military aviation only uses ATF

For the LTO-cycle calculations, emissions per LTO cycle are required for each of a number of representative aircraft types.  Emission factors for the LTO cycle of aircraft operation have been taken from the International Civil Aviation Organization (ICAO) database.  The cruise emissions have been taken from CORINAIR data (which are themselves developed from the same original ICAO dataset).

Table A3.3.9b	Non-CO2 Emission Factors for Civil and Military Aviation

|  | Fuel | Units | CH4 | N2O | NOx | CO | NMVOC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Civil aviation |  |  |  |  |  |  |  |
| Domestic LTO | AS | kt/Mt | 1.53 | 0.1 | 5.12 | 943.0 | 14.0 |
| Domestic LTO | ATF | kt/Mt | 0.22 | 0.1 | 9.68 | 12.8 | 2.31 |
| Domestic Cruise | ATF | kt/Mt | 0.0 | 0.1 | 12.6 | 3.08 | 0.84 |
| International LTO | AS | kt/Mt | - | - | - | - | - |
| International LTO | ATF | kt/Mt | 0.13 | 0.1 | 12.6 | 9.31 | 1.39 |
| International Cruise | ATF | kt/Mt | 0.0 | 0.1 | 14.4 | 1.12 | 0.51 |
| Military aviation | ATF | kt/Mt | 0.1 | 0.1 | 8.50 | 8.20 | 1.1 |

Notes

AS – Aviation Spirit

ATF – Aviation Turbine Fuel

Use of all aviation spirit assigned to the LTO cycle

Method used to estimate emissions from the LTO cycle – civil aviation – domestic and international

The basic approach to estimating emissions from the LTO cycle is as follows.  The contribution to aircraft exhaust emissions (in kg) arising from a given mode of aircraft operation (see list below) is given by the product of the duration (seconds) of the operation, the engine fuel flow rate at the appropriate thrust setting (kg fuel per second) and the emission factor for the pollutant of interest (kg pollutant per kg fuel).  The annual emissions total for the mode (kg per year) is obtained by summing contributions over all engines for all aircraft movements in the year.

The time in each mode of operation for each type of airport and aircraft has been taken from individual airport studies.  The time in mode is multiplied by an emission rate (the product of fuel flow rate and emission factor) at the appropriate engine thrust setting in order to estimate emissions for phase of the aircraft flight.  The sum of the emissions from all the modes provides the total emissions for a particular aircraft journey.  The modes considered are:

- Taxi-out

- Hold

- Take-off Roll (start of roll to wheels-off)

- Initial-climb (wheels-off to 450 m altitude)

- Climb-out (450 m to 1000 m altitude)

- Approach (from 1000 m altitude)

- Landing-roll

- Taxi-in

- APU use after arrival

- Auxiliary Power Unit (APU) use prior to departure

Departure movements comprise the following LTO modes: taxi-out, hold, take-off roll, initial-climb, climb-out and APU use prior to departure.  Arrivals comprise: approach, landing-roll, taxi-in and APU use after arrival.

Method used to estimate emissions in the cruise – civil aviation - domestic and international

The approaches to estimating emissions in the cruise are summarised below.  Cruise emissions are only calculated for aircraft departures from UK airports (emissions therefore associated with the departure airport), which gives a total fuel consumption compatible with recorded deliveries of aviation fuel to the UK.  This procedure prevents double counting of emissions allocated to international aviation.

Estimating emissions of the indirect and non-greenhouse gases.  The EMEP/CORINAIR Emission Inventory Guidebook (EMEP/CORINAIR, 1996) provides fuel consumption and emissions of non-GHGs (NOx, HC and CO) for a number of aircraft modes in the cruise.  The data are given for a selection of generic aircraft type and for a number of standard flight distances.

The breakdown of the CAA movement by aircraft type contains a more detailed list of aircraft types than in the EMEP/CORINAIR Emission Inventory Guidebook.  Therefore, each specific aircraft type in the CAA data has been assigned to a generic type in the Guidebook.  Details of this mapping are given in Watterson et al. (2004).

A linear regression has been applied to these data to give emissions (and fuel consumption) as a function of distance:

ECruised,g,p=mg,pd+cg,p

Where:

ECruised,g,p	is the emissions in cruise of pollutant p for generic aircraft type g and flight distance d (kg)

d	is the flight distance

g	is the generic aircraft type

p	is the pollutant (or fuel consumption)

mg,p	is the slope of regression for generic aircraft type g and pollutant p (kg / km)

cg,p	is the intercept of regression for generic aircraft type g and pollutant p (kg)

Emissions of SO2 and metals are derived from estimates of fuels consumed in the cruise (see equation above) multiplied by the sulphur and metals contents of the aviation fuels for a given year.

Estimating emissions of the direct greenhouse gases.  Estimates of CO2 were derived from estimates of fuel consumed in the cruise (see equation above) and the carbon contents of the aviation fuels.

Methane emissions are believed to be negligible at cruise altitudes, and the emission factors listed in EMEP/CORINAIR guidance are zero (EMEP/CORINAIR, 1996); we have also assumed them to be zero.  This was the assumption in the previous aviation calculation method also.

Estimates of N2O have been derived from an emission factor recommended by the IPCC (IPCC, 1997c) and the estimates of fuel consumed in the cruise (see equation above).

Classification of domestic and international flights

The UK CAA has provided the aircraft movement data used to estimate emissions from civil aviation.  The definitions the CAA use to categorise whether a movement is international or domestic are (CAA, per. comm.)

- Domestic	a flight is domestic if the initial point on the service is a domestic and the final point is a domestic airport

- International	a flight is international if either the initial point or the final point on the service is an international airport

Take, for example, a flight (service) that travels the following route: Glasgow (within the UK) – Birmingham (within the UK) – Paris (outside the UK).  The airport reporting the aircraft movement in this example is Glasgow, and the final airport on the service is Paris.  The CAA categorises this flight as international, as the final point on the service is outside the UK.

Flights to the Channel Islands and the Isle of Man are considered to be within the UK in the CAA aircraft movement data.

By following the IPCC Good Practice guidance (IPCC, 2000), it is necessary to know whether passengers or freight are put down before deciding whether the whole journey is considered as an international flight or consisting of a (or several) domestic flight(s) and an international flight.  We feel the consequence of the difference between CAA and IPCC definitions will have a small impact on total emissions.

The CAA definitions above are also used by the CAA to generate national statistics of international and domestic aircraft movements.  Therefore, the aircraft movement data used in this updated aviation methodology are consistent with national statistical datasets on aircraft movements.

Overview of method to estimate emission from military aviation

LTO data are not available for military aircraft movements, so a simple approach is used to estimate emissions from military aviation.  A first estimate of military emissions is made using military fuel consumption data and IPCC (1997) and EMEP/CORINAIR (1999) cruise defaults shown in Table 1 of EMEP/CORINAIR (1999) (see Table A3.3.9b).  The EMEP/CORINAIR (1999) factors used are appropriate for military aircraft.  The military fuel data include fuel consumption by all military services in the UK.  It also includes fuel shipped to overseas garrisons, casual uplift at civilian airports, but not fuel uplifted at foreign military airfields or ad hoc uplift from civilian airfields.

Emissions from military aircraft are reported under IPCC category 1A5 Other.

Fuel reconciliation

The estimates of aviation fuels consumed in the commodity balance table in the DTI publication DUKES are the national statistics on fuel consumption, and IPCC guidance states that national total emissions must be on the basis of fuel sales.  Therefore, the estimates of emissions have been re-normalised based on the results of the comparison between the fuel consumption data in DUKES and the estimate of fuel consumed produced from the civil aviation emissions model.  The ATF fuel consumptions presented in DTI DUKES include the use of both civil and military ATF, and the military ATF use must be subtracted from the DUKES total to provide an estimate of the civil aviation consumption.  This estimate of civil ATF consumption has been used in the fuel reconciliation.  Emissions will be re-normalised each time the aircraft movement data is modified or data for another year added.

Geographical coverage of aviation emission estimates

According to the IPCC Guidelines, "inventories should include greenhouse gas emissions and removals taking place within national (including administered) territories and offshore areas over which the country has jurisdiction." IPCC, (1997c); (IPPC Reference Manual, Overview, Page 5).

The national estimates of aviation fuels consumed in the UK are taken from DTI DUKES.  The current (and future) methods used to estimate emissions from aviation rely on these data, and so the geographical coverage of the estimates of emissions will be determined by the geographical coverage of DUKES.

The UK DTI has confirmed that the coverage of the energy statistics in DUKES is England, Wales, Scotland and Northern Ireland plus any oil supplied from the UK to the Channel Islands and the Isle of Man.  This clarification was necessary since this information cannot be gained from UK trade statistics.  The DTI have confirmed estimates in DUKES exclude Gibraltar and the other UK overseas territories.  The DTI definition fully accords with that of the "economic territory of the United Kingdom" used by the UK Office for National Statistics (ONS), which in turn accords with the definition required to be used under the European System of Accounts (ESA95).

      - Railways

The UK GHGI reports emissions from both stationary and mobile sources.  The source, railways (stationary), reports emissions from the combustion of burning oil, fuel oil and natural gas by the railway sector.  The natural gas emission derives from generation plant used for the London Underground.  These stationary emissions are reported under 1A4a Commercial/Institutional in the IPCC reporting system.  Most of the electricity used by the railways for electric traction is supplied from the public distribution system, so the emissions arising from its generation are reported under 1A1a Public Electricity.  These emissions are based on fuel consumption data from DTI (2004).  Emission factors are reported in Tables A3.3.1 to A3.3.3.

The NAEI reports emissions from diesel trains as railways (freight), railways (intercity) and railways (regional).  These estimates are based on the gas oil consumption for railways reported in DTI (2004).  Emissions from diesel trains are reported under the IPCC category 1A3c Railways.

Diesel train journeys have been split into three categories: freight, intercity and regional.  Carbon dioxide, sulphur dioxide and N2O emissions are calculated based on fuel based emission factors using fuel consumption data from DTI (2004).  This fuel consumption is distributed according to railway km data from DETR (1996c) on the three types of journey, an assumed mix of locomotives for each journey type, and fuel consumption factors for the different types of locomotive (LRC (1998), BR (1994) and Hawkins & Coad (2004)).  The detailed railway km data are only available up to 1995 and later years are interpolated using 1995 data.  Emissions of CO, NMVOC, NOx and methane are based on the railway km estimates and emission factors for the various types of locomotive used. The emission factors shown in Table A3.3.10 are aggregate factors so that all factors are reported on the common basis of fuel consumption.

Table A3.3.10	Railway Emission Factors (kt/Mt)

|  | C1 | CH4 | N2O | NOx | CO | NMVOC | SO2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Freight | 870 | 0.12 | 1.2 | 18.8 | 5.8 | 3.0 | 3.1 |
| Intercity | 870 | 0.17 | 1.2 | 31.6 | 9.9 | 4.3 | 3.1 |
| Regional | 870 | 0.12 | 1.2 | 75.7 | 6.5 | 3.0 | 3.1 |

1	Emission factor as ktonnes carbon per Mtonne fuel

      - Road Transport

Emissions from road transport are calculated either from a combination of total fuel consumption data and fuel properties or from a combination of drive related emission factors and road traffic data.

        - Improvements in the 2003 inventory

A number of changes have been made to the methodologies and data used for compiling the 2002 inventory for the road transport sector while compiling the 2003 inventory.  Some of these changes are due to minor changes in the traffic activity data used, to changes in the values of the carbon content of road diesel fuel and to use of an improved modelling methodology that uses more detailed information on traffic speeds and information on composition of the vehicle fleet.  These changes have led to revisions to some of the emission estimates for back years of the inventory and are summarised below:

- Minor revisions by DfT in the vehicle km data for years 1998-2002

- Minor change in assumptions about penetration of Euro IV petrol cars in the fleet from 2001-2003, on the basis of evidence from DfT suggesting a lower penetration rate than previously assumed.

- Revisions to the data from DTi on petrol and road diesel deliveries in years 2000-2002 that affect CO2 emissions

- A revision to the carbon content of road diesel fuel sold in the UK from 1990-2002 on the basis of new industry information from UKPIA.

        - Fuel-based emissions

- Emissions of carbon dioxide and sulphur dioxide from road transport are calculated from the consumption of petrol and diesel fuels and the sulphur content of the fuels consumed.  Data on petrol and diesel fuels consumed by road transport in the UK are taken from the Digest of UK Energy Statistics published by the DTI and corrected for consumption by off-road vehicles.

In 2003, 19.92 Mtonnes of petrol and 17.71 Mtonnes of diesel fuel (DERV) were consumed in the UK.  It was estimated that of this, around 1.0% of petrol was consumed by off-road vehicles and machinery, leaving 19.73 Mtonnes of petrol consumed by road vehicles in 2003.  According to figures in DUKES (DTI, 2004), 0.104 Mtonnes of LPG were used for transport in 2003, up from 0.086 Mtonnes the previous year.

Emissions of CO2, expressed as kg carbon per tonne of fuel, are based on the H/C ratio of the fuel; emissions of SO2 are based on the sulphur content of the fuel.  The carbon content of petroleum fuels have been revised this year based on a survey carried out by UKPIA on UK fuels since 1990 (UKPIA, 2004).  The survey found that no changes were necessary to the carbon content used for petrol fuel at a constant level of 855 kg/tonne, but the previous value used for road diesel (857 kg/tonne) needed to be increased to 864 kg/tonne in 1990-1998 and to 863 kg/tonne in years from 1999-2003.  Values of the fuel-based emission factors for CO2 and SO2 from consumption of petrol and diesel fuels are shown in Table A3.3.11.  Values for SO2 vary annually as the sulphur-content of fuels change, and are shown in Table A3.3.11 for 2003 fuels based on data from UKPIA (2004).

Table A3.3.11	Fuel-Based Emission Factors for Road Transport (kg/tonne fuel)

| Fuel | Ca | SO2b |
| --- | --- | --- |
| Petrol | 855 | 0.08 |
| Diesel | 863 | 0.08 |

a	Emission factor in kg carbon/tonne, based on UKPIA (2004)

- b	2003 emission factor calculated from UKPIA (2004) – figures on the weighted average sulphur-content of fuels delivered in the UK in 2003

Emissions of CO2 and SO2 can be broken down by vehicle type based on estimated fuel consumption factors and traffic data in a manner similar to the traffic-based emissions described below for other pollutants.   The important equations relating fuel consumption to average speed based on the set of tailpipe CO2, CO and total hydrocarbon (THC) emission-speed equations were developed by TRL (Barlow et al, 2001).  The TRL equations were derived from their large database of emission measurements compiled from different sources covering different vehicle types and drive cycles.  A substantial part of the emission measurements for Euro I and II standard vehicles come from test programmes funded by DfT and Defra and carried out in UK test laboratories between 1999 and 2001.  The measurements were made on dynamometer test facilities under simulated real-world drive cycles.

For cars, average fuel consumption factors were calculated from UK fleet-averaged CO2 emission factors for different car vintages (years of production) provided by DfT (2004a) following consultation with the Society of Motor Manufacturers and Traders (SMMT).  Their dependence on speed used the TRL-based speed relations for vehicles categorised into each Euro standard.  For LGVs, buses and motorcycles, the inventory used fuel consumption factors expressed as g fuel per kilometre for each vehicle type and road type calculated directly from the TRL equations.  Data on the average fuel efficiency of HGVs in the fleet since 1990 were taken from DfT (2004b) who, through the Continuous Survey of Road Goods Transport (CSRGT), obtain data each year on the fuel efficiency of HGVs of different weights from surveys among goods hauliers.  These relate to fuel consumption (e.g. in litre/100km or in miles per gallon) that are averaged over all operational drive cycles, so their dependence on speed were estimated using the TRL-based speed relations for HGVs categorised into each Euro standard.

Average fuel consumption factors are shown in Table A3.3.12 for each vehicle type, emission regulation and road type in the UK.  The different emission standards are described in a later section.  A normalisation procedure was used to ensure that the breakdown of petrol and diesel consumption by each vehicle type calculated on the basis of the fuel consumption factors added up to the DTI figures for total fuel consumption in the UK (adjusted for off-road consumption).

Total CO2 emissions from vehicles running on LPG are estimated on the basis of national figures (from DTI) on the consumption of this fuel by road transport.  The CO2 emissions from LPG consumption cannot be broken down by vehicle type because there are no figures available on the total number of vehicles or types of vehicles running on this fuel.  This is unlike vehicles running on petrol and diesel where the DfT has statistics on the numbers and types of vehicles registered as running on these fuels.  It is believed that many vehicles running on LPG are cars and vans converted by their owners and that these conversions are not necessarily reported to vehicle licensing agencies.  It is for this same reason that LPG vehicle emission estimates are not possible for other pollutant types, because these would need to be based on traffic data and emission factors for different vehicle types rather than on fuel consumption.

Emissions from vehicles running on natural gas are not estimated at present, although the number of such vehicles in the UK is very small.  Estimates are not made as there are no separate figures from DTI on the amount of natural gas used by road transport, nor are there useable data on the total numbers and types of vehicles equipped to run on natural gas.

        - Traffic-based emissions

Emissions of the pollutants NMVOCs, NOx, CO, CH4 and N2O are calculated from measured emission factors expressed in grammes per kilometre and road traffic statistics from the Department for Transport (DfT, 2004c). The emission factors are based on experimental measurements of emissions from in-service vehicles of different types driven under test cycles with different average speeds.  The road traffic data used are vehicle kilometre estimates for the different vehicle types and different road classifications in the UK road network.  These data have to be further broken down by composition of each vehicle fleet in terms of the fraction of diesel- and petrol-fuelled vehicles on the road and in terms of the fraction of vehicles on the road made to the different emission regulations which applied when the vehicle was first registered.  These are related to the age profile of the vehicle parc.

Emissions from motor vehicles fall into three different types, which are each calculated in a different manner.  These are hot exhaust emissions, cold-start emissions and, for NMVOCs, evaporative emissions.

          - Hot exhaust emissions

Hot exhaust emissions are emissions from the vehicle exhaust when the engine has warmed up to its normal operating temperature.  Emissions depend on the type of vehicle, the type of fuel its engine runs on, the driving profile of the vehicle on a journey and the emission regulations which applied when the vehicle was first registered as this defines the type of technology the vehicle is equipped with affecting emissions.

Table A3.3.12	Fuel Consumption Factors for Road Transport (in g fuel/km)

For a particular vehicle, the drive cycle over a journey is the key factor that determines the amount of pollutant emitted.  Key parameters affecting emissions are the acceleration, deceleration, steady speed and idling characteristics of the journey, as well as other factors affecting load on the engine such as road gradient and vehicle weight.  However, work has shown that for modelling vehicle emissions for an inventory covering a road network on a national scale, it is sufficient to calculate emissions from emission factors in g/km related to the average speed of the vehicle in the drive cycle (Zachariadis and Samaras, 1997).  Emission factors for average speeds on the road network are then combined with the national road traffic data.

Vehicle and fuel type

Emissions are calculated for vehicles of the following types:

- Petrol cars

- Diesel cars

- Petrol Light Goods Vehicles (Gross Vehicle Weight (GVW) ≤ 3.5 tonnes)

- Diesel Light Goods Vehicles (Gross Vehicle Weight (GVW) ≤ 3.5 tonnes)

- Rigid-axle Heavy Goods Vehicles (GVW > 3.5 tonnes)

- Articulated Heavy Goods Vehicles (GVW > 3.5 tonnes)

- Buses and coaches

- Motorcycles

Total emission rates are calculated by multiplying emission factors in g/km with annual vehicle kilometre figures for each of these vehicle types on different types of roads.

Vehicle kilometres by road type

Hot exhaust emission factors are dependent on average vehicle speed and therefore the type of road the vehicle is travelling on.  Average emission factors are combined with the number of vehicle kilometres travelled by each type of vehicle on many different types of urban roads with different average speeds and the emission results combined to yield emissions on each of these main road types:

- Urban

- Rural single carriageway

- Motorway/dual carriageway

DfT estimates annual vehicle kilometres for the road network in Great Britain by vehicle type on roads classified as trunk, principal and minor roads in built-up areas (urban) and non-built-up areas (rural) and motorways (DfT, 2004c).  The DfT Report “Transport Statistics Great Britain” (DfT, 2004c) explains some of the developments made in recent years to improve its traffic estimates from measurements of traffic flows on the national road network. Some minor changes were made to the vehicle kilometres data from 1998-2002 in the 2004 publication used for this year’s emission estimates.  The vehicle kilometre estimates are based on traffic counts from the rotating census and core census surveys.

Vehicle kilometre data for Northern Ireland became available for 1999, 2001, 2002 and 2003 from survey reports provided by Department for Regional Development, Northern Ireland, Road Services Headquarters (DRDNI, 2002, 2003a, 2003b, 2004).  Data for 2000 were not available, but were estimated from the 1999 and 2001 figures.  The Northern Ireland data have been combined with the DfT data for Great Britain to produce a time-series of total UK vehicle kilometres by vehicle and road type from 1970 to 2003.

The vehicle kilometre data were grouped into the three road types mentioned above for combination with the associated hot exhaust emission factors.

Vehicle speeds by road type

Average speed data for traffic in a number of different urban areas have been published in a series of DETR reports based on measured traffic speed surveys (DETR (1998a, 1998b, 1998c, 1998d), DfT (2004c)).  These data were rationalised with speed data from other DETR sources, including the 1997 National Road Traffic Forecasts (DETR, 1997), which give average speeds for different urban area sizes, and consolidated with average speed data for unconstrained rural roads and motorways published in Transport Statistics Great Britain (DfT, 2004c).  They are shown in Table A3.3.13.  The speeds are averages of speeds at different times of day and week, weighted by the level of traffic at each of these time periods where this information is known.

Weighting by the number of vehicle kilometres on each of the urban road types gives an overall average speed for urban roads of 43 kph.

Vehicle fleet composition: by age, technology and fuel type

The vehicle kilometres data based on traffic surveys do not distinguish between the type of fuels the vehicles are being run on (petrol and diesel) nor on their age.  The latter determines the type of emission regulation that applied when the vehicle was first registered.  These have successively entailed the introduction of tighter emission control technologies, for example three-way catalysts and better fuel injection and engine management systems.

Table A3.3.14 shows the regulations that have come into force up to 2003 for each vehicle type.

The average age profile and the fraction of petrol and diesel cars and LGVs in the traffic flow each year are based on the composition of the UK vehicle fleet using DfT Vehicle Licensing Statistics.  The Transport Statistics Bulletin “Vehicle Licensing Statistics: 2003” (DfT, 2004d) either gives historic trends in the composition of the UK fleet directly or provides sufficient information for this to be calculated from new vehicle registrations and average vehicle survival rates.  The vehicle licensing data are combined with data on the change in annual vehicle mileage with age to take account of the fact that newer vehicles on average travel a greater number of kilometres in a year than older vehicles.  For cars and LGVs, such mileage data are from the National Travel Survey (DETR, 1998e); data for HGVs of different weights are taken from the Continuous Survey of Road Goods Transport (DETR, 1996a).

The fraction of diesel cars and LGVs in the fleet was taken from data in “Vehicle Licensing Statistics: 2003” (DfT, 2004d).

Year-of-first registration data for vehicles licensed in each year from 1990 to 2003 taken from DfT’s Vehicle Licensing Statistics reflect the age distribution of the fleet in these years.

Table A3.3.13	Average Traffic Speeds in Great Britain

| URBAN ROADS |  | kph |
| --- | --- | --- |
| Central London | Major/trunk A roads | 18 |
|  | Other A roads | 14 |
|  | Minor roads | 16 |
| Inner London | Major/trunk A roads | 28 |
|  | Other A roads | 20 |
|  | Minor roads | 20 |
| Outer London | Major/trunk A roads | 45 |
|  | Other A roads | 26 |
|  | Minor roads | 29 |
| Urban motorways |  | 95 |
| Large conurbations | Central | 34 |
|  | Outer trunk/A roads | 45 |
|  | Outer minor roads | 34 |
| Urban, pop >200,000 | Central | 37 |
|  | Outer trunk/A roads | 50 |
|  | Outer minor roads | 37 |
| Urban, pop >100,000 | Central | 40 |
|  | Outer trunk/A roads | 54 |
|  | Outer minor roads | 40 |
| Urban >25 sq km | Major roads | 46 |
|  | Minor roads | 42 |
| Urban 15-25 sq km | Major roads | 49 |
|  | Minor roads | 46 |
| Urban 5-15 sq km | Major roads | 51 |
|  | Minor roads | 48 |
| Urban < 5sq km | Major roads | 52 |
|  | Minor roads | 48 |

Table A3.3.13	Average Traffic Speeds in Great Britain (continued)

| RURAL ROADS |  | Lights kph | Heavies kph |
| --- | --- | --- | --- |
| Rural single carriageway | Major roads | 80 | 75 |
|  | Minor roads | 67 | 63 |
| Rural dual carriageway |  | 113 | 89 |
| Rural motorway |  | 113 | 92 |

Table A3.3.14a	Vehicles types and regulation classes

| Vehicle Type | Fuel | Regulation | Approx. date into service in UK |
| --- | --- | --- | --- |
| Cars | Petrol | Pre ECE-15.00 |  |
|  |  | ECE-15.00 | 1/1/1971 |
|  |  | ECE-15.01 | 1/7/1975 |
|  |  | ECE-15.02 | 1/7/1976 |
|  |  | ECE-15.03 | 1/7/1979 |
|  |  | ECE-15.04 | 1/7/1983 |
|  |  | 91/441/EEC (Euro I) | 1/7/1992 |
|  |  | 94/12/EC (Euro II) | 1/1/1997 |
|  |  | 98/69/EC (Euro III) | 1/1/2001 |
|  |  | 98/69/EC (Euro IV) | 1/1/2006 |
|  | Diesel | Pre-Euro I |  |
|  |  | 91/441/EEC (Euro I) | 1/1/1993 |
|  |  | 94/12/EC (Euro II) | 1/1/1997 |
|  |  | 98/69/EC (Euro III) | 1/1/2001 |
| LGVs | Petrol | Pre-Euro I |  |
|  |  | 93/59/EEC (Euro I) | 1/7/1994 |
|  |  | 96/69/EEC (Euro II) | 1/7/1997 |
|  |  | 98/69/EC (Euro III) | 1/1/2001 (<1.3t) 1/1/2002 (>1.3t) |
|  | Diesel | Pre-Euro I |  |
|  |  | 93/59/EEC (Euro I) | 1/7/1994 |
|  |  | 96/69/EEC (Euro II) | 1/7/1997 |
|  |  | 98/69/EC (Euro III) | 1/1/2001 (<1.3t) 1/1/2002 (>1.3t) |
| HGVs and | Diesel (All types) | Old |  |
| buses |  | 88/77/EEC (Pre-Euro I) | 1/10/1988 |
|  |  | 91/542/EEC (Euro I) | 1/10/1993 |
|  |  | 91/542/EEC (Euro II) | 1/10/1996 |
|  |  | 99/96/EC (Euro III) | 1/10/2001 |
| Motorcycles | Petrol | Pre-2000: < 50cc, >50cc (2 st, 4st) |  |
|  |  | 97/24/EC: all sizes | 1/1/2000 |

Note: Euro IV standards for petrol cars are shown because some new cars models sold from 2001 already meet Euro IV standards even they are not required to until 2006.

Statistics are also available on the number of new registrations in each year up to 2003, reflecting the number of new vehicles entering into service in previous years.  The two sets of data combined allow an average survival rate to be determined for each type of vehicle.  Particularly detailed information is available on the composition of the HGV stock by age and size.

Assumptions are made about the proportion of failing catalysts in the petrol car fleet.  For first-generation catalyst cars (Euro I), it is assumed that the catalysts fail in 5% of cars fitted with them each year (for example due to mechanical damage of the catalyst unit) and that 95% of failed catalysts are repaired each year, but only for cars more than three years in age, when they first reach the age for MOT testing.  Following discussions with DfT, a review of information from the Vehicle Inspectorate, TRL, the Cleaner Vehicles Task Force, industry experts and other considerations concerning durability and emission conformity requirements in in-service tests, lower failure rates are assigned to Euro II, III and IV cars manufactured since 1996.  The following failure rates are assumed in the inventory:

- Euro I		5%

- Euro II	1.5%

- Euro III, IV	0.5%

The inventory takes account of the early introduction of certain emission and fuel quality standards and additional voluntary measures to reduce emissions from road vehicles in the UK fleet.  The Euro III emission standards for passenger cars (98/69/EC) came into effect from January 2001 (new registrations).  However, some makes of cars sold in the UK already met the Euro III standards prior to this (DfT, 2001).  Figures from the Society of Motor Manufacturers and Traders suggested that 3.7% of new cars sold in 1998 met Euro III standards (SMMT, 1999).  Figures were not available for 1999 and 2000, but it was assumed that 5% of new car sales met Euro III standards in 1999 increasing to 10% in 2000.  In 2001, an assumption was made that 15% of all new petrol cars sold in the UK met Euro IV standards, increasing to 43% in 2003 even though the mandatory date of introduction of this standard is not until 2006 (DfT, 2004e).  The remaining new petrol car registrations in 2001 - 2003 would meet Euro III standards.

In January 2000, European Council Directive 98/70/EC came into effect relating to the quality of petrol and diesel fuels.  This introduced tighter standards on a number of fuel properties affecting emissions.  The principle changes in UK market fuels were the sulphur content and density of diesel and the sulphur and benzene content of petrol.  The volatility of summer blends of petrol was also reduced, affecting evaporative losses.  During 2000-2003, virtually all the diesel sold in the UK was of ultra-low sulphur grade (<50 ppmS), even though this low level of sulphur content is not required by the Directive until 2005.  Similarly, ultra-low sulphur petrol (ULSP) became on-line in filling stations in 2000, with around one-third of sales being of ULSP quality during 2000, the remainder being of the quality specified by the Directive. In 2001-2003, virtually all unleaded petrol sold was of ULSP grade (UKPIA, 2004).  These factors and their effect on emissions were taken into account in the inventory.  It is assumed that prior to 2000, only buses had made a significant switch to ULSD, as this fuel was not widely available in UK filling stations.

Freight haulage operators are now looking at incentives to upgrade the engines in their HGVs or retrofit them with particle traps.  DETR estimated that around 4000 HGVs and buses were retrofitted with particulate traps in 2000, rising to 10,000 vehicles by the end of 2003 (DETR, 2000).  This was accounted for in the 2003 inventory.

Detailed information from DVLA was used on the composition of the motorcycle fleet in terms of engine capacity (DfT, 2004d).  The information was used to calculate the proportion of motorcycles on the road less than 50cc (i.e. mopeds), >50cc, 2-stroke and >50cc, 4-stroke.

          - Hot emission factors

The emission factors for NOx, CO and NMVOCs used for pre-Euro I vehicles in the 2001 inventory are based on data from TRL (Hickman, 1998) and COPERT II, “Computer Programme to Calculate Emissions from Road Transport” produced by the European Topic Centre on Air Emissions for the European Environment Agency (1997).  Both these sources provide emission functions and coefficients relating emission factor (in g/km) to average speed for each vehicle type and Euro emission standard derived by fitting experimental measurements to some polynomial functional form.

Emission factors for Euro I and Euro II vehicles are based on speed-emission factor relationships derived by TRL from emission test programmes carried out in the UK (Barlow et al, 2001).  The tests were carried out on in-service vehicles on dynamometer facilities under simulated real-world drive cycles.  These provided a more robust source of emission factors for these vehicle classes than had hitherto been available.  The factors for NMVOCs are actually based on emission equations for total hydrocarbons (THC), the group of species that are measured in the emission tests.  To derive factors for non-methane VOCS, the calculated g/km factors for methane were subtracted from the corresponding THC emission factors.

Due to lack of measured data, emission factors for Euro III vehicles (and Euro IV petrol cars) were estimated by applying scaling factors to the Euro II factors.  The scale factors for light duty vehicles take into consideration the requirement for new vehicles to meet certain durability standards set in the Directives.  Scaling factors were first estimated by considering how much emissions from Euro II vehicles would need to be reduced to meet the Euro III and IV limit values taking account of the characteristics and average speed of the regulatory test cycles used for type-approval of the vehicle’s engine. It was then assumed that emissions from new vehicles would be a certain percentage lower than the limit value-derived figure when new so that the vehicle would not have emissions that degrade to levels higher than the limit value over the durability period of the vehicle set in the Directives.  The emission degradation rates permitted for Euro III and IV light duty vehicles by Directive 98/69/EC are as follows:

Table A3.3.14b 	Emission Degradation rates permitted for Euro III and IV Light-Duty Vehicles by Directive 98/69/EC

|  |  |  | Degradation rate |
| --- | --- | --- | --- |
| Petrol vehicles | NOx, HC and CO | Euro III | x1.2 over 80,000km |
|  |  | Euro IV | x1.2 over 100,000km |
| Diesel vehicles | PM | Euro III | x1.2 over 80,000km |
|  |  | Euro IV | x1.2 over 100,000km |
|  | CO | Euro III | x1.1 over 80,000km |
|  |  | Euro IV | x1.1 over 100,000km |

For heavy-duty vehicles, the emission scaling factors were taken from COPERT III (European Environment Agency, 2000).

The speed-emission factor equations were used to calculate emission factor values for each vehicle type and Euro emission standard at each of the average speeds of the road and area types shown in Table A3.3.13.  The calculated values were averaged to produce single emission factors for the three main road classes described earlier (urban, rural single carriageway and motorway/dual carriageway), weighted by the estimated vehicle kilometres on each of the detailed road types taken from the 1997 NRTF (DETR, 1997).

For each type of vehicle, both TRL and COPERT II provide equations for different ranges of vehicle engine capacity or vehicle weight.  Emission factors calculated from these equations were therefore averaged, weighted according to the proportion of the different vehicle sizes in the UK fleet, to produce a single average emission factor for each vehicle type and road type.  These average emission factors are given in Tables A3.3.17 to 21 for each of the different vehicle types and emission regulations.

Speed-dependent functions provided by TRL (Hickman, 1998) for different sizes of motorcycles were used.  Prior to 2000, all motorcycles are assumed to be uncontrolled. It was also assumed that mopeds (<50cc) operate only in urban areas, while the only motorcycles on motorways are the type more than 50cc, 4-stroke.  Otherwise, the number of vehicle kilometres driven on each road type was disaggregated by motorcycle type according to the proportions in the fleet.  Motorcycles sold since the beginning of 2000 were assumed to meet the Directive 97/24/EC and their emission factors were reduced according to the factors given in the latest version of COPERT III (European Environment Agency, 2000).

Emissions from buses were scaled down according to the proportion running on ultra-low sulphur diesel fuel in each year, the proportion fitted with oxidation catalysts or particulate traps (CRTs) and the effectiveness of these measures in reducing emissions from the vehicles.  The effectiveness of these measures in reducing emissions from a Euro II bus varies for each pollutant and is shown in Table A3.3.15.  It is assumed that a bus fitted with an oxidation catalyst or CRT is also running on ULS diesel.

Table A3.3.15	Scale Factors for Emissions from a Euro II Bus Running on Ultra-Low Sulphur Diesel and Fitted with an Oxidation Catalyst or CRT

|  |  | NOx | CO | NMVOCs |
| --- | --- | --- | --- | --- |
| ULS diesel only | Urban | 1.01 | 0.91 | 0.72 |
|  | Rural | 0.99 | 1.01 | 1.02 |
| ULS diesel + Oxy catalyst | Urban | 0.97 | 0.20 | 0.39 |
|  | Rural | 0.95 | 0.22 | 0.55 |
| ULS diesel + CRT | Urban | 0.90 | 0.17 | 0.19 |
|  | Rural | 0.88 | 0.19 | 0.27 |

These scale factors are relative to emissions from a bus running on 500ppm S diesel and are based on analysis of fuel quality effects by Murrells (2000) and data on the effectiveness of oxidation catalysts on bus emissions by LT Buses (1998).

Similarly, the small numbers of HGVs equipped with CRTs have their emissions reduced by the amounts shown in Table A3.3.16.  Again these vehicles will also be running on ULS diesel.

Table A3.3.16	Scale Factors for Emissions from a Euro II HGV Running on Ultra-Low Sulphur Diesel and Fitted with an Oxidation Catalyst or CRT

|  |  | NOx | CO | NMVOCs |
| --- | --- | --- | --- | --- |
| ULS diesel only | Urban | 0.94 | 0.96 | 0.97 |
|  | Rural | 0.99 | 1.01 | 1.02 |
| ULS diesel + CRT | Urban | 0.81 | 0.10 | 0.12 |
|  | Rural | 0.85 | 0.10 | 0.12 |

The older in-service vehicles in the test surveys that were manufactured to a particular emission standard would have covered a range of different ages.  Therefore, an emission factor calculated for a particular emission standard (e.g. ECE 15.04) from the emission functions and coefficients from TRL and COPERT II is effectively an average value for vehicles of different ages which inherently takes account of possible degradation in emissions with vehicle age.  However, for the more recent emission standards (Euro I and II), the vehicles would have been fairly new when the emissions were measured.  Therefore, based on data from the European Auto-Oil study, the deterioration in emissions with age or mileage was taken into account for catalyst cars.  It was assumed that emissions of CO and NOX increase by 60% over 80,000 km, while emissions of NMVOCs increase by 30% over the same mileage (DETR, 1996b).  Based on the average annual mileage of cars, 80,000 km corresponds to a time period of 6.15 years.  Emissions from Euro III and IV light duty vehicles were assumed to degrade at rates described earlier, consideration given to the durability requirements of the Directive 98/69/EC.

For methane, factors for pre-Euro I and/or Euro I standards for each vehicle type were taken from COPERT III which provided either full speed-emission factor equations or single average factors for urban, rural and highway roads.  Methane emission factors for other Euro standards were scaled according to the ratio in the THC emission factors between the corresponding Euro standards.  This assumes that methane emissions are changed between each standard to the same extent as total hydrocarbons so that the methane fraction remains constant.

Emission factors for nitrous oxide (N2O) are the road-type factors taken from COPERT III.  Due to lack of available data, no distinction between different Euro standards can be discerned, except for the higher N2O emissions arising from petrol vehicles fitted with a three-way catalyst (Euro I and on).

The uncertainties in the CH4 and N2O factors can be expected to be quite large.  However, the emission factors used reflect the fact that three-way catalysts are less efficient in removing methane from the exhausts than other hydrocarbons and also lead to higher N2O emissions than non-catalyst vehicles.

          - Cold-Start Emissions

When a vehicle’s engine is cold it emits at a higher rate than when it has warmed up to its designed operating temperature.  This is particularly true for petrol engines and the effect is even more severe for cars fitted with three-way catalysts, as the catalyst does not function properly until the catalyst is also warmed up.  Emission factors have been derived for cars and LGVs from tests performed with the engine starting cold and warmed up.  The difference between the two measurements can be regarded as an additional cold-start penalty, paid on each trip a vehicle is started with the engine (and catalyst) cold.

The procedure for estimating cold-start emissions is taken from COPERT II (European Environment Agency, 1997), taking account of the effects of ambient temperature on emission factors for different vehicle technologies and its effect on the distance travelled with the engine cold.  A factor, the ratio of cold to hot emissions, is used and applied to the fraction of kilometres driven with cold engines to estimate the cold start emissions from a particular vehicle type using the following formula:

Ecold =  . Ehot . (ecold/ehot - 1)

where

Ehot 		= hot exhaust emissions from the vehicle type

	= fraction of kilometres driven with cold engines

ecold/ehot 	= ratio of cold to hot emissions for the particular pollutant and vehicle type

The parameters  and ecold/ehot are both dependent on ambient temperature and  is also dependent on driving behaviour in, particular the average trip length, as this determines the time available for the engine and catalyst to warm up.  The equations relating ecold/ehot to ambient temperature for each pollutant and vehicle type were taken from COPERT II and were used with an annual mean temperature for the UK of 11oC.  This is based on historic trends in Met Office data for ambient temperatures over different parts of the UK.

The factor  is related to ambient temperature and average trip length by the following equation taken from COPERT II:

 = 0.698 - 0.051 . ltrip - (0.01051 - 0.000770 . ltrip) . ta

where

ltrip 		= average trip length

ta				= average temperature

An average trip length for the UK of 8.4 km was used, taken from Andre et al (1993).  This gives a value for  of 0.23.

This methodology was used to estimate annual UK cold start emissions of NOx, CO and NMVOCs from petrol and diesel cars and LGVs.  Emissions were calculated separately for catalyst and non-catalyst petrol vehicles.  Cold start emissions data are not available for heavy-duty vehicles, but these are thought to be negligible (Boulter, 1996).

All the cold start emissions are assumed to apply to urban driving.

Cold-start emissions data are not available for the pollutants methane and nitrous oxide.

          - Evaporative Emissions

Evaporative emissions of petrol fuel vapour from the tank and fuel delivery system in vehicles constitute a significant fraction of total NMVOC emissions from road transport.  The procedure for estimating evaporative emissions of NMVOCs takes account of changes in ambient temperature and fuel volatility.

There are three different mechanisms by which gasoline fuel evaporates from vehicles:

i) Diurnal loss

This arises from the increase in the volatility of the fuel and expansion of the vapour in the fuel tank due to the diurnal rise in ambient temperature.  Evaporation through “tank breathing” will occur each day for all vehicles with gasoline fuel in the tank, even when stationary.

ii) Hot soak loss

This represents evaporation from the fuel delivery system when a hot engine is turned off and the vehicle is stationary.  It arises from transfer of heat from the engine and hot exhaust to the fuel system where fuel is no longer flowing.  Carburettor float bowls contribute significantly to hot soak losses.

iii) Running loss

These are evaporative losses that occur while the vehicle is in motion.

Evaporative emissions are dependent on ambient temperature and the volatility of the fuel and, in the case of diurnal losses, on the daily rise in ambient temperature.  Fuel volatility is usually expressed by the empirical fuel parameter known as Reid vapour pressure (RVP).  For each of these mechanisms, equations relating evaporative emissions to ambient temperature and RVP were developed by analysis of empirically based formulae derived in a series of CONCAWE research studies in combination with UK measurements data reported by TRL.  Separate equations were developed for vehicles with and without evaporative control systems fitted such as carbon canister devices.  The overall methodology is similar to that reported by COPERT II (European Environment Agency, 1997), but the data are considered to be more UK-biased.

Evaporative emissions are calculated using monthly average temperature and RVP data.  Using this information, evaporative emissions are calculated from the car fleet for each month of the year and the values summed to derive the annual emission rates.  Calculating emissions on a monthly basis enables subtle differences in the seasonal fuel volatility trends and differences in monthly temperatures to be better accounted for.  Monthly mean temperatures from 1970-2003 were used for the calculations based on Met Office for Central England (CET data).  The monthly average, monthly average daily maximum and monthly average diurnal rise in temperatures were required.  The monthly average RVP of petrol sold in the UK used historic trends data on RVP and information from UKPIA on the RVP of summer and winter blends of fuels supplied in recent years and their turnover patterns at filling stations (Watson, 2001, 2003).  The average RVP of summer blends of petrol in the UK in 2003 was 68 kPa, 2kPa below the limit set by European Council Directive 98/70/EC for Member States with “arctic” summer conditions (UKPIA, 2004).

All the equations for diurnal, hot soak and running loss evaporative emissions from vehicles with and without control systems fitted developed for the inventory are shown in Table A3.3.22.  The inventory uses equations for Euro I cars with “first generation” canister technology, based on early measurements, but equations taken from COPERT III leading to lower emissions were used for Euro II-IV cars as these better reflected the fact that modern cars must meet the 2g per test limit on evaporative emissions by the diurnal loss and hot soak cycles under Directive 98/69/EC.

For diurnal losses, the equations for pre-Euro I (non-canister) and Euro I cars were developed from data and formulae reported by CONCAWE (1987), TRL (1993) and ACEA (1995).  Equations for Euro II-IV cars were taken from COPERT III.  The equations specified in Table A3.3.22 give diurnal loss emissions in g/vehicle.day for uncontrolled (DLuncontrolled) and Euro I and Euro II-IV canister controlled vehicles (DLEU1, DLEUII-IV).  Total annual diurnal losses were calculated from the equation:

Ediurnal	  = 365 . N . (DLuncontrolled . Funcontrolled + DLEU1 . FEUI + DLEUII-IV . FEUII-IV)

where:

N	  = 	number of petrol vehicles (cars and LGVs) in the UK parc

Funcontrolled	  = 	fraction of vehicles not fitted with carbon canisters, assumed to be the same as the fraction of pre-Euro I vehicles

FEUI	  = 					fraction of Euro I vehicles in the fleet

FEUII-IV	  = 	fraction of Euro II-IV vehicles in the fleet

For hot soak losses, the equations were developed from data and formulae reported by CONCAWE (1990), TRL (1993) and COPERT II.  The equations specified in Table A3.3.22 give hot soak loss emissions in g/vehicle.trip for uncontrolled (HSuncontrolled) and Euro I and Euro II-IV canister controlled (HSEUI, HSEUII-IV) vehicles.  Total annual hot soak losses were calculated from the equation:

Ehot soak	     =  (VKM/ ltrip) . (HSuncontrolled . Funcontrolled + HSEU1 . FEUI + HSEUII-IV . FEUII-IV)

where

VKM	= 	total number of vehicle kilometres driven in the UK by the petrol vehicles (cars and LGVs)

ltrip	= 	average trip length (8.4 km in the UK)

For running losses, the equations were developed from data and formulae reported by CONCAWE (1990) and COPERT II.

The equations specified in Table A3.3.22 give running loss emissions in g/vehicle.km for uncontrolled (RLuncontrolled) and canister controlled (RLcontrolled) vehicles with no distinction made between Euro I and Euro II-IV canister cars.  Total annual running losses were calculated from the equation:

Erunning loss	  = VKM. (RLuncontrolled . Funcontrolled + RLcontrolled . Fcontrolled)

where

Fcontrolled = FEUI + FEUII-IV

Table A3.3.17	NMVOC Emission Factors for Road Transport (in g/km)

Table A3.3.18	NOx Emission Factors for Road Transport (in g/km)

Table A3.3.19	CO Emission Factors for Road Transport (in g/km)

Table A3.3.20	Methane Emission Factors for Road Transport (in g/km)

Table A3.3.21	N2O Emission Factors for Road Transport (in g/km)

Table A3.3.22	Equations for diurnal, hot soak and running loss evaporative emissions from vehicles with and without control systems fitted

| Emission factor | Units | Uncontrolled vehicle (pre-Euro I) |
| --- | --- | --- |
| Diurnal loss (DLuncontrolled) | g/vehicle.day | 1.54 * (0.51*Trise + 0.62*Tmax + 0.22*RVP - 24.89) |
| Hot soak (HSuncontrolled) | g/vehicle.trip | exp(-1.644 + 0.02*RVP + 0.0752*Tmean) |
| Running loss (RLuncontrolled) | g/vehicle.km | 0.022 * exp(-5.967 + 0.04259*RVP + 0.1773*Tmean) |

| Emission factor | Units | Carbon canister controlled vehicle (Euro I) |
| --- | --- | --- |
| Diurnal loss (DLEUI) | g/vehicle.day | 0.3 * (DLuncontrolled) |
| Hot soak (HSEUI) | g/vehicle.trip | 0.3 * exp(-2.41 + 0.02302*RVP + 0.09408*Tmean) |
| Running loss (RLcontrolled) | g/vehicle.km | 0.1 * (RLuncontrolled) |

| Emission factor | Units | Carbon canister controlled vehicle (Euro II-IV) |
| --- | --- | --- |
| Diurnal loss (DLEUII-IV) | g/vehicle.day | 0.2 * 9.1 * exp(0.0158*(RVP-61.2) + 0.0574*(Tmax-Trise-22.5) + 0.0614*(Trise-11.7)) |
| Hot soak (HSEUII-IV) | g/vehicle.trip | 0 |
| Running loss (RLcontrolled) | g/vehicle.km | 0.1 * (RLuncontrolled) |

where

Trise 	= 	diurnal rise in temperature in oC

Tmax 	= 	maximum daily temperature in oC

Tmean 	= 	annual mean temperature in oC

RVP 	= 	Reid Vapour Pressure of petrol in kPa

      - Navigation

The NAEI estimates emissions from coastal shipping, fishing, naval shipping and international marine.  Coastal shipping has been mapped onto 1A3dii National Navigation and fishing onto 1A4ciii Fishing.  The category coastal shipping does contain emissions from offshore diesel oil use.  A proportion of this will be marine transport associated with the offshore industry but some is diesel oil use in turbines, motors and heaters on offshore installations.  The emissions reported under coastal shipping, naval shipping and fishing are estimated according to the base combustion module using the emission factors given in Table A3.3.1.

The NAEI category International Marine is the same as the IPCC category 1A3i International Marine.  The estimate used is based on the following information and assumptions:

(i)	Total deliveries of fuel oil, gas oil and marine diesel oil to marine bunkers are given in DTI (2004).

(ii)	Naval fuel consumption is assumed to be marine diesel oil (MOD, 2004).  Emissions from this source are not included here but are reported under 1A5 Other.

(iii)	The fuel consumption associated with international marine is the marine bunkers total minus the naval consumption.  The emissions were estimated using the emission factors shown in Table A3.3.1.

Emissions from 1A3i International Marine are reported for information only and are not included in national totals.  Bunker fuels data for shipping are provided to the DTI by UKPIA, and are based on sale of fuels to UK operators going abroad and overseas operators (who it is presumed were also heading abroad) (DTI, per. comm).

    - Other Sectors (1A4)

The mapping of NAEI categories to 1A4 Other Sectors is shown in Section A3.2.  The estimation procedure follows that of the base combustion module.  The NAEI category public service is mapped onto 1A4a Commercial and Institutional.  This contains emissions from stationary combustion at military installations, which should be reported under 1A5a Stationary.  Also included are stationary combustion emissions from the railway sector, including generating plant dedicated to railways.  Also included in 1A4 are emissions from the ‘miscellaneous’ sector.  This includes emissions from the commercial sector and some service industries.

Emissions from 1A4b Residential and 1A4c Agriculture/Forestry/Fishing are disaggregated into those arising from stationary combustion and those from off-road vehicles and other machinery.  The estimation of emissions from off-road sources is discussed in Section A3.7.7.1.  Emissions from fishing vessels are estimated from fuel consumption data (DTI, 2004) and emission factors are shown in Table A3.3.1.

    - Other (1A5)

Emissions from military aircraft and naval vessels are reported under 1A5b Mobile.  The method of estimation is discussed in Sections A3.3.5.1 and A3.3.5.4 with emission factors given Table A3.3.1.  Military stationary combustion is included under 1A4a Commercial and Institutional.  Emissions from off-road sources are estimated and are reported under the relevant sectors, i.e. Other Industry, Residential, Agriculture and Other Transport.  The methodology of these estimates is discussed in Section A3.3.7.1.

      - Estimation of other offroad sources

These cover emissions from a range of portable or mobile equipment powered by reciprocating diesel or petrol driven engines.  They include agricultural equipment such as tractors and combine harvesters; construction equipment such as bulldozers and excavators; domestic lawn mowers; aircraft support equipment; and industrial machines such as portable generators and compressors.  In the NAEI they are grouped into four main categories:

- domestic house & garden

- agricultural power units (includes forestry)

- industrial off-road (includes construction and quarrying)

- aircraft support.

The mapping of these categories to the appropriate IPCC classes is shown in Section 3.2.  Aircraft support is mapped to Other Transport and the other categories map to the off-road vehicle subcategories of Residential, Agriculture and Manufacturing Industries and Construction.

The estimates are calculated using a modification of the methodology given in EMEP/ CORINAIR (1996).  This involves the estimation of emissions from around seventy classes of off-road source using the following equation for each class:

Ej	=	Nj . Hj . Pj . Lj . Wj.(1 + Yj . aj /2). ej

where

Ej	=	Emission of pollutant from class j 			(kg/y)

Nj	=	Population of class j.

Hj	=	Annual usage of class j				(hours/year)

Pj	=	Average power rating of class j			(kW)

Lj	=	Load factor of class j				(-)

Yj 	=	Lifetime of class j 					(years)

Wj	=	Engine design factor of class j			(-)

aj	=	Age factor of class j				(y-1 )

ej	=	Emission factor of class j				(kg/kWh)

For petrol-engined sources, evaporative NMVOC emissions are also estimated as:

Evj	=	Nj . Hj . evj

where

Evj	=	Evaporative emission from class j			kg

evj	=	Evaporative emission factor for class j		kg/h

The population, usage and lifetime of different types of off-road machinery were updated following a study carried out recently by NETCEN on behalf of the Department for Transport (NETCEN, 2004a).  This study researched the current UK population, annual usage rates, lifetime and average engine power for a range of different types of diesel-engined non-road mobile machinery.  Additional information including data for earlier years were based on research by Off Highway Research (2000) and market research polls amongst equipment suppliers and trade associations by Precision Research International on behalf of the former DoE (Department of the Environment) (PRI, 1995, 1998).   Usage rates from data published by Samaras et al (1993, 1994) were also used.

The emission factors used came mostly from EMEP/CORINAIR (1996) though a few of the more obscure classes were taken from Samaras & Zierock (1993).  The load factors were taken from Samaras (1996).  Emission factors for garden machinery, such as lawnmowers and chainsaws were updated following a recent review by NETCEN (2004b), considering the impact of Directive 2002/88/EC on emissions from these types of machinery.

It was possible to calculate fuel consumptions for each class of machinery based on fuel consumption factors given in EMEP/CORINAIR (1996).  Comparison with known fuel consumption for certain groups of classes (e.g. agriculture and construction) suggested that the population method over estimated fuel consumption by factors of 5-12.  Hence the methodology was modified in the following way:

1) 	Aggregate emission factors were calculated for each of the four main categories listed above as:

ep	=	E

F

where

ep	=	Aggregate emission factor for main NAEI category 	(kg/t fuel)

E	=	Sum of emissions of pollutant from classes within main NAEI category calculated from the population approach (kg)

F	=	Sum of fuel consumption from classes within main NAEI category calculated from the population approach (tonnes)

2)	Estimates were derived for the fuel consumptions for the years 1990-2003 for each of the four main categories:

A.	Agricultural power units:  Data on gas oil consumption were taken from DTI (2004).  The consumption of petrol was estimated using the population method for 1995 without correction.  The same estimate was used for 1990 to 2003.

B.	Aircraft support: Data on diesel oil consumption at Heathrow Airport were extrapolated on the basis of the number of takeoffs and landings (DfT, 2004c; Leech, 1994)

C.	Industrial off-road: The construction component of the gas oil consumption was calculated from DUKES data (DTI, 2004) on building and contracting; mines and quarrying and water giving a time series for 1990-2003.  Gas oil consumption by other types of industrial machinery was estimated from the population approach.  This gave an estimate for 1995, which was used for all years.  The petrol consumption was estimated from the population approach for 1995 and used for the period 1990-2003.  Earlier years were extrapolated based on the building, contracting, mines, quarrying and water diesel consumption.

D.	Domestic house & garden: Petrol and diesel oil consumption were estimated from the EMEP/CORINAIR population approach for 1995 and the same value used for all years.

3)	The emission for each of the four main NAEI categories was estimated as:

Ep	=	ep . Ap

where

Ap	=	Fuel consumption of NAEI main category p (tonnes)

Emissions from off-road sources are particularly uncertain.  The population and usage data found by a recent study (NETCEN 2004a) produced higher gas-oil fuel consumption estimates than the DTi data for sector groups by between 5 and 12 times.  The DTi gas-oil fuel data are currently mainly used in estimation of off-road emissions.  This is under review.

The petrol consumed by off-road machinery is still only a tiny proportion of total petrol sales.

The aggregate emission factors calculated for each NAEI category are shown in Table A3.3.23.

Table A3.3.23	Aggregate Emission Factors for Off-Road Source Categories1 in 2003 (t/kt fuel)

| Source | Fuel | C2 | CH4 | N2O | NOx | CO | NMVOC | SO23 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Domestic House&Garden | DERV | 863 | 0.165 | 1.36 | 50.4 | 6.6 | 3.57 | 0.08 |
| Domestic House&Garden | Petrol | 855 | 5.47 | 0.045 | 5.92 | 1043 | 227 | 0.08 |
| Agricultural Power Units | Gas Oil | 870 | 0.158 | 1.28 | 48.7 | 16.1 | 6.9 | 3.08 |
| Agricultural Power Units | Petrol | 855 | 7.53 | 0.02 | 1.98 | 1367 | 753 | 0.08 |
| Industrial Off-road | Gas Oil | 870 | 0.162 | 1.35 | 42.56 | 17.4 | 6.69 | 3.08 |
| Industrial Off-road | Petrol | 855 | 6.03 | 0.067 | 8.15 | 2388 | 125 | 0.08 |
| Aircraft Support | Gas Oil | 870 | 0.167 | 1.34 | 45.5 | 12.5 | 5.4 | 3.08 |

- Emission factors reported are for 2003

- Emission factor as kg carbon/t, UKPIA (2004)

- UKPIA (2004), Factor for 2003

The emission factors used for carbon dioxide were the standard emission factors for DERV, gas oil and petrol given in Table A3.3.1.

    - Fugitive Emissions From fuels (1B)

      - Solid fuels (1B1)

        - Coal Mining

Emissions for IPCC categories 1B1ai Underground Mines-mining, 1B1ai Underground Mines-post-mining and 1B1aii Surface Mines are calculated from saleable coal production statistics reported in DTI, (2004).  Licensed mines referred to privately owned mines and were generally smaller and shallower than previously nationalised mines.  The distinction was sufficiently marked to allow the use of a separate emission factor.  Data on the shallower licensed mines are supplied by Barty (1995) up to 1994.  Following privatisation, the distinction between licensed mines and deep mines no longer exists and all domestically produced coal that is not open-cast is assumed to be deep mined.  For 1995, data from 1994 were used but in subsequent years the distinction has been abandoned.  The emission factors used are shown in Table A3.3.24.

Table A3.3.24	Methane Emission Factors for Coal Mining (kg/t coal)

|  | Deep Mined | Coal Storage & Transporta | Licensed Mined | Open Castc |
| --- | --- | --- | --- | --- |
| 1990 | 10.0a | 1.16 | 1.36 | 0.34 |
| 1991 | 10.2 a | 1.16 | 1.36 | 0.34 |
| 1992 | 11.0 a | 1.16 | 1.36 | 0.34 |
| 1993 | 13.1 a | 1.16 | 1.36 | 0.34 |
| 1994 | 13.0 a | 1.16 | 1.36 | 0.34 |
| 1995 | 13.0 a | 1.16 | 1.36 | 0.34 |
| 1996 | 13.4 a | 1.16 | 1.36 | 0.34 |
| 1997 | 13.4 a | 1.16 | 1.36 | 0.34 |
| 1998 | 13.4 a | 1.16 | - | 0.34 |
| 1999 | 13.5 | 1.16 | - | 0.34 |
| 2000 | 14.0b | 1.16 | - | 0.34 |
| 2001 | 12.6 b | 1.16 | - | 0.34 |
| 2002 | 13.5 b | 1.16 | - | 0.34 |
| 2003 | 9.1 b | 1.16 | - | 0.34 |

a	Bennet et al (1995)

b	Factor based on UK Coal Mining Ltd data

c	Williams (1993)

The licensed and open cast factors are taken from Williams (1993).  The deep mined factors for 1990 -1992 and the coal storage factor are taken from Bennet et al (1995).  This was a study on deep mines which produced estimates of emissions for the period 1990-93.  This was a period over which significant numbers of mines were being closed, hence the variation in emission factors.  The emission factors for 1998-2003 are based on operator's measurements of the methane extracted by the mine ventilation systems.  The mines surveyed cover around 90% of deep mined production.  No time series data are available for 1993-97, so the 1998 factor was used.  Methane extracted is either emitted to atmosphere or utilised for energy production.  Methane is not flared for safety reasons.  The factors reported in Table A3.3.24 refer to emissions and exclude the methane utilised.  The coal storage and transport factor is only applied to deep mined coal production.

The activity data for the coal mining emissions are reported in the CRF tables attached as a CD ROM to this report.  It should be noted that the number of active deep mines reported is defined as the number of mines producing at any one time during the period (Coal Authority, 2004).  Hence, this would include large mines as well as small ones or those that only produced for part of the year.  The colliery methane utilisation data are taken from DTI (2004).

A new source, methane from closed coal mines, is now included in sector 1B1a of the UK inventory following publication of the findings of a recent study by White Young Green funded by Defra (Kershaw, 2005).

Methane emissions from abandoned mines arise at the surface through many possible flow paths: vents into the workings to relieve pressure, old mine entries, diffuse emission through fractured and permeable strata.  Direct measurement of the total quantity of gas released from abandoned mines is not practical.  Emission estimates for 1990 to 2004 have been calculated using a relationship between emission and the quantity of the underlying methane gas within the abandoned mine workings.

Methane reserves have been calculated for all UK coalfields that are not totally flooded between 1990 and 2005.  The gas reserves are calculated by totalling all the gas quantities in individual seams likely to have been disturbed by mining activity.  To enable calculation of the reserves over time, it has been necessary to calculate the rises in water levels in the abandoned mines due to water inflow.  As workings become flooded they cease to release significant amounts of methane to the surface.

Monitoring has been carried out to measure methane emission from vents and more diffuse sources.  Monitoring of vents involved measurement of the flows and concentrations of the gas flowing out of the mine.  Monitoring of more diffuse sources required collection of long-term gas samples to measure any increases in background atmospheric methane level in the locality.

Methane flows measured by both methods showed a general increase with the size of the underlying gas reserve. The data indicated an emission of 0.74% of the reserve per year as a suitable factor to apply to the methane reserve data in order to derive methane emission estimates for abandoned UK coalfields for 1990 to 2004.

        - Solid Fuel Transformation

This section discusses fugitive emissions from solid fuel transformation processes.  These emissions are reported in IPCC category 1B1b.  The IPCC Revised 1996 Guidelines do not provide any methodology for such estimates, hence emissions are largely based on default emission factors.  Combustion emissions from these processes have already been discussed in Section A3.3.3.

In a coke oven, coal is transformed into coke and coke oven gas.  The coke oven gas is used as a fuel to heat the coke oven or elsewhere on the site.  The coke may be used elsewhere as a fuel or as a reducing agent in metallurgical processes.  A carbon balance is performed over the coke oven on the carbon contents of the fuels input and the fuels produced as described in Section A.3.3.1.

Process emissions of other pollutants from coke ovens are estimated either on the basis of total production of coke or the coal consumed.  The emission factors used are given in Table A3.3.25.

Emissions of carbon from solid smokeless fuel production are calculated using a mass balance approach, described previously in Section A.3.3.1.  A similar mass balance is carried out for SO2. For emissions of other pollutants, a mass balance approach is not used.  It is likely that emissions will arise from the combustion of the gases produced by some SSF retorts but this combustion is not identified in the energy statistics.  Process emissions from SSF plant are estimated on the basis of total production of SSF.  The emission factors used are given in Table A3.3.25 and are based on USEPA (2004) factors for coke ovens.  There are a number of processes used in the UK ranging from processes similar to coking to briquetting of anthracite dust and other smokeless fuels.  Given the number of processes in use these estimates will be very uncertain.

Data are available on the production of SSF and the fuels used (DTI, 2004), however it is clear that in recent years both coke and imported petroleum coke have been used in the production of smokeless fuels.  Data on the total UK imports and exports of petroleum coke are available but little information is available on its consumption.  In the GHGI, it is assumed that 100 kt per annum of petroleum coke were used in SSF production from 1992 to 1998 based on DETR (1998).  For 1999-2003 approximate estimates by DTI (2004) were used.  The carbon content of the petroleum coke consumed is not included in the SSF carbon balance – instead it is allocated to the domestic sector as a separate fuel.  Coke used by SSF manufacturers is assumed to be burnt as a fuel and is also not included in the carbon balance.  The model used is not entirely satisfactory but further information would be required before a more accurate carbon balance could be developed.

Emissions from the combustion of fuels to heat the smokeless fuel retorts are reported under 1A1ci Manufacture of Solid Fuels, however process emissions and the residual carbon emission discussed above are considered to be fugitives and are reported under 1B1b Solid Fuel Transformation.

Table A3.3.25	Emission Factors Used for Coke and Solid Smokeless Fuel Production

|  | Units | CH4 | CO | NOx | SO2 | NMVOC |
| --- | --- | --- | --- | --- | --- | --- |
| Coke | kt/Mt coke made | 0.081a | 2.51c | - | 1.15c | 0.024c |
| Coke | kt/Mt coal consumed | - | - | 0.02b |  | - |
| SSF | kt/Mt SSF made | 0.081a | - | - | - | 0.018a |
| SSF | kt/Mt coal consumed | - | 0.3b | 0.005b | 0d | - |

a	EIPPCB, (2000)

b	USEPA (2004)

c	Factor for 2003 based on Environment Agency (2004)

d	Based on mass balance but zero for 2002 (because calculated sulphur content of SSF produced was higher than the sulphur content of coal used to make the SSF).

      - Oil and Natural Gas (1B2)

The emissions reported in this sector pertain to the offshore platforms and onshore terminals on the UK Continental Shelf Area and represented by the United Kingdom Offshore Operators Association (UKOOA).

Emissions estimates for the offshore industry are based on data provided by the trade organisation, UKOOA, through their annual emissions reporting mechanism to UK regulators, the Environmental Emissions Monitoring System (EEMS).  This system provides a detailed inventory of point source emissions estimates, based on operator returns for the years 1995-2003.  Additional, more detailed data on CO2 emissions from some offshore combustion processes has become available as a result of the development of the industry’s National Allocation Plan pertaining to the EU Emission Trading Scheme.  Therefore, for the main combustion sources in the offshore oil & gas sector, the UKOOA data from 1998 onwards is sourced via NAP estimates, replacing historic estimates previously reported via EEMS.

For years prior to 1995 (i.e. pre-EEMS), emission totals are based on an internal UKOOA summary report produced in 1998.  These data were revised and reported with the 1995-2003 datasets in the UKOOA 2005 submision to the inventory.

The 1998 UKOOA report presented data from detailed industry studies in 1991 and 1995 to derive emission estimates for 1990 from available operator estimates.  Emission estimates for 1991-1994 were then calculated using production-weighted interpolations.  Only limited data were available from operators in 1990-1994, and emission totals could only be estimated in broadly aggregated sectors of: drilling (offshore), production (offshore), loading (offshore) and total emissions onshore.  Estimates of the more detailed oil & gas processing source sectors for 1990-1994 are therefore based on applying the fraction of total emissions derived from the 1995 data from EEMS.

The data provided in 2005 by UKOOA (Russell, 2005) therefore constitutes a significant revision of the time-series of emissions from the offshore oil & gas sector, with the 1995 split of emissions between the different sector sources applied to the 1990-1994 emission totals, replacing data that were based on a range of assumptions and estimates that were not as detailed as those that are now available.

UKOOA has periodically produced operator guidance for estimating and reporting emissions from offshore oil & gas processes, with recent guidance covering CO2 emissions from combustion processes (Russell, 2004) and other guidance for emissions of other pollutants such as CH4, VOCs, NOX and SO2 (UKOOA, 2001).  A spreadsheet-based tool has also been developed to improve quality and consistency of operator reports. The operators are required to report flaring and venting masses and compositions.  Methodologies have been developed to estimate combustion emissions from turbines, boilers and heaters and fugitive emissions.  These are calculated on a plant item-by-item basis.

For some sections of the EEMS data prior to 1999 where detailed data were not provided, estimates have been made on the basis of oil and gas throughput. However, since improvements in the reporting system have minimised the need for such assumptions, since 1999 the majority of emission estimates are based on detailed data.  For example, prior to 1999, N2O emissions were not monitored by the plant operators and hence the 1990-1998 N2O emissions data are based on emission factors (derived from the 1999 data) and estimated production throughput data.

The data reported in the EEMS database must be reconciled with the UK Energy Statistics and integrated into the NAEI without double counting emissions.  The diesel oil consumption by offshore installations is not reported separately in the UK Energy Statistics but is included under coastal shipping.  In order to avoid double counts the UKOOA estimates have been corrected to remove diesel oil emissions.

In the NAEI, offshore emissions are estimated in the following categories each with its own methodology:

- Offshore flaring

- Offshore Oil & Gas (well testing)

- Offshore Oil & Gas (venting)

- Offshore Oil & Gas

- Offshore Loading

- Onshore loading

- Oil Terminal Storage

- Offshore own gas use (reported under 1A1c Other Energy Industries)

- Gas Separation Plant (Combustion) (reported under 1A1c Other Energy Industries)

The mapping of these sources to IPCC categories is described in Section A3.2.  Activity data are reported in the CRF Background Table 1B2, however in most cases these data are not used to calculate the emissions, but are provided for comparison with other inventories.

        - Offshore flaring

This includes flaring from offshore platforms and onshore terminals.  Flaring emission data for CO2, SO2, NOx, CO, NMVOC, and CH4 were taken from the UKOOA (2005) dataset.  Data from 1995-2003 are based on detailed operator returns, whilst 1990-1994 data are calculated from extrapolation of total emissions data and the use of 1995 data splits between sources. N2O emissions are based on operator information from 1999-2003, and on emission factors and production throughput data for 1990-1998.

The aggregate emission factors are given in Table A3.3.26 and the activity data in Table A3.3.27.  The aggregate emission factors for 1990-2003 are reported as kg pollutant/kg gas flared and are calculated from the emissions and activity data reported within the UKOOA 2005 dataset.

The UKOOA data from the EEMS database do not include flaring on onshore oil production fields.  These emissions are estimated by extrapolation using flaring volume data collected by DTI (2004a) and the offshore flaring factors.  The onshore flaring data are shown in Table A3.3.27 though the contribution is very small.

Table A3.3.26	Aggregate Emission Factors for Offshore Gas Flaring

|  | CO2 | CH4 | NOx | CO | NMVOC | SO2 | N2O | Units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2003 | 2.64 | 0.0101 | 0.0012 | 0.00676 | 0.00664 | 0.00016 | 0.00008 | Kg/kg |
| 2002 | 2.64 | 0.0097 | 0.00160 | 0.00681 | 0.00698 | 0.00015 | 0.00008 | Kg/kg |
| 2001 | 2.63 | 0.0099 | 0.00128 | 0.00662 | 0.00700 | 0.00120 | 0.00008 | Kg/kg |
| 2000 | 2.55 | 0.0109 | 0.00117 | 0.00647 | 0.00642 | 0.00095 | 0.00008 | Kg/kg |
| 1999 | 2.66 | 0.0103 | 0.00127 | 0.00687 | 0.00819 | 0.00168 | 0.00008 | kg/kg |
| 1998 | 2.70 | 0.0107 | 0.00157 | 0.00836 | 0.00901 | 0.00033 | 0.00009 | kg/kg |
| 1997 | 2.70 | 0.0107 | 0.00165 | 0.00866 | 0.00969 | 0.00040 | 0.00009 | kg/kg |
| 1996 | 2.70 | 0.0104 | 0.00150 | 0.00870 | 0.00961 | 0.00002 | 0.00010 | kg/kg |
| 1995 | 2.63 | 0.0102 | 0.00171 | 0.00871 | 0.00979 | 0.00009 | 0.00010 | kg/kg |
| 1994 | 2.60 | 0.0101 | 0.00181 | 0.00895 | 0.01134 | 0.00007 | 0.00011 | kg/kg |
| 1993 | 2.60 | 0.0107 | 0.00193 | 0.00915 | 0.01185 | 0.00007 | 0.00011 | kg/kg |
| 1992 | 2.60 | 0.0125 | 0.00008 | 0.00939 | 0.01244 | 0.00008 | 0.00011 | kg/kg |
| 1991 | 2.60 | 0.0134 | 0.00008 | 0.00965 | 0.01310 | 0.00008 | 0.00011 | kg/kg |
| 1990 | 2.60 | 0.0140 | 0.00008 | 0.00965 | 0.01254 | 0.00008 | 0.00011 | kg/kg |

Flaring is reported under 1B2ciii Flaring – Combined, since many of the platforms produce both oil and gas.  An estimate of NMVOC emissions from refinery flares is reported in 1B2ci Venting and Flaring: Oil.  This is based on estimates supplied by UKPIA (2004).

Table A3.3.27	Activity Data for Offshore Gas Flaring

|  | Gas Flared (kt)1 | Gas Flared, Offshore Fields & Terminals (Mm3)2 | Gas Flared, Onshore Fields (Mm3)2 |
| --- | --- | --- | --- |
| 2003 | 1478 | 1697 | 21.4 |
| 2002 | 1710 | 1665 | 19.1 |
| 2001 | 1869 | 1808 | 11.6 |
| 2000 | 1987 | 1814 | 15.0 |
| 1999 | 2113 | 2206 | 4.6 |
| 1998 | 20563 | 2110 | 0.0 |
| 1997 | 20423 | 2122 | 7.0 |
| 1996 | 23083 | 2539 | 15.0 |
| 1995 | 22723 | 2388 | 11.0 |
| 1994 | 2164 | 3282 | 10 |
| 1993 | 2034 | 2461 | 16 |
| 1992 | 1905 | 2468 | 12 |
| 1991 | 1775 | 2531 | 7 |
| 1990 | 1796 | 2793 | 7 |

- EEMS (2005)

- DTI (2004) and previous editions.

- A correction has been applied for non-reporting operators

NE	Not Estimated

        - Offshore gas use

This refers to the use of unrefined natural gas on offshore platforms and onshore terminals as a fuel in heaters, boilers, turbines and reciprocating engines.  Gas combustion emission data for CO2, SO2, NOx, CO, NMVOC, and CH4 were taken from the UKOOA (2005) dataset.  Data from 1995-2003 are based on detailed operator returns, whilst 1990-1994 data are calculated from extrapolation of total emissions data and the use of 1995 data splits between sources. N2O emissions are based on operator information from 1999-2003, and on emission factors and production throughput data for 1990-1998.

The aggregate emission factors are given in Table A3.3.28. The aggregate emission factors for 1990-2003 are reported as kg pollutant/ kg gas used and are calculated from the emissions and activity data reported within the UKOOA 2005 dataset.

Table A3.3.28	Aggregate Emission Factors for Offshore Own Gas Use

|  | CO2 | CH4 | NOx | CO | NMVOC | SO2 | N2O | Units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2003 | 2.71 | 0.00120 | 0.00803 | 0.00286 | 0.00010 | 0.00008 | 0.00022 | kg/kg |
| 2002 | 2.63 | 0.00123 | 0.00830 | 0.00278 | 0.00011 | 0.00008 | 0.00024 | kg/kg |
| 2001 | 2.67 | 0.00121 | 0.00653 | 0.00290 | 0.00010 | 0.00048 | 0.00021 | kg/kg |
| 2000 | 2.68 | 0.00126 | 0.00657 | 0.00281 | 0.00009 | 0.00051 | 0.00021 | kg/kg |
| 1999 | 2.50 | 0.00129 | 0.00656 | 0.00278 | 0.00009 | 0.00067 | 0.00017 | kg/kg |
| 1998 | 2.62 | 0.00048 | 0.00670 | 0.00261 | 0.00008 | 0.00013 | 0.00021 | kg/kg |
| 1997 | 2.74 | 0.00063 | 0.00705 | 0.00275 | 0.00011 | 0.00052 | 0.00021 | kg/kg |
| 1996 | 2.68 | 0.00044 | 0.00816 | 0.00303 | 0.00011 | 0.00003 | 0.00021 | kg/kg |
| 1995 | 2.69 | 0.00045 | 0.00827 | 0.00312 | 0.00009 | 0.00018 | 0.00021 | kg/kg |
| 1994 | 2.69 | 0.00044 | 0.00876 | 0.00316 | 0.00010 | 0.00017 | 0.00024 | kg/kg |
| 1993 | 2.69 | 0.00047 | 0.00934 | 0.00323 | 0.00011 | 0.00017 | 0.00024 | kg/kg |
| 1992 | 2.69 | 0.00055 | 0.01000 | 0.00331 | 0.00011 | 0.00018 | 0.00024 | kg/kg |
| 1991 | 2.69 | 0.00058 | 0.01075 | 0.00620 | 0.00012 | 0.00019 | 0.00024 | kg/kg |
| 1990 | 2.69 | 0.00061 | 0.01075 | 0.00341 | 0.00011 | 0.00019 | 0.00024 | kg/kg |

These emissions apply to the mixture of methane, ethane, propane and butane used.  In the NAEI database they are reported in the categories:

- Offshore own gas use: natural gas

- Gas separation plant: LPG

- Gas separation plant: OPG

Emissions are reported under 1A1cii Other Energy Industries.

        - Well testing

This activity involves the combustion of crude oil and crude gas during well testing, and is an activity that is not included in UK Energy Statistics from the DTI.  Combustion emission data for CO2, SO2, NOx, CO, NMVOC, and CH4 were taken from the UKOOA (2005) dataset.  Data from 1995-2003 are based on detailed operator returns, whilst 1990-1994 data are calculated from extrapolation of total emissions data and the use of 1995 data splits between sources. N2O emissions are based on operator information from 1999-2003, and on emission factors and production throughput data for 1990-1998.

The estimates of the amounts of crude oil and gas burnt are considered to be fairly speculative. Data is provided by the DTI regarding the number of wells tested annually, but the number of wells tested is only a small proportion of the number of wells explored and that proportion may vary from year to year.  Also the number of wells explored varies considerably from year to year.

The aggregate emission factors are given in Table A3.3.29.  Well testing is reported under 1B2a Oil Production since many of the wells produce oil and gas.

Table A3.3.29	Aggregate Emission Factors for Offshore Well Testing (kt/well explored)

|  | C | SO2 | NOx | CO | NMVOC | CH4 | N2O | Units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2003 | 1.40 | 0.0000 | 0.0038 | 0.0192 | 0.0222 | 0.0646 | 0.0001 | Kt/well |
| 2002 | 2.51 | 0.0000 | 0.0083 | 0.0412 | 0.0522 | 0.0988 | 0.0002 | Kt/well |
| 2001 | 1.23 | 0.0000 | 0.0043 | 0.0212 | 0.0274 | 0.0462 | 0.0001 | Kt/well |
| 2000 | 1.45 | 0.0000 | 0.0052 | 0.0256 | 0.0335 | 0.0524 | 0.0001 | Kt/well |
| 1999 | 3.61 | 0.0001 | 0.0111 | 0.0558 | 0.0685 | 0.1513 | 0.0004 | Kt/well |
| 1998 | 3.66 | 0.1290 | 0.0119 | 0.0609 | 0.0648 | 0.1563 | 0.0004 | Kt/well |
| 1997 | 2.77 | 0.1031 | 0.0089 | 0.0459 | 0.0516 | 0.1203 | 0.0003 | Kt/well |
| 1996 | 2.34 | 0.0923 | 0.0080 | 0.0411 | 0.0462 | 0.1077 | 0.0002 | Kt/well |
| 1995 | 2.69 | 0.1000 | 0.0087 | 0.0445 | 0.0500 | 0.1167 | 0.0003 | Kt/well |
| 1994 | 3.07 | 0.1015 | 0.0081 | 0.0470 | 0.0492 | 0.1002 | 0.0003 | Kt/well |
| 1993 | 3.51 | 0.1202 | 0.0098 | 0.0549 | 0.0588 | 0.1215 | 0.0003 | Kt/well |
| 1992 | 2.26 | 0.0806 | 0.0068 | 0.0363 | 0.0398 | 0.0917 | 0.0002 | Kt/well |
| 1991 | 1.46 | 0.0542 | 0.0047 | 0.1460 | 0.0270 | 0.0633 | 0.0001 | Kt/well |
| 1990 | 0.99 | 0.0369 | 0.0032 | 0.0164 | 0.0176 | 0.0451 | 0.0001 | Kt/well |

        - Other emissions from platforms and terminals

These include emissions of CH4 and NMVOC from platforms and terminals arising from cold venting, other fugitive emissions and also from storage of crude oil at terminals.  Emissions data were taken from the UKOOA (2005) dataset.  Data from 1995-2003 are based on detailed operator returns, whilst 1990-1994 data are calculated from extrapolation of total emissions data and the use of 1995 data splits between sources.

These other emissions from platforms and terminals are reported in the following NAEI categories, all mapped to 1B2a Oil ii Production: offshore oil & gas (fugitive and process emissions), offshore venting and oil terminal storage. It is not possible to split oil and gas production emissions since oil and gas are frequently produced on the same platform.

Table A3.3.30	Aggregate Emission Factors used for Emissions from Platforms and Terminals

|  | Period | Units | CH4 | NMVOC |
| --- | --- | --- | --- | --- |
| Gas Platforms Oil Platforms Oil/Gas Platforms | 1970-92 1970-92 1970-92 | kt/installation kt/installation kt/installation | 0.589 0.327 0.763 | 0.0754 0.393 0.686 |
| Gas Terminals Oil Terminals | 1970-92 1970-92 | kt/installation kt/installation | 3.0 0.076 | 0.425 0.315 |

        - Loading emissions

This sector includes emissions of CH4 and NMVOCs from tanker loading and unloading based on data from the UKOOA (2005) dataset.  Data from 1995-2003 are based on detailed operator returns, whilst 1990-1994 data are calculated from extrapolation of total emissions data and the use of 1995 data splits between sources.

These data are derived from operator information on the tonnage of oil shipped and site-specific emission factors, taking account for the application of abatement measures. A correction is made regarding emissions from the Seal Sands Refinery to take account of discrepancies across the time series between data provided by the operator and the Pollution Inventory (Environment Agency, 2004).

Emissions data are reported using oil shipment data taken from DTI (2004), covering the amount of crude oil shipped by tanker from:

- production sites to UK users and export

- onshore terminals to UK users and export

It is assumed that no emission occurs from the amounts of crude oil transported by pipeline.  Table A3.3.31 shows aggregate factors calculated from the amounts of oil loaded.  Oil shipment data are reported in the CRF tables.

Table A3.3.31	Aggregate Emission Factors for Crude Oil Loading and Unloading

|  | Onshore CH4 | Onshore NMVOC | Offshore CH4 | Offshore NMVOC | Units |
| --- | --- | --- | --- | --- | --- |
| 2003 | 0.007 | 0.618 | 0.076 | 1.430 | t/kt oil |
| 2002 | 0.007 | 0.700 | 0.115 | 1.643 | t/kt oil |
| 2001 | 0.012 | 0.706 | 0.113 | 1.542 | t/kt oil |
| 2000 | 0.012 | 0.747 | 0.110 | 1.695 | t/kt oil |
| 1999 | 0.006 | 0.691 | 0.071 | 1.382 | t/kt oil |
| 1998 | 0.013 | 0.787 | 0.043 | 1.440 | t/kt oil |
| 1997 | 0.004 | 0.712 | 0.036 | 1.978 | t/kt oil |
| 1996 | 0.002 | 0.723 | 0.035 | 1.956 | t/kt oil |
| 1995 | 0.018 | 0.716 | 0.036 | 2.000 | t/kt oil |
| 1994 | 0.021 | 0.881 | 0.033 | 2.143 | t/kt oil |
| 1993 | 0.022 | 0.908 | 0.036 | 2.240 | t/kt oil |
| 1992 | 0.026 | 0.941 | 0.042 | 2.350 | t/kt oil |
| 1991 | 0.028 | 0.995 | 0.045 | 2.476 | t/kt oil |
| 1990 | 0.029 | 0.973 | 0.047 | 2.369 | t/kt oil |

        - Leakage from the gas transmission system

The NAEI category Gas Leakage covers emissions of CH4 and NMVOC from the UK gas transmission and distribution system.  This is mapped on to the IPCC category 1B2b Natural Gas ii Transmission/Distribution.  The estimates are derived from specific leakage rates measured on the various types of gas mains and services together with data on the total lengths and types of the gas mains and services (British Gas, 1993; Williams, 1993).  The emissions for 1997 to 2003 are based on the results of a model (Transco 2000-2004) that reflects recent conditions in the gas industry. The emissions from 1990-96 are based on the British Gas model which provided historical data for 1991-94 but projected estimates for 1995-96.  The methane content of natural gas is shown in Table A3.3.32.  Following discussions with Transco there have been significant revisions in the composition data used, resulting from clarification of the units used i.e. weight % as opposed to volume %.  An extra emission of 40 kt methane is been included to account for high-pressure mains, which is not included in the British Gas and Transco models (DETR, 1999).

Table A3.3.32	Methane and NMVOC Composition of Natural Gas

| Period | CH4 weight % | NMVOC weight % |
| --- | --- | --- |
| 1990-961 | 84.3 | 8.9 |
| 1997-992 | 77.1 | 14.73 |
| 20002 | 77.6 | 14.7 |
| 20012 | 77.1 | 14.2 |
| 20022 | 77.3 | 15.5 |
| 20032 | 77.4 | 15.4 |

- British Gas (1994)

- Transco (2004)

        - Petrol distribution

The NAEI reports emissions from the storage, distribution and sale of petrol in the following categories each of which is further divided into emissions of leaded and unleaded petrol:

- Refineries (Road/Rail Loading).  Emissions during loading of petrol on to road and rail tankers at refineries

- Petrol Terminals (Storage).  Emissions from storage tanks at petrol distribution terminals.

- Petrol Terminals (Tanker Loading).  Emissions during loading of petrol on to road and rail tankers at petrol terminals.

- Petrol Stations (Petrol Delivery).  Emissions during loading of petrol from road tankers into storage tanks at petrol stations.

- Petrol Stations (Storage Tanks).  Emissions from storage tanks at petrol stations.

- Petrol Stations (Vehicle Refuelling).  Emissions due to displacement of vapour during the refuelling of motor vehicle at petrol stations.

- Petrol Stations (Spillages).  Emissions due to spillages during refuelling of motor vehicles at petrol stations.

Emissions would also occur from storage tanks at refineries.  This source is included together with emissions from the storage of crude oil and other volatile materials in the NAEI source category, refineries (tankage).

The estimate for road and rail tanker loading at refineries are supplied by UKPIA (2004).  The remaining estimates are based on methodologies published by the Institute of Petroleum (2000) or, in the case of petrol terminal storage, based on methods given by CONCAWE (1986).  The calculations require information on petrol density, given in DTI (2004), and petrol Reid Vapour Pressure (RVP), data for which have been obtained from a series of surveys carried out by the Associated Octel company between 1970 and 1994.  More recent, detailed, RVP data are not available however UKPIA have suggested values used for 1999 onwards.  Central England Temperature (CET) data (Met Office, 2004) are used for ambient UK temperatures. The methodology also requires assumptions to be made on the level of vapour recovery in place at terminals and petrol stations.  These assumptions have been made taking into account surveys carried out by Petroleum Review (2000, 2002, 2003, 2004) which included questions on petrol station controls, and the timescales recommended in Secretary of State’s Guidance for petrol terminals (PG 1/13 (97)). The activity data are the sales of leaded and unleaded petrol taken from DTI (2004).

        - Refineries and petroleum processes

The IPCC category 1B2aiv Refining and Storage reports estimates of NMVOC emissions from oil refineries.  In the NAEI these are split into:

- Refineries (drainage)

- Refineries (tankage)

- Refineries (Process)

All are based on UKPIA (2004) estimates for 1994-2003.  The UKPIA data refer to the following installations:

- Texaco, Milford Haven

- Elf, Milford Haven

- BP, Coryton

- Shell, Shell Haven (closed during 1999)

- Conoco, South Killingholme

- Lindsey, Killingholme

- Shell, Stanlow

- PIP, North Tees

- Esso, Fawley

- BP, Grangemouth

- Gulf, Milford Haven (closed during 1997)

UKPIA also supply estimates for loading of petrol into road and rail tankers at refineries – see Section A3.3.8.2.7

Prior to 1994, process emissions are estimated by extrapolation from the 1994 figure on the basis of refinery throughput, whereas emissions from tankage, flares and drainage systems are assumed to be constant.

Also included under 1B2aiv Refining and Storage are NMVOC emissions from the NAEI category petroleum processes.  This reports NMVOC emissions from specialist refineries (Llandarcy, Eastham, Dundee, & Harwich), onshore oil production facilities, and miscellaneous petroleum processes not covered elsewhere in the inventory (most significant of which are the Tetney Lock and Tranmere oil terminals).  Emissions are taken from the Pollution Inventory,  (Environment Agency, 2004).  No emissions data have been found for the Dundee refinery.

        - Gasification processes

The NAEI also reports NMVOC emissions from on shore gas production facilities, refining and odourisation of natural gas, natural gas storage facilities, and processes involving reforming of natural gas and other feedstocks to produce carbon monoxide and hydrogen gases.  Emissions are taken from the Pollution Inventory (Environment Agency, 2004).  For the years prior to 1994, they are extrapolated based on gas throughput.  Care is taken to avoid double counting with the offshore emissions.

    - Stored carbon

- The UK Inventory does not use the IPCC default methodology for stored carbon in products because it is not clear what processes it represents or if it is applicable to the UK.  The procedure adopted, is to report emissions from the combustion of fuels only with emissions from the non-energy use of fuels assumed to be zero (i.e. the carbon is assumed to be sequestrated as products).  The only exceptions to this approach are those instances where an emission can be identified.  This is true in the following cases, for which emissions are included in the inventory:

- Catalytic crackers – regeneration of catalysts

- Ammonia production

- Aluminium production – consumption of anodes

- Benzoles and tars – produced in coke ovens and emissions assigned to the waste sector

- Combustion of waste lubricants and waste solvents

- Incineration of fossil carbon in products disposed of as waste.

The approach for benzoles and tars needs to be verified.  Previously, benzoles and tars were ignored as a separate output from coke ovens on the basis that they were assumed to be used on site with their carbon content assumed to be emitted from the process.  The revised methodology for fuel transformation processes recognises that tars and benzoles are recovered and processed into a number of products including creosote.  The current method therefore assumes that no emission of carbon occurs from coke ovens but that the carbon is ultimately released when the products made from the benzoles and tars are disposed of e.g. when creosoted wood is burnt, incinerated or landfilled.  More information is required on the uses of benzoles and tars and their ultimate fate so that the methodology can be refined.

Netcen estimates of the quantities of lubricants burnt are based on data from Recycling Advisory Unit, 1999; Oakdene Hollins Ltd, 2001 & BLF/UKPIA/CORA, 1994.  Separate estimates are produced for the following sources:

- Power stations

- Cement kilns

- Other industry.

In addition, an estimate is made of lubricants burnt in vehicle engines.  Carbon emissions from these sources are calculated using a carbon factor derived from analysis of eight samples of waste oil (Passant, 2004).

Fossil carbon destroyed in MSW incinerators and clinical waste incinerators is included in the GHGI, as is carbon emitted by chemical waste incinerators.  However, no estimates are included for process wastes flared or used as fuels in boilers etc at chemical manufacturing facilities.

  - INdustrial Processes (crf sector 2)

    - Mineral Processes (2A)

      - Cement Production (2A1)

Emission factors for the production of cement, as described in Chapter 4 are as follows:

Table A3.4.1a	Emission Factors for Cement Kilns based on Fuel Consumption, 2003

| Fuel | C a | CH4 | N2O | Units |
| --- | --- | --- | --- | --- |
| Coal | 749.7b | 0.3e | 0.108h | Kt / Mt fuel |
| Fuel Oil | 879 b | 0.0406f | NE | Kt / Mt fuel |
| Natural Gas | 1.48b | 0.0001f | NE | Kt / Mtherm |
| Petro-Coke | 829.6b | 0.442i | 0.158j | Kt / Mt fuel |
| Scrap Tyres | 757 c | 0.026 e | NE | Kt / Mt fuel |
| Waste Oils | 864.8d | 0.0406g | NE | Kt / Mt fuel |

a	Emission factor as mass carbon per unit fuel consumed

b	Baggott et al (2004)

c	Emission factor derived from emissions reported in the PI

d	Passant, N.R., 2004

e	Brain, SA et al. British Coal Corp, CRE (1994)

f	EMEP/CORINAIR (1996)

g	Jockel & Hartje (1991)

h	Fynes et al (1994)

i	DTI (1994)

j	Fynes et al(1994)

Table A3.4.1b	Emission Factors for Cement Kilns based on Clinker Production, 1990-2003

| Year | CO | NOx | NMVOC | SO2 | Units |
| --- | --- | --- | --- | --- | --- |
| 1990-94 | 2.96 | 5.65 | 0.135 | 3.05 | kt/Mt Clinker |
| 1995 | 2.86 | 5.13 | 0.135 | 3.33 | kt/Mt Clinker |
| 1996 | 4.39 | 3.43 | 0.135 | 2.11 | kt/Mt Clinker |
| 1997 | 1.91 | 3.72 | 0.135 | 2.37 | kt/Mt Clinker |
| 1998 | 1.45 | 3.86 | 0.135 | 2.26 | kt/Mt Clinker |
| 1999 | 2.58 | 3.61 | 0.121 | 2.26 | kt/Mt Clinker |
| 2000 | 2.49 | 3.42 | 0.124 | 1.88 | kt/Mt Clinker |
| 2001 | 2.31 | 3.06 | 0.156 | 1.93 | kt/Mt Clinker |
| 2002 | 2.40 | 2.89 | 0.111 | 2.07 | kt/Mt Clinker |
| 2003 | NR | NR | NR | NR | kt/Mt Clinker |

NR – 2003 emission factor data are not reported due to issues of commercial confidentiality raised by the BCA.

      - Lime Production (2A2)

Emission factors for the production of lime, as discussed in Chapter 4, Section 4.3:

Table A3.4.1c	Emission Factors for Lime Kilns based on Fuel Consumption, 2003

| Fuel | C a | CH4 | N2O | Units |
| --- | --- | --- | --- | --- |
| Coal | 671.5b | 0.011c | 0.213e | Kt / Mt fuel |
| Natural Gas | 1.48b | 0.00038d | 0.0000095f | Kt / Mtherm |
| Coke | 746.2 b | 0.011c | 0.221e | Kt / Mt fuel |

a	Emission factor as mass carbon per unit fuel consumed

b	Baggott et al (2004)

c	Brain, SA et al. British Coal Corp, CRE (1994)

d	CORINAIR (1992)

e	Fynes et al (1994)

f	IPCC(1997) IPCC Revised 1996 Guidelines

Table A3.4.1d 2003 Emission Factors for Lime Kilns: Indirect GHGs

| Fuel | CO | NOx | NMVOC | Units |
| --- | --- | --- | --- | --- |
| Coal | 25.05 | 48.3 | 0.05 | Kt / Mt fuel |
| Natural Gas | 0.095 | 0.017 | 0.00023 | Kt / Mtherm |
| Coke | 12.58 | 0.088 | 0.05 | Kt / Mt fuel |

    - Chemical Industry (2B)

      - Nitric Acid Production (2B2)

Table A3.4.2	Summary of Nitric Acid Production in the UK, 1990-2003

|  | 1990 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No of Sites | 8 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 5 | 4 | 4 |
| Production (Mt 100% Nitric Acid) | 2.41 | 2.43 | 2.40 | 2.44 | 2.35 | 2.61 | 2.44 | 2.03 | 1.65 | 1.57 | 1.63 |
| Aggregate EF (kt N2O / Mt Acid) | 5.54 | 5.71 | 4.14 | 4.19 | 4.12 | 4.27 | 6.56 | 7.33 | 7.04 | 4.97 | 5.20 |
| Aggregate EF (kt NOX / Mt Acid) | 3.0 | 1.93 | 0.81 | 0.74 | 0.90 | 0.73 | 0.91 | 0.99 | 0.66 | 0.41 | 0.45 |

      - Adipic Acid Production (2B3)

There is only one company manufacturing adipic acid in the UK.  Production data are not provided in the NIR because it may be commercially confidential.

UK production figures and emission estimates have been provided by the process operator (Invista, 2004).  The emission estimates are based on extrapolation of on-line measurement of the concentration of nitrous oxide in the flue gases, conducted for regulatory compliance monitoring purposes since 1998. Prior to 1998, the operators have provided estimates of emissions using appropriate emission factor data.

A small nitric acid plant is associated with the adipic acid plant that also emits nitrous oxide.  From 1994 onwards this emission is reported as nitric acid production but prior to 1994 it is included under adipic acid production.  This will cause a variation in reported effective emission factor for these years. This allocation reflects the availability of data.  In 1998 an N2O abatement system was fitted to the plant.  The abatement system is a thermal oxidation unit and is reported by the operators to be 99.99% efficient at N2O destruction. In 2003 it was operational 95.4% of the time (when compared to plant operation), and in previous years (1998-2002) an estimate of 89% abatement plant availability has been provided.

The level of uncertainty associated with reported emissions of N2O is not currently known.  However these data are considered to be reliable since they are subject to internal QA/QC checks within the company producing the adipic acid, and QA/QC checks by the Environment Agency before being reported in the Pollution Inventory.

Fluctuations in the N2O EF from this plant are apparent since the installation of the abatement plant. The reason for these fluctuations are unknown, and a study has been commissioned to investigate this matter during 2005.

    - Metal Production (2C)

      - Iron and Steel (2C1)

The following emissions are reported under 2C1 Iron and Steel Production.

- Blast furnaces: process emissions of CO, NOX, and SO2

- Flaring of blast furnace gas/basic oxygen furnace gas

- Electric arc furnace emissions

- Basic oxygen furnaces: process emissions of CO and NOX.

- Rolling mill process emissions of VOC

- Slag processing: process emissions of SO2

Emissions arising from the combustion of blast furnace gas and other fuels used for heating the blast furnace are reported under 1A2a Iron and Steel.  Emissions of CO, NOX, and SO2 from integrated steelworks, and the flaring of blast furnace gas and basic oxygen furnace gas are reported under 2C1 Iron and Steel Production.  Carbon dioxide emissions from limestone and dolomite use in iron and steel production are reported under 2A3 Limestone and Dolomite use.

        - Carbon Dioxide Emissions

Carbon emissions from flaring of blast furnace gas (BFG) and basic oxygen furnace gas (BOFG) are calculated using an emission factor of 70,180 g C/GJ, a figure which is calculated as part of the carbon balance used to estimate emissions from CRF category 1A2a.  Emissions from electric arc furnaces are 3.6 kt C/Mt steel (BISPA, 1997).

        - Other Pollutants

Emissions from blast furnaces of other pollutants are partly based on the methodology described in IPCC (1997) for blast furnace charging and pig iron tapping.  However, a revision has been made to the SO2 factors based on data supplied by Corus (2001) who use EIPPCB (2000) defaults. The emission factors are expressed in terms of the emission per Mt of pig iron produced and are given in Table A3.4.3a.  Data on iron production are reported in ISSB (2003).

Table A3.4.3a	Emission Factors for Blast Furnaces (BF), Electric Arc Furnaces (EAF) and Basic Oxygen Furnaces (BOF), 2003

|  | C a | CH4 | N2O | NOx | SO2 | NMVOC | CO | Units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Blast furnaces | IE | NE | NE | NE | 0.90b | 0.12c | 1.44c | kt/Mt pig iron |
| Electric arc furnaces | 3.6d | 0.01e | 0.005e | 0.02e | 0.24b | 0.09e | 0.71b | kt/Mt Steel |
| Basic oxygen furnaces | IE | NE | NE | 0.012f | IE | NE | 7.15b | kt/Mt Steel |
| Losses of BFG/BOFG | 7.4g | NE | NE | NE | NE | NE | NE | kt/Mtherm gas |
| Slag processing | NE | NE | NE | NE | 0.0037b | NE | NE | kt/Mt Pig iron |

a	Emission factor as kt carbon/unit activity

b	Emission factor for 2003 based on Environment Agency (2004)

c	IPCC (1997)

d	BISPA (1997)

e	EMEP/CORINAIR(1999)

f	EIPPCB(2000), Corus (2001, 2000)

g	Netcen estimate based on carbon balance

NE	Not estimated

IE	Emission included elsewhere.

Emissions from electric arc furnaces are calculated mainly using default emission factors taken from EMEP/CORINAIR (1999).  The CO2 emission arises from the consumption of a graphite anode and is based on manufacturer’s data.

Emissions from basic oxygen furnaces have been revised based on more detailed data supplied by Corus (2004).  The NOx emission is based on an EIPPCB default and the CO emission is based on plant estimates provided by Corus (2004).

Emissions of NMVOC are estimated from the hot rolling and cold rolling of steel using emission factors 1 g/tonne product and 25 g/tonne product respectively (EMEP/CORINAIR, 1996).  Activity data were taken from ISSB (2004).

There is insufficient activity or emission factor data to make an estimate for emissions from ferroalloys.  Emissions of CO2 will be included in 1A2a, since the fuels used as reducing agents are included in the energy statistics.

      - Aluminium production (2C3)

Details of the method used to estimate emissions of F-gases from this source are given in AEAT (2004). Emission factors for aluminium production, as discussed in Chapter 4, Section 4.16, are shown in Table A3.4.3b.

Table A3.4.3b	Emission Factors for Aluminium Production, 2003

|  | C a | SO2 b | NOx b | CO b | Units |
| --- | --- | --- | --- | --- | --- |
| Prebake | 420 | 13.12 | 0.81 | 80.24 | Kt / Mt Al |
| Anode Baking | IE | 1.18 | 0.40 | 2.86 | Kt / Mt anode |

a		Emission factor as kt carbon per unit activity, Walker, 1997.

b		Environment Agency Pollution Inventory (2004), SEPA (2004) & Alcan International (2004)

IE	Emission included elsewhere.

      - SF6 used in Aluminium and Magnesium Foundries (2C4)

The method used to estimate emissions of SF6 from this source is described in AEAT (2004).

      - Food and Drink (2D2)

NMVOC emission factors for food and drink, as discussed in Chapter 4, Section 4.20.

Table A3.4.3c	NMVOC Emission Factors for Food and Drink Processing, 2003

| Food/Drink | Process | Emission Factor | Units |
| --- | --- | --- | --- |
| Beer | Barley Malting Wort Boiling Fermentation | 0.6c 0.0048c 0.02c | g/L beer |
| Cider | Fermentation | 0.02c | g/L cider |
| Wine | Fermentation | 0.2c | kg/m3 |
| Spirits | Fermentation Distillation Casking Spent grain drying Barley Malting Maturation | 1.58d 0.79g 0.40h 1.31i 4.8c 15.78d | g/ L alcohol g/ L alcohol g/ L whiskey kg/ t grain kg/ t grain g/ L alcohol |
| Bread Baking |  | 1a | kg/tonne |
| Meat, Fish & Poultry |  | 0.3f | kg/tonne |
| Sugar |  | 0.020b | kg/tonne |
| Margarine and solid cooking fat |  | 10f | kg/tonne |
| Cakes, biscuits, breakfast cereal, animal feed |  | 1f | kg/tonne |
| Malt production (exports) |  | 4.8c | kg/ t grain |
| Coffee Roasting |  | 0.55f | kg/tonne |

a	Federation of Bakers (2000)

b	Environment Agency Pollution Inventory (2004)

c	Gibson et al (1995)

d	Passant et al (1993)

e	Assumes 0.1% loss of alcohol based on advice from distiller

f	EMEP/CORINAIR, 2003

g    Unpublished figure provided by industry

h	Based on loss rate allowed by HMCE during casking operations

i	US EPA, 2004

    - Production of Halocarbons and SF6 (2E)

Details of the method used to estimate emissions of F-gases from this source are given in AEAT (2004).

    - Consumption of Halocarbons and SF6 (2F)

      - Refrigeration and Air Conditioning Equipment (2F1)

Details of the method used to estimate emissions of F-gases from this source are given in AEAT (2004).

      - Foam Blowing (2F2)

Details of the method used to estimate emissions of F-gases from this source are given in AEAT (2004).

      - Fire Extinguishers (2F3)

Details of the method used to estimate emissions of F-gases from this source are given in AEAT (2004).

      - Aerosols/ Metered Dose Inhalers (2F4)

Details of the method used to estimate emissions of F-gases from this source are given in AEAT (2004).

      - Solvents (2F5)

Details of the method used to estimate emissions of F-gases from this source are given in AEAT (2004).

      - Semiconductor Manufacture (2F6)

Details of the method used to estimate emissions of F-gases from this source are given in AEAT (2004).

      - Electrical Equipment (2F7)

Details of the method used to estimate emissions of F-gases from this source are given in AEAT (2004).

      - One Component Foams (2F8A)

Details of the method used to estimate emissions of F-gases from this source are given in AEAT (2004).

      - Semiconductors, Electrical and Production of Trainers (2F8B)

Details of the method used to estimate emissions of F-gases from this source are given in AEAT (2004).

  - solvent and other product use (crf sector 3)

There is currently no additional information for this sector in this Annex.

  - agriculture (crf sector 4)

    - Enteric Fermentation (4A)

Methane is produced in herbivores as a by-product of enteric fermentation, a digestive process by which carbohydrates are broken down by microorganisms.  Emissions are calculated from animal population data (Table A3.6.1) collected in the June Agricultural Census and published in Defra (2004a) and the appropriate emission factors.  Data for earlier years are often revised so information was taken from the Defra agricultural statistics database. Table A3.6.2 shows the emission factors used.  Apart from cattle, lambs and deer, the methane emission factors are IPCC Tier I defaults (IPCC, 1997) and do not change from year to year.  The dairy cattle emission factors are estimated following the IPCC Tier 2 procedure (IPCC, 1997) and vary from year to year.  For dairy cattle, the calculations are based on the population of the ‘dairy breeding herd’ rather than ‘dairy cattle in milk’.  The former definition includes ‘cows in calf but not in milk’.  The emission factors for beef and other cattle were also calculated using the IPCC Tier 2 procedure (Table A3.6.4), but do not vary from year to year.  The enteric emission factors for beef cattle were almost identical to the IPCC Tier I default so the default was used in the estimates.  The base data and emission factors for 1990-2001 are given in Table A3.6.3 and Table A3.6.4.  The emission factor for lambs is assumed to be 40% of that for adult sheep (Sneath et al. 1997).  In using the animal population data, it is assumed that the reported numbers of animals are alive for that whole year.  The exception is the treatment of sheep where it is normal practice to slaughter lambs and other non-breeding sheep after 6 to 9 months.  Hence it is assumed that breeding sheep are alive the whole year but that lambs and other non-breeding sheep are only alive 6 months of a given year (based on Smith and Frost 2000).  The sheep emission factors in Table A3.6.2 are reported on the basis that the animals are alive the whole year.

Table A3.6.1	Livestock Population Data for 2003 by Animal Type

| Animal Type | Number |
| --- | --- |
| Cattle: |  |
| Dairy Breeding Herd | 2,192,258 |
| Dairy Heifers | 441,099 |
| Beef Herd | 1,700,065 |
| Other cattle >2 years | 658,332 |
| Other cattle 1-2 years | 2,458,862 |
| Other cattle < 1 year | 2,828,278 |
| Pigs: |  |
| All breeding pigs | 597,395 |
| Other pigs > 50 kg | 1,821,459 |
| Other pigs 20-50 kg | 1,231,615 |
| Pigs <20 kg | 1,396,366 |
| Sheep: |  |
| Breeding sheep | 17,599,464 |
| Other sheep | 910,944 |
| Lambs < 1 year | 17,335,321 |
| Goats | 88,453 |
| Horses | 299,886 |
| Deer: |  |
| Stags | 4,717 |
| Hinds | 16,982 |
| Calves | 9,749 |
| Poultry (000 head): |  |
| Broilers | 116,792 |
| Breeders | 10,990 |
| Layers | 29,274 |
| Growing Pullets | 8,286 |
| Ducks, geese and guinea fowl | 2,550 |
| Turkeys | 7,522 |

Table A3.6.2	Methane Emission Factors for Livestock Emissions

|  | Enteric methanea kg CH4/head/year | Methane from manuresa kg CH4/head/year |
| --- | --- | --- |
| Dairy Breeding Herd | 103.8b | 25.49b |
| Beef Herd | 48 | 2.74 |
| Other Cattle >1 year, Dairy Heifers | 48 | 6 |
| Other Cattle <1 year | 32.8 | 2.96 |
| Pigs | 1.5 | 3 |
| Breeding Sheep | 8 | 0.19 |
| Other Sheep | 8e | 0.19e |
| Lambs < 1 year | 3.2ce | 0.076ce |
| Goats | 5 | 0.12 |
| Horses | 18 | 1.4 |
| Deer: Stags & Hinds | 10.4c | 0.26c |
| Deer: Calves | 5.2c | 0.13c |
| Poultryd | NE | 0.078 |

a	IPCC (1997)

b	Emission factor for year 2003

c	Sneath et al. (1997)

d	Chickens, turkeys, geese, ducks and guinea fowl

e	Factor quoted assumes animal lives for a year; emission calculation assumes animal lives for 6 months

Table A3.6.3	Dairy Cattle Methane Emission Factorsa

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average Weight of cow (kg) b | 550 | 556 | 561 | 567 | 572 | 578 | 584 | 590 | 596 | 602 | 608 | 614 | 614 | 614 |
| Average Rate of Milk Production (litre/d) | 14.3 | 14.2 | 14.5 | 14.7 | 14.7 | 15.0 | 15.1 | 15.9 | 16.1 | 16.4 | 16.6 | 16.7 | 17.9 | 18.3 |
| Average Fat Content (%) | 4.01 | 4.04 | 4.06 | 4.07 | 4.05 | 4.05 | 4.08 | 4.07 | 4.07 | 4.03 | 4.03 | 4.01 | 3.97 | 3.96 |
| Enteric Emission Factor (kg CH4/head/y) | 88.1 | 88.4 | 90.1 | 90.8 | 90.9 | 92.3 | 93.2 | 96.0 | 96.8 | 97.8 | 98.7 | 99.4 | 102.7 | 103.8 |
| Manure Emission Factor (kg CH4/head/y) | 21.6 | 21.7 | 22.1 | 22.3 | 22.3 | 22.7 | 22.8 | 23.6 | 23.8 | 24.0 | 24.2 | 24.4 | 25.2 | 25.5 |

a	In 2003, 46% of animals graze on good quality pasture, rest confined

Gestation period 281 days

Digestible energy 74% (Bruce Cottrill, ADAS, pers. comm.)

Methane conversion rate 6%

Ash content of manure 8%

Methane producing capacity of manure 0.24 m3/kg VS

b	Weight assuming annual growth of 1% from 1990 to 2001

Table A3.6.4	Beef and Other Cattle Methane Emission Factorsa

|  | Beef Cattle | Other Cattle |
| --- | --- | --- |
| Average Weight of Animal (kg) | 500 | 180 |
| Time Spent Grazing | 50% | 46% |
| GE (MJ/d) | 123.3 | 83.4 |
| Enteric Emission Factor (kg CH4/head/y) | 48.5b | 32.8 |
| Manure Emission Factor (kg CH4/head/y) | 2.74 | 2.96 |

a	Digestible Energy 65%, 	Ash content of manure 8%

Methane producing capacity of manure 0.24 m3/kg VS

b	IPCC (1997) default (48 kg/head/y) used since calculated factor is very close to default and the difference under the Tier II method will not affect the accuracy of the emission factor at the required level of precision

    - Manure Management (4B)

      - Methane emissions from animal manures

Methane is produced from the decomposition of manure under anaerobic conditions.  When manure is stored or treated as a liquid in a lagoon, pond or tank it tends to decompose anaerobically and produce a significant quantity of methane.  When manure is handled as a solid or when it is deposited on pastures, it tends to decompose aerobically and little or no methane is produced.  Hence the system of manure management used affects emission rates.  Emissions of methane from animal manures are calculated from animal population data (Defra, 2004a) in the same way as the enteric emissions.  The emission factors are listed in Table A3.6.2.  Apart from cattle, lambs and deer, these are all IPCC Tier I defaults (IPCC, 1997) and do not change from year to year.  The emission factors for lambs are assumed to be 40% of that for adult sheep.  Emission factors for dairy cattle were calculated from the IPCC Tier 2 procedure using data shown in Table A3.6.3 and Table A3.6.5 (Defra, 2002).  There was a revision (in 2002) of the allocation of manure to the different management systems based on new data. This is detailed in Section 6.3.2.2.  For dairy cattle, the calculations are based on the population of the ‘dairy breeding herd’ rather than ‘dairy cattle in milk’ used in earlier inventories.  The former definition includes ‘cows in calf but not in milk’.  The waste factors used for beef and other cattle are now calculated from the IPCC Tier 2 procedure but do not vary from year to year.  Emission factors and base data for beef and other cattle are given in Table A3.6.4.

Table A3.6.5	Cattle Manure Management Systems in the UK

| Manure Handling System | Methane Conversion Factor %a | Fraction of manure handled using manure system % | Fraction of manure handled using manure system % |
| --- | --- | --- | --- |
|  |  | Dairy | Beef and Other |
| Pasture Range | 1 | 45.5 | 50.5 |
| Liquid System | 39 | 30.6 | 6 |
| Solid Storage | 1 | 9.8 | 20.7 |
| Daily Spread | 0.1 | 14.1 | 23 |

a	IPCC (2000)

      - Nitrous Oxide emissions from Animal Waste Management Systems

Animals are assumed not to give rise to nitrous oxide emissions directly, but emissions from their manures during storage are calculated for a number of animal waste management systems (AWMS) defined by IPCC.  Emissions from the following AWMS are reported under the Manure Management IPCC category:

- Flushing anaerobic lagoons. These are assumed not to be in use in the UK.

- Liquid systems

- Solid storage and dry lot (including farm-yard manure)

- Other systems (including poultry litter, stables)

According to IPCC (1997) guidelines, the following AWMS are reported in the Agricultural Soils category:

- All applied animal manures and slurries

- Pasture range and paddock

Emissions from the combustion of poultry litter for electricity generation are reported under power stations.

The IPCC (1997) method for calculating emissions of N2O from animal waste management systems can be expressed as:

N2O(AWMS)	=	44/28 .  N(T) . Nex(T) . AWMS(T) . EF(AWMS)

where

N2O(AWMS)	=	N2O emissions from animal waste management systems (kg N2O/yr)

N(T)	=	Number of animals of type T

Nex(T)	=	N excretion of animals of type T (kg N/animal/yr)

AWMS(W)	=	Fraction of Nex that is managed in one of the different 		waste management systems of type W

EF(AWMS)	=	N2O emission factor for an AWMS (kg N2O-N/kg of Nex in AWMS)

The summation takes place over all animal types and the AWMS of interest.  Animal population data are taken from Agricultural Statistics (Defra, 2004a).  Table A3.6.6 shows emission factors for nitrogen excretion per head for domestic livestock in the UK (Nex).  These are based on N balance figures of Smith and Frost (2000) and Smith et al. (2000), using linear regression to interpolate between animal weights reported in their paper.

The UK methodology assumes that 20% of the total N emitted by livestock volatilises as NOx and NH3 and therefore does not contribute to N2O emissions from AWMS.  This is because in the absence of a more detailed split of NH3 losses at the different stages of the manure handling process it has been assumed that NH3 loss occurs prior to major N2O losses.  Thus, the Nex factors used in the AWMS estimates (and those reported in Table A3.6.6 and Table A3.6.7) exclude the fraction of N volatilising and are 20% less than if they were reported on the same basis as the ‘total’ Nex factors reported in the IPCC Guidelines.  Values of total N excreted shown in the Common Reporting Format are not corrected in this way and are estimates of total N excreted from livestock.  Nex factors for dairy cattle take account of the assumed growth in the average cow weight by 1% per annum (1990–2001) and are shown in Table A3.6.7.

The conversion of excreted N into N2O emissions is determined by the type of manure management system used.  The distribution of waste management systems for each animal type (AWMS(T)) is given in Table A3.6.8.  The distributions used were revised for cattle and poultry in the 2000 Inventory.  The change related to the way that data on ‘no significant storage capacity’ of farmyard manure (FYM) were allocated.  This could have a large effect on emissions because it amounted to around 50% of manure and the ‘Daily spread (DS)’ category has an emission factor of zero, compared to 0.02 for the ‘Solid storage and dry lot (SSD)’ category.  However, we are advised (Smith, 2002) that:

In terms of slurry, it seems likely that where a proportion of the estimated slurry production is attributed with “nil” or little storage (<1 month capacity), as above, it can be assumed that such units will rely on a significant amount of daily – weekly spreading activity, according to land availability and trafficability, throughout.  With FYM and poultry manure, however, significant storage capacity exists within the house and so, “no storage” generally implies that manure is cleared from the house/straw littered yard and spread direct on land.  Storage capacity within the house or yard might comprise between 7 weeks – 12 months (poultry) or several months (cattle) and is unlikely to require “daily” spreading activity.

Therefore, assigning this ‘stored in house’ manure to ‘daily spread’ is acceptable only if emissions from the housing phase are thought to be very small.  Calculations were performed with the N2O Inventory of Farmed Livestock to compare housing and storage phases (Sneath et al. 1997).  For pigs and poultry, the emission factor for housing is the same as or greater than that of storage.  It would therefore lead to significant underestimation to use the daily spread emission factor.  All of the FYM in this case has therefore been re-allocated to SSD.

For dairy and non-dairy cattle, the emission factor for the housing phase is around 10% of the storage phase, so the non-stored FYM has been split between SSD and DS to account for this.

Table A3.6.9 gives the N2O emission factor for each animal waste management system (EF(AWMS)).  These are expressed as the emission of N2O-N per mass of excreted N processed by the waste management system.

Emissions from grazing animals (pasture range and paddock) and daily spread are calculated in the same way as the other AWMS.  However, emissions from land spreading of manure that has previously been stored in a) liquid systems, b) solid storage and dry lot and c) other systems, are treated differently.  These are discussed in Section A3.6.3.

Table A3.6.6	   Nitrogen Excretion Factors for Animals in the UKa

| Animal Type | Emission Factor kg N/animal/yearb |
| --- | --- |
| Dairy Cows | 90.0c |
| Other Cattle > 2 year | 60 |
| Other Cattle 1-2 year | 47 |
| Other Cattle <1 year | 11.8 |
| Pigs < 20kg | 3.0 |
| Other Pigs 20-50 kg | 7.1 |
| Fattening & Other Pigs > 50 kg | 10.7 |
| Breeding Pigs > 50 kg | 14.3 |
| Breeding Sheep | 9.2 |
| Other Sheep <1 year | 9.2e |
| Lambs | 3.36e |
| Goats | 7.2 |
| Broilers | 0.495 |
| Broiler Breeders | 0.899 |
| Layers | 0.589 |
| Ducks, | 0.984 |
| Turkeys | 1.052 |
| Growing Pullets | 0.106 |
| Horses | 32 |
| Deer: Stagsd | 17.5 |
| Deer: Hindsd | 11.7 |
| Deer: Calvesd | 8.64 |

a	Smith and Frost (2000), Smith et al. (2000)

b	Nex  factors exclude 20% N volatilising as NOx and NH3

c	Weighted average of dairy breeding herd (based on liveweight) and dairy heifers, 2003

d	Sneath et al. (1997)

e	Factor quoted assumes animal lives for a year.  Emission calculation assumes animal lives for 6 months (Smith and Frost, 2000)

Table A3.6.7	Nitrogen Excretion Factors for Dairy Cattlea

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Emission Factor kg N/animal/year | 80.9 | 81.6 | 82.2 | 82.9 | 83.9 | 84.5 | 84.9 | 85.7 | 86.6 | 87.6 | 88.5 | 89.4 | 89.7 | 90.0 |

a				Nex factors exclude 20% N volatilising as NOx and NH3.   Weighted average of dairy breeding herd and dairy heifers in first calf.

Table A3.6.8	Distribution of Animal Waste Management Systems used for Different Animal typesc

| Animal Type | Liquid System | Daily Spread | Solid Storage and Dry Lota | Pasture Range and Paddock | Otherb | Fuel |
| --- | --- | --- | --- | --- | --- | --- |
| Dairy Cows | 30.6 | 14.1 | 9.8 | 45.5 |  |  |
| Other Cattle >1 year | 6.0 | 23.0 | 20.4 | 50.5 |  |  |
| Other Cattle <1 year |  | 22.9 | 22.3 | 54.8 |  |  |
| Fattening & Other Pigs > 20 kg, | 29.2 | 5.8 | 64.0 | 1.0 |  |  |
| Breeding sows | 35.5 | 7.1 | 28 | 29.3 |  |  |
| Pigs <20 kg | 38.3 | 7.7 | 46.0 | 8.0 |  |  |
| Sheep |  |  | 2.0 | 98.0 |  |  |
| Goats |  |  |  | 96.0 | 4.0 |  |
| Broilers & Table Fowl (2003) |  |  |  | 1.0 | 63.0 | 36.0 |
| Breeders |  |  |  | 1.0 | 99.0 |  |
| Layerse |  |  |  | 10.0 | 90.0 |  |
| Pulletse |  |  |  | 10.0 | 90.0 |  |
| Ducks, Geese & Guinea Fowle |  |  |  | 50.0 | 50.0 |  |
| Turkeyse |  |  |  | 8.0 | 92.0 |  |
| Horses |  |  |  | 96.0 | 4.0 |  |
| Deer: Stagsd |  |  |  | 100 |  |  |
| Deer: Hinds & Calvesd |  |  |  | 75.0 | 25.0 |  |

a	Farmyard manure

b	Poultry litter, Stables

c	ADAS (1995a), Smith (2002)

d	Sneath et al. (1997)

e	Tucker and Canning (1997)

Table A3.6.9	Nitrous Oxide Emission Factors for Animal Waste Handling Systemsa

| Waste Handling System | Emission Factor kg N2O-N per kg N excreted |
| --- | --- |
| Liquid System | 0.001 |
| Daily Spreadb | 0 |
| Solid Storage and Dry Lot | 0.02 |
| Pasture, Range and Paddockb | 0.02 |
| Fuel | - |
| Other | 0.005 |

a	IPCC (1997)

b	Reported under Agricultural Soils

    - Agricultural Soils (4D)

      - Source category description

Direct emissions of nitrous oxide from agricultural soils are estimated using the IPCC recommended methodology (IPCC, 1997) but incorporating some UK specific parameters.  The IPCC method involves estimating contributions from:

- The use of inorganic fertilizer

- Biological fixation of nitrogen by crops

- Ploughing in crop residues

- Cultivation of Histosols (organic soils)

- Spreading animal manures on land

- Manures dropped by animals grazing in the field

In addition to these, the following indirect emission sources are estimated:

- Emission of N2O from atmospheric deposition of agricultural NOx and NH3

- Emission of N2O from leaching of agricultural nitrate and runoff

Descriptions of the methods used are described in Section 6.5.2.

      - Inorganic Fertiliser

Emissions from the application of inorganic fertilizer are calculated using the IPCC (1997) methodology and IPCC default emission factors.  They are given by:

N2O(SN) 	=	44/28 . N(FERT) . (1-Frac(GASF)) . EF1

where

N2O(SN)	=	Emission of N2O from synthetic fertiliser application 		(kg N2O/yr)

N(FERT)	=	Total use of synthetic fertiliser (kg N/yr)

Frac(GASF)	=	Fraction of synthetic fertiliser emitted as NOx  + NH3

=	0.1 kg NH3-N+NOx -N / kg synthetic N applied

EF1	=	Emission Factor for direct soil emissions

=  	0.0125 kg N2O-N/kg N input

annual consumption of synthetic fertilizer is estimated based on crop areas (Defra, 2004a) and fertilizer application rates (BSFP, 2004).

      - Biological Fixation of Nitrogen by crops

Emissions of nitrous oxide from the biological fixation of nitrogen by crops are calculated using the IPCC (2000) Tier 1a methodology and IPCC default emission factors. They are given by:

N2O(BF)	=	44/28 . 2 . Crop(BF) . FracDM . Frac(NCRBF) . EF1

where

N2O(BF)	=	Emission of N2O from biological fixation (kg N2O/yr)

Crop(BF)	=	Production of legumes (kg /yr)

FracDM	=	Dry matter fraction of crop

Frac(NCRBF)	=	Fraction of nitrogen in N fixing crop

=	0.03 kg N/ kg dry mass

EF1	=	Emission Factor for direct soil emissions

=	0.0125 kg N2O-N/kg N input

The factor of 2 converts the edible portion of the crop reported in agricultural statistics to the total biomass.  The fraction of dry mass for the crops considered is given in Table A3.6.10.

Table A3.6.10	Dry Mass Content and Residue Fraction of UK Crops

| Crop Type | Fraction dry massb | Residue/Crop |
| --- | --- | --- |
| Broad Beans, Green Peas | 0.08 | 1.1 |
| Field Beand, Peas(harvest dry) | 0.86 | 1.1 |
| Rye, Mixed corn, Triticale | 0.855a | 1.6 |
| Wheat, Oats | 0.855a | 1.3 |
| Barley | 0.855a | 1.2 |
| Oilseed Rape, Linseed | 0.91a | 1.2 |
| Maize | 0.50 | 1 |
| Hopsc | 0.20 | 1.2 |
| Potatoes | 0.20 | 0.4 |
| Roots, Onions | 0.07 | 1.2 |
| Brassicas | 0.06 | 1.2 |
| Sugar Beet | 0.1 | 0.2 |
| Other | 0.05 | 1.2 |
| Phaseolus beans | 0.08 | 1.2 |

a	Defra (2002)

b	Burton (1982), Nix (1997) or Defra estimates

c	Hops dry mass from Brewers Licensed Retail Association (1998)

d	Field beans dry mass from PGRE (1998)

The data for the ratio residue/crop are default values found under Agricultural Soils or derived from Table 4.17 in Field Burning of Agricultural Residues (IPCC, 1997). Crop production data are taken from Defra (2004a, 2004b). The total nitrous oxide emission reported also includes a contribution from improved grass calculated using a fixation rate of 4 kg N/ha/year (Lord, 1997).

      - Crop Residues

Emissions of nitrous oxide from the ploughing in of crop residues are calculated using a combination of the IPCC (2000) Tier 1b and 1a methodology, for non-N fixing and N-fixing crops, respectively, and IPCC default emission factors. They are given by:

N2O(CR)	=	i (CropO . Resoi/Cropoi . FracDMi . Frac(NCRO)  . (1-FracB) + j (2 . Crop(BFj) . FracDMj . Frac(NCRBFj)) . (1-FracRj) . (1-FracBj) )) . EF1	. 44/28

where

N2O(CR)	=	Emission of N2O from crop residues (kg N2O/yr)

CropOi	=	Production of non-N fixing crop i (kg /yr)

Frac(NCRO)	=	Fraction of nitrogen in non-N fixing crops

=	0.015 kg N/ kg dry mass

FracDMi,j	=	dry matter fraction of crop i, j.

FracR	=	Fraction of crop that is remove from field as crop

FracB	=	Fraction of crop residue that is burnt rather than left on field

EF1	=	Emission Factor for direct soil emissions

=	0.0125 kg N2O-N/kg N input

Crop(BFj)	=	Production of legume crop j (kg /year)

Frac(NCRBF)	=	Fraction of nitrogen in N fixing crop

= 	0.03 kg N/ kg dry mass

Production data of crops are taken from Defra (2004a, 2004b).  The dry mass fraction of crops and residue fraction are given in Table A3.6.10.  Field burning has largely ceased in the UK since 1993.  For years prior to 1993, field-burning data were taken from the annual MAFF Straw Disposal Survey (MAFF, 1995).

      - Histosols

Emissions from Histosols were estimated using the IPCC (2000) default factor of 8 kg N2O-N/ha/yr.  The area of cultivated Histosols is assumed to be equal to that of eutric organic soils in the UK and is based on a FAO soil map figure supplied by SSLRC (now NSRI).

      - Grazing Animals

Emissions from manure deposited by grazing animals are reported under agricultural soils by IPCC.  The method of calculation is the same as that for AWMS (see Section A3.6.2.2), using factors for pasture range and paddock.

      - Organic Fertilizers

Emissions from animal manures and slurries used as organic fertilizers are reported under agricultural soils by IPCC.  The calculation involves estimating the amount of nitrogen applied to the land and applying IPCC emission factors.  For daily spreading of manure, the emission is given by:

N2O(DS)	=	44/28 . T (NT . Nex(T) . AWMS(DS)) . EF1

where

N2O(DS)	= 	N2O emissions from daily spreading of wastes (kg N2O/yr)

NT	= 	Number of animals of type T

Nex(T)	= 	N excretion of animals of type T (kg N/animal/yr), net of N volatilising

as NOx and NH3 (values in Table A3.6.6)

AWMS(DS)	= 	Fraction of Nex that is daily spread

EF1	=	Emission Factor for direct soil emissions

=	0.0125 kg N2O-N/kg N input

For the application of previously stored manures to land, a correction is applied to account for previous N2O losses during storage.

N2O(FAW)	=	44/28 . T (NT . Nex(T) . AWMS(T) - N(AWMS) ) . EF1

where

N2O(FAW)	=	N2O emission from organic fertiliser application

NT	= 	Number of animals of type T

Nex(T)	= 	N excretion of animals of type T (kg N/animal/yr) net of N 		volatilising 		as NOx and NH3 (values in Table A3.6.6)

AWMS(W)	= 	Fraction of Nex that is managed in one of the different 		waste management systems of type W

N(AWMS)	= 	N2O emissions from animal waste management systems as 		nitrogen

(kg N2O-N/yr)

The summation is for all animal types and manure previously stored in categories defined as a) liquid, b) solid storage and dry lot and c) other.

      - Atmospheric deposition of NOx and NH3

Indirect emissions of N2O from the atmospheric deposition of ammonia and NOx are estimated according to the IPCC (1997) methodology but with corrections to avoid double counting N.  The sources of ammonia and NOx considered are synthetic fertiliser application and animal manures applied as fertiliser.

The contribution from synthetic fertilisers is given by:

N2O(DSN)	=	44/28 . N(FERT) . Frac(GASF) . EF4

where

N2O(DSN)	=	Atmospheric deposition emission of N2O arising from synthetic fertiliser application (kg N2O/yr)

N(FERT)	=	Total mass of nitrogen applied as synthetic fertiliser (kg N/yr)

Frac(GASF)	=	Fraction of total synthetic fertiliser nitrogen that is emitted 		as NOx + NH3

=	0.1 kg N/ kg N

EF4	=	N deposition emission factor

=	0.01 kg N2O-N/kg NH3-N and NOx-N emitted

The indirect contribution from waste management systems is given by:

N2O(DWS)	=	44/28. (N(EX) /(1-Frac(GASM)) -N(F) ) . Frac(GASM) . EF4

where

N2O(DWS)	=	Atmospheric deposition emission of N2O arising from animal wastes

(kg N2O/yr)

N(EX)	=	Total N excreted by animals (kg N/yr), net of N volatilising 		as NOx and NH3 (values in Table A3.6.6)

Frac(GASM)	=	Fraction of livestock nitrogen excretion that volatilises as 		NH3  and NOx

=	0.2 kg N/kg N

N(F) 	=	Total N content of wastes used as fuel (kg N/yr)

The equation corrects for the N content of manures used as fuel but no longer for the N lost in the direct emission of N2O from animal manures as previously.  The nitrogen excretion data in Table A.3.6.6 already exclude volatilisation losses, and hence a correction is included for this.

      - Leaching and runoff

Indirect emissions of N2O from leaching and runoff are estimated according the IPCC methodology but with corrections to avoid double counting N.  The sources of nitrogen considered, are synthetic fertiliser application and animal manures applied as fertiliser.

The contribution from synthetic fertilisers is given by:

N2O(LSN)	=	44/28 . (N(FERT) . (1-Frac(GASF) )- N(SN)) . Frac(LEACH) . EF5

where

N2O(LSN)	=	Leaching and runoff emission of N2O arising from synthetic fertiliser application (kg N2O/yr)

N(FERT)	=	Total mass of nitrogen applied as synthetic fertiliser (kg N/yr)

N(SN)	=	Direct emission of N2O(SN) as nitrogen (kg N2O-N/yr)

Frac(GASF)	=	Fraction of total synthetic fertiliser nitrogen emitted 		as NOx + NH3

=	0.1 kg N/ kg N

Frac(LEACH)	=	Fraction of nitrogen input to soils lost through leaching and runoff

=	0.3 kg N/ kg fertiliser or manure N

EF5	=	Nitrogen leaching/runoff factor

=	0.025 kg N2O-N /kg N leaching/runoff

The estimate includes a correction to avoid double counting N2O emitted from synthetic fertiliser use.

The indirect contribution from waste management systems is given by:

N2O(LWS)	=	44/28. ((N(EX)  -N(F) -N(AWMS) ) . Frac(LEACH) . EF5

where

N2O(LWS)	=	Leaching and runoff emission of N2O from animal wastes (kg N2O/yr)

N(EX)	=	Total N excreted by animals (kg N/yr), net of N volatilising 		as NOx and NH3 (values in Table A3.6.6) net of N volatilising 		as NOx and NH3 (values in Table A3.6.6)

N(F) 	=	Total N content of wastes used as fuel (kg N/yr)

N(AWMS)  	=	Total N content of N2O emissions from waste management systems including daily spread and pasture range and paddock (kg N2O-N/yr)

Frac(LEACH)	=	Fraction of nitrogen input to soils that is lost through leaching and runoff

=	0.3 kg N/ kg fertiliser or manure N

EF5	=	Nitrogen leaching/runoff factor

=	0.025 kg N2O-N /kg N leaching/runoff

The equation corrects both for the N lost in the direct emission of N2O from animal wastes and the N content of wastes used as fuel.

    - Field Burning of Agricultural Residues (4F)

The National Atmospheric Emissions Inventory reports emissions from field burning under the category agricultural incineration.  The estimates are derived from emission factors calculated according to IPCC (1997) and from USEPA (1997) shown in Table A3.6.11.

Table A3.6.11	Emission Factors for Field Burning (kg/t)

|  | CH4 | CO | NOx | N2O | NMVOC |
| --- | --- | --- | --- | --- | --- |
| Barley | 3.05a | 63.9a | 2.18a | 0.060a | 7.5b |
| Other | 3.24a | 67.9a | 2.32a | 0.064a | 9.0b |

a	IPCC (1997)

b	USEPA (1997)

The estimates of the masses of residue burnt of barley, oats, wheat and linseed are based on crop production data (Defra, 2004b) and data on the fraction of crop residues burnt (MAFF, 1995; ADAS, 1995b).  Field burning ceased in 1993 in England and Wales.  Burning in Scotland and Northern Ireland is considered negligible, as is grouse moor burning, so no estimates are reported from 1993 onwards.  The carbon dioxide emissions are not estimated because under the IPCC Guidelines they are considered to be part of the annual carbon cycle.

  - Land Use Change and Forestry (CRF Sector 5)

The following section describes in detail the methodology behind emission predictions from the Land-Use Change and Forestry sector.  Other details regarding this sector can be found in Chapter 7.

    - Changes in Forest and Other Woody Biomass Stocks - Temperate Forests (5A2)

The carbon uptake by the forests planted since 1920 is calculated by a carbon accounting model (Dewar and Cannell, 1992, Cannell and Dewar, 1995, Milne et al., 1998) as the net change in pools of carbon in standing trees, litter, soil in conifer and broadleaf forests and products.  Restocking is assumed in all forests.  The method of the IPCC 1996 Guidelines is not used.  The UK carbon accounting model forests calculates the mass of carbon in trees, litter, soil and wood products from harvested material in new even-aged plantations that were clearfelled and then replanted at the time of Maximum Area Increment (MAI).  Two types of input data and two parameter sets were required for the model (Cannell and Dewar, 1995).  The input data are a) areas of new forest planted in each year in the past and b) the stemwood growth rate and harvesting pattern.  Parameter values were required to estimate i) stemwood, foliage, branch and root masses from the stemwood volume and ii) the decomposition rates of litter, soil carbon and wood products.

For the estimates described here we used the combined area of new private and state planting from 1920 to 2003 for England, Scotland, Wales and Northern Ireland sub-divided into conifers and broadleaves.  Restocking was dealt with in the model through the second and subsequent rotations and hence areas restocked each year did not need to be considered separately.  The implicit assumption is therefore that the forests are felled according to standard management tables.  Data on variation in management, i.e. felling/replanting dates, from that recommended in the standard tables is not available to the Inventory compilers.

The carbon flow model uses Forestry Commission Yield Tables (Edwards and Christie, 1981) to describe forest growth after thinning and by an expo-linear curve before thinning.  It was assumed that all new conifer plantations have the same growth characteristics as Sitka spruce (Picea sitchensis (Bong.) Carr.) under an intermediate thinning management.  Milne et al. (1998) have shown that mean Yield Class for Sitka spruce varied across Great Britain from 10-16 m3 ha-1 a-1 but with no obvious geographical pattern, and that this variation had a less than 10% effect on estimated carbon uptake for the country as a whole.  The Inventory data has therefore been estimated by assuming all conifers in Great Britain followed the growth pattern of Yield Class 12 m3 ha-1 a-1, but in Northern Ireland Yield Class 14 m3 ha-1 a-1, Sitka spruce.  Milne et al. (1998) also showed little effect of different assumptions on broadleaf species.  It is assumed here, that broadleaf forests had the characteristics of beech (Fagus sylvatica L.) of Yield Class 6 m3 ha-1 a-1.  Data in the most recent inventory of British woodlands (Forestry Commission, 2002) shows that beech is only about 8% of broadleaf forest (all ages).  Although sensitivity analysis of the carbon accounting model shows that different assumptions on the broadleaf species planted have little effect on overall carbon uptake, the assumption of using beech as the representative species will be reviewed.  Using oak or the sycamore-ash-birch group Yield Class data instead of beech data is likely to have a less than 10% effect on the value of removal of carbon to UK forests.  Irrespective of the assumption on representative species, the variation in removals from 1990 to the present is determined by the afforestation rate in earlier decades and the effect this has on the age structure in the present forest estate and hence the average growth rate.  This afforestation is all on ground that has not been wooded for many decades.  Table A3.7.1 shows the afforestation rate since 1922 and the present age structure of these forests.  In addition to these planted forests, there are about 820,000 ha of woodland planted prior to 1922 or not of commercial importance.  Variation from year to year in the reported removals to woody biomass, soils and harvested products reflect the changing pattern of afforestation over the period of available data.  For example, there are increases in removals to harvested products about 50 years (the conifer forest rotation cycle) after a period of increased planting of conifers.  It can be shown that if forest expansion continues at the present rate, removals of atmospheric carbon will continue to increase until about 2005 and then will begin to decrease, reflecting the reduction in afforestation rate after the 1970s.

Table A3.7.1	Afforestation rate and age distribution of conifers and broadleaves in the United Kingdom since 1922

| Period | Planting rate (000 ha a-1) |  | Age distribution |  |
| --- | --- | --- | --- | --- |
|  | Conifers | Broadleaves | Conifers | Broadleaves |
| 1922-1929 | 4.9 | 2.4 | 2.9% | 6.9% |
| 1930-1939 | 7.2 | 2.2 | 5.3% | 8.1% |
| 1940-1949 | 6.3 | 1.9 | 4.6% | 6.9% |
| 1950-1959 | 20.0 | 3.0 | 14.8% | 11.1% |
| 1960-1969 | 28.4 | 2.9 | 21.0% | 10.7% |
| 1970-1979 | 33.2 | 1.5 | 24.6% | 5.5% |
| 1980-1989 | 22.5 | 1.4 | 16.7% | 5.1% |
| 1990 | 26.8 | 3.1 | 2.0% | 1.1% |
| 1991 | 15.4 | 5.8 | 1.1% | 2.1% |
| 1992 | 13.4 | 6.8 | 1.0% | 2.5% |
| 1993 | 11.6 | 6.5 | 0.9% | 2.4% |
| 1994 | 10.1 | 8.9 | 0.7% | 3.2% |
| 1995 | 7.4 | 11.2 | 0.5% | 4.1% |
| 1996 | 9.5 | 10.5 | 0.7% | 3.8% |
| 1997 | 7.4 | 8.9 | 0.6% | 3.3% |
| 1998 | 7.0 | 9.7 | 0.5% | 3.6% |
| 1999 | 6.6 | 10.1 | 0.5% | 3.7% |
| 2000 | 6.5 | 10.9 | 0.5% | 4.0% |
| 2001 | 4.9 | 13.4 | 0.4% | 4.9% |
| 2002 | 3.9 | 10.0 | 0.3% | 3.7% |
| 2003 | 3.7 | 9.3 | 0.3% | 3.4% |

Increases in stemwood volume were based on standard Yield Tables, as in Dewar and Cannell (1992) and Cannell and Dewar (1995). These Tables do not provide information for years prior to first thinning so a curve was developed to bridge the gap (Hargreaves et al. 2003).  The pattern fitted to the stemwood volume between planting and first thinning from the Yield Tables follows a smooth curve from planting to first thinning rather.  The formulation begins with an exponential pattern but progresses to a linear trend that merges with the pattern in forest management tables after first thinning.

The mass of carbon in a forest was calculated from volume by multiplying by species-specific wood density, stem to branch and root mass ratios and the fraction of carbon in wood (0.5 assumed).  The values used for these parameters for conifers and broadleaves are given in Table A3.7.2.

Table A3.7.2	Main parameters for forest carbon flow model for species used to estimates carbon uptake by planting of forests of Sitka spruce (P. sitchensis and beech (F. sylvatica) in the United Kingdom (Dewar and Cannell, 1992)

|  |  | P. sitchensis | P. sitchensis | F. sylvatica |
| --- | --- | --- | --- | --- |
|  |  | YC12 | YC14 | YC6 |
|  | Rotation (years) | 59 | 57 | 92 |
|  | Initial spacing (m) | 2 | 2 | 1.2 |
|  | Year of first thinning | 25 | 23 | 30 |
|  | Stemwood density (t m-3) | 0.36 | 0.35 | 0.55 |
|  | Maximum carbon in foliage (t ha-1) | 5.4 | 6.3 | 1.8 |
|  | Maximum carbon in fine roots (t ha-1) | 2.7 | 2.7 | 2.7 |
|  | Fraction of wood in branches | 0.09 | 0.09 | 0.18 |
|  | Fraction of wood in woody roots | 0.19 | 0.19 | 0.16 |
|  | Maximum foliage litterfall (t ha-1 a-1) | 1.1 | 1.3 | 2 |
|  | Maximum fine root litter loss (t ha-1 a-1) | 2.7 | 2.7 | 2.7 |
|  | Dead foliage decay rate (a-1) | 1 | 1 | 3 |
|  | Dead wood decay rate (a-1) | 0.06 | 0.06 | 0.04 |
|  | Dead fine root decay rate (a-1) | 1.5 | 1.5 | 1.5 |
|  | Soil organic carbon decay rate (a-1) | 0.03 | 0.03 | 0.03 |
|  | Fraction of litter lost to soil organic matter | 0.5 | 0.5 | 0.5 |
|  | Lifetime of wood products | 57 | 59 | 92 |

The parameters controlling the transfer of carbon into the litter pools and its subsequent decay are given in Table A3.7.2.  Litter transfer rate from foliage and fine roots increased to a maximum at canopy closure.  A fraction of the litter was assumed to decay each year, half of which added to the soil organic matter pool that then decayed at a slower rate.  Tree species and Yield Class, but not other factors which varied with location, were assumed to control the decay of litter and soil matter.  Additional litter was generated at times of thinning and felling.

Estimates of carbon losses from the soils afforested are based on measurements taken at deep peat moorland locations covering afforestation of peat from 1 to 9 years previously and at a 26 year old conifer forest (Hargreaves et al. 2003).  These measurements suggest that long term losses from afforested peatlands are not as great as had been previously thought, settling to about 0.3 tC ha-1 a-1 thirty years after afforestation.  In addition, a short burst of regrowth of moorland plant species occurs before forest canopy closure.  The pattern of carbon loss and gain from afforested deep peat moorland was summarized in Table 7.1.

Carbon incorporated into the soil under all new forests is included, and losses from pre-existing soil layers are described by the general pattern measured for afforestation of deep peat with conifers.  Account is taken of the relative amounts of afforestation on deep peat and other soils in the decades since 1920.  For planting on organo-mineral and mineral soils, it is assumed that the pattern of emissions after planting will follow that measured for peat, but the emissions from the pre-existing soil layers will broadly be in proportion to the soil carbon density of the top 30 cm relative to that same depth of deep peat.  A simplified approach was taken to deciding on the proportionality factors, and it is assumed that emissions from pre-existing soil layers will be equal to that from the field measurements for all planting in Scotland and Northern Ireland and for conifer planting on peat in England and Wales.  Losses from broadleaf planting in England and Wales are however assumed to proceed at half the rate of the field measurements.  These assumptions are based on consideration of mean soil carbon densities for non-forest in the fully revised UK soil carbon database (See Appendix 3, Section 7.3).  The temporary re-growth of grasses is however assumed to occur for all planting at the same rate as in afforested peat moorland.  This assumption agrees with qualitative field observations at plantings on agricultural land in England.

It is assumed in the carbon accounting model that harvested material from thinning and felling is made into wood products.  These products are then assumed to decay over a period equal to the rotation of the forest, conifer or broadleaf as appropriate, since products from broadleaves (e.g. furniture) will decay more slowly than those from conifers (e.g. paper, building timber).  The net change in the carbon in this pool of wood products is reported in Category 5A.  Calculated in this way, that part of the total wood products pool from UK forests is presently increasing due to continuing expansion in forest area.  Dewar and Cannell (1992) and Cannell and Dewar (1995) provided a detailed description of all the assumptions in the model.

    - Forest and Grassland Conversion - Temperate Forests (5B2)

Levy and Milne (2004) discuss methods for estimating deforestation using a number of data sources. Here we use their approach of combining Forestry Commission felling licence data for rural areas with Ordnance Survey data for non-rural areas.

In Great Britain, some activities that involve tree felling require permission from the Forestry Commission, in the form of a felling licence, or a felling application within the Woodland Grant Scheme.  Under the Forestry Act 1967, there is a presumption that the felled areas will be restocked, usually by replanting.  Thus, in the 1990s, around 14,000 ha a –1 were felled and restocked.  However, some licences are granted without the requirement to restock, where there is good reason – so-called unconditional felling licences.  Most of these areas are small (1-20 ha), but their summation gives some indication of areas deforested.  These areas are not published, but recent figures from the Forestry Commission have been collated.  These provide estimates of rural deforestation rates in England for 1990 to 2002 and for GB in 1999 to 2001.

Only local planning authorities hold documentation for allowed felling for urban development, and the need for collation makes estimating the national total difficult.  However, in England, The Ordnance Survey (national mapping agency) makes an annual assessment of land use change (Office of The Deputy Prime Minister, 2003) from the data it collects for map updating.  Eleven broad land-use categories are defined, with a number of sub-categories.  The data for England (1990 to 2002) were available to produce a land-use change matrix, quantifying the transitions between land-use classes.  Deforestation rate was calculated as the sum of transitions from all forest classes to all non-forest classes providing estimates on non-rural deforestation.

The rural and non-rural values for England were each scaled up to GB scale, assuming that England accounted for 72 per cent of deforestation, based on the distribution of licenced felling between England and the rest of GB in 1999 to 2001.  However, the Ordnance Survey data come from a continuous rolling survey programme, both on the ground and from aerial photography.  The changes reported each year may have actually occurred in any of the preceding 1-5 years (the survey frequency varies among areas, and can be up to 10 years for moorland/mountain areas).  Consequently, a three-year moving average was applied to the data to smooth out the between-year variation appropriately, to give a suitable estimate with annual resolution.  The most recent deforestation rate available (1134 ha a-1 for 2002) is made up of 243 ha a-1 from non-rural areas and 891 ha a-1 from rural areas.  The rate for 2003 was estimated by extrapolating forwards from the rates for 1999 to 2002.  Deforestation is not currently estimated for Northern Ireland.  The annual area loss rates were used in the method described in the IPCC 1996 guidelines (IPCC, 1997a, b, c) to estimate immediate emissions of CO2, CH4 and N2O. Only immediate losses are considered because sites are normally completely cleared for development, leaving no debris to decay.  It is assumed that 60% of the standing biomass is removed as timber products.

    - CO2 Emissions and Removals from Soil (5D1 and 5D2)

The basic method for assessing changes in soil carbon due to land use change is to use a matrix of change from surveys of land linked to a dynamic model of gain or loss of carbon.  For Great Britain (England, Scotland and Wales), matrices from the Monitoring Landscape Change (MLC) data from 1947 & 1980 (MLC, 1986) and the Countryside Surveys (CS) of 1984, 1990 and 1998 (Haines-Young et al. 2000) are used.  In Northern Ireland, less data is available to build matrices of land use change, but for 1990 to 1998 a matrix for the whole of Northern Ireland was available from the Northern Ireland Countryside Survey (Cooper and McCann, 2002).  The only data available for Northern Ireland pre-1990 is land use areas from The Agricultural Census and The Forest Service and processed by Cruickshank and Tomlinson (2000).  Matrices of land use change had then to be estimated for 1970-79 and 1980-89 using area data.  The basis of the method devised was to assume that the relationship between the matrix of land use transitions for 1990 to 1998 and the area data for 1990 is the same as the relationship between the matrix and area data for each of two earlier periods – 1970-79 and 1980-89.  The matrices developed by this approach were used to extrapolate areas of land use transition back to 1950 to match the start year in the rest of the UK.

The Good Practice Guidance for Land Use, Land Use Change and Forestry (IPCC, 2003) recommends use of six classes of land for descriptive purposes: Forest, Grassland, Cropland, Settlements, Wetlands and Other Land.  The data presently available for the UK does not distinguish wetlands from other types, so land in the UK has been placed into the five other types.  The more detailed categories for the two surveys in Great Britain were combined as shown in Tables A3.7.3a for MLC and A3.7.3b for CS.

The database of soil carbon density for the UK (Milne and Brown, 1997, Cruickshank et al. 1998) used in previous GHG Inventories has been extensively revised recently (Bradley et al. In prep).  There are three soil survey groups covering the UK and the field data, soil classifications and laboratory methods were harmonized to reduce uncertainty in the final data.  The depth of soil considered was also restricted to 1 m at maximum as part of this process. Table A3.7.4 shows total stock of soil carbon (1990) for different land types in the four devolved areas of the UK.

Table A3.7.3a	Grouping of MLC land cover types for soil carbon change modelling

| CROPLAND | GRASSLAND | FORESTLAND | SETTLEMENTS (URBAN) | OTHER |
| --- | --- | --- | --- | --- |
| Crops | Upland heath | Broadleaved wood | Built up | Bare rock |
| Market garden | Upland smooth grass | Conifer wood | Urban open | Sand/shingle |
|  | Upland coarse grass | Mixed wood | Transport | Inland water |
|  | Blanket bog | Orchards | Mineral workings | Coastal water |
|  | Bracken |  | Derelict |  |
|  | Lowland rough grass |  |  |  |
|  | Lowland heather |  |  |  |
|  | Gorse |  |  |  |
|  | Neglected grassland |  |  |  |
|  | Marsh |  |  |  |
|  | Improved grassland |  |  |  |
|  | Rough pasture |  |  |  |
|  | Peat bog |  |  |  |
|  | Fresh Marsh |  |  |  |
|  | Salt Marsh |  |  |  |

Table A3.7.3b	Grouping of Countryside Survey Broad Habitat types for soil carbon change modelling

| CROPLAND | GRASSLAND | FORESTLAND | SETTLEMENTS (URBAN) | OTHER |
| --- | --- | --- | --- | --- |
| Arable | Improved grassland | Broadleaved/mixed | Built up areas | Inland rock |
| Horticulture | Neutral grassland | Coniferous | Gardens | Supra littoral rock |
|  | Calcareous grassland |  |  | Littoral rock |
|  | Acid grassland |  |  | Standing waters |
|  | Bracken |  |  | Rivers |
|  | Dwarf shrub heath |  |  | Sea |
|  | Fen, marsh, swamp |  |  |  |
|  | Bogs |  |  |  |
|  | Montane |  |  |  |
|  | Supra littoral sediment |  |  |  |
|  | Littoral sediment |  |  |  |

Table A3.7.4	Soil carbon stock (TgC = MtC) for depths to 1 m in different land types in the UK

| Region Type | England | Scotland | Wales | N. Ireland | UK |
| --- | --- | --- | --- | --- | --- |
| Forestland | 108 | 227 | 45 | 20 | 400 |
| Grassland | 995 | 1,839 | 283 | 242 | 3,359 |
| Cropland | 583 | 110 | 8 | 33 | 734 |
| Settlements | 54 | 10 | 3 | 1 | 69 |
| Other | 0 | 0 | 0 | 0 | 0 |
| TOTAL | 1,740 | 2,187 | 340 | 296 | 4,562 |

The effect of land use change from 1950 to the present on stocks of soil carbon is taken into account. Area data exist for various periods between 1947 and 1998 and how these are used is shown in Table A3.7.5. The land use change data over the different periods were used to estimate annual changes by assuming that these were uniform across the measurement period. Examples of these annual changes (for the period 1990 to 1999) are given in Tables 3.7.6a-d. The data for afforestation and deforestation shown in the Tables are adjusted before use for estimating carbon changes to harmonise the values with those used in the calculations described in Sections A3.7.1 and A3.7.2.

Table A3.7.5a	Sources of land use change data in Great Britain for different periods in estimation of changes in soil carbon

| Year or Period | Method | Change matrix data |
| --- | --- | --- |
| 1950-1979 | Measured LUC matrix | MLC 1947->MLC1980 |
| - 1980 - 1984 | - Interpolated | - CS1984->CS1990 |
| 1984 - 1989 | Measured LUC matrix | CS1984->CS1990 |
| 1990 - 1998 | Measured LUC matrix | CS1990->CS1998 |
| - 1999-2003 | Extrapolated | - CS1990->CS1998 |

Table A3.7.5b	Sources of land use change data in Northern Ireland for different periods in estimation of changes in soil carbon

| Year or Period | Method | Change matrix data |
| --- | --- | --- |
| - 1950 - 1969 | - Extrapolation and ratio method | - NICS1990->NICS1998 |
| - 1970 - 1989 | - Land use areas and ratio method | - NICS1990->NICS1998 |
| 1990 - 1998 | Measured LUC matrix | NICS1990->NICS1998 |
| - 1999-2003 | Extrapolated | - NICS1990->NICS1998 |

Footnote:  NICS = Northern Ireland Countryside Survey

The core equation describing changes in soil carbon with time for any land use transition is:

Ct=Cf−(Cf−C0)e−kt

where

Ct is carbon density at time t

C0 is carbon density initially

Cf is carbon density after change to new land use

k is time constant of change

By differentiating we obtain the equation for flux ft (emission or removal) per unit area:

ft=k(Cf−Co)e−kt

From this equation we obtain, for any inventory year, the land use change effects from any specific year in the past.  If AT is area in a particular land use transition in year T considered from 1950 onwards then total carbon lost or gained in an inventory year, e.g. 1990, is given by:

F1990=T=1950t=1990kAT(Cf−Co)(e−k(1990−T))

This equation is used with k, AT and (Cf-C0) chosen by Monte Carlo methods within ranges set by prior knowledge e.g. literature, soil carbon database, agricultural census, LUC matrices.

The land use transitions considered are each of those between the Forestland, Grassland, Cropland and Settlements. It is assumed there are no conversions between these and Other Land.  Scotland, England, Northern Ireland and Wales are treated separately.

Table A3.7.6a	Annual changes (000 ha) in land use in England in matrix form for 1990 to 1999.  Based on land use change between 1990 and 1998 from Countryside Surveys (Haines-Young et al. 2000).  Data have been rounded to 100 ha.

| From To | Forestland | Grassland | Cropland | Settlements |
| --- | --- | --- | --- | --- |
| Forestland |  | 8.9 | 3.4 | 2.1 |
| Grassland | 8.7 |  | 55.3 | 3.4 |
| Cropland | 0.5 | 62.9 |  | 0.6 |
| Settlements | 1.2 | 8.5 | 2.1 |  |

Table A3.7.6b	Annual changes (000 ha) in land use in Scotland in matrix form for 1990 to 1999.  Based on land use change between 1990 and 1998 from Countryside Surveys (Haines-Young et al. 2000).  Data have been rounded to 100 ha.

| From To | Forestland | Grassland | Cropland | Settlements |
| --- | --- | --- | --- | --- |
| Forestland |  | 11.1 | 0.6 | 0.2 |
| Grassland | 5.0 |  | 16.8 | 0.7 |
| Cropland | 0.1 | 21.4 |  | 0.3 |
| Settlements | 0.3 | 2.2 | 0.1 |  |

Table A3.7.6c	Annual changes (000 ha) in land use in Wales in matrix form for 1990 to 1999.  Based on land use change between 1990 and 1998 from Countryside Surveys (Haines-Young et al. 2000).  Data have been rounded to 100 ha.

| From To | Forestland | Grassland | Cropland | Settlements |
| --- | --- | --- | --- | --- |
| Forestland |  | 2.4 | 0.2 | 0.2 |
| Grassland | 1.5 |  | 5.5 | 0.6 |
| Cropland | 0.0 | 8.0 |  | 0.0 |
| Settlements | 0.1 | 1.8 | 0.2 |  |

Table A3.7.6d	Annual changes (000 ha) in land use in Northern Ireland in matrix form for 1990 to 1999.  Based on land use change between 1990 and 1998 from Northern Ireland Countryside Surveys (Cooper and McCann, 2002).  Data have been rounded to 100 ha.

| From To | Forestland | Grassland | Cropland | Settlements |
| --- | --- | --- | --- | --- |
| Forestland |  | 1.6 | 0.0 | 0.0 |
| Grassland | 0.3 |  | 5.9 | 0.0 |
| Cropland | 0.0 | 3.7 |  | 0.0 |
| Settlements | 0.1 | 1.0 | 0.0 |  |

In the model, the change is required in equilibrium carbon density from the initial to the final land use during a transition.  Here, these are calculated for each land use category as averages for Scotland, England, Wales and Northern Ireland.  These averages are weighted by the area of Land Use Change occurring in four broad soil groups (organic, organo-mineral, mineral, unclassified) in order to account for the actual carbon density where change has occurred.

Hence mean soil carbon density change are calculated as:

Cijc=s=16(CsijcLsijc)s=16Lsijc

which is the weighted mean, for each country, of change in equilibrium soil carbon when land use changes, where:

i = initial land use (Forestland, grassland, cropland, settlements)

j = new land use (Forestland, grassland, cropland, settlements)

c = country (Scotland, England, N. Ireland & Wales)

s = soil group (organic, organo-mineral, mineral, unclassified)

Csijc is change in equilibrium soil carbon for a specific land use transition

The most recent land use data (1990 to 1998) is used in the weighting. The averages calculated are presented in Tables A3.7.7a-d.

The rate of loss or gain of carbon is dependent on the type of land use transition (Table A3.7.8).  For transitions where carbon is lost e.g. transition from Grassland to Cropland, a ‘fast’ rate is applied whilst a transition that gains carbon occurs much more slowly.  A literature search for information on measured rates of changes of soil carbon due to land use was carried out and, in combination with expert judgement, ranges of possible times for completion of different transitions were selected.  These are shown in Table A3.7.9.

Table A3.7.7a  	Weighted average change in equilibrium soil carbon density (kg m-2) to 1 m deep for changes between different land types in England

| From To | Forestland | Grassland | Cropland | Settlements |
| --- | --- | --- | --- | --- |
| Forestland | 0 | 25 | 32 | 83 |
| Grassland | -21 | 0 | 23 | 79 |
| Cropland | -31 | -23 | 0 | 52 |
| Settlements | -87 | -76 | -54 | 0 |

Table A3.7.7b  	Weighted average change in equilibrium soil carbon density (kg m-2) to 1 m deep for changes between different land types in Scotland

| From To | Forestland | Grassland | Cropland | Settlements |
| --- | --- | --- | --- | --- |
| Forestland | 0 | 35 | 133 | 206 |
| Grassland | -39 | 0 | 77 | 157 |
| Cropland | -140 | -78 | 0 | 81 |
| Settlements | -200 | -156 | -62 | 0 |

Table A3.7.7c  	Weighted average change in equilibrium soil carbon density (kg m-2) to 1 m deep for changes between different land types in Wales

| From To | Forestland | Grassland | Cropland | Settlements |
| --- | --- | --- | --- | --- |
| Forestland | 0 | 23 | 57 | 114 |
| Grassland | -18 | 0 | 36 | 101 |
| Cropland | -53 | -38 | 0 | 48 |
| Settlements | -110 | -95 | -73 | 0 |

Table A3.7.7d  	Weighted average change in equilibrium soil carbon density (kg m-2) to 1 m deep for changes between different land types in Northern Ireland

| From To | Forestland | Grassland | Cropland | Settlements |
| --- | --- | --- | --- | --- |
| Forestland | 0 | 94 | 168 | 244 |
| Grassland | -94 | 0 | 74 | 150 |
| Cropland | -168 | -74 | 0 | 76 |
| Settlements | -244 | -150 | -76 | 0 |

Table A3.7.8	Rates of change of soil carbon for land use change transitions. (“Fast” & “Slow” refer to 99% of change occurring in times shown in Table A3.7.9)

|  |  | Initial |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cropland | Grassland | Settlement | Forestland |
| Final | Cropland |  | slow | slow | slow |
|  | Grassland | fast |  | slow | slow |
|  | Settlement | fast | fast |  | slow |
|  | Forestland | fast | fast | fast |  |

Table A3.7.9	Range of times for soil carbon to reach 99% of a new value after a change in land use in England (E), Scotland (S) and Wales (W)

|  | Low (years) | High (years) |
| --- | --- | --- |
| Carbon loss (“fast”) E, S, W | 50 | 150 |
| Carbon gain (“slow”) E, W | 100 | 300 |
| Carbon gain (“slow”) S | 300 | 750 |

Changes in soil carbon from equilibrium to equilibrium (Cf-Co) were assumed to fall in ranges based on 2004 database values for each transition and uncertainty indicted from this source (up to +/- 11% of mean).  The areas of land use change for each transition were assumed to fall a range of uncertainty of +/- 30% of mean.

The model of change was run 500 times with the time constant for change in soil carbon, land use change areas and equilibrium carbon change being selected separately using a Monte Carlo approach for England, Scotland N. Ireland and Wales from within the ranges described above.  The mean carbon flux for each region resulting from this imposed random variation is reported as the estimate for the Inventory.  An adjustment was made to these calculations for each country to remove increases in soil carbon due to afforestation, as the C-Flow model (See Section A3.7.1) provides a better estimate of these fluxes.

Variations from year to year in the reported net emissions reflect the trend in land use change as described by the matrices of change.

    - Liming of Agricultural Soils (5D3)

The method for estimating CO2 emissions due to application of lime and related compounds is that described in the IPCC 1996 Guidelines.  For limestone and chalk, an emission factor of 120 tC/kt applied is used, and for dolomite application, 130 tC/kt.  These factors are based on the stoichiometry of the reaction and assume pure limestone/chalk and dolomite.

Only dolomite is subjected to calcination.  However, some of this calcinated dolomite is not suitable for steel making and is returned for addition to agricultural dolomite – this fraction is reported in PA1007 as ‘material for calcination’ under agricultural end use.  Calcinated dolomite, having already had its CO2 removed, will therefore not cause the emissions of CO2 and hence is not included here.  Lime (calcinated limestone) is also used for carbonation in the refining of sugar but this is not specifically dealt with in the UK LUCF GHG Inventory.

    - Other: Lowland Drainage (5D5)

Lowland wetlands in England were drained many years ago for agricultural purposes and continue to emit carbon from the soil.  Bradley (1997) described the methods used to estimate these emissions.  The baseline (1990) for the area of drained lowland wetland for the UK was taken as 150,000 ha.  This represents all of the East Anglian Fen and Skirtland and limited areas in the rest of England.  This total consists of 24,000 ha of land with thick peat (more than 1 m deep) and the rest with thinner peat.  Different loss rates were assumed for these two thicknesses as shown in Table A3.7.10.

Table A3.7.10	Area and carbon loss rates of UK fen wetland in 1990

|  | Area | Organic carbon content | Bulk density | Volume loss rate | Carbon mass loss | Implied emission factor |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | kg m-3 | m3 m-2 a-1 | GgC a-1 | gC m-2 a-1 |
| ‘Thick’ peat | 24x107 m2 (24,000 ha) | 21% | 480 | 0.0127 | 307 | 109 |
| ‘Thin’ peat | 126x107 m2 (126,000 ha) | 12% | 480 | 0.0019 | 138 | 1280 |
| Total | 150x107 m2 (150 kha) |  |  |  | 445 | 297 |

The trend in emissions after 1990 was estimated on the assumption that no more area has been drained since then but the existing areas have continued to lose carbon.  The annual loss decreases for a specific location in proportion to the amount of carbon remaining.  But, in addition to this, as the peat loses carbon it will become more mineral in structure. Burton (1995) provides data on how these soil structure changes proceed with time.  The Century model of plant and soil carbon was used to average the carbon losses for the areas of component soils as they thinned to lose peat, become humose and possibly even mineral (Bradley 1997).

    - Other: Changes in Non-forest Biomass, Peat Extraction (5E)

      - Changes in non-forest biomass

Adger and Subak (1996) estimated recent changes in carbon storage in biomass on non-forest lands in the UK, including land used for agriculture, horticulture and urbanisation.  The land area converted to forest was specifically excluded to avoid overlap with estimates for Category 5A and 5B.  They used agricultural census statistics for the period 1988-1992 published by the Ministry of Agriculture, Fisheries and Food.  These statistics are strongly correlated with agricultural land cover data in 1984 and 1990 U.K. Countryside Surveys, which were used to calculate changes in soil carbon on non-forest lands, so the two estimates are considered to be compatible.

Two carbon sinks were quantified.  First, 0.23 MtC a-1 was estimated to be accumulating in biomass as a result, mainly, of (i) the transfer of land from arable crops with 2.2 tC ha-1 biomass to set-aside land with 5.0 tC ha-1 biomass, (ii) the establishment of woodlands on farms in response to financial incentives (Farm Woodland Scheme and Farm Woodland Premium), assuming that these woodlands increased in biomass by 2.8 tC ha-1 a-1, (iii) the transfer of agricultural land to urban uses, assuming that urban land has an average carbon density of 3 tC ha-1  and (iv) the transfer of rough grass to permanent grass.

Second, 0.14 MtC a-1 was estimated to be accumulating on agricultural land, without a change in crop type, on the assumption that the annual average standing biomass has increased linearly with yield. Most of this component was due to increases in cereal yields.

Thus, the total increase in biomass on agricultural land was estimated to be 0.37 MtC a-1.  However, this is an upper bound, because some of the farm woodlands were also counted in Forestry Commission statistics which were used to calculate the forest biomass carbon for Category 5A, and because increases in ‘harvest index’ mean that crop biomass generally increases proportionately less than yield.  Thus, the lower estimate for this component of 0.3 MtC a-1 +/- 30% has been adopted for non-forest biomass changes.  From the 1998 Inventory onwards, more recent data from the Agricultural Census were considered but did not support any change to the existing estimate.  This rate is therefore reported for all years from 1990 to 2003.

      - Peat extraction

Cruickshank et al. (1997) provide initial estimates of Emissions due to peat extraction. Since their work, trends in peat extraction in Scotland and England over the period 1990 to 2003 have been estimated from activity data taken from the UK Minerals Handbook (BGS, 2004).   In Northern Ireland, no new data on use of peat for horticultural use has been available but a recent survey of extraction for fuel use suggested that there is no significant trend for this purpose. The contribution of emissions due to peat extraction in Northern Ireland is therefore incorporated as constant from 1990 to 2003.  Peat extraction is negligible in Wales.   Emissions factors are from Cruickshank et al. (1997) and are shown in Table A3.7.11.

Table A3.7.11	Emission Factors for Peat Extraction

|  | Emission | Factor |
| --- | --- | --- |
|  | kg C m-3 | Gg C/Gg |
| GB Horticultural Peat | 55.7 | - |
| GB Fuel Peat | 55.7 | - |
| NI Horticultural Peat | 44.1 | - |
| NI Fuel Peat | - | 0.3 |

Footnote:  GB = Great Britain, NI = Northern Ireland

  - Waste (CRF sector 6)

The detailed description for the 2005 NIR is provided in Section 8 of the main report and by the references it provides.

- ANNEX 4: Comparison of CO2 Reference and Sectoral Approaches

- This annex presents information about the Reference Approach calculations, and its comparison with the Sectoral Approach.

  - Estimation of CO2 from the reference approach

- The UK greenhouse gas inventory uses the bottom-up (sectoral) approach based on the combustion of fuels in different economic sectors and estimates of non-combustion emissions from other known sectors to produce detailed sectoral inventories of the 10 pollutants.  In addition, estimates are also provided of carbon dioxide emissions using the IPCC Reference Approach.  This is a top down inventory calculated from national statistics on production, imports, exports and stock changes of crude oil, natural gas and solid fuels.  It is based on a different set of statistics and methodology and produces estimates around between 1% lower to 3 % higher than the bottom-up approach when categories not included in the reference approach are removed from the sectoral approach estimate.

  - Discrepancies between the IPCC Reference and Sectoral APPROACH

The UK GHGI contains a number of sources not accounted for in the IPCC Reference Approach and so gives a higher estimate of CO2 emissions.  The sources not included in the reference approach are:

- Land use change and forestry

- Offshore flaring and well testing

- Waste incineration

- Non-Fuel industrial processes

In principle the IPCC Reference Total can be compared with the IPCC Table 1A Total plus the emissions arising from fuel consumption in 1B1 Solid Fuel Transformation and Table 2 Industrial Processes (Iron and Steel and Ammonia Production).  The IPCC Reference totals range between 1% lower to 3 % higher than the comparable bottom up totals.

- The IPCC Reference Approach is based on statistics of production, imports, exports and stock changes of fuels whilst the sectoral approach uses fuel consumption data.  The two sets of statistics can be related using mass balances (see the publication ‘Digest of UK Energy Statistics’ DTI, 2004), but these show that some fuel is unaccounted for.  This fuel is reported in DUKES as statistical differences – these differences consist of measurement errors and losses.  The system of energy statistics operated by the DTI aims to keep UK statistical differences (without normalisation) at less than 0.5% of energy supply, and generally manages to meet this target, not only for total supply but by fuel.  Time series of UK statistical differences can be found in Table 1.1.2 at:

www.dti.gov.uk/energy/inform/energy_stats/total_energy/index.shtml

Nevertheless a proportion of the difference between the Reference Approach and the sectoral totals will be accounted for by statistical differences, particularly for liquid fuels.

- The sectoral approach only includes emissions from the non-energy use of fuel where they can be specifically identified and estimated such as with fertilizer production and iron and steel production.  The IPCC Reference approach implicitly treats the non-energy use of fuel as if it were combustion.  A correction is then applied by deducting an estimate of carbon stored from non-energy fuel use.  The carbon stored is estimated from an approximate procedure that does not identify specific processes.  The result is that the IPCC Reference approach is based on a higher estimate of non-energy use emissions.

- The IPCC Reference Approach uses data on primary fuels such as crude oil and natural gas liquids, which are then corrected for imports, exports and stock changes of secondary fuels.  Thus the estimates obtained will be highly dependent on the default carbon contents used for the primary fuels.  The sectoral approach is based wholly on the consumption of secondary fuels where the carbon contents are known with greater certainty.  In particular the carbon contents of the primary liquid fuels are likely to vary more than those of secondary fuels.

  - Time series of differences IN THE IPCC REFERENCE AND SECTORAL INVENTORIES

Table A4.1 shows the percentage differences between the IPCC Reference Approach and the National Approach.  These percentages include a correction for the fact that a significant proportion of fuel consumption emissions occur in the 2C Metal Production and 2B1 Ammonia Production sectors.

Table A4.1	Modified comparison of the IPCC Reference Approach and the National Approach

| Year | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 |
| --- | --- | --- | --- | --- | --- | --- |
| Percentage difference | -0.7 | 0.6 | 1.4 | 0.9 | 1.0 | 2.2 |

| Year | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- |
| Percentage difference | 0.4 | 0.3 | 0.9 | 1.6 | 2.8 | 1.2 |

| Year | 2002 | 2003 |
| --- | --- | --- |
| Percentage difference | 0.6 | 0.2 |

- ANNEX 5: Assessment of Completeness

Table A5.1 shows sources of GHGs that are not estimated in the UK GHG inventory, and the reasons for those sources being omitted.  This table is taken from the CRF; Table “Table9s1”.

Table A5.1	GHGs and sources not considered in the UK GHG inventory

| GHG | CRF sector | Source/sink category | Reason |
| --- | --- | --- | --- |
| CO2 | 2. Industrial Processes | 2A5/6 Asphalt Roofing/Paving | No methodology available |
| CO2 | 3. Solvent and Other Product Use |  | Carbon equivalent of solvent use not included in total - provided for information |
| CO2 | 5. Land-Use Change and Forestry | 5C2/5C4 Abandonment of Managed Lands | Considered negligible |
| CH4 | 2. Industrial Processes | 2B1 Ammonia Production | Manufacturers do not report emission - believed negligible |
| CH4 | 2. Industrial Processes | 2C1 Iron and Steel | EAF emission and flaring only estimated - methodology not available for other sources |
| CH4 | 2. Industrial Processes | 2C2 Ferroalloys | Methodology not available |
| CH4 | 2. Industrial Processes | 2C3 Aluminium | Methodology not available |
| CH4 | 6. Waste | 6B1 Industrial Waste Water | Activity data unavailable - most waste water treated in public system- believed small |
| N2O | 3. Solvent and Other Product Use | 3D Other -Anaesthesia | Activity not readily available – believed small |

- ANNEX 6: Additional Information - Quantitative Discussion of 2003 Inventory

This Annex discusses the emission estimates made in the 1990-2003 Greenhouse Gas Inventory.  Each IPCC sector is described in detail with significant points noted for each pollutant where appropriate.  It should be noted that tables show rounded percentages only.  All calculations are based on IPCC categorisation, which in all cases apart from Land-use Change and Forestry (LUCF) is the same as CRF format.  For LUCF, emissions for IPCC and CRF are categorised slightly differently and so figures shown in this annex can only be derived from IPCC tables.

  - Energy Sector (1)

Figure A6.1 and A6.2 show both emissions of direct and indirect Greenhouse Gases for the Energy sector (category 1) in the UK for the years 1990-2003.  Emissions from direct greenhouse gases in this sector have declined 7% since 1990, although an increase of just over 4% was seen between 2002 and 2003.

Tables A6.1 to A6.4 summarise the changes observed through the time series for each pollutant, as well as the contribution the emissions make to both sector 1 and the overall emissions in the UK during 2003.

    - Carbon Dioxide

Analysing emissions by pollutant shows that 95% of total CO2 emissions in 2003 came from the Energy sector (Table A6.4), making this sector by far the most important source of CO2 emissions in the UK.  Overall, CO2 emissions from sector 1 have declined by 5% since 1990 (Table A6.1).

Energy industries (category 1A1) were responsible for 39% of the sector’s CO2 emissions in 2003 (Table A6.3).  Although emissions from energy industries increased by 5% since 2002, there has been an overall decline of 10% since 1990 (Table A6.1).  Since the privatisation of the power industry in 1990, there has been a move away from coal and oil generation towards combined cycle gas turbines (CCGT) and nuclear power, the latter through greater availability. During this time there has been an increase of around 20% in the amount of electricity generated, but a decrease in CO2 emissions from Power stations (1A1a) of 15%.  This can be attributed to several reasons.  Firstly, the greater efficiency of the CCGT stations compared with conventional stations – around 46% as opposed to 36%.  Secondly, the calorific value of natural gas per unit mass carbon being higher than that of coal and oil and thirdly, the proportion of nuclear generated electricity supplied increasing from 21% to 24%.

Emissions of from category 1A2 – Manufacturing Industries and Construction contributed 15% (Table A6.4) to overall CO2 emissions in the UK in 2003.  Since 1990, these emissions have declined by 10%, (Table A6.1) mostly as a result of a decline in the emissions from the Iron and steel industry.  This sector has seen a significant decrease in coke, coal and fuel oil usage, with an increase occurring in the emissions of natural gas from combustion.

Emissions of CO2 from 1A3 (Transport) have increased by 7% since 1990 (Table A6.1).  In 2003, this sector contributed 22% (Table A6.4) to overall CO2 emissions within the UK.  Emissions from transport are dominated by the contribution from road transport (1A3b), which in 2003 contributed almost 95% to the emissions from transport.  Since 1990, emissions from road transport have increased by 8%.  In recent years (since around 1998), although the vehicle kilometres driven have continued to increase, the rate of increase in emissions of CO2 from road transport has slowed.  In part this is due to the increasing fuel efficiency of new cars.

Emissions of CO2 from 1A4 (Other) have increased by less than 1% since 1990 (Table A6.1), although between 2002 and 2003 decreased by 3% (Table A6.2).  During this period, residential emissions have increased by almost 11% and emissions from the commercial/institutional subsector have decreased by 18%.  Fuel consumption data shows a trend away from coal, coke, fuel oil and gas oil towards burning oil and natural gas usage.

Emissions of CO2 from 1A5 (Fuel Combustion; Other), 1B1 (Fugitive Emissions from Fuels; Solid fuels) and 1B2 (Fugitive Emissions from Fuels; Oil and Natural Gas) show large decreases between 1990-2003, although they only contribute a small percentage towards emissions from the energy sector.

    - Methane

In 2003, 32% (see Table A6.4) of total methane emissions came from the energy sector, the majority (48%, Table A6.3) from fugitive emissions from oil and natural gas (1B2).  Emissions from this category have decreased by 42% since 1990 (Table A6.1).  Sources include leakage from the gas transmission and distribution system and offshore emissions.  Estimates of leakage from the gas distribution system are based on leakage measurements made by Transco together with data on their gas main replacement programme, and have declined since 1990 as old mains are replaced.  The major sources of emissions from the offshore oil and gas industry are venting, fugitive emissions and loading and flaring from offshore platforms.

    - Nitrous Oxide

The energy sector accounted for 20% of total N2O emissions in the UK during 2003.  Of this, a majority (57%, Table A6.3) arose from the transport sector (1A3).  Between 1990 and 2003, emissions increased by over 277% (Table A6.1).  This is because of the increasing numbers of petrol driven cars fitted with three-way catalytic converters.  Catalytic converters are used to reduce emissions of nitrogen oxides, carbon monoxide and non-methane volatile organic compounds.  However, nitrous oxide is produced as a by-product and hence emissions from this sector have increased.

The other major contribution towards N2O emissions within the energy sector comes from the energy industries (1A1).  Within this category, emissions from both the public electricity and petroleum refining industries have remained fairly constant and so no particular trend is apparent.  Emissions from 1A1c (Manufacture of Solid Fuels and Other Energy Industries), however, have steadily increased between 1990 and 2003, and while there was a 13% decrease between 2002-2003, N2O emissions have increased overall by 44% since 1990.  Over this period the use of coal has decreased and the use of natural gas increased.

    - Nitrogen Oxides

In 2003, over 99% of NOx emissions in the UK came from the energy sector.  Since 1990 emissions from this sector have decreased by 44% (Table A6.1), mostly as a result of abatement measures on power stations, three-way catalytic converters fitted to cars and stricter emission regulations on trucks.  The main source of NOx emissions is transport: in 2003, emissions from transport contributed 45% (Table 6.4) to the total emissions of NOx in the UK, with 40% arising from road transport (1A3b).  From 1970, emissions from transport increased (especially during the 1980s) and reached a peak in 1989 before falling by 50% (Table A6.1) since 1990.  This reduction in emissions is due to the requirement since the early 1990s for new petrol cars to be fitted with catalytic converters and the further tightening up of emission standards on these and all types of new diesel vehicles over the last decade.

Emissions from the energy industries (1A1) contributed 30% (Table A6.4) to total NOx emissions in the UK during 2003.  Between 1990 and 2003, emissions from this sector decreased by 46% (Table A6.1).  The main reason for this was a decrease in emissions from public electricity and heat (1A1a) of 51%.  Since 1998 the electricity generators adopted a programme of progressively fitting low NOx burners to their 500 Mwe coal fired units.  Since 1990, further changes in the electricity supply industry such as the increased use of nuclear generation and the introduction of CCGT plant have resulted in additional reduction in NOx emissions.

Emissions from Manufacturing, Industry and Construction (1A2) have fallen by 28% (Table A6.1) since 1990.  In 2003, emissions from this sector contributed 14% (Table A6.4) to overall emissions of NOx.  Over this period, the iron and steel sector has seen a move away from the use of coal, coke and fuel oil towards natural gas and gas oil usage.

    - Carbon Monoxide

Emissions of carbon monoxide from the energy sector contributed 93% (Table A6.4) to overall UK CO emissions in 2003.  Of this, 51% of emissions (Table A6.4) occur from the transport sector.  Since 1990, emissions from 1A3 have declined by almost 75% (Table A6.1), which is mainly because of the increased use of catalytic converters, although a proportion is a consequence of fuel switching in moving from petrol to diesel cars.

Emissions from sector 1A2 contributed 21% (Table A6.4) to overall emissions of CO in 2003.  Emissions from within this category mostly come from the Iron and Steel industry and from petrol use in off-road vehicles within the Manufacturing, industry and combustion sector.

    - Non Methane Volatile Organic Compounds

In 2003, 51% (Table A6.4) of non-methane volatile organic compound emissions came from the energy sector.  Of these, the largest contribution arises from the fugitive emissions of oil and natural gas (1B2), which contributed 28% (Table A6.4) towards the overall UK emissions of NMVOCs in 1990.  This includes emissions from gas leakage, which comprise around 8% of the total.  Remaining emissions arise from oil transportation, refining, storage and offshore.

Emissions from transport (1A3) contribute 15% (Table A6.4) to overall emissions of NMVOC in the UK in 2003.  Since 1990, emissions from this sector have decreased by 81% (Table A6.1) due to the increased use of catalytic converters on cars.

    - Sulphur Dioxide

96% (Table A6.4) of emissions of sulphur dioxide came from the energy sector in 2003.  76% (Table A6.4) of these emissions arose from the energy industries sector (1A1).   A majority of these emissions are from the public electricity and heat production category (1A1a).  Since 1990, emissions from the power stations have declined by 76%.  This decline has been due to the increase in the proportion of electricity generated in nuclear plant and the use of CCGT stations and other gas fired plant.  CCGTs run on natural gas and are more efficient (see Section A6.1.1) than conventional coal and oil stations and have negligible SO2 emissions.

Emissions from Manufacturing, Industry and Construction were responsible for 12% (Table A6.4) of UK emissions of SO2 in 2003.  Since 1990, emissions from this sector have declined by 73% (Table A6.1).  This decline is due to the reduction in the use of coal and oil in favour of natural gas, and also some improvement in energy efficiency.

Table A6.1

Table A6.2

Table A6.3

Table A6.4

  - Industrial Processes sector (2)

Figure A6.3 and A6.4 show both emissions of direct and indirect Greenhouse Gases for the UK industrial processes sector in 1990-2003.  Emissions from direct Greenhouse gases within this sector have decreased by 53% since 1990. Tables A6.5 to A6.8 summarise the changes observed through the time series for each pollutant as well as the contribution the emissions make to Sector 2 and total UK emissions during 2003.

    - Carbon Dioxide

The industrial sector is not a major source of emissions in the UK for carbon dioxide.  In 2003, just 2% (Table A6.8) of UK emissions originated from this sector.

    - Methane

Emissions of methane from the industrial sector are very small and have a negligible effect on overall methane emissions in the UK.

    - Nitrous Oxide

In 2003, 8% (Table A6.8) of N2O emissions in the UK came from the industrial sector.  Between 1990 and 2003, emissions from this sector declined by almost 90% (Table A6.5) due to reductions in emissions from adipic acid manufacture (a feedstock for nylon) and nitric acid production. N2O emissions from nitric acid manufacture show a fall in 1995 due to the installation of an abatement system at one of the plants. Emissions from adipic acid manufacture were reduced significantly from 1998 onwards due to the retrofitting of an emissions abatement system to the only adipic acid plant in the UK.

    - Hydrofluorocarbons

Table A6.8 shows that the industrial processes sector was responsible for 100% of emissions of HFCs in the UK in 2003.  Since 1990, emissions of HFCs have decreased by 7% (Table A6.5).  The largest contribution to this sector in 2003 arises from category 2F1 – refrigeration and air conditioning equipment.  In 2003, these contributed 48% (Table A6.8) to the overall emissions of HFCs.  Emissions from this category arise due to leakage from refrigeration and air conditioning equipment during its manufacture and lifetime.  Emissions from aerosols contribute the next largest percentage (36%, Table A6.8) to overall HFC emissions.  In this category, it is assumed that all the fluid is emitted in the year of manufacture.  This category contains mainly industrial aerosols and also the medical use in metered does inhalers (MDI).

The remaining emissions arise from foam blowing (6%, Table A6.8), by-product emissions (4%, Table A6.8) and fire extinguishers (2%, Table A6.8).

    - Perfluorocarbons

In 2003, 100% (Table A6.8) of PFC emissions came from the industrial processes sector.  Since 1990, emissions from this sector have declined by 72% (Table A6.5), with a 12% decrease (Table A6.6) since 2002.  Within this sector, the main contribution to emissions comes from aluminium production (58%, Table A6.8).  During the process of aluminium smelting, PFC is formed as a by-product.  The emissions are caused by the anode effect, which occurs when alumina concentrations become too low in the smelter.  This can cause very high electrical current and decomposition of the salt – fluorine bath.  The fluorine released then reacts with the carbon anode to create CF4 and C2F6.  Since 1990, emissions arising from aluminium production have shown an 85% decrease (Table A6.5) due to significant improvements in process control and an increase in the rate of aluminium recycling.

The next largest emissions occur from 2F8, which includes a range of sources including the semiconductor and electronics industries.  In 2003, this sector contributed 24% (Table A6.8) to overall PFC emissions in the UK. The remaining contribution arises from fugitives.  In 2003, this contributed 17% (Table A6.8) to overall PFC totals in the UK.

    - Sulphur Hexaflouride

In 2003, the industrial sectors category contributed 100% (Table A6.8) towards the emissions of SF6 in the UK.  Emissions arise from two sectors.  The use of SF6 in aluminium and magnesium foundries contributed 58% (Table A6.8) towards total emissions in 2003.  Emissions from 2F6 – Other contributed 42% (Table A6.8) towards emissions, which includes emissions from electrical insulation.  Emissions arise during the manufacture and filling of circuit breakers and from leakage and maintenance during the equipment lifetime.  It also includes emissions from applications in the electronics industry and sports shoes.  Since 1990, emissions from SF6 have increased by 44% (Table A6.5).

    - Nitrogen Oxides

Although emissions of NOX from this sector do occur, overall they have little impact on emissions of NOX in the UK (see Table A6.8).

    - Carbon Monoxide

During 2003, emissions from the industrial sector contributed 6% (Table A6.8) to overall CO emissions in the UK.  Contributions within this sector arise mainly from the chemical industry, iron and steel production, and aluminium production.  For details see Table A6.7.  Since 1990, emissions from this sector have decreased by 8% (Table A6.5).

    - Non Methane Volatile Organic Compounds

In 2003, emissions from the industrial sector contributed 13% (Table A6.8) to overall UK emissions of NMVOCs.  The majority of emissions within this category come from the pulp and paper sector.  Emissions also arise from the chemical industry.

    - Sulphur Dioxide

In 2003, SO2 emissions from the industrial sector contributed just 4% (Table A6.8) to overall emissions in the UK.  Emissions arise from a variety of sources including the chemical industry, metal production and mineral products (Fletton brick production).  Since 1990, SO2 emissions from this sector have declined by over 24% (Table A6.5), showing a 5% reduction on 2002 emissions.

Table A6.5

Table A6.6

Table A6.7

Table A6.8

  - Solvents and Other Product Use sector (3)

Only emissions of NMVOCs occur from the solvents category.  Figure A6.5 displays total NMVOC emissions for 1990-2003.  Tables A6.9-6.12 summarise the changes observed through the time series as well as the contribution the emissions make to both sector 3 and the overall emissions in the UK during 2003.  Emissions from this sector contribute 35% to overall emissions of NMVOC in the UK (Table A6.12), and since 1990 emissions have declined by 44% (Table A6.9).

The largest source of emissions within the solvents sector is category 3D (solvent and other product use: other), contributing 59% of NMVOC emissions in this sector (Table A6.11).

Table A6.9

Table A6.10

Table A6.11

Table A6.12

  - Agriculture Sector (4)

Figures A6.6 and A6.7 show both emissions of direct and indirect greenhouse gases for the agricultural sector (category 4) in the UK for the years 1990-2003.  Emissions of direct greenhouse gases from this sector have decreased by 14% since 1990.

Tables A6.13-A6.16 summarise the changes observed through the time series for each pollutant emitted from the agricultural sector, as well as the contribution emissions make to both the sector and the overall UK estimates during 2003.

    - Methane

Agriculture is the largest source of methane in the UK, and in 2003 emissions from this sector totalled 46% (Table A6.16) of the UK total.  Since 1990, methane emissions from agriculture have declined by 12% (Table A6.13).  The largest single source within the agricultural sector is 4A1 – enteric fermentation from cattle.  This accounts for 65% of methane emissions from this sector (Table A6.15), and 30% of total methane emissions in 2003 (Table A6.16).  Since 1990, emissions from cattle have declined by 9% (Table A6.13) and this is due to a decline in cattle numbers over this period.

    - Nitrous Oxide

In 2003, nitrous oxide emissions from agriculture contributed 67% (Table A6.16) to the UK total emission.  Of this, 64% (Table A6.16) came from the agricultural soils sector, 4D.  Since 1990, emissions of N2O from the agricultural sector have declined by 15% (Table A6.13), driven by a fall in synthetic fertiliser application and a decline in animal population over this period.

    - Nitrogen Oxides

Emissions from the agricultural sector occur for NOX until 1993 only.  During 1993, agricultural stubble burning was stopped and therefore emissions of NOX became zero after this time.

    - Carbon Monoxide

Emissions from the agricultural sector occur for CO until 1993 only.  During 1993, agricultural stubble burning was stopped and therefore emissions of CO became zero after this time.

    - Non-Methane Volatile Organic Compounds

Emissions from the agricultural sector occur for NMVOC until 1993 only.  During 1993, agricultural stubble burning was stopped and therefore emissions of NMVOC became zero after this time.

Table A6.13

Table A6.14

Table A6.15

Table A6.16

  - Land Use Change and forestry (5)

Figures A6.8 and A6.9 show both emissions of direct and indirect Greenhouse gases for the Land-use change and forestry sector (sector 5) in the UK for the years 1990-2003.

Tables A6.17 to A6.20 summarise the changes observed through the time series for each pollutant, as well as the contribution the emissions make to both sector 5 and the overall emissions in the UK during 2003.  These tables refer to CO2 emissions only.

    - Carbon Dioxide

Figure 6.8 shows both emissions and removals (plotted as a negative quantity) of carbon dioxide.  Emissions from Land Use Change and Forestry were approximately 3% (Table A6.20) of the UK total in 2003.  Since 1990, emissions have declined by 16% (Table A6.17).  Most emissions from land-use change and forestry arise from the emissions of CO2 from soil (5D), which includes the cultivation of mineral soils, liming of agricultural soils and drainage.

    - Methane

Emissions of methane from Land Use Change and Forestry are emitted entirely from the Forest and Grassland conversion (5B) category and arise from emission from temperate forests (5B2).  Emissions from this sector have declined by 22% since 2002 (Table A6.18), and declined overall by 14% since 1990 (Table A6.17).

    - Nitrous Oxide

Emissions of nitrous oxide from Land Use Change and Forestry are emitted entirely from the Forest and Grassland conversion (5B) category and arise from emission from temperate forests (5B2).  Emissions of nitrous oxide from this sector have declined by 14% since 1990 (Table A6.17), and shown a decline of 22% since 2002 (Table A6.18).

    - Nitrogen Oxides

Emissions of nitrogen oxides from Land Use Change and Forestry are emitted entirely from the Forest and Grassland conversion (5B) category and arise from emission from temperate forests (5B2).  Emissions from this sector have declined by 22% since 2002 (Table A6.18), and declined overall by 14% since 1990 (Table A6.17).

    - Carbon Monoxide

Emissions of carbon monoxide from Land Use Change and Forestry are emitted entirely from the Forest and Grassland conversion (5B) category and arise from emission from temperate forests (5B2).  Carbon monoxide emissions from this sector have declined by 22% since 2002 (Table A6.18), and declined overall by 14% since 1990 (Table A6.17).

Table A6.17

Table A6.18

Table A6.19

Table A6.20

  - Waste (6)

Figures A6.10 and A6.11 show emissions of both direct and indirect greenhouse gases from the waste category (sector 6) in the UK for the years 1990-2003.  Emissions from direct greenhouse gases in this sector have declined by 60% since 1990.  This is mostly as a result of a decline in methane emissions, although emissions of nitrous oxide have shown an increase.

Tables A6.21 to A6.24 summarise the changes observed through the time series for each pollutant, as well as the contribution the emissions make to both sector 6 and the overall emissions in the UK during 2003.

    - Carbon Dioxide

Emissions of carbon dioxide from the waste sector occur from waste incineration only.  These emissions are small in comparison to CO2 emissions from other sectors and have a negligible effect on overall CO2 emissions in the UK (see Table A6.24).  Since 1990, CO2 emissions arising from the waste sector have decreased by 54% (Table A6.21), but have shown only a small decrease since 2002 (2%, Table A6.22).

    - Methane

Emissions of methane from the waste sector accounted for around 22% (Table A6.24) of total CH4 emissions in the UK during 2003.  Emissions from methane occur from landfills, waste water treatment and waste incineration.  The largest single source is landfill (6A1), with emissions from wastewater treatment and incineration being small in comparison (see Table A6.23).  Emissions estimates from landfill are derived from the amount of putrescible waste disposed of to landfill and are based on a model of the kinetics of anaerobic digestion involving four classifications of landfill site.  The model accounts for the effects of methane recovery, utilisation and flaring.  Since 1990, methane emissions from landfill have declined by 66% (Table 6.21) due to the implementation of methane recovery systems.  This trend is likely to continue as all new landfill sites are required to have these systems and many existing sites may have systems retrofitted.

    - Nitrous Oxide

Nearly all nitrous oxide waste emissions in the UK occur from the wastewater handling sector (see Table A6.23).  Since 1990, N2O emissions from this sector have increased by 17% (Table A6.21).  Overall, this sector contributes just 3% (Table A6.24) to overall nitrous oxide emissions.

    - Nitrogen Oxides

Emissions of NOX from the waste category have a negligible effect on overall UK emissions.

    - Carbon Monoxide

Emissions of CO from the waste category have a negligible effect on overall UK emissions, contributing less than 1% during 2003 (Table A6.24).

    - Non-Methane Volatile Organic Compounds

Emissions of NMVOC from the waste category have a very small influence (1%, Table A6.24) on overall UK emissions.

    - Sulphur Dioxide

Emissions of SO2 from the waste category have a negligible effect on overall UK emissions.

Table A6.21

Table A6.22

Table A6.23

Table A6.24

- ANNEX 7: Uncertainties

Uncertainty estimates are provided using both the Tier 1 and Tier 2 methods described by the IPCC and this submission introduces a number of improvements, including presenting estimates of uncertainties according to IPCC sector in addition to presenting estimates by direct greenhouse gas.

Further refinements are planned for next submission, including a review of the uncertainties allocated to the activity data and emission factors.

  - Estimation of Uncertainty by Simulation

Quantitative estimates of the uncertainties in the emissions were calculated using Monte Carlo simulation.  This corresponds to the IPCC Tier 2 approach discussed in the Good Practice Guidance (IPCC, 2000).  This work is described in detail by Eggleston et al (1998) with the estimates reported here revised to reflect changes in the 2003 inventory.  This section gives a brief summary of the methodology, assumptions and results of the simulation.

The computational procedure was:

- An probability distribution function (PDF) was allocated to each emission factor and piece of activity data.  The PDFS were mostly normal, log-normal or uniform.  The parameters of the PDFs were set by analysing the available data on emission factors and activity data or by expert judgement.

- A calculation was set up to estimate the emission of each gas, carbon dioxide sink, and the global warming potential for the years 1990 and 2003.  In the 2005 NIR, this analysis was extended to include both 1990 and the base year emissions.  For the UK, the base year comprises the emissions of CO2, CH4 and N2O for 1990, and the emissions of HFC, PFC and SF6 for 1995.

- Using the software tool @RISK™, each PDF was sampled 20,000 times and the emission calculations performed to produce a converged output distribution.

- It was assumed that the distribution of errors in the parameter values was normal.  The quoted range of possible error of uncertainty is taken as 2s, where s is the standard deviation.  If the expected value of a parameter is E and the standard deviation is s, then the uncertainty is quoted as 2s/E expressed as a percentage.  For a normal distribution the probability of the parameter being less than E-2s is 0.025 and the probability of the emission being less than E+2s is 0.975.

- For methane and nitrous oxide, it was assumed that there was no correlation between emission factors for the same fuels applied to different sources.  For CO2 emission factors for LPG, orimulsion and MSW and were correlated with those for the same fuel used in different sources.

- The uncertainties used for the fuel activity data were estimated from the statistical difference between supply and demand for each fuel.  This means that the quoted uncertainty in Table A7.1 refers to the total fuel consumption rather than the consumption by a particular sector, e.g. residential coal.  Hence, to avoid underestimating uncertainties, it was necessary to correlate the uncertainties used for the same fuel in different sectors.  A further refinement was to correlate the data used for the same fuels to calculate emissions of carbon dioxide, methane and nitrous oxide.  These modifications to the methodology were introduced in the 2000 inventory.

- The uncertainty in the trend between 1990 and 2003 according to gas was also estimated.  This will be influenced by the degree of correlation of activity data and emission factors between 1990 and 2003.  Generally it was assumed that activity data from different years were not correlated, but certain emission factors were.  These correlations are discussed in subsequent sections.

- The uncertainties for total halocarbon and SF6 emissions were taken from the recent study on emissions and projections of HFCs, PFCs and SF6 for the UK and constituent countries (AEAT, 2004).

    - Carbon Dioxide Emission Uncertainties

It was necessary to estimate the uncertainties in the activity data and the emission factors for the main sources and then combine them.

The uncertainties in the fuel activity data for major fuels were estimated from the statistical differences data in DTI (1996).  These are effectively the residuals when a mass balance is performed on the production, imports, exports and consumption of fuels.  For solid and liquid fuels both positive and negative results are obtained indicating that these are uncertainties rather than losses.  For gaseous fuels these figures include losses and tended to be negative.  For natural gas, a correction was made to take account of leakage from the gas transmission system but for other gases this was not possible.  The uncertainties in activity data for minor fuels (colliery methane, orimulsion, SSF, petroleum coke) and non-fuels (limestone, dolomite and clinker) were estimated based on judgement comparing their relative uncertainty with that of the known fuels.  The high uncertainty in the aviation fuel consumption reflects the uncertainty in the split between domestic and international aviation fuel consumption.

The uncertainties in the emission factors were based largely on expert judgement.  It was possible to compare the coal emission factors used in the inventory with measurements (Fynes, 1994).  Also, Transco (1998) data allowed an estimate of the uncertainty in the carbon content of natural gas.  The time series data of the gross calorific value of fuels used in the UK (DTI, 1996) would also give some indication of the relative variability in the carbon contents.  Thus the uncertainties in the fuel emission factors were based on judgements on whether they were likely to be similar or less than those of coal or natural gas.

In the case of non-fuel sources, the uncertainty depended on the purity of limestone or the lime content of clinker so the uncertainties estimated were speculative.

The uncertainties in certain sources were estimated directly.  Offshore flaring uncertainties were estimated by comparing the UKOOA (2005) flaring time series data with the flaring volumes reported by DTI (2001).  The uncertainty in the activity data was found to be around 16%.  This uncertainty will be an over estimate since it was assumed that the flaring volume data reported by DTI should be in a fixed proportion to the mass data reported by UKOOA.  The uncertainty in the carbon emission factor was estimated by the variation in the time series to be around 6%.  Again this will be an over estimate since it was assumed that the carbon emission factor is constant.  Uncertainties for fuel gas combustion were estimated in a similar way.  Uncertainties in the land use change sources were recalculated (Milne, 1999) for the revised source categories in the IPCC 1996 Guidelines using data from Eggleston et al (1998).  A new carbon source – Fletton bricks – has been added, and the uncertainty based on expert assessment of the data used to make the estimate.  There has been a very slight revision to the uncertainty used for cement production, based on the estimates reported in IPCC (2000).  Clinical waste incineration was assumed to have the same uncertainty as MSW incineration.

The overall uncertainty was estimated as around 2.0% in 1990 and 2.4% in 2003.

The uncertainty in the trend between 1990 and 2003 was also estimated.  In running this simulation it was necessary to make assumptions about the degree of correlation between sources in 1990 and 2003.  If source emission factors are correlated this will have the effect of reducing the trend uncertainty.  The assumptions were:

- Activity data are uncorrelated

- Emission factors of similar fuels are correlated (i.e. gas oil with gas oil, coke with coke etc)

- Land Use Change and forestry emissions are correlated (i.e. 5A with 5A etc)

- Offshore emissions are not correlated since they are based on separate studies using emission factors appropriate for the time.

- Emission factors covered by the Carbon Factors Review (Baggott et al, 2004) are not correlated

- Process emissions from blast furnaces, coke ovens and ammonia plant were not correlated.

The trend was found to range between –4.2% and –7.8% - that is to say this analysis indicated 95% probability that CO2 emissions in 2003 were between 4.2% and 7.8% below the level in 1990.

Table A7.1	Estimated Uncertainties in Carbon Dioxide Emissions (only major sources are listed)

| Source | Activity Uncertainty % | Emission Factor Uncertainty % | Uncertainty in Emission % |
| --- | --- | --- | --- |
| Coal (including derived gases) | 1.2 | 6 | ‡ |
| Coke | 5.6 | 3 | ‡ |
| Petroleum Coke | 5 | 3 | ‡ |
| SSF | 3 | 3 | ‡ |
| Burning Oil | 6 | 2 | ‡ |
| Fuel Oil | 4 | 2 | ‡ |
| Gas Oil/Diesel Oil | 1.4 | 2 | ‡ |
| Motor Spirit (Gasoline) | 0.8 | 2 | ‡ |
| Orimulsion | 1 | 2 | ‡ |
| Aviation Fuel (Domestic) | 50 | 2 | ‡ |
| Lubricants | 25 | 5 | ‡ |
| Natural Gas | 2.4 | 1 | ‡ |
| Colliery Methane | 5 | 5 | ‡ |
| LPG | 24 | 3 | ‡ |
| OPG | 1.1 | 3 | ‡ |
| Scrap Tyres | 15 | 10 | ‡ |
| Waste Oils | 15 | 5 | ‡ |
| Ammonia Production | - | - | 5 |
| Cement | 1 | 2.2 | ‡ |
| Lime/Limestone/Dolomite | 1 | 5 | ‡ |
| Soda Ash Use | 15 | 2 | ‡ |
| Fletton Bricks | 20 | 70 | ‡ |
| Flaring | 16 | 6 | ‡ |
| Other Offshore | - | - | 28 |
| Natural Gas (offshore) | 2.4 | 6 | ‡ |
| Iron & Steel Processes | 1 | 20 | ‡ |
| Aluminium Production | 1 | 5 | ‡ |
| Waste (MSW and Clinical) | 7 | 20 | 21 |
| 5A Forest Biomass Change2 | - | - | 30 |
| 5B Deforestation | 30 | 40 | ‡ |
| 5D Soils2 | - | - | 60 |
| 5E Other2 | - | - | 50 |

1	Expressed as 2s/E

2	Uniform distribution used

‡	Input parameters were uncertainties of activity data and emission factors.

    - Methane Emission Uncertainties

In the methane inventory, combustion sources are a minor source of emissions.  The uncertainty in methane combustion emission factors will outweigh the activity errors so an uncertainty of 50% was assumed for combustion sources as a whole.  The errors in the major sources are listed in Table A7.2.  These are mainly derived from the source documents for the estimates or from the Watt Committee Report (Williams, 1993).  The uncertainty in offshore emissions was revised for the 2000 inventory using improved estimates of the activity data.  The methane factors were assumed to have an uncertainty of 20% since the flaring factors are based on test measurements.

Table A7.2	Estimated Uncertainties in Methane Emissions (only major sources are listed)

| Source | Reference | Activity % | Emission Factor % | Source Uncertainty % |
| --- | --- | --- | --- | --- |
| Fuel Combustion | ‡ |  | 50 |  |
| Field Burning | ‡ | - | - | 50 |
| Landfill | Brown et al 1999 | - | - | ~481 |
| Livestock: enteric | Williams, 1993 | 0.1 | 20 |  |
| Livestock: wastes | Williams, 1993 | 0.1 | 30 |  |
| Coal Mining | Bennett et al, 1995 | 1.2 | 13 |  |
| Offshore | ‡ | 16 | 20 |  |
| Gas Leakage | Williams, 1993 | - | - | 17-752 |
| Chemical Industry | ‡ | 20 | 20 |  |
| Fletton Bricks | ‡ | 20 | 100 |  |
| Sewage Sludge | Hobson et al, 1996 | - | - | 50 |

1	Skewed distribution

2	Various uncertainties for different types of main and service

‡	See text

- Input parameters were uncertainties of activity data and emission factors

The sources quoted in Table A7.2 are assumed to have normal distributions of uncertainties with the exception of landfills.  Brown et al. (1999) estimated the uncertainty distribution for landfill emissions using Monte Carlo analysis and found it to be skewed.  For normal distributions there is always a probability of negative values of the emission factors arising.  For narrow distributions this probability is negligible; however with wide distributions the probability may be significant.  In the original work (Eggleston et al, 1998) this problem was avoided by using truncated distributions.  However, it was found that this refinement made very little difference to the final estimates, so in these estimates normal distributions were used rather than truncated normal.

The total emission of methane in 2003 was estimated as 1,933 Gg.  The Monte Carlo analysis suggested that 95% of trials were between 1,730 Gg and 2,260 Gg.  The total uncertainty was around 13%.  The emission of methane in 1990 was estimated as 3,690 Gg.  The Monte Carlo analysis suggested that 95% of trials were between 3,156 Gg and 4,649 Gg.  The uncertainty was around 19%.

The uncertainty in the trend between 1990 and 2003 was also estimated.  In running this simulation it was necessary to make assumptions about the degree of correlation between sources in 1990 and 2003.  If source emission factors are correlated this will have the effect of reducing the emissions.  The assumptions were:

- Activity data are uncorrelated between years, but activity data for major fuels were correlated in the same year in a similar manner to that described above for carbon.

- Emission factors for animals are correlated across years for a given species.

- Landfill emissions were partly correlated across years in the simulation.  It is likely that the emission factors used in the model will be correlated, and also the historical estimates of waste arisings will be correlated since they are estimated by extrapolation from the year of the study.  However, the reduction in emissions is due to flaring and utilisation systems installed since 1990 and this is unlikely to be correlated.  As a crude estimate it was assumed that the degree of correlation should reflect the reduction.  Emissions have reduced by 63% hence the degree of correlation was 37%.

- Offshore emissions are not correlated across years since they are based on separate studies using emission factors that reflected the processes in use at the time.

- Gas leakage emissions were fully correlated across years.

- Emissions from deep mines were not correlated across years as they were based on different studies, and a different selection of mines.  Open cast and coal storage and transport were correlated since they are based on default emission factors.

On the basis of this analysis there is 95% probability that methane emissions in 2003 were between 33% and 53% below the level in 1990.

    - Nitrous Oxide Emission Uncertainties

The analysis of the uncertainties in the nitrous oxide emissions is particularly difficult because emissions sources are diverse, and few data are available to form an assessment of the uncertainties in each source.  Emission factor data for the combustion sources are scarce and for some fuels are not available.  The parameter uncertainties are shown in Table A7.3.  The uncertainty assumed for agricultural soils uses a lognormal distribution since the range of possible values is so high.  Here it is assumed that the 97.5 percentile is greater by a factor of 100 than the 2.5 percentile based on advice from the Land Management Improvement Division of DEFRA (per. comm.).  The uncertainty distribution of the calculated emission was heavily skewed with a mean emission of 130 Gg in 2003 with 95% of the values found to lie between 32 Gg and 478 Gg N2O.

The uncertainty in the trend between 1990 and 2003 was also estimated.  In running this simulation it was necessary to make assumptions about the degree of correlation between sources in 1990 and 2002.  If sources are correlated this will have the effect of reducing the emissions.  The assumptions were:

- Activity data are uncorrelated between years, but similar fuels are correlated in the same year.

- Emissions from agricultural soils were correlated

- The emission factor used for sewage treatment was assumed to be correlated, though the protein consumption data used as activity data were assumed not to be correlated.

- Nitric acid production emission factors were assumed not to be correlated, for reasons explained in the 2000 National Inventory Report.

- Adipic acid emissions were assumed not to be correlated because of the large reduction in emissions due to the installation of abatement plant in 1998.

95% of the values for the trend were found to lie between  -19% and -76%, that is to say the analysis indicates a 95% probability that emissions in 2003 were between 19 and 73% below the level in 1990.

Table A7.3	Estimated Uncertainties in the Nitrous Oxide Emissions1 (only major sources are listed)

|  | Emission Factor Uncertainty % | Activity Rate Uncertainty % |
| --- | --- | --- |
| Agricultural Soils | Log-normal2 | 0 |
| Wastewater Treatment | Log-normal2 | 10 |
| Adipic Acid | 15 | 0.5 |
| Nitric Acid | 230 | 10 |
| Coal | 195 | 1.2 |
| Anthracite | 387 | 1.2 |
| Coke | 118 | 5.6 |
| Patent Fuel | 118 | 3 |
| Burning Oil | 140 | 6 |
| Gas Oil | 140 | 1.4 |
| Fuel Oil | 140 | 4 |
| Gasoline | 170 | 0.8 |
| Auto Diesel | 170 | 1.4 |
| Orimulsion | 140 | 1 |
| LPG | 110 | 24 |
| OPG | 110 | 1.1 |
| Aviation Fuel (domestic) | 170 | 50 |
| Natural Gas | 110 | 2.4 |
| Colliery Methane | 110 | 5 |
| Lubricants | 140 | 25 |
| Biogas | 110 | 5 |
| Offshore Sources | 110 | 1 |
| Field burning | 230 | 25 |
| Poultry Litter | 230 | 7 |
| Scrap Tyres, Waste Oils | 140 | 15 |
| Sewage, MSW and Clinical Incineration | 230 | 7 |
| Wood | 230 | 30 |
| Straw | 230 | 50 |

1	Expressed as 2s/E

2	With 97.5 percentile 100 times the 2.5 percentile

    - Halocarbons and SF6

The uncertainties in the emissions of HFCs, PFCs and SF6 were taken from a recent publication on (AEAT, 2004).  The uncertainties were estimated as 21% for HFCs, 13% for PFCs and 16% for SF6, and were assumed uncorrelated between 1990 and 2003.  Trend uncertainties are reported in Table A7.4a.

Uncertainties and trends were also calculated for HFC’s, PFC’s and SF6 against the base year 1995.  The uncertainties were estimated as 21% for HFCs, 13% for PFCs and 16% for SF6, and again were assumed uncorrelated between 1995 and 2003.  Trend uncertainties reported in Table A7.4b.

    - GWP Weighted emissions

The uncertainty in the combined GWP weighted emission of all the greenhouse gases in both 2003 and in 1990 was estimated as 14%.  The trend in the total GWP is -13%, with 95% of the values found to lie within the range -11% and -16%.  The uncertainty estimates for all gases are summarised in Table A7.4a.  The source which makes the major contribution to the overall uncertainty is 4D Agricultural Soils.  This source shows little change over the years, but other sources have fallen since 1990.

Table A7.4a	Summary of Tier 2 Uncertainty Estimates 1990 - 2003

| A | B | C | D | E | F | G | H | I | J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPCC | Gas | 1990 | 2003 | Uncertainty in 2003 emissions |  | Uncertainty | % change in | Range of likely % change |  |
| Source |  | Emissions | Emissions | as % of emissions |  | Introduced | emissions | between 2003 and 1990 |  |
| Category |  |  |  | in category |  | on national total | between 2003 |  |  |
|  |  |  |  |  |  | in 2003 | and 1990 |  |  |
|  |  |  |  | 2.5 percentile | 97.5 percentile |  |  | 2.5 percentile | 97.5 percentile |
|  |  | Gg CO2 | Gg CO2 | Gg CO2 | Gg CO2 | % | % | % | % |
|  |  | equivalent | equivalent | equivalent | equivalent |  |  |  |  |
| TOTAL | CO2 (net) | 591416 | 555927 | 542869 | 568949 | 2.4% | -6.0% | -8% | -4% |
|  | CH4 | 77480 | 40583 | 36339 | 47460 | 13% | -48% | -57% | -38% |
|  | N2O | 67893 | 40423 | 9841 | 148279 | 226% | -40% | -76% | -19% |
|  | HFC | 11375 | 10699 | 8521 | 12875 | 21% | -6% | -28% | 20% |
|  | PFC | 1393.7 | 377.1 | 329.6 | 424.4 | 13% | -73% | -77% | -69% |
|  | SF6 | 1081.9 | 1559.5 | 1310.0 | 1808.9 | 16% | 44% | 9% | 97% |
|  | All | 750641 | 649568 | 613338 | 758602 | 14% | -13% | -16% | -11% |

a	Calculated as 2s/E where s is the standard deviation and E is the mean, calculated in the simulation.

b	Not quoted because distribution is highly skewed.

Emissions of CO2 are net emissions (i.e. sum of emissions and removals)

Table A7.4b	Summary of Tier 2 Uncertainty Estimates - Base Year - 2003

| A | B | C | D | E | F | G | H | I | J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPCC | Gas | Base year* | 2003 | Uncertainty in 2003 emissions |  | Uncertainty | % change in | Range of likely % change |  |
| Source |  | Emissions | Emissions | as % of emissions |  | Introduced | emissions | between 2003 and base year |  |
| Category |  |  |  | in category |  | on national total | between 2003 |  |  |
|  |  |  |  |  |  | in 2003 | and base year |  |  |
|  |  |  |  | 2.5 percentile | 97.5 percentile |  |  | 2.5 percentile | 97.5 percentile |
|  |  | Gg CO2 | Gg CO2 | Gg CO2 | Gg CO2 | % | % | % | % |
|  |  | equivalent | equivalent | equivalent | equivalent |  |  |  |  |
| TOTAL | CO2 (net) | 591416 | 555927 | 542868 | 568877 | 2.4% | -6.0% | -8.7% | -3.2% |
|  | CH4 | 77480 | 40583 | 36330 | 47512 | 13% | -48% | -57% | -38% |
|  | N2O | 67893 | 40423 | 10171 | 149110 | 227% | -40% | -76% | -19% |
|  | HFC | 15491 | 10699 | 8521 | 12876 | 21% | -31% | -47% | -12% |
|  | PFC | 457.4 | 377.1 | 329.7 | 424.4 | 13% | -18% | -29% | -6% |
|  | SF6 | 1291.3 | 1559.5 | 1310.0 | 1808.9 | 16% | 21% | -8% | 65% |
|  | All | 754029 | 649568 | 613359 | 758745 | 14% | -14% | -8% | -11% |

* Base year for CO2, CH4 and N2O is 1990 and for HFC, PFC and SF6 the base year is 1995

    - Sectoral Uncertainties

The emissions from all sectors were calculated by summing the GWP weighted emissions of carbon, N2O, methane, the halocarbons and SF6.  It was then possible to estimate uncertainties for most of the sectors for which we have emissions.  In some cases, such as refrigeration and air conditioning, data were not available for the base year emissions hence some uncertainties could not be calculated.  Where the Monte Carlo simulation returned equal values for the mean and the 95% confidence intervals, the uncertainty introduced on the national total cannot be calculated.  The emissions and uncertainties per sector are presented in Table A7.4c.

Table A7.4c	Sectoral Uncertainty Estimates

| A | C | D | E | F | G |
| --- | --- | --- | --- | --- | --- |
| IPCC | 1990 | 2003 | Uncertainty in 2003 emissions |  | Uncertainty |
| Source | Emissions | Emissions | as % of emissions |  | Introduced |
| Category |  |  | in category |  | on national total |
|  |  |  |  |  | in 2003 |
|  |  |  | 2.5 percentile | 97.5 percentile |  |
|  | Gg CO2 | Gg CO2 | Gg CO2 | Gg CO2 | % |
|  | equivalent | equivalent | equivalent | equivalent |  |
| 1A1a | 206290 | 177518 | 169840 | 185197 | 4.4% |
| 1A1b | 18401 | 18389 | 17978 | 18800 | 2.3% |
| 1A1c | 13612 | 20591 | 19980 | 21202 | 3.0% |
| 1A2a | 24285 | 19771 | 19141 | 20401 | 3.3% |
| 1A2f | 74369 | 68948 | 67630 | 70266 | 2.0% |
| 1A3a | 1297 | 2138 | 1398 | 2878 | 35.3% |
| 1A3b | 112109 | 124597 | 117952 | 131241 | 5.4% |
| 1A3c | 2141 | 1077 | 973 | 1180 | 9.8% |
| 1A3d | 3586 | 3367 | 3273 | 3461 | 2.8% |
| 1A3e | 39 | 58 | 49 | 67 | 16.1% |
| 1A4a | 29610 | 24139 | 23610 | 24669 | 2.2% |
| 1A4b | 79160 | 86270 | 84361 | 88179 | 2.3% |
| 1A4c | 3472 | 1300 | 1233 | 1366 | 5.2% |
| 1A5b | 5356 | 2828 | 1713 | 3942 | 40.2% |
| 1B1a | 18271 | 4781 | 4338 | 5224 | 9.4% |
| 1B1b | 878 | 124 | 118 | 130 | 5.3% |
| 1B2a | 3125 | 1479 | 1164 | 1795 | 21.8% |
| 1B2b | 8360 | 5143 | 5112 | 5173 | 0.6% |
| 1B2c_Flaring | 5264 | 4305 | 3634 | 4976 | 15.9% |
| 1B2c_Venting | 740 | 506 | 380 | 632 | 25.4% |
| 2A1 | 6659 | 5356 | 5229 | 5483 | 2.4% |
| 2A2 | 1192 | 901 | 856 | 946 | 5.1% |
| 2A3 | 1369 | 1327 | 1280 | 1374 | 3.6% |
| 2A4 | 167 | 152 | 130 | 175 | 15.1% |
| 2A7 | 189 | 143 | 49 | 236 | 67.1% |
| 2B1 | 1322 | 1164 | 1135 | 1194 | 2.6% |
| 2B2 | 4134 | 2618 | -3291 | 8524 | 230.4% |
| 2B3 | 25136 | 582 | 496 | 667 | 15.0% |
| 2B5 | 141 | 43 | 34 | 52 | 20.7% |
| 2C1 | 1881 | 1335 | 1258 | 1413 | 5.9% |
| 2C3 | 1777 | 731 | 705 | 758 | 3.7% |
| 2C4 | 478 | 908 | 908 | 908 | ‡ |
| 2E1 | 11374 | 2191 | 2191 | 2191 | ‡ |
| 2E2 | 9 | 69 | 69 | 69 | ‡ |
| 2F1 | 0 | 4952 | 4952 | 4952 | ‡ |
| 2F2 | 0 | 406 | 406 | 406 | ‡ |
| 2F3 | 0 | 356 | 356 | 356 | ‡ |
| 2F4 | 2 | 2669 | 2669 | 2669 | ‡ |
| 2F5 | 0 | 21 | 21 | 21 | ‡ |
| 2F8 | 662 | 860 | 860 | 860 | ‡ |
| 4A1 | 13484 | 12267 | 10527 | 14007 | 14.5% |
| 4A10 | 9 | 6 | 5 | 7 | 20.0% |
| 4A3 | 4354 | 3616 | 2907 | 4324 | 20.0% |
| 4A4 | 12 | 9 | 7 | 11 | 20.0% |
| 4A6 | 76 | 113 | 91 | 136 | 20.0% |
| 4A8 | 238 | 159 | 128 | 190 | 20.0% |
| 4B1 | 2114 | 1909 | 1496 | 2323 | 22.1% |
| 4B3 | 103 | 86 | 60 | 112 | 30.5% |
| 4B4 | 0 | 0 | 0 | 0 | 30.5% |
| 4B6 | 6 | 9 | 6 | 11 | 30.5% |
| 4B8 | 476 | 318 | 223 | 413 | 30.5% |
| 4B9 | 224 | 287 | 219 | 356 | 24.3% |
| 4B9a | 0 | 0 | 0 | 0 | 30.5% |
| 4D | 31924 | 27069 | -63347 | 117480 | 340.9% |
| 4F1 | 342 | 0 | 0 | 0 | ‡ |
| 4F5 | 2 | 0 | 0 | 0 | ‡ |
| 5A2_(Harvested_Wood) | -1587 | 248 | 164 | 332 | 34.7% |
| 5A2_(Temperate) | -12226 | -15418 | -20655 | -10181 | -34.7% |
| 5B2 | 181 | 155 | 85 | 224 | 45.8% |
| 5D_(Emission) | 14952 | 12482 | 4629 | 20336 | 64.2% |
| 5E_(Emission) | 2442 | 2111 | 916 | 3306 | 57.7% |
| 5E_(Removal) | -1100 | -1100 | -1722 | -478 | -57.7% |
| 6A1 | 24778 | 8618 | 3905 | 13330 | 55.8% |
| 6B2 | 1734 | 1997 | -2210 | 6205 | 214.9% |
| 6C | 1245 | 511 | 347 | 676 | 32.8% |

‡	Value could not be calculated or was equal to 0 (see text)

  - Estimation of Uncertainties using a Tier 1 Approach

The IPCC Good Practice Guidance (IPCC, 2000) defines Tier 1 and Tier 2 approaches to estimating uncertainties in national greenhouse gas inventories.  The Monte Carlo approach described above corresponds to Tier 2 whilst Tier 1 provides for a simplified calculation method based on the error propagation equations.  The results of the Tier 1 approach are shown in Tables A7.5a and A7.5 b.  In the Tier 1 approach the emission sources are aggregated up to a level broadly similar to the IPCC Summary Table 7A.  Uncertainties are then estimated for these categories.  The uncertainties used in the Tier 1 approach are not exactly the same as those used in the Monte Carlo Simulation since the Tier 1 source categorisation is far less detailed.  However, the values used were chosen to agree approximately with those used in the Monte Carlo Simulation.  The Tier 1 approach is only able to model normal distributions.  This presented a problem in how to estimate a normal distribution approximation of the lognormal distribution used for agricultural soils and wastewater treatment.  The approach adopted was to use a normal distribution with the same mean as the lognormal distribution.  The standard deviation was then estimated as (97.5 percentile –mean)/2.

The Tier 1 analysis suggests an uncertainty of 17% in the combined GWP total emission in 2003 (GWP emission uncertainty of 18% in the 2002 inventory - 2004 NIR).  The analysis also estimates an uncertainty of 2% in the trend between 1990 and 2003 (trend uncertainty of 2% in the 2002 inventory - 2004 NIR).

In the UK inventory, certain source categories are particularly significant in terms of their contribution to the overall uncertainty of the inventory.  These key source categories have been identified so that the resources available for inventory preparation may be prioritised, and the best possible estimates prepared for the most significant source categories.  We have used the method set out in Section 7.2 of the IPCC Good Practice Guidance (2000) (Determining national key source categories) to determine the key source categories.  The results of this key source analysis can be found in Annex 1.

Table A7.5a	Summary of Tier 1 Uncertainty Estimates

Table A7.5b	Summary of Tier 1 Uncertainty Estimates

- ANNEX 8: Verification

This Annex discusses the verification of the UK Estimates of the Kyoto Gases.

  - Modelling approach used for the verification of the UK GHGI

GAD has established continuous high-frequency observations of the Kyoto gases under the supervision of Professor Peter Simmonds of the University of Bristol at the Mace Head Atmospheric Research Station on the Atlantic Ocean coastline of Ireland (Simmonds et al. 1996).  This is a contribution to international programmes on monitoring atmospheric concentrations of greenhouse gases and other pollutants and the results have also been used to estimate emissions for the UK and other European countries by inverse modelling.

This is undertaken by the Met Office also under contract to GAD.  The approach uses the Lagrangian dispersion model NAME (Numerical Atmospheric dispersion Modelling Environment) (Ryall et al. 1998) driven by 3D synoptic meteorology from the Unified Model (Cullen, 1993) to sort the observations made at Mace Head into those that represent Northern Hemisphere baseline air masses and those that represent regionally-polluted air masses arriving from Europe.  The Mace Head observations and the hourly air origin maps are applied in an inversion algorithm to estimate the magnitude and spatial distribution of the European emissions that best support the observations (Manning et al. 2003).  The technique has been applied to 2-yearly rolling subsets of the data and used to estimate longer term averages.

The inversion (best-fit) technique, simulated annealing, is used to fit the model emissions to the observations.  It assumes that the emissions from each grid box are uniform in both time and space over the duration of the data.  This in turn implies that the release is assumed independent of meteorological factors such as temperature and diurnal or annual cycles, and that, in so far as the emission relates to industrial production or other anthropogenic activity, use there is no definite cycle or intermittency. The estimated releases will include any natural release as well as anthropogenic emissions.

  - Results

Table A8.1 shows long run averages over the period since 1995 as estimated by the inventory and the NAME model for the greenhouse gases presently covered.  Results are rounded to two significant figures.

Table A8.1	Verification of the UK emission inventory estimates in Gg yr-1 for 1995 to 2003

|  | Inventory average (Gg/yr) | NAME average (Gg/yr) |
| --- | --- | --- |
| Methane | 2600 | 2000 |
| Nitrous Oxide | 160 | 110 |
| HFC-134a | 2.2 | 1.0 |
| HFC-152a | 0.06 | 0.14 |

The NAME estimates are subject to an uncertainty of order ±50% and the inventory estimates for these gases are uncertain, in 2003, to around:

- CO2	2.4%

- CH4	13%

- N2O	230%

(see Annex 7).  The uncertainty ranges therefore overlap.

When applied to the trend in emissions the NAME estimates presently show agreement to within the band of uncertainties and have similar slope at the European level.  Country estimates including those of the UK, France and Germany can have opposite slope with the present analysis, although this is still likely to be within the uncertainty range.

Details of this work can be found in Manning et al. (2005).  The UK believes that techniques of this kind are promising for verification purposes and plans to continue working on their development.

- ANNEX 9: IPCC Sectoral Tables of GHG Emissions

The tables in this Annex present summary data for UK greenhouse gas emissions for the years 1990-2003, inclusive.  The data are given in IPCC reporting format.  These data are updated annually to reflect revisions in the methodology and the availability of new information.  These adjustments are applied retrospectively to earlier years, which accounts for any differences in data published in previous reports, to ensure a consistent time series.

There are three series of tables in this Annex:

- Tables A9.1.1 to Tables A9.1.12 present UK GHG emissions as summary reports for national greenhouse gas inventories (IPCC Table 7A).

- Tables A9.2.1 to Tables A9.2.24 present UK GHG emissions in IPCC sectoral tables.

- Tables A9.2.1 to Tables A9.2.24 present UK GHG emissions in IPCC sectoral tables.

- Tables A9.3.1	 to Tables A9.3.14	 present Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

The footnotes referred to in first two sets of tables are given after the second set of tables.

TABLE A9.1.1	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 1990

TABLE A9.1.2		SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 1991

TABLE A9.1.3	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 1992

TABLE A9.1.4	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 1993

TABLE A9.1.5	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 1994

TABLE A9.1.6	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 1995

TABLE A9.1.7	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 1996

TABLE A9.1.8	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 1997

TABLE A9.1.9	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 1998

TABLE A9.1.10	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 1999

TABLE A9.1.11	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 2000

TABLE A9.1.12	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 2001

TABLE A9.1.13	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 2002

TABLE A9.1.14	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 2003

TABLE A9.2.1	SECTORAL REPORT FOR ENERGY - 1990

TABLE A9.2.1	SECTORAL REPORT FOR ENERGY - 1990

TABLE A9.2.2	SECTORAL REPORT FOR INDUSTRIAL PROCESSES - 1990

TABLE A9.2.2	SECTORAL REPORT FOR INDUSTRIAL PROCESSES - 1990

TABLE A9.2.3	SECTORAL REPORT FOR SOLVENT AND OTHER PRODUCT USE - 1990

TABLE A9.2.4	SECTORAL REPORT FOR AGRICULTURE - 1990

TABLE A9.2.4	SECTORAL REPORT FOR AGRICULTURE - 1990

TABLE A9.2.5	SECTORAL REPORT FOR LAND-USE CHANGE AND FORESTRY - 1990

TABLE A9.2.6	SECTORAL REPORT FOR WASTE - 1990

TABLE A9.2.7	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 1990

TABLE A9.2.7	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 1990

TABLE A9.2.7	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 1990

TABLE A9.2.8	SECTORAL REPORT FOR ENERGY - 2003

TABLE A9.2.8	SECTORAL REPORT FOR ENERGY – 2003

TABLE A9.2.9	SECTORAL REPORT FOR INDUSTRIAL PROCESSES - 2003

TABLE A9.2.9	SECTORAL REPORT FOR INDUSTRIAL PROCESSES - 2003

TABLE A9.2.10	SECTORAL REPORT FOR SOLVENT AND OTHER PRODUCT USE - 2003

TABLE A9.2.11	SECTORAL REPORT FOR AGRICULTURE - 2003

TABLE A9.2.11	SECTORAL REPORT FOR AGRICULTURE - 2003

TABLE A9.2.12	SECTORAL REPORT FOR LAND-USE AND FORESTRY - 2003

TABLE A9.2.13	SECTORAL REPORT FOR WASTE - 2003

TABLE A9.2.14	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 2003

TABLE A9.2.14	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 2003

TABLE A9.2.14	SUMMARY REPORT FOR NATIONAL GREENHOUSE GAS INVENTORIES (IPCC TABLE 7A) - 2003

Footnotes to tables

| a | Net flux may be estimated as the sum of emissions and removals |
| --- | --- |
| b | Naval vessels and military aircraft |
| c | Emissions arise from refrigeration, electronics applications, electrical insulation, foams, aerosols, training shoes and solvent cleaning |
| d | The CO2 equivalent of solvent NMVOC (excluding emissions from source category 3C) is 1515 Gg in 1990 and 880 Gg in 2003 |
| e | Field burning ceased in 1994 |
| f | 5A removals are sum of removals to forest biomass, forest litter, forest soil and harvested wood products |
| g | 5D Emissions are sum of emissions from soils and removals to soils due to land use change (not forestry), Set Aside and liming of agricultural land. |
| h | 5E Emissions are sum of emissions from soils due to upland drainage, lowland drainage and peat extraction |
| i | 5E Removals are increases in crop biomass |
| j | Emissions from own wastewater treatment by industry are not estimated |
| k | Emissions are for information only and are not totalled |
| l | Emissions arise from wood, straw, biogases and poultry litter combustion for energy production |
| m | 5B2 Emissions from forest conversion – burning of waste wood |
| NE | Not estimated |
| NO | Not occurring |
| IE | Included elsewhere |

Table A9.3.1	Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1990 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

Table A9.3.2	Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1991 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

Table A9.3.3	Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1992 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

Table A9.3.4	Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1993 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

Table A9.3.5	Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1994 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

Table A9.3.6	Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1995 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

Table A9.3.7	Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1996 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

Table A9.3.8	Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1997 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

Table A9.3.9	Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1998 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

Table A9.3.10	Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 1999 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

Table A9.3.11	Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 2000 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

Table A9.3.12	Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 2001 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

Table A9.3.13	Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 2002 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

Table A9.3.14	Emissions and Removals by Land Use, Land Use Change and Forestry (Sector 5) in 2003 for United Kingdom in Sectoral Report Table Format recommended by IPCC Good Practice Guidance for LULUCF

1 Excluding removals.  This is the basis for percentages quoted throughout the report unless otherwise indicated.

Methane emissions from working mines were already included, in accordance with the IPCC Guidelines.

Decision No 280/2004/EC of the European Parliament and of the Council of 11 February 2004 concerning a mechanism for monitoring Community greenhouse gas emissions and for implementing the Kyoto Protocol

Decision No 280/2004/EC of the European Parliament and of the Council of 11 February 2004 concerning a mechanism for monitoring Community greenhouse gas emissions and for implementing the Kyoto Protocol.

Gross emissions are displayed here.  Net emission and removals for CO2 can be found in tables ES1 and ES2

As distinct from the  NAEI category air transport  which gives an estimation of emissions within a 1000 m ceiling of landing and take-off (LTO), because of the reporting requiremnts of other international treaties.

Under category 5A and 5D/E in the 1996 Guidelines classification.

The UK greenhouse gas inventory is part of the UK National Atmospheric Emissions Inventory contract.  The UK National Atmospheric Emissions Inventory is funded by the UK Department for Environment, Food & Rural Affairs and is contracted to AEA Technology.

Global Atmosphere Division, Department for Environment, Food & Rural Affairs

Global Atmosphere Division, Department for Environment, Food & Rural Affairs

These are the factors used the latest inventory year.  The corresponding time series of emission factors and calorific values may are available electronically [on the CD accompanying this report].

These are the factors used the latest inventory year.  The corresponding time series of emission factors and calorific values may are available electronically [on the CD accompanying this report].

These are the factors used the latest inventory year.  The corresponding time series of emission factors and calorific values may are available electronically [on the CD accompanying this report].

These are the factors used the latest inventory year.  The corresponding time series of emission factors and calorific values may are available electronically [on the CD accompanying this report].

Making use, from 2000 onwards, of supplementary data from DTI because of a revision to the DUKES reporting format.

DTI (2004) Personal communication from Martin Young, DTI.

 Electricity generated by Major Power Producers, Table 5.1.3, DTI (2004)

 Plant loads, demand and efficiency, Table 5.10, DTI (2004)